3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503904

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on RRM requirements for RedCap UE in NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.29.4.1

Document for:	Discussion

Introduction

RRM requirements for RedCap UE in NTN are discussed in RAN4#114, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues.

- RRM requirements

- Impact of 1Rx

- Impact of HD-FDD

In this paper, we will provide our views on RRM requirements for RedCap UE in NTN.

Discussion

RRM requirements

Issue 1-2-3: SMTCs and 'in parallel' related

Issue 1-2-3-1: Feature 25-1(parallelSMTC)

In RAN4#114, following two options are listed for further discussion.

<Way Forward>: Options for further discussion: If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is: - Option 1: [1 SMTC or] 2 SMTCs not in parallel - Option 2: 2 parallel SMTCs - ‘Parallel’ means the SMTCs are TDM with proximity distance.

We support option 1. In last meeting, some companies commented that parallel measurement of 2 non-overlapping SMTCs may not be an issue if the distance between the 2 SMTCs is large enough, and it can be considered as if NW configured a smaller SMTC period. We agree that using a larger proximity distance could be helpful, but it is difficult to keep the UE complexity not increased due to support of 2 SMTCs in parallel because the max proximity distance is 80ms (which resembles the case of 80ms SMTC period).

The reason is that cell search delay is lower bounded by 600ms, meaning UE is not expected to use every SMTC occasion for cell search when SMTC period is <= 80ms. With single SMTC of 80ms period, UE is required to do PSS/SSS detection over 1 set of SMTC within 600ms. With 2 SMTCs of 160ms period and 80ms proximity distance, UE is required to do PSS/SSS detection over 2 sets of SMTC within 600ms, which means the cell search activity is doubled within a 600ms period even with 80ms proximity distance.

It can be seen that supporting parallel measurement of 2 non-overlapping SMTCs will increase the UE complexity. As low complexity is important for RedCap UE, we prefer not to increase the measurement complexity to trade-off measurement delay for multiple STMCs, at least not as a mandatory requirement.

There are two ways to fully address the complexity issue:

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO,

- Option 2: scale measurement period with number of SMTCs no matter if the SMTCs collide or not.

In RAN4#114, some companies commented that supporting 2 SMTCs per MO is better than supporting only 2 SMTC. In this case, we suggest that RedCap UE is still required to mandatorily supports 2 SMTCs, but not in parallel, i.e. measurement period is scaled with number of SMTCs no matter if the SMTCs collide or not. On the other hand, UE can optionally support 2 or more SMTCs in parallel as in existing requirements for normal UE. In this way, the existing capability 25-1 is not impacted as it only indicates number of SMTCs UE can support, and RAN4 can introduce a new capability on whether UE supports parallel measurement of multiple SMTCs per MO.

Proposal 1: RedCap UE mandatorily supports 2 SMTCs but not in parallel, and can optimally support 2 SMTCs in parallel.

Issue 1-2-3-1a: The proximity distance between two SMTCs

We support to consider relaxation for proximity condition for parallel measurement of multiple SMTCs for (e)RedCap UE. Due to low complexity, RedCap UE may have lower processing capability compared to normal UE, and it may take longer time to switch from one measurement of SMTC1 to another for SMTC2. We are open to discuss the exact relaxation x.

Proposal 2: Consider relaxation for proximity condition for parallel measurement of multiple SMTCs on a SAN carrier for (e)RedCap UE.

Issue 1-2-3-3: The number of cells/SSBs/supported satellites

We suggest to re-use the requirements for normal UE in NTN on number of cells/SSBs/LEO satellites for RedCap UE in NTN. In TN, same requirements on number of cells/SSBs are defined for RedCap UE and normal UE. In RAN4#114, some companies suggested to define requirements on number of satellites related to number of SMTC for RedCap. We do not support to define new requirements for RedCap UE.

Proposal 3: For number of cells/SSBs/LEO satellites, requirements for normal UE in NTN for re-used for (e)RedCap UE in NTN.

Issue 1-2-6: NCD-SSB

We support P2 on the terminology for NCD-SSB. In RAN4#112 meeting, it was agreed not to define NCD-SSB specific measurement requirements because with the typical BW in FR1-NTN the use of NCD-SSB is limited. On the other hand, we do not see the need to preclude use of NCD-SSB for RedCap in spec. It can be left to NW implementation.

Proposal 4: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and no need to add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

Issue 1-2-7: Other features

For the features or requirements mentioned in P1 of Issue 1-2-8 that are not discussed in other issues,

- We support to define requirements for SMTC adjustment in Idle/Inactive as mandatory. In our view, this is a rather important functionality for RRM measurement in NTN in Idle/Inactive, and it does not require high complexity at UE side (mainly determining the PDD between the target cell and serving cell). Also, RAN4 has not defined UE capability for it.

- We suggest RAN4 not to further discuss location-based measurement report trigger. This is an optional feature for normal UE, and we think it is reasonable to define it as an optional feature for RedCap UE also, as the functionality mainly relies on GNSS. However, there is no requirement related to this feature in RAN4 spec, and the UE capability was not defined by RAN4. We do not think RAN4 should make decision on the applicability of this feature to RedCap UE.

Proposal 5: For the applicability of other NTN features to RedCap UE,

- RAN4 to define mandatory requirements for SMTC adjustment in Idle/Inactive

- RAN4 not to further discuss applicability of location-based measurement report trigger

Impact of 1Rx

Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

RAN4 agreed to define enhanced cell reselection measurement requirements for 1Rx and 2Rx RedCap UE. Comparing the normal requirements and enhanced requirements for normal UEs, Tdetect is tightened most. Considering cell detection is the most complex part is cell reselection measurement, and power saving is important for RedCap UE, we suggest to relax Tdetect for RedCap UE. On the exact value, we suggest to extend Tdetect by [2] samples, and we are open to other views.

Tmeasure and Tevaluate are also tightened in enhanced requirements, but to us the enhanced requirements are still acceptable for RedCap in terms of power saving. After all, enhanced cell reselection measurement is an optional feature, and further relaxing the enhanced requirements will make it not much different from the normal requirements.

We suggest to apply the same enhanced requirements for 1Rx and 2Rx UEs, following the same principle as normal requirements, where the relaxation for 1Rx UE is reflected in the measurement accuracy.

Proposal 6: To define enhanced cell reselection requirements for RedCap UE,

- Re-use the requirements for normal UE except that Tdetect is extended by [2] samples

- Same delay requirements for 1Rx and 2Rx UEs.

Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

In RAN4#114 it was agreed to define cell re-selection requirements of inter-RAT E-UTRAN TN carrier. It is straightforward that R18 requirements on inter-RAT E-UTRAN measurement for normal UE in NTN in cl. 4.2C.2.11 are re-used for 2Rx RedCap UE in NTN. In our view, there are two issues to be further discussed.

One issue is whether to relax the delay or accuracy/margin for 1Rx UE. Checking R17 RedCap requirements for inter-RAT E-UTRAN measurement, it can be found that delay and accuracy/margin are sane for 1Rx UE and 2Rx UE. Therefore, we suggest to define same requirements for 1Rx and 2Rx UE for RedCap in NTN.

Another issue is eDRX related requirements as R18 requirements for normal UE in NTN do not account for eDRX. Checking the eDRX related requirements in R17 RedCap requirements for inter-RAT E-UTRAN measurement in cl. 4.2B.2.5, we understand they can be re-used for RedCap UE in NTN, for both 2Rx and 1Rx UEs.

Proposal 7a: For normal DRX, requirements in cl. 4.2C.2.11 for inter-RAT E-UTRAN measurement for normal UE in NTN are re-used for RedCap UE in NTN, for both 2Rx and 1Rx UEs.

Proposal 7b: For eDRX, requirements in cl. 4.2B.2.5 for inter-RAT E-UTRAN measurement for RedCap UE in TN are re-used for RedCap UE in NTN, for both 2Rx and 1Rx UEs.

Issue 2-4-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

We support the WF in [1] for this issue, as they clearly capture all the impacts of 1RX in TN, and we do not see any issue to re-use them for RedCap in NTN.

Proposal 8: Adopt the WF [1] for L1/L3 measurement accuracy requirements for RedCap UE.

Impact of HD-FDD

Issue 3-1-1: General HD-FDD related impact

P1a lists all the impacted requirements due to HD-FDD support in TN, and we do not see any issue to re-use them for RedCap in NTN. One exception is “MAC-CE/DCI/RRC based uplink spatial relation switch delay”, for which there is no requirement for NTN. Another exception is “Scheduling availability of UE during L1-RSRP measurement”, which is related to RAN1 discussion on TA mismatching as we will discuss next.

Proposal 9: Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE during L1-RSRP measurement

There is ongoing discussion in RAN1 related to TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB. Agreements that are relevant to RRM requirements in our view are copied below.

Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

It can be seen that there are possibilities that semi-static DL reception of CSI-RS and PRS is de-prioritized compared to UL transmission, e.g. when NW indicates UL overriding DL. In this case, the measurement period of CSI-RS and PRS based measurement may be impacted if the CSI-RS and PRS resources collide with UL transmission based on actual TA used by the UE because of the punctured measurement occasions. For such cases, we suggest RAN4 to clarify that the measurement period requirements apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission. Another alternative is to define the extended measurement period, but considering that such cases are anyway “semi-error” cases that NW would try to avoid, we do not prefer to spend much efforts to define exact requirements for them.

In existing requirements for scheduling availability of UE during L1-RSRP measurement, it is specified that CBD evaluation is prioritized over UL transmission. This may not be always true based on the latest RAN1 agreement. Our suggestion is to remove those 2 paragraphs because the UL/DL prioritization is anyway defined in RAN1 spec. This is why we list “Scheduling availability of UE during L1-RSRP measurement” as an exception for re-using the TN requirements.

9.5B.6.1 Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1 For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1. #### 9.5B.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 - For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1.

Proposal 10: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

Issue 3-2-1: The impact on NR Handover due to HD-FDD

We support P1 in WF [1] as it is similar to the impacts of HD-FDD in TN, and we do not see any issue to re-use them for mobility procedures in NTN.

Proposal 11: Adopt P1 in WF [1] for the impact on HO due to HD-FDD.

Conclusions

In this paper we provided our views on RRM requirements for RedCap UE in NTN.

Proposal 1: RedCap UE mandatorily supports 2 SMTCs but not in parallel, and can optimally support 2 SMTCs in parallel.

Proposal 2: Consider relaxation for proximity condition for parallel measurement of multiple SMTCs on a SAN carrier for (e)RedCap UE.

Proposal 3: For number of cells/SSBs/LEO satellites, requirements for normal UE in NTN for re-used for (e)RedCap UE in NTN.

Proposal 4: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and no need to add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

Proposal 5: For the applicability of other NTN features to RedCap UE,

- RAN4 to define mandatory requirements for SMTC adjustment in Idle/Inactive

- RAN4 not to further discuss applicability of location-based measurement report trigger

Proposal 6: To define enhanced cell reselection requirements for RedCap UE,

- Re-use the requirements for normal UE except that Tdetect is extended by [2] samples

- Same delay requirements for 1Rx and 2Rx UEs.

Proposal 7a: For normal DRX, requirements in cl. 4.2C.2.11 for inter-RAT E-UTRAN measurement for normal UE in NTN are re-used for RedCap UE in NTN, for both 2Rx and 1Rx UEs.

Proposal 7b: For eDRX, requirements in cl. 4.2B.2.5 for inter-RAT E-UTRAN measurement for RedCap UE in TN are re-used for RedCap UE in NTN, for both 2Rx and 1Rx UEs.

Proposal 8: Adopt the WF [1] for L1/L3 measurement accuracy requirements for RedCap UE.

Proposal 9: Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE during L1-RSRP measurement

Proposal 10: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

Proposal 11: Adopt P1 in WF [1] for the impact on HO due to HD-FDD.

Reference

- R4-2502604, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)