3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503910

Wuhan, China, 7th – 11th April 2025

Source:	Ericsson

Title:	Discussion on Rel-19 NTN UE/SAN demodulation requirements for less than 5 MHz channel bandwidth

Agenda item:	7.10.3.6

Document for:	Discussion

# 1	Introduction

Objective of Core part WI

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signalling to support the above objectives

According to WID [1], it is the time to kick off the NR Satellite Access Node (SAN) and UE demodulation performance discussion for this WI. The corresponding objective for core part and performance part are copied below.

Objective of Performance part WI

- Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In this contribution, a general view for UE and SAN demodulation requirements for less than 5 MHz channel bandwidth (CBW) is provided.

# 2	SAN demodulation requirements

## 2.1 	General

According to the WID, the Rel-18 feature ‘Terrestrial Networks (TN) to support less than 5 MHz channel bandwidth’ enables more efficient deployment of FR1 Non-Terrestrial Networks (NTN) bands, particularly where satellite spectrum availability is limited. Currently, the minimum channel bandwidth requirement for NTN FR1 is 5 MHz, as defined in [2]. Given that the scope of this WI is to extend the work from Rel-18 to support NTN FR1 bands, our initial approach would be to define the same requirements, such as PUCCH format 2, as specified in Rel-18 [3]. However, with different NTN channel models and FRC parameters, we may need to consider other PUCCH formats and PUSCH channel.

Observation 1: Consider introducing new PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS, due to different channel models and FRC parameters for NTN.

Further, from the perspective of SAN demodulation, current applicability rule of PUSCH for [4] is:

8.1.2.2.2	Applicability of requirements for different channel bandwidths

For each subcarrier spacing declared to be supported, the test requirements for a specific channel bandwidth shall apply only if the SAN supports it (see D.7 in table 4.6-1).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth.

Therefore, to prevent a situation where no requirements apply if a SAN supports only minimum channel bandwidth, defining new PUSCH demodulation requirements for 3 MHz is a logical step, theoretically. However, it seems unlikely for a SAN to support only 3 MHz capability, and this could be furtherly discussed among the companies. Also, from the Rel-18 WI in TN, there was no significant performance difference between 3 and 5 MHz for PUSCH. Thus, RAN4 should perform simulations to identify if there is enough performance difference to define normal PUSCH requirements for NTN.

Proposal 1: Introduce new PUSCH demodulation requirements for NTN with 3 MHz CBW and 15 KHz SCS, if 3 MHz only support for SAN is needed or performance difference is observed from simulations between 3 and 5 MHz CBW.

Further for PUCCH, the applicability rule for SAN [4] is same as PUSCH:

8.1.2.3.3	Applicability of requirements for different channel bandwidths

For each subcarrier spacing declared to be supported by the SAN, the test requirements for a specific channel bandwidth shall apply only if the SAN supports it (see D.7 in table 4.6-1).

Unless otherwise stated, for each subcarrier spacing declared to be supported, the tests shall be done only for the widest supported channel bandwidth. If performance requirement is not specified for this widest supported channel bandwidth, the tests shall be done by using performance requirement for the closest channel bandwidth lower than this widest supported bandwidth; the tested PRBs shall then be centered in this widest supported channel bandwidth

According to this applicability rule, the same reasoning as given for PUSCH above, is valid for PUCCH also, to define the PUCCH demodulation requirements. Further, in Rel-18 TN requirements, 3 MHz CBW requirements were created for PUCCH format 2 with frequency hopping, due to clear performance difference between 3 and 5 MHz CBW. With narrow channel bandwidth, the frequency diversity achieved via intra-slot frequency hopping is reduced, which causes loss in the PUCCH detection. Further, due to narrower channel bandwidth a smaller number of sub-carriers are available for hoping and thereby can cause overlap of PUCCH transmissions in the same band causing interference. Therefore, it seems necessary to simulate and analyze similar aspects for NTN.

Proposal 2: Introduce new PUCCH demodulation requirements for NTN with 3 MHz CBW and 15 KHz SCS, if 3 MHz only support for SAN is needed or performance difference is observed from simulations between 3 and 5 MHz CBW.

Further, the SAN demodulation requirements for PRACH in [2] are defined for any CBW per SCS. Therefore, RAN4 does not need to define requirements for PRACH with 3 MHz CBW.

Proposal 3: Do not consider PRACH to define SAN demodulation requirements for NTN with 3 MHz CBW.

## 2.2	Scope of the SAN demodulation requirements

As per the WID [1], NR-NTN requirements should be defined for FR1 bands- n254, n255 and n256. However, given there are no physical layer enhancements in RAN1, the limited test cases of PUSCH and PUCCH requirements with 3 MHz could be checked.

Observation 2: Introduce limited test cases of PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS.

Following cases from existing SAN PUSCH requirements [2] can be considered:

1) Requirements for PUSCH with transform precoding disabled: Considering the legacy and baseline requirements, it is reasonable to consider normal PUSCH demodulation requirements.

2) Requirements for PUSCH with transform precoding enabled: NTN application is expected to provide network in the coverage limited scenario. Therefore, considering larger propagation delays and path loss between SAN and UE, and high doppler due to satellite speed, for very low SNR regions in 3 MHz CBW, transform precoding (DFT-s-OFDM) seems suitable.

3) Requirements for PUSCH repetition Type A: This transmission scheme repeats the PUSCH transmission across multiple slots to improve decoding reliability and enhances robustness of the uplink transmission. Considering adverse channel conditions in NTN deployment, redundant transmissions across slots seems appropriate.

Other cases are not precluded but can be de-prioritized:

1) Requirements for UL timing adjustment: For the test of UL timing adjustment, the performance is determined by a moving UE and stationary UE, with 12 PRBs assigned for each UE for 5 MHz CBW / 15 KHz SCS. For 3 MHz CBW, the RB allocation should be scaled and we don’t expect reasonable performance difference in this case.

2) Requirements for PUSCH with DM-RS bundling: In this transmission scheme, the DM-RS are bundled across multiple transmissions to improve reception coverage and reliability. However, the existing test case has 6 PRBs in the middle of the test bandwidth for DM-RS Bundling, which may not fit well within 3 MHz (which has ~12 to 15 PRBs at 15 kHz SCS).

Proposal 4: Consider defining new PUSCH FR1 3 MHz demodulation requirements for SAN:

- Requirements for PUSCH with transform precoding disabled

- Requirements for PUSCH with transform precoding enabled

- Requirements for PUSCH repetition Type A

Observation 3: Regarding test parameter configuration, existing SAN FR1 could be followed as the starting point.

For PUCCH SAN demodulation requirements, PUCCH formats with enabled frequency hopping can be prioritized, but other formats are not precluded.

Proposal 5: Consider defining new PUCCH FR1 3 MHz SAN demodulation requirements for format 0/1/2/3/4.

# 3	UE demodulation requirements

## 3.1	General

3GPP introduced new UE feature “support-3MHz-ChannelBW-r18” as the result of Rel-18 WI [5]. For the WI less than 5MHz, RAN4 defined PDCCH/PBCH demodulation requirements assuming PRB puncturing [6]. However, RAN4 did not define new PDSCH demodulation requirements for 3MHz CBW, because it is assumed the UE supporting ‘Dedicated Spectrum of Less than 5MHz (support-3MHz-ChannelBW-r18)’ should support other band with 10MHz CBW, according to the operators.

## 3.2	Scope of the UE demodulation requirements

### 3.2.1	PDSCH demodulation requirements

RAN4 added to support 3MHz CBW in the existing FR1-NTN bands (n254, n255, and n256) with SCS=15kHz in this WI Enhanced requirements and test methodology for NR and IoT NTN. Since RAN4 added 3MHz CBW support in the existing bands, we think the NTN UE supporting 3MHz should also support the existing CBWs such as 10MHz.

Since there is no NTN UEs supporting 3MHz only, RAN4 does not need to define a new requirement as we concluded in Rel-18.

Observation 4: RAN4 RF added 3MHz CBW with SCS=15kHz in FR1-NTN bands n254, n255, n256.

Observation 5: RAN4 did not define PDSCH demodulation requirements for 3MHz CBW in Rel-18 because RAN4 assumed UE supporting 3MHz CBW should also support wider channel bandwidth such as 5MHz.

Proposal 6: No new PDSCH demodulation requirements with 3MHz CBW in TS 38.101-5, as same as Rel-18. UE supporting 3MHz in NTN bands should pass the existing PDSCH demodulation requirements.

### 3.2.2	PDCCH/PBCH demodulation requirements

In Rel-18 less than 5MHz WI, RAN4 defined PBCH/PDCCH demodulation requirements with puncturing to fit to 3MHz CBW according to the RAN1 specification.

According to TS 38.101-5 Table 8.2.1.1.2-1, it is specified:

- “For UE supporting NR NTN access (nonTerrestrialNetwork-r17), the requirements in TS38.101-4 also applies to UE according to applicability rules in TS38.101-4 Clause 5.1, 6.1, 7.1 and 8.1”

This means the UE supporting 3MHz in NTN bands should pass the PDCCH/PBCH demodulation requirements specified for 3MHz CBW in Rel-18. We therefore don’t think RAN4 need to add new PDCCH/PBCH demodulation requirements in TS 38.101-5.

Proposal 7: No new PDCCH/PBCH demodulation requirements with puncturing in TS 38.101-5. UE supporting 3MHz in NTN bands should pass the PDCCH/PBCH demodulation requirements specified for 3MHz CBW in Rel-18 TS38.101-4.

### 3.2.3	CSI reporting requirements

RAN4 did not define new CSI reporting requirements with 3MHz CBW in Rel-18. We therefore don’t think RAN4 need to add new CSI reporting requirements in TS 38.101-5.

Proposal 8: No new CSI reporting for 3MHz CBW in TS 38.101-5.

# 4	Conclusions

## 4.1	SAN demodulation requirements

Based on the above discussion, following observations are made:

Observation 1: Consider introducing new PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS, due to different channel models and FRC parameters for NTN.

Observation 2: Introduce limited test cases of PUSCH and PUCCH demodulation requirements for 3 MHz CBW with 15 KHz SCS.

Observation 3: Regarding test parameter configuration, existing SAN FR1 could be followed as the starting point.

Based on the above discussion, following proposals are made:

Proposal 1: Introduce new PUSCH demodulation requirements for NTN with 3 MHz CBW and 15 KHz SCS, if 3 MHz only support for SAN is needed or performance difference is observed from simulations between 3 and 5 MHz CBW.

Proposal 2: Introduce new PUCCH demodulation requirements for NTN with 3 MHz CBW and 15 KHz SCS, if 3 MHz only support for SAN is needed or performance difference is observed from simulations between 3 and 5 MHz CBW.

Proposal 3: Do not consider PRACH to define SAN demodulation requirements for NTN with 3 MHz CBW.

Proposal 4: Consider defining new PUSCH FR1 3 MHz demodulation requirements for SAN:

- Requirements for PUSCH with transform precoding disabled

- Requirements for PUSCH with transform precoding enabled

- Requirements for PUSCH repetition Type A

Proposal 5: Consider defining new PUCCH FR1 3 MHz SAN demodulation requirements for format 0/1/2/3/4.

## 4.2	UE demodulation requirements

Based on the above discussion, following observations are made:

Observation 4: RAN4 RF added 3MHz CBW with SCS=15kHz in FR1-NTN bands n254, n255, n256.

Observation 5: RAN4 did not define PDSCH demodulation requirements for 3MHz CBW in Rel-18 because RAN4 assumed UE supporting 3MHz CBW should also support wider channel bandwidth such as 5MHz.

Based on the above discussion, following proposals are made:

Proposal 6: No new PDSCH demodulation requirements with 3MHz CBW in TS 38.101-5, as same as Rel-18. UE supporting 3MHz in NTN bands should pass the existing PDSCH demodulation requirements.

Proposal 7: No new PDCCH/PBCH demodulation requirements with puncturing in TS 38.101-5. UE supporting 3MHz in NTN bands should pass the PDCCH/PBCH demodulation requirements specified for 3MHz CBW in Rel-18 TS38.101-4.

Proposal 8: No new CSI reporting for 3MHz CBW in TS 38.101-5.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, 3GPP TSG RAN Meeting #106.

- 3GPP TS 38.108, “Satellite Access Node radio transmission and reception (Release 18)”

- 3GPP TS 38.104, “Sec. 8.3.4: Performance requirements for PUCCH format 2, Base Station (BS) radio transmission and reception (Release 18)”

- 3GPP TS 38.181, “Satellite Access Node (SAN) conformance testing (Release 18)”

- RP-234062, “Summary for WI NR support for dedicated spectrum less than 5MHz for FR1”, Nokia.

- R4-2409843, “BigCR for 38.101-4 on Lessthan5MHz UE demod perf and CSI requirements”, Nokia