# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503913

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.17.4

Source:	China Telecom

Title:	Discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode

Document for:	Discussion

- Introduction

In this paper, we present our initial discussion on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode.

- Discussion

In the core part discussion, two sub-topics for FR2-1 L3 measurement delay enhancement are discussed including Rx BSF and CSSF. For FR2-1 L3 measurement delay reduction by optimizing Rx BSF, Scenario 1/2/3/4/5/7 were agreed to be considered as first priority in previous RAN4 meetings, including Scenario 1 SSB based Intra-frequency measurement without MG, Scenario 2 SSB based Intra-frequency measurement with MG, Scenario 3 SSB based Inter-frequency measurement without MG, Scenario 4 SSB based Inter-frequency measurement with MG, Scenario 5 Handover and Scenario 7 RRC Re-establishment/RRC Connection Release with Redirection. Therefore, in our views, it’s proposed to define test cases for Scenario 1/2/3/4/5/7.

Proposal 1: For L3 measurement delay reduction by optimizing Rx BSF, it’s proposed to define test cases for Scenario 1/2/3/4/5/7, including:

  - Scenario 1: SSB based Intra-frequency measurement without MG;

  - Scenario 2: SSB based Intra-frequency measurement with MG;

  - Scenario 3: SSB based Inter-frequency measurement without MG;

  - Scenario 4: SSB based Inter-frequency measurement with MG;

  - Scenario 5: Handover;

  - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection.

For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, 3 aspects are considered and no priority is defined for the 3 aspects. Therefore, it’s proposed to define test cases for 3 aspects including Aspect 1 SSB based Intra-frequency measurement without MG, Aspect 2 SSB based Inter-frequency measurement without MG and Aspect 3 Inter-RAT SSB measurement without MG.

Besides, two solutions are agreed to be considered including Solution 1 measure one serving CC per band and Solution 3 3 searchers for CSSF, it’s proposed to define test cases for the two solutions.

Proposal 2: For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, it’s proposed to define test cases for 3 aspects, including:

  - Aspect 1: SSB based Intra-frequency measurement without MG;

  - Aspect 2: SSB based Inter-frequency measurement without MG;

  - Aspect 3: Inter-RAT SSB measurement without MG.

Proposal 3: For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, it’s proposed to define test cases for two solutions, including:

  - Solution 1: measure one serving CC per band;

  - Solution 3: 3 searchers for CSSF.

- Conclusion

The contribution presents our views on RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode, with the following proposals:

Proposal 1: For L3 measurement delay reduction by optimizing Rx BSF, it’s proposed to define test cases for Scenario 1/2/3/4/5/7, including:

  - Scenario 1: SSB based Intra-frequency measurement without MG;

  - Scenario 2: SSB based Intra-frequency measurement with MG;

  - Scenario 3: SSB based Inter-frequency measurement without MG;

  - Scenario 4: SSB based Inter-frequency measurement with MG;

  - Scenario 5: Handover;

  - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection.

Proposal 2: For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, it’s proposed to define test cases for 3 aspects, including:

  - Aspect 1: SSB based Intra-frequency measurement without MG;

  - Aspect 2: SSB based Inter-frequency measurement without MG;

  - Aspect 3: Inter-RAT SSB measurement without MG.

Proposal 3: For FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, it’s proposed to define test cases for two solutions, including:

  - Solution 1: measure one serving CC per band;

  - Solution 3: 3 searchers for CSSF.