3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503920

Wuhan, China, April 7th – 11th 2025

Agenda Item:	7.19.1

Source:	Ericsson

Title:	On general issues for AI/ML

Document for:	Discussion

# 1	Introduction

In this contribution, post-deployment and LCM issues are being discussed.

From RAN4#114, a WF on post-deployment issues and LCM can be found in [1].

# 2	Post-deployment testing

Post-deployment testing refers to the ability of the AI/ML framework to ensure that deployed models that are updated can continue to be expected to perform similarly to models passing RAN5 conformance testing. It is clearly not possible to re-apply conformance testing to UEs that are already in use, and hence the framework should aim to achieve confidence that performance has not degraded due to model update.

Two possibilities for post-deployment testing have been considered:

- Pre-activation functionality/model update testing

  - This would be a testing of a functionality/model by some means prior to it’s actual use in a UE. The testing could be e.g. on equivalent hardware in a lab, or by simulation, or possibly even within a UE

- Post deployment management based on LCM

  - This basically means not doing any kind of testing, but relying on the “monitoring” framework. The question is whether monitoring can always be robust in a complex environment.

It may be difficult to evaluate these two options until further details emerge of the monitoring framework and its efficiency.

In the following, views are provided on some of the open issues listed in the table in [1].

## 2.1 Pre-activation functionality/model update testing

Whether the testing of a new model is on a device or in a lab.

Assuming that a model update is created within the computing facilities of a UE vendor (or other AI/ML model vendor), then it would make sense for validation/testing of the model to take place within the model development facility. It is unlikely to be preferable to provide additional hardware and software support to emulate model operation within a device. Within the development facility, there would be a need to emulate/test the model considering the realistic performance of the device hardware. Assuming that the AI/ML facility is aiming to produce a useful model for the device in question, it is reasonable to expect that there is a means of emulating performance on the actual device.

- Proposal 1 (Pre-activation functionality/model update testing): Post deployment validation should be at the facility that creates the update.

Whether models that have been updated post conformance testing should be kept in a device.

Assuming that post-deployment “testing” has already been performed then this question does not appear to be related to post-deployment testing, more on how the device manages it’s already deployed models and can be considered to be outside of the scope of discussing post-deployment validation.

Whether parallel operating of one model and (in-device) testing of another can be assumed.

As discussed above, it would be natural to validate a model update in the same location that the update is created, and it would be reasonable to assume that a facility producing a model update for a specific device would have a means to emulate the device sufficiently well.

It is not obvious why it would be necessary and whether it would even be feasible to run “testing” within a device itself.

Whether signaling from a UE of changes/updates are needed.

In case the download of functionality would be managed from the network, the signaling would be natural. For UE side management of model updates, signaling is not necessary. What would be necessary would be a framework for the AI/ML model vendor to perform validation and declare validation completed. The role of standardization would be to create a set of conditions and potentially test data that could be used for validation and would create confidence that the model could provide reasonable performance in conditions similar to those envisaged for the actual conformance testing.

Whether there is any relation to GCF procedures.

As discussed at RAN4#114, GCF incorporates procedures for handling UE software updates. These procedures may or may not be suited for AI model updates; it is important to take account of the procedures if they are useful, but ensure a reasonable impact if any to UEs, adapt if needed and not use if inappropriate. As a first step, we recommend to ask RAN5 to consider and clarify the procedures today.

- Proposal 2 (Pre-activation functionality/model update testing): To move the discussion on post-deployment validation forward, ask RAN5 to clarify the GCF procedure and whether anything similar might be considered.

Whether it is possible to differentiate “major” and “minor” model changes and only apply option 1 to major changes.

It is not fully clear how the scale of a model change could be considered. Clearly updating on a very frequent and short time basis would not be suitable for “Pre-activation functionality/model update testing”, whereas occasional and large changes could be.

- Proposal 3 (Pre-activation functionality/model update testing): Further discuss “major” and “minor” model changes.

## 2.2	Post deployment management based on LCM

Conceptually, “monitoring” of functionality appears to be a practical way to manage post-deployment performance. The main questions are whether monitoring that enables specific functionality issues to be remedied is really feasible, and whether monitoring is interoperable.

With regard to feasibility, it is important to consider that in real life, a very large number of factors influence the behavior of a link, for example:

- Changing channel conditions

- Changing user conditions, e.g. Doppler etc.

- Interference variations

- Traffic and scheduling

- Behaviour of analogue components in some circumstances; e.g. BS CFR/DPD, UE AGC, power control etc.

- If there would be multiple AI functionalities present, behaviour of other AI functionalities (e.g. if there would be AI based channel estimation, CSI prediction, CSI compression, decoding etc.)

- ...

A monitoring needs to be able to specifically identify that a performance issue arises within an AI model and could be remedied by some kind of fallback in real world conditions.

Regarding interoperability, in particular if a UE makes reports relating to functionality monitoring, it is important that the reports are comparable between different UE implementations and models. If the interpretation of a report would need an understanding of which particular type of UE has made the report, then interoperability of monitoring would be lost, which would vastly increase monitoring complexity for the network. Examples of variation between UEs could be:

- Confidence metrics that actually lead to different levels of performance, or performance variations in different timescales between different UEs.

- Metrics that may vary in different ways between different UEs because of other factors than the model (e.g. differences in analogue performance etc.)

- Different timescales, averaging etc. when making the reports.

- Observation 1 (post-deployment management based on LCM): It is not obvious as yet whether monitoring can useful identify individual model behavior and whether it can be interoperable.

- Proposal 4 (post-deployment management based on LCM): Continue to work on monitoring to understand how useful and interoperable it can be before deciding on whether it could be a sole solution for post-deployment performance assurance.

# 3	LCM requirements

In RAN4#114, no agreements were made for LCM requirements. In [1], two issues are listed with some open questions:

- Necessity of delay and tolerance requirements for input data for monitoring,

- Is this related to the delay before a monitoring report update reflects the change in input conditions, or is it related to the delay before LCM has taken action due to the change in conditions?

- If it is related to the delay before LCM has taken action, if the LCM is on the UE side, is it visible in the 3GPP specifications?  If it is on the network side, how would a requirement be defined on the UE?

- What is meant by tolerance?  Is it the accuracy of the monitoring report ? Is it for monitoring operation or is it TE tolerance?

- Different AI models may have different generality; in one case a model may accommodate a change in input data with no actions; in another case there may be a need for LCM action when input data changes. How to take into account of this variation?

- Delay requirements for activation and de-activation of functionality

- Alignment of terminology on “activation” of AI/ML functionality, “de-activation” of AI/ML functionality, “switching” AI/ML functionality etc… with RAN1/RAN2

- Definition of the starting point for <activation/de-activation> delay

- What de-activation delay means; e.g. model no longer operative, or non-AI actived.

- Which use cases delay requirements are needed for (BM, CSI prediction, positioning).

## 3.1	On delay requirements for activation/deactivation/fallback

RAN2 has earlier agreed on the following definitions, which are relevant for all use cases:

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

- Applicable functionalities refer to functionalities that the UE is ready to apply for inference.

- Activated functionalities refers to functionalities already enabled for performing inference.

For a functionality to be applicable at least there should at least one model available within it. RAN2 also agreed that the UE will indicate to the gNB/LMF whether the AI/ML functionality is available/applicable.

Although the granularity of functionality (e.g., whether it is a use case such as beam management, whether it is a sub-use case such as beam management Case 1, or others) is still under discussion in RAN1, “RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.”

AI/ML functionality switching: There is no explicit definition of AI/ML functionality switching in RAN1 or RAN2. However, it is reasonable to assume that switching refers to a situation when one AI/ML functionality is deactivated and another functionality is activated by the same message. Such definition can be assumed by RAN4 for progressing the discussions.

- Proposal 5: The following definition for AI/ML functionality switching can be assumed by RAN4 in its AI/ML discussions:

AI/ML functionality switching is a situation when one AI/ML functionality is deactivated and another functionality is activated by the same message.

Non-AI/ML operation: There is no explicit definition of non-AI/ML mode. However, it is natural to assume that non-AI/ML mode is characterized by a state when non-AI/ML procedures are followed, which by configuration can happen in parallel to AI/ML procedures (e.g., when a non-AI/ML method or measurement is configured, if possible, in parallel to an AI/ML method or measurement) or at a fallback, when one or more AI/ML functionalities are deactivated and there is no relevant activated AI/ML functionality left. It can be useful to clarify the fallback definition for progressing the RAN4 discussions.

- Proposal 6: The following definition for a fallback from AI/ML operation to non-AI/ML operation can be assumed by RAN4 in its AI/ML discussions:

Fallback from AI/ML operation to non-AI/ML operation is a situation when one or more AI/ML functionalities are deactivated and there is no relevant activated AI/ML functionality left.

Whether a delay requirement is needed for a fallback to non-AI/ML operation, depends on the use case and how AI/ML operation is configured.

### AI/ML positioning

For the AI/ML positioning use case 1 (UE-side model), an AI/ML positioning method is configured as a separate positioning method, similar to the existing non-AI/ML positioning methods, for which the UE has to report the result or an error – for this case, no additional fallback delay requirements are necessary, since no unspecified UE behaviour is involved, and the existing messages are also reused for activation/deactivation of AI/ML positioning functionalities.

The related RAN2 agreements from RAN2#129 (Feb. 2025) are supporting the above observations:

1:	Introduce AI/ML positioning Case 1 as a new positioning method.

2:	Existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing and requesting the results of the UE sided model inference operation. The detail stage 3 message extention can be disucssed while drafting the stage 3 CR.

3:	FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed.  FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation,

4: 	The content of error cause is discussed while drafting stage3 CRs.

5: 	As a baseline, UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the inference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

6: 	UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality.   FFS if any additional LMF control is needed.

Below is the LCM procedure for AI/ML positioning use case 1 agreed by now in RAN2.

- For Case 1, the baseline LCM procedure is:

  - Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP RequestCapabilities message.

  - Step 2: UE sends LPP ProvideCapabilities message to LMF with the supported functionalities at the UE side.

  - Step 3: LMF sends the LPP ProvideAssistanceData message (which may contain network side additional condition).

  - Step 4: UE reports the applicable functionality to the LMF by the LPP ProvideCapabilities message in response to Step 3 (“reactive” way).

    - The UE can report the applicable functionality also in “proactive” way by sending an unsolicited LPP ProvideCapabilities message to LMF when the applicability changes.

  - Step 5: The LMF requests the inferred location information using the LPP RequestLocationInformation message.

  - Step 6: UE reports the inferred location using LPP ProvideLocationInformation message.

- As a baseline, if a functionality (e.g., AIML-based positioning method) becomes non-applicable in the UE, when LMF requests UE location estimation, the UE cannot perform the AIML-based positioning and reply with LPP ProvidelocationInformation message with error cause. FFS in RAN2 if other fallback options are considered.

- Proposal 7: No fallback delay requirement for AI/ML positioning use case 1 is necessary, since AI/ML positioning use case 1 is configured by LMF as a separate positioning method, based on the current RAN2 agreements. (The statement can be rediscussed if there will be new related RAN2 agreements impacting the procedure.)

### AI/ML beam management

The LCM procedure for AI/ML beam management is different from that for AI/ML positioning. For the beam management use case, UE-sided model, the LCM steps are as below.

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

- 1) UE is allowed to do UAI reporting via OtherConfig.

- 2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

- 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

- 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

  - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.

- Step 5:

- 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

- 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.  FFS the initial activation state.  FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation.  FFS if multiple applicable functionalities can be activated at the same time.   FFS what is the granularity of functionality

- Applicable periodic CSI Report (for beam management) is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported, e.g., by L1 and MAC signalling.

- When a functionality becomes non-applicable the UE reports this to NW, but does not autonomously deactivate the functionality. NW is expected to deactivate active functionality upon receiving the UE’s report.

Further related agreements were made by RAN2 in RAN2#129 (Feb. 2025):

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities FFS option B Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

From the above agreements, for periodic CSI reporting, the UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete (Step 4). To prevent long activation delays, activation requirements can be well justified at least for periodic CSI reporting with AI/ML.

- Proposal 8: RAN4 will consider defining functionality activation delay requirement for the beam management use case at least with periodic CSI reporting, FFS for aperiodic and semi-persistent reporting.

On functionality deactivation: when a functionality becomes non-applicable the UE reports this to network, and network is expected to deactivate the activated functionality upon receiving the UE’s report. Upon receiving the deactivation command, the UE may need to go to non-AI/ML operation, and in that case it is important that this is completed as soon as possible. Therefore, a deactivation delay requirement is justified in this case.

- Proposal 9: RAN4 will consider defining functionality deactivation delay requirement for the beam management use case.

- Proposal 10: FFS: whether fallback delay includes other delays than AI/ML functionality deactivation delay for beam management (e.g., a delay until the next non-AI/ML report).

## 3.2	Data collection for training

For beam management data collection configuration for UE-side model training, the UE can send a request for data collection.  The following methods for network control of the initiation and configuration for data collection were agreed in RAN2:

  - The network can decide when to start/stop the data collection and send configuration.

  - The network can configure whether UE is allowed to initiate request for data collection.

When the network decides the start/stop of the data collection, it is important that the data collection is not delayed by the UE, which can justify a requirement ensuring the timely start of UE measurements for data collection. One reason for this is that the network may need to configure radio signals for these measurements, and they should not be wasted. Another reason could be that the network is interested in that the UE is measuring during a specific time, for the data collection to serve the purpose properly.

- Proposal 11: A requirement for the maximum delay until the UE actually starts the data collection after receiving the network configuration can be considered for data collection, to ensure that the data collection is not delayed. Alternatively, a UE behaviour can be specified that the UE shall start, e.g., from a first signal occasion available after receiving and decoding the command.

# Summary

- Proposal 1 (pre-activation functionality/model update testing): Post deployment validation should be at the facility that creates the update.

- Proposal 2 (pre-activation functionality/model update testing): To move the discussion on post-deployment validation forward, ask RAN5 to clarify the GCF procedure and whether anything similar might be considered.

- Proposal 3 (pre-activation functionality/model update testing): Further discuss “major” and “minor” model changes.

- Observation 1 (post-deployment management based on LCM): It is not obvious as yet whether monitoring can useful identify individual model behavior and whether it can be interoperable.

- Proposal 4 (post-deployment management based on LCM): Continue to work on monitoring to understand how useful and interoperable it can be before deciding on whether it could be a sole solution for post-deployment performance assurance.

- Proposal 5 (LCM requirements): The following definition for AI/ML functionality switching can be assumed by RAN4 in its AI/ML discussions:

AI/ML functionality switching is a situation when one AI/ML functionality is deactivated and another functionality is activated by the same message.

- Proposal 6 (LCM requirements): The following definition for a fallback from AI/ML operation to non-AI/ML operation can be assumed by RAN4 in its AI/ML discussions:

Fallback from AI/ML operation to non-AI/ML operation is a situation when one or more AI/ML functionalities are deactivated and there is no relevant activated AI/ML functionality left.

- Proposal 7 (LCM requirements): No fallback delay requirement for AI/ML positioning use case 1 is necessary, since AI/ML positioning use case 1 is configured by LMF as a separate positioning method, based on the current RAN2 agreements. (The statement can be rediscussed if there will be new related RAN2 agreements impacting the procedure.)

- Proposal 8 (LCM requirements): RAN4 will consider defining functionality activation delay requirement for the beam management use case at least with periodic CSI reporting, FFS for aperiodic and semi-persistent reporting.

- Proposal 9 (LCM requirements): RAN4 will consider defining functionality deactivation delay requirement for the beam management use case.

- Proposal 10 (LCM requirements): FFS: whether fallback delay includes other delays than AI/ML functionality deactivation delay for beam management (e.g., a delay until the next non-AI/ML report).

- Proposal 11 (LCM requirements): A requirement for the maximum delay until the UE actually starts the data collection after receiving the network configuration can be considered for data collection, to ensure that the data collection is not delayed. Alternatively, a UE behaviour can be specified that the UE shall start, e.g., from a first signal occasion available after receiving and decoding the command.

# References

R4-2502953, WF on AI/ML air interface study, Ericsson, Feb. 2025.

R4-2500683, Topic summary for [114][131] NR_AIML_air_part1, Qualcomm, Feb. 2025.

R4-2500684, Topic summary for [114][132] NR_AIML_air_part2, Ericsson, Feb. 2025.