3GPP TSG-RAN WG4 Meeting #114bis	R4-2503925

Wuhan, China, Apr 7th – Apr 11th, 2025

Source:	Nokia

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	7.18.2

Document for:	Discussion

Introduction

In RAN4#114 the topic of intercell and intracell inter user interference for 8Rx configuration were further discussed with focus on down selecting the testcases as well as which applicability rules to introduce.

In the following we will discuss the remaining open issues and provide our view with observations and proposals to enable further progress on the topic

Discussion

General

Definition of MMSE-IRC receiver capability for 8Rx UE

In RAN4#114 the definition of the MMSE-IRC receiver capability for 8Rx UEs were discussed (see [3]):

Definition of MMSE-IRC receiver capability for 8Rx UE Agreement: - If agreed to change the receiver definition, it also needs to be changed in Rel-18 maintenance. - If receiver definition is agreed to be changed, an LS will be sent to RAN2 for updating TS38.306. Proposals: - Option 1: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 2: Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following: - Baseline 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. - Simplified 8Rx MMSE-IRC Receiver: 8Rx MMSE-IRC receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors. - Option 3: Not to introduce new UE capability in Rel-19 or to update the existing Rel-18 UE capability definition. Instead, clearly state in the applicability rule that the new Rel-19 requirements under MU-MIMO are applicable for UE declares supporting SU-MIMO 8Rx receiver capability

As we highlighted in earlier contributions, it is our view that the current definition of baseline and simplified receiver in 38.101-4 [3] cannot be used directly for intra-cell inter-user interference configuration as the definitions focus on SU-MIMO. The definitions in 38.101-4 [5] are as follows:

Baseline SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector. Simplified SU-MIMO 8Rx Receiver: 8Rx receivers for SU-MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

Several options to rectify the issue were proposed in RAN4#114 and captured in the WF and can be condensed to the following:

- Do not introduce new UE capabilities but instead state in applicability rules that also intra-cell inter-user interference is applicable.

- Introduce new UE capabilities for intra-cell inter-user interference.

- Update existing definition in 38.101-4 [3] from Rel-18.

We do not see any reason why the definition has to contain the specific focus on MIMO type (i.e. SU-MIMO). Even for existing Rel-18 8Rx requirements removing the SU-MIMO restriction from the definitions will not make a difference as the requirements in Rel-18 are anyway focusing on SU-MIMO only. Hence, we conclude that there is no problem in simply removing SU-MIMO from the existing definitions.

Therefor our view is that in Rel-19 the definition shall be changed to be more generic to also cover the interference cases discussed in this work item.

The existing definition of baseline and simplified receivers in 38.101-4 is focused on SU-MIMO configuration. A change is needed to also cover the discussed interference cases for Rel-19.

Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

8Rx UE MMSE-IRC receiver coverage

In RAN4#114 the receiver coverage was discussed (see [3]):

8Rx UE MMSE-IRC receiver coverage Agreement: - Criteria for decision if one or two sets of PDSCH requirements are to be introduced: - Secure the span is <=2.5 dB for each receiver type by removal of outliers - If difference in the average of each receiver type for a specific configuration is > [TBD:limit] then define two sets of requirements for the specific configuration. - Further discuss the actual value for the [limit]. - Proposals: - Option 1: Set [limit] to 0.5dB - Option 2: Set [limit] to 1.0dB - Option 3: Set [limit] to 1.2dB - Other options not precluded.

In RAN4 the most used margin for defining requirements for a new feature has been if the new feature shows the average of companies simulation results to be more than 1dB of improvement compared to the average of the used reference. Similar criteria but between performance of the two receiver types being discussed were agreed in RAN4#114 but without deciding on the actual limit used.
Since the normal procedure for RAN4 to decide on high enough gain for a feature compared to existing requirements is to have at least 1dB of difference, we think that at least 1dB of difference between the individual average of the two receiver types must be seen. In addition, we feel that even a 1.2dB limit will still provide enough overlap between the results of the two receiver types to ensure that acceptable requirements are defined with only one set of requirements.

Set the limit to 1.2dB (Option 3).

PDSCH requirements with inter cell interference

Receiver type coverage

Based on the simulation results from RAN4#114 and our updated simulation results [7] and assuming potential updates from companies does not introduce significant changes, we see it as quite clear that a very high [limit] will be required to mandate only defining one set of requirements. In addition, we do not see any obstacles in the RAN4#114 simulation results nor in our updated simulation results [7] for introducing requirements for all agreed testcases.

The simulation results from RAN4#114 and our updated simulation results show it feasible to define performance requirements for all agreed testcases. In addition, the results also show, that a much higher limit is needed than the currently proposed options, for only one set of requirements to be defined. Hence, we see it very likely that two sets of requirements are to be defined independent of what limit is selected.

Define two sets of performance requirements for all previously agreed PDSCH with inter-cell interference testcases.

PDSCH requirements with intra-cell inter-user interference

Rank and receiver type coverage

In RAN4#114 it was agreed to introduce requirements for Rank 2+2 with MCS 17 however it was kept open if requirements for rank 4+4 are to be introduced.

Rank 4+4 Proposals: - Option 1: Do not define Rank 4+4 requirements. - Option 2: Define requirements for Rank 4+4 MCS 13 for baseline receiver.

Rank 2+2

Based on the simulation results from RAN4#114 and our updated simulation results [7] and assuming potential updates from companies does not introduce significant changes, we see it as quite clear that a very high [limit] will be required to mandate only defining one set of requirements for rank 2+2.

Based on the results from RAN4#114 and our updated simulation results, we see it very likely that two sets of requirements are to be defined for rank 2+2 independent of what limit is selected.

Define two sets of performance requirements for PDSCH with intra-cell inter-user interference with rank 2+2.

Rank 4+4

We have further investigated the need from network perspective with the configuration of rank 4+4. On one hand, we see a lower probability that rank 4+4 will be present in deployment. On the other hand, a scheduling with 4 layers of interference is likely to occur.	
Since in previous discussions there was opposition to define requirements with rank 2+4 (i.e. where the target UE and coUE ranks are not the same) we see having requirements for rank 4+4 as a viable option to cover the UE performance for 4 interference layers and show the UE capability for the baseline receiver. In addition, it will cover the possible, but less likely to occur, situations where rank 4+4 is scheduled.

Additionally, the following two use cases can be imagined, where the NW would still be able to co-schedule UEs with a total rank higher than the reported maximum rank from the UE and the UE would still be able to do interference cancellation of the co-scheduled layers:

- UE implementation of simplified 8Rx receiver, with channel estimation exchange between the two joint 4Rx MIMO detectors enabling the UE to cancel addition layers.

- UE implementation of a baseline 8Rx receiver, which will always be able to handle a total of 8 layers independently of how they are split between target UE and co-scheduled UE(s).

In a situation, where the UE is switching between receiver implementations to manage delay, complexity, or power, sharing of scheduling information with the UE concerning the presence of co-scheduled layers would be useful to make switching choices.

Based on above we see possible scenarios where a UE would be able to support up to rank 4 with additional co-scheduled layers (1 from above) or make full use of the UE baseline receiver if UE only reports rank4 with additional co-scheduled layers (2 from above). Note that the same would apply if UE reports max rank 2 but is co-scheduled with for example 4 layers.

We find it relevant in deployment to consider configurations with more than 4 layers (target + coUE). Using the already available simulation results we see it feasible to define performance requirements for rank4+4, which represents a situation with 4 co-scheduled layers. Furthermore, it will provide a test for the specific UE implementation where the UE supports and fully utilizes the baseline receiver in intra-cell inter-user interference cases.

Define PDSCH requirements with intra-cell inter-user interference for rank 4+4 with MCS13 to also cover deployment scenarios with 4 interference layers.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

In RAN4#114 it was decided to cover both baseline and simplified receiver for initial analysis and decide in this meeting if one or two sets of requirements will be defined (see [3])

CQI calculation algorithm Proposals: - Option 1: RAN4 to define CQI requirements for two receiver types: - Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. - Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing. - Option 2: If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined. Further discuss when more simulation results are available.

It has not yet been agreed how to determine if one or two sets of requirements will be needed for CQI reporting requirements.

For AWGN we expect no difference between simplified and baseline performance with the discussed rank 1 and rank 2.

For fading conditions, we expect the simplified receiver will perform worse than baseline receiver, however for the discussed rank 1 and rank 2 the difference is likely not much.

If the difference in gamma is below 0.1 between simplified and baseline receiver we expect only one set of requirements are needed. Based on the difference in gamma for the existing requirements for 2Rx and 4Rx in Rel-17, it is our expectation that this will be the case.

If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined. Further discuss when more simulation results are available.

CSI-IM and NZP CSI-RS configurations

In RAN4#114 the actual configuration was discussed (see [3]):

Issue 1-4-3: CSI-IM and NZP CSI-RS configurations Agreement: - Use the following CSI-RS configurations for FDD and TDD CQI requirements simulations: ZP CSI-RS configuration; CSI-RS resource Type; Periodic; Periodic. Number of CSI-RS ports (X); 4; 4. CDM Type; FD-CDM2; FD-CDM2. Density (ρ); 1; 1. First subcarrier index in the PRB used for CSI-RS (k0); Row 5,(4); Row 5,(6). First OFDM symbol in the PRB used for CSI-RS (l0); Row 5,(9); Row 5,(9). CSI-RS periodicity and offset; slot; 10/1; Same as serving cell. NZP CSI-RS for CSI acquisition; CSI-RS resource Type; Periodic; Periodic. Number of CSI-RS ports (X); 2; 1. CDM Type; FD-CDM2; noCDM. Density (ρ); 1; 1. First subcarrier index in the PRB used for CSI-RS (k0); Row 3,(6); Row 2,(6). First OFDM symbol in the PRB used for CSI-RS (l0); Row 3,(13); Row 2,(13). NZP CSI-RS-timeConfig periodicity and offset; slot; 10/1; Same as serving cell. CSI-IM configuration; CSI-IM resource Type; Periodic; Periodic. CSI-IM RE pattern; 0; 0. CSI-IM Resource Mapping (kCSI-IM,lCSI-IM); (4, 9); (6,9). CSI-IM timeConfig periodicity and offset; slot; 10/1; Same as serving cell. Proposals: - Option 1: Simulation assumptions for CQI requirements: (CSI configurations are from above agreements but kept for completeness) Parameter; Unit; Test1. Cell 1; Cell 2. Bandwidth; MHz; 10; 10. Duplex Mode; FDD; FDD. Subcarrier spacing; kHz; 15; 15. SINR; dB. Beamforming Model; As specified in Annex B.4.1. Interference Model; As specified in B.6.2. ZP CSI-RS configuration; CSI-RS resource Type; Periodic; Periodic. Number of CSI-RS ports (X); 4; 4. CDM Type; FD-CDM2; FD-CDM2. Density (ρ); 1; 1. First subcarrier index in the PRB used for CSI-RS (k0); Row 5,4; Row 5,6. First OFDM symbol in the PRB used for CSI-RS (l0); 9; 9. CSI-RS periodicity and offset; slot; 5/1; Same as serving cell. NZP CSI-RS for CSI acquisition; CSI-RS resource Type; Periodic; Periodic. Number of CSI-RS ports (X); 2; 1. CDM Type; FD-CDM2; noCDM. Density (ρ); 1; 1. First subcarrier index in the PRB used for CSI-RS (k0, k1 ); Row 3(6, -); Row 2(6, -). First OFDM symbol in the PRB used for CSI-RS (l0); 13; 13. NZP CSI-RS-timeConfig periodicity and offset; slot; 5/1; Same as serving cell. CSI-IM configuration; CSI-IM resource Type; Periodic; Periodic. CSI-IM RE pattern; 0; 0. CSI-IM Resource Mapping (kCSI-IM,lCSI-IM); (4, 9); (6, 9). CSI-IM timeConfig periodicity and offset; slot; 5/1; Same as serving cell. ReportConfigType; Periodic; Not configured. CQI-table; Table 2; Table 2. reportQuantity; cri-RI-PMI-CQI; Not configured. timeRestrictionForChannelMeasurements; Not configured; Not configured. timeRestrictionForInterferenceMeasurements; Not configured; Not configured. cqi-FormatIndicator; Wideband; Wideband. pmi-FormatIndicator; Wideband; Wideband. Sub-band Size; RB; 8; -. Csi-ReportingBand; 1111111; Not configured. CSI-Report periodicity and offset; slot; 5/0; Not configured. aperiodicTriggeringOffset; Not configured; Not configured. Codebook configuration; Codebook Type; typeI-SinglePanel; typeI-SinglePanel. Codebook Mode; 1; 1. (CodebookConfig-N1,CodebookConfig-N2); Not configured; Not configured. CodebookSubsetRestriction; Rank 1: 000001 Rank 2: 010000; Not configured. RI Restriction; N/A; Not configured. Physical channel for CSI report; PUCCH; Not configured. CQI/RI/PMI delay; ms; 8; Not configured. Maximum number of HARQ transmission; 1; Not configured. Measurement channel; For Rank 1: As specified in Table A.4-2, TBS.2-1 For Rank 2: As specified in Table A.4-2, TBS.2-2; -. INR (Note 6); dB; N/A; 10.04. Propagation condition; TDLA30-5; AWGN. Antenna configuration; 2x8; 1x8. Correlation configuration; ULA Low; N/A. Note 1: The respective received power spectral density of each interfering cell relative to Noc´ is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to EsNoc´ of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1..

We see no issue with the proposed option 1 which also includes previous agreements.

Use option 1 for defining wideband CQI reproting requirements with inter-cell interference

Applicability rule and release independency

Test applicability to skip 2/4Rx requirements

In RAN4#114 the test applicability to skip 2/4Rx requirements were discussed (see [3]):

Test applicability to skip 2/4Rx requirements Proposals: - Option 1: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, for both demodulation performance tests and CQI reporting tests, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, no matter same test configurations as 2RX/4RX are defined or not. - Option 2: For UEs supporting both 8Rx and 2/4Rx, the 2/4Rx tests under inter cell interference and under intra cell inter user interference can be skipped if the corresponding 8Rx requirements are passed. TBD: Clarification of what “corresponding 8Rx requirements” covers.

Looking at the two options from RAN4#114 there is a consensus in the two options to introduce test applicability rules to skip 2/4Rx tests in case of an 8Rx UE. There is however some inconsistency with relation to the understanding about the test configuration comparison between 2/4Rx and 8Rx requirements which can mandate the applicability rules. The following are proposed:

- All 2/4Rx requirements can be skipped if 8Rx tests are passed without looking at which 2/4Rx requirements exists.

- 2/4Rx requirements can be skipped if similar/corresponding 8Rx tests are passed.

In RAN4#114 WF it is FFT what “corresponding 8Rx requirements” would cover. From our perspective corresponding requirements relates mainly to rank (both inter-cell and intra-cell inter-user interference) and number of interference cells (inter-cell interference). For example, if the configuration of intra-cell inter-user interference with Rank 1+1 exist in 2/4Rx but not in 8Rx, then UE must pass also the 2/4Rx requirement(s) for rank 1+1. However, if the same configuration like for example rank 2+2 exists for 4Rx and 8Rx antenna configurations, then the UE only needs to pass the 8Rx requirement for said rank 2+2 (i.e. it can skip the requirements for 4Rx).

If a specific test for 8Rx configuration is done with a higher MCS compared to 2/4Rx we do not see a problem for the 8Rx UE to skip the corresponding 2/4Rx requirements as we assume the UE will perform equally well in 2/4Rx.

If an introduced 8Rx performance requirement matches the configuration of a 2Rx and/or 4Rx requirement with relation to rank and interference cells/interference UEs and the 8Rx requirement is passed, we expect the UE would also be able to pass the equivalent 2Rx and/or 4Rx requirement.

Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test:
- UE support both 8Rx and 2/4Rx
- The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement.

Release independency

In RAN4#114 the question to introduce release independency was brought up, but not further discussed, hence the options provided by companies were captured in the WF [3]:

Release independency Proposals: - Option 1: Discuss release independence requirements at a later stage in the WI. - Option 2: The new Rel-19 requirements for 8Rx should be release independent from Rel-17. - Option 3: The performance requirements for 8Rx MMSE-IRC should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17. - Option 4: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18. - Option 5: The requirement should at least be released independent from Rel-18 if no new signalling or UE capability introduced.

There are already 8Rx requirements defined in Rel-18 and it is expected that a Rel-19 UE will perform equal in interference scenarios to a Rel-18 UE, hence we see no issue for new requirements for 8Rx to also be applicable from Rel-18.

In addition, the following applies:

- The 2Rx and 4Rx with inter-cell and intra-cell inter-user interference requirements are defined in Rel-17, hence a Rel-17 UE is expected to have IRC receiver implemented.

- The 8Rx requirements defined in Rel-18 are release independent from Rel-17 (TS38.307 Table 5.4-1)

Since 8Rx SU-MIMO requirements are release independent from Rel-17 and requirements for 2Rx and 4Rx with inter-cell and intra-cell inter-user interference are introduced in Rel-17, the interference requirements defined for 8Rx can be release independent from Rel-17.

The new Rel-19 requirements for 8Rx shall be release independent from Rel-17. Further discuss if the requirements shall be mandatory for Rel-18 and optional for Rel-17

Conclusion

In this paper we have provided our view on the topic of requirement definition for 8Rx inter-cell and intra-cell inter-user requirements.

The following Observations and Proposals were made:

General

Definition of MMSE-IRC receiver capability for 8Rx UE

- The existing definition of baseline and simplified receivers in 38.101-4 is focused on SU-MIMO configuration. A change is needed to also cover the discussed interference cases for Rel-19.

- Redefine the simplified and baseline receiver definitions in 38.101-4 section 3.1 to the following:
- Baseline 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of up to 8 layers with joint 8Rx MIMO detector.
- Simplified 8Rx Receiver: 8Rx receivers for MIMO transmissions with support of only up to 4 layers with two joint 4Rx MIMO detectors.

8Rx UE MMSE-IRC receiver coverage

Set the limit to 1.2dB (Option 3)

PDSCH requirements with inter cell interference

Receiver coverage

The simulation results from RAN4#114 and our updated simulation results show it feasible to define performance requirements for all agreed testcases. In addition, the results also show, that a much higher limit is needed than the currently proposed options, for only one set of requirements to be defined. Hence, we see it very likely that two sets of requirements are to be defined independent of what limit is selected.

Define two sets of performance requirements for all previously agreed PDSCH with inter-cell interference testcases.

PDSCH requirements with intra cell inter user interference

Rank and receiver coverage

Based on the results from RAN4#114 and our updated simulation results, we see it very likely that two sets of requirements are to be defined for rank 2+2 independent of what limit is selected.

Define two sets of performance requirements for PDSCH with intra-cell inter-user interference with rank 2+2.

We find it relevant in deployment to consider configurations with more than 4 layers (target + coUE). Using the already available simulation results we see it feasible to define performance requirements for rank4+4, which represents a situation with 4 co-scheduled layers. Furthermore, it will provide a test for the specific UE implementation where the UE supports and fully utilizes the baseline receiver in intra-cell inter-user interference cases.

Define PDSCH requirements with intra-cell inter-user interference for rank 4+4 with MCS13 to also cover deployment scenarios with 4 interference layers.

Wideband CQI reporting requirements with inter-cell interference

CQI calculation algorithm

If the difference in gamma between baseline and simplified receiver is 0.1 or lower only one set of requirements shall be defined. Further discuss when more simulation results are available.

CSI-IM and NZP CSI-RS configurations

Use option 1 for defining wideband CQI reproting requirements with inter-cell interference.

Applicability rule and release independency

Test applicability to skip 2/4Rx requirements

If an introduced 8Rx performance requirement matches the configuration of a 2Rx and/or 4Rx requirement with relation to rank and interference cells/interference UEs and the 8Rx requirement is passed, we expect the UE would also be able to pass the equivalent 2Rx and/or 4Rx requirement.

Introduce test applicability under inter-cell interference and under intra-cell inter user interference to skip a 2/4Rx requirement if at least the following applies for each considered 8Rx test:
- UE support both 8Rx and 2/4Rx
- The UE passes an 8Rx requirement with same rank and interference configuration as the 2/4Rx requirement.

Release independency

Since 8Rx SU-MIMO requirements are release independent from Rel-17 and requirements for 2Rx and 4Rx with inter-cell and intra-cell inter-user interference are introduced in Rel-17, the interference requirements defined for 8Rx can be release independent from Rel-17.

The new Rel-19 requirements for 8Rx shall be release independent from Rel-17. Further discuss if the requirements shall be mandatory for Rel-18 and optional for Rel-17

- References

- R4-2416555 - WF for [112bis][320] NR_demod_Ph5_Part2_UE, Nokia

- R4-2419784 - WF for [113][322] NR_demod_Ph5_Part2_UE, Nokia

- R4-2502275 - WF for [114][324] NR_demod_Ph5_Part2_UE

- TS38.212 - Multiplexing and channel coding (Release 18)

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- RP-241297, “NR demodulation performance: Phase 5”, RAN#104, June 2024

- R4-2503926 - Simulations for UE demodulation performance req. for 8Rx with MMSE-IRC