3GPP TSG-RAN WG4 Meeting #114bis 	    R4-2503930

Wuhan, China, 07 – 11 April 2025

Agenda Item:	7.21.2.2

Source: 	Ericsson

Title:	On requirements for event prediction for AI/ML based mobility

Document for:	Approval

# Introduction

In the last RAN4 meeting it was agreed to start the discussion on measurement event prediction use case in RAN4#114bis meeting. In this contribution issues related to direct and indirect event prediction use cases are highlighted and the impact on RAN4 requirement due to event predicted measurement reporting are discussed.

Agreement from RAN4#114 [1]

- RAN4 will start the discussion on measurement event prediction use case in RAN4#114bis meeting.

# Discussion

In legacy event triggered reporting, UE is configured with the event and the associated condition for measurement reporting. The UE continuously monitors if the event condition is met. When the event condition is met, UE starts TTT timer and starts the measurement reporting to the network over the air interface given the event condition persists until the TTT timer expires. The measurement reporting delay in this case is defined as the time between the occurrence of an event and the point in time when the UE starts to transmit the measurement report over the air interface to the network. An example timeline of legacy event triggered reporting is shown in Figure 1.

Figure 1. Elaboration of legacy event triggered reporting.

To enable event triggered reporting for AI/ML based mobility, indirect and direct event predictions are being considered for evaluations in RAN2. For indirect event prediction, the UE is expected to perform the prediction of an event by exploiting the RRM measurements predicted in temporal domain. In direct event prediction, the model output is the probability of event occurrence within a time window.

In both prediction-based approaches, the UE predicts the time when the condition for the event occurrence will be met and whether the event triggered reporting condition persists until the TTT timer expires. The event condition holding true until the expiry of TTT timer is based on predictions, such that the event and event condition holding true until the expiry of the TTT timer is evaluated in advance by the UE before the actual event condition is met and the event condition has persisted until the expiry of the TTT timer. Based on the prediction, if the UE determines that the prediction of the event condition holds true until the expiry of the TTT timer, UE reports the measurement to the network. Unlike in the legacy event triggered reporting, reporting of measurements based on the event prediction, in principle, occurs earlier than the actual reporting of the measurements based on the occurrence of the actual event. In this regard, the reporting delay requirement defined for the legacy event triggered reporting may not hold true for the event prediction cases.

Observation 1: Reporting delay requirement defined for the legacy event triggered reporting may not hold true for the event prediction case.

Proposal 1: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting.

Figure 2. Implication of event prediction on event triggered reporting for mobility.

Event triggered reporting was introduced for mobility in NR to allow the network to perceive real-time radio conditions at UE when it is needed. Measurement reporting is only triggered upon occurrence of an event, thus reducing the signaling overhead while RRM measurement is reported by the UE for the mobility purposes. In our view, this principle should be kept intact when AI/ML model is used by the UE to predict event. To ensure this, measurement reporting based on event prediction should not result into additional signaling overhead compared to the signaling for legacy event triggered based measurement reporting. Furthermore, it should also be ensured that the measurement reporting based on event prediction should not result into the degradation of HO performance. For example, false detection of an event may result into additional signaling and missed detection of an event may result into degradation in HO performance. To ensure additional signaling overhead is not introduced and HO performance is not degraded due to the measurement reporting based on the event prediction, relevant performance requirement for event prediction needs to be studied in RAN4. The RAN2 agreements from RAN2#127bis meeting further validates the need for performance requirement for event prediction.

Agreements from RAN2#127bis [2]

- For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighbouring cells.  The final output is the expected occurrence time of a certain measurement event (ex. event A3).

- For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

- A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT).

- As baseline, we will use RLF event prediction KPI:

  - for indirect: F1 score.  the following can be reported: RSRP difference, missed event detection, false event detection.  FFS how to define F1 score.

  - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction.

  - for direct: F1 score.  The following can be reported: missed event detection, false event detection,

- Continue discussion over email discussion to see if there is a difference.

In RAN2#129 meeting, results for measurement predictions were analyzed and the following were concluded:

Agreement from RAN2#129 [3]

- Indirect prediction: Case A

  - Most of the results show that the F1 score for prediction of measurement event is very good.

  - For handover decision option 3 and 2 AI/ML performs better than baseline (legacy) in terms of HO failure rate.

- Indirect prediction: Case B

  - Measurement event case B can have very good F1 score depending on filtering approach/PW

  - F1 score is good for the PW window for case B (i.e. low PW)

  - F1 score decreases with an increasing MRRT value

  - We will focus on finding cases that bring benefits to the system rather than trying to compare

  - With indirect measurement event prediction based on temporal domain Case B (MRRT=50%), the AI-based HO has a minor/no system-level performance (i.e., HOF rate and HO number) decrease compared with the legacy HO mechanism.

Indirect prediction case-A in the RAN2 agreement above refers to the scenario where the predicted RRM measurements for temporal prediction for case-A are used for event prediction. And the indirect prediction case-B in the RAN2 agreement above refers to the scenario where the predicted RRM measurements for temporal prediction for case-B are used for event prediction. The accuracy of indirect prediction relies on the accuracy of temporal prediction of RRM measurements. Therefore, for indirect event prediction the requirements for RRM measurement prediction in temporal can be reused. Potentially no new or additional requirement may need to be defined for the indirect event prediction use case. However, relevant requirements for the direct event prediction may need to be defined provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility.

Proposal 2: Requirements for RRM measurement prediction in temporal domain is reused for indirect event prediction use cases.

Proposal 3: Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility.

# Summary

In this contribution issues related to event prediction use cases are discussed. The observations and proposals below summarize the discussion presented in this paper.

Observation 1: Reporting delay requirement defined for the legacy event triggered reporting may not hold true for the event prediction case.

Proposal 1: RAN4 to consider impact on reporting delay requirement due to predicted event triggered reporting.

Proposal 2: Requirements for RRM measurement prediction in temporal domain is reused for indirect event prediction use cases.

Proposal 3: Performance requirement for direct event prediction needs to be defined for event prediction provided direct event prediction is supported by the framework designed by RAN2 for AI/ML based mobility.

# References

- R4-2502594, WF on AI/ML Mobility, Moderator (Nokia).

- Chair’s note from RAN2#127bis, RAN2 Chair (InterDigital).

- Chair’s note from RAN2#129, RAN2 Chair (InterDigital).