# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503940

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			NTN Ku bands – General aspects on bands

Agenda item:		7.11.1

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised ([1]) including support for 15 kHz SCS.

In last RAN4#114 meeting, RAN4 agreed on the frequency range definition of the NTN Ku-bands in both FR1-NTN and FR2-NTN and their corresponding system parameters ([4]).

In this contribution, we are clarifying the applicability of the NTN Ku-bands and initiate some preliminary discussion on the 2nd priority frequency range described in the WID ([1]).

# Discussion

## Applicability of NTN Ku-bands

In last RAN4#114 meeting, the following frequency ranges for the FR1-NTN bands and corresponding FR2-NTN bands have been agreed [4].

Table 1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 | 14000 MHz - 14500 MHz | 10700 MHz – 127504 MHz | FDD |
| n508 | 13750 MHz - 14000 MHz | 10700 MHz – 127504 MHz | FDD |
| NOTE 4: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. |  |  |  |

Table 2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n247 | 13750 MHz - 14000 MHz | 10700 MHz – 127501 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT.. |  |  |  |

From the previous RAN4 agreements ([2] and [3]):

- The n248/n509 bands were defined for the CEPT countries, North America, Taiwan, Japan and Australia.

- The n247/n508 bands (13.75-14.0 GHz frequency range) were defined for the studied regulations (i.e. Taiwan, Japan, Australia) in the region 3.

This applicability should be reflected in the bands table, at least for n247/n508.

Also, following the WI updates made in RAN#106, it should be clarified that, for the time being, those bands have been specified in 3GPP for operation with Mobile VSAT only. This clarification would be needed to avoid any potential misunderstanding in the future and should be removed of course as soon as fixed VSAT operation in the NTN Ku-bands will be analyzed in RAN4.

For the above reasons, we make the following proposal.

Proposal1: Update to the 2 NTN Ku-bands tables according to the following:

Table 1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 2 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n508 2, 3 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. NOTE 2: In this version of the specification, only operation with Mobile VSAT is supported in this band. NOTE 3: This band is applicable in Region 3 (Taiwan, Japan and Australia) only. |  |  |  |

Table 2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 2 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n247 2, 3 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. NOTE 2: In this version of the specification, only operation with Mobile VSAT is supported in this band. NOTE 3: This band is applicable in [Region 3 (Taiwan, Japan and Australia)] only. |  |  |  |

## Work Item - 2nd priority Frequency Range

The WI specified another set of frequency ranges to be considered as 2nd priority, as shown in Figure 1.

Figure 1: Extract from the Work Item - 2nd priority frequency range

It’s too early to study in details the corresponding regulations, RAN4 should better focused on progressing on the consistency of FR1-NTN and FR2-NTN requirements for SAN and Mobile VSAT. Nevertheless, RAN4 should better have some initial discussion on those frequency ranges to clarify some aspects.

First, it should be noted that US is excluded from this 2nd priority frequency range. Checking with ISED, there is no ESIM allocation in the 12.75-13.25 GHz frequency range (Figure 2).

Figure 2: ISED allocation

Based on those observations, the 2nd priority frequency range proposed in the WI might not be relevant for North America in the scope of this revised WI (Mobile VSAT only).

Observation1: The 12.75-13.25 GHz frequency range is excluded for USA (WI’s scope) and seems not relevant for Canada (no Mobile VSAT allocation).

In Region 1, looking at CEPT/EC Decision, only ECC Decision (19)04 covers that 2nd priority frequency range, but it’s restricted to earth station on-board aircraft. Regarding ETSI Harmonized Standards, this ECC Decision refers to:

- EN 302 186 for operation with GSO. A version 2.2.1 has been published in 2021 but has never been cited in the Official Journal. Instead, there is a revision which has just been initiated, with an expected publication in November 2025 (according to ETSI Work Program for DEN/SES-00477 WI). As ETSI initiated a revision and the initial version was not cited, we should expect there was something not aligned with the Legislation in this initial version.

- EN 303 984 for operation with NGSO. But only a mature draft of this EN is available, its publication is planned for May 2026 (according to ETSI Work Program for DEN/SES-00449 WI).

It looks then premature to specify this 2nd priority frequency range for CEPT countries.

Observation2: It looks premature to consider the WI’s 2nd priority frequency range (12.75-13.25 GHz) for CEPT countries.

Proposal2: It should be clarified which Regulation should be considered for the WI 2nd priority frequency range (12.75-13.25 GHz) with the understanding that:

- USA and Canada don’t allocate ESIM operation in this frequency range

- It would be premature to consider CEPT Regulation observing that ETSI Harmonized Standards are not yet ready.

# Conclusion

In this contribution, we analyzed the applicability of the agreed NTN Ku-bands and initiated some preliminary discussion on the 2nd priority frequency range described in the  WID. We made the following observations and proposals:

Proposal1: Update to the 2 NTN Ku-bands tables according to the following:

Table 1: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n509 2 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n508 2, 3 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. NOTE 2: In this version of the specification, only operation with Mobile VSAT is supported in this band. NOTE 3: This band is applicable in Region 3 (Taiwan, Japan and Australia) only. |  |  |  |

Table 2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n248 2 | 14000 MHz - 14500 MHz | 10700 MHz – 127501 MHz | FDD |
| n247 2, 3 | 13750 MHz - 14000 MHz | 10700 MHz – 12750 MHz | FDD |
| NOTE 1: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT. NOTE 2: In this version of the specification, only operation with Mobile VSAT is supported in this band. NOTE 3: This band is applicable in [Region 3 (Taiwan, Japan and Australia)] only. |  |  |  |

Observation1: The 12.75-13.25 GHz frequency range is excluded for USA (WI’s scope) and seems not relevant for Canada (no Mobile VSAT allocation).

Observation2: It looks premature to consider the WI’s 2nd priority frequency range (12.75-13.25 GHz) for CEPT countries.

Proposal2: It should be clarified which Regulation should be considered for the WI 2nd priority frequency range (12.75-13.25 GHz) with the understanding that:

- USA and Canada don’t allocate ESIM operation in this frequency range.

- It would be premature to consider CEPT Regulation observing that ETSI Harmonized Standards are not yet ready.

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Eutelsat

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Eutelsat