# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503942

# Wuhan, Hubei, China, 07th – 11th April, 2025

Source:			Ericsson

Title:			NTN Ku bands –VSAT RF

Agenda item:		7.11.4

Document for:		Approval

# Introduction

In last RAN#107, the NTN KU-band WI was revised ([1]) including support for 15 kHz SCS.

In last RAN4#114 meeting, RAN4 agreed on the frequency range definition of the NTN Ku-bands in both FR1-NTN and FR2-NTN and their corresponding system parameters ([2]), and some agreements were also reached on NTN VSAT ([4]).

In this contribution, we are further studying the existing regulations, clarifying the context and corresponding requirements.

# Discussion

## ESIM types and corresponding regulation

### Background

In last RAN4#114 meeting, it was agreed ([4]) to consider the 3 types of ESIM for the NTN Ku-bands.

As we initially focused on Land ESIM in our previous contributions, we are also checking the Regulations for A-ESIM and M-ESIM in the following sub-sections.

### FCC

Most of the requirements are common for all ESIM types, except for A-ESIM for which FCC specified additional PFD limits in 25.228(i) to protect fixed service network (when they have primary allocation in that band), as shown in the following:

### ISED

From our first investigation, ISED has not specified any specific requirements for one type of ESIM.

### CEPT and ETSI

The ECC Decisions applicable for each type of ESIM and for the considered frequency ranges are captures in the following table.

|  | 10.7-12.75 GHz and 14.0-14.5 GHz |  | 10.7-12.75 GHz and 12.75-13.25 GHz |  |
| --- | --- | --- | --- | --- |
|  | NGSO | GSO | NGSO | GSO |
| Land | ECC Dec(18)05 | ECC Dec(18)04 |  |  |
| Aeronautical | ECC Dec(18)05 |  | ECC Dec(19)04 | ECC Dec(19)04 |
| Maritime | ECC Dec(18)05 | ECC Dec(05)10 |  |  |

The ETSI Harmonized Standards applicable for each type of ESIM and for the considered frequency ranges are captures in the following table.

|  | 10.7-12.75 GHz and 14.0-14.5 GHz |  | 10.7-12.75 GHz and 12.75-13.25 GHz |  |
| --- | --- | --- | --- | --- |
|  | NGSO | GSO | NGSO | GSO |
| Land | EN 303 980 EN 303 981 | EN 302 977 |  |  |
| Aeronautical |  | EN 302 186 (*) | EN 303 984 (**) | EN 302 186 (*) |
| Maritime |  | EN 302 340 |  |  |

(*) A version 2.2.1 has been published in 2021 but has never been cited in the Official Journal. Instead, there is a revision which has just been initiated, with an expected publication in November 2025.

(**) Only a mature draft of this EN is available, its publication is planned for May 2026.

## NS

RAN4 agreed to specify each NTN Ku-band in both FR1-NTN and FR2-NTN ([2]). Consequently, to restrict the number of bands when introducing a new NTN KU-band, it was agreed to specify only 2 bands in FR1-NTN (n247 and n248) and 2 corresponding bands in FR2-NTN (n508 and n509). More bands could still be specified later one (e.g. when considering the 2nd priority frequency range listed in the WI).

As the NTN Ku-bands are not country specific, RAN4 should also specify new NSs to handle the Regulatory requirements specific to each country/Region. So far, only FCC, ISED and CEPT Regulations have been studied and would require specifying new NSs. It’s assumed that Taiwan and Australia regulations are fully aligned with FCC one while Japan Regulation is fully aligned with CEPT one. If not, additional NSs would be needed.

Proposal1: Specify at least the following new NSs:

- In FR1-NTN:

  - NS_06N: to handle FCC and ISED Regulation.

  - NS_07N: to handle CEPT Regulation.

- In FR2-NTN:

  - NS_203N: to handle FCC and ISED Regulation.

  - NS_204N: to handle CEPT Regulation.

## PFD limits

In last RAN4#114 meeting, capturing PFD limits in TS 38.101-5 was left FFS ([4]):

If PFD limits for SAN might not be relevant for 3GPP as it’s depending on SAN deployment, the situation is not the same for the VSAT as the PFD limits are usually not depending on the VSAT location.

Moreover, as it can be observed in EN 302 186, the PFD limits are specified, strengthening the need to capture those PFD limits in 3GPP specifications as well.

For the reasons mentioned above, we make the following proposal:

Proposal2: Capture PFD limits for NTN VSAT when specified by the Regulation.

Based on this proposal, the following PFD should then be captured:

- FCC:

  - A-ESIM, to protect fixed service network (when they have primary allocation in that band). 25.228(i):

- CEPT:

  - A-ESIM with NGSO – ECC Dec(18)05

    - In 14.25-14.5 GHz where FS systems are authorized in that band:

  - L-ESIM with NGSO – ECC Dec(18)05

    - In 14.25-14.5 GHz where FS systems are authorized in that band:

PFD limit of -116 dBW/m²/MHz at 30 m height above ground

    - In 14.47-14.5 GHz:

PFD threshold value at the observatory of -169 dBW/m²/(150 kHz), not to be exceeded more than 2% of the time (Ref: Recommendation ITU-R RA.769

  - M-ESIM with NGSO – ECC Dec(18)05

    - In 14.25-14.5 GHz where FS systems are authorized in that band:

the PFD threshold value is -116 dBW/m²/MHz at a height of 80 metres above mean sea level at the low-water mark

    - In 14.47-14.5 GHz:

PFD threshold value at the observatory of -169 dBW/m²/(150 kHz), not to be exceeded more than 2% of the time (Ref: Recommendation ITU-R RA.769)

## Remaining open issues for CEPT countries

### On-axis spurious requirements

In last RAN4#114bis meeting, the following issue was captured ([4]):

As we explained in [5], to remain consistent with 3GPP approach, when the same requirement is specified in different way in the ETSI Harmonized Standards, 3GPP shall consider the most stringent option to make sure 3GPP specification is compliant with all ENs. Therefore, the [] should be removed from the above issue:

Proposal3: For CEPT countries, specify the On-axis spurious requirements for the “emissions disabled” and “carrier-off” states according to the following table:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 9 | 100 |

And, as also justified in [5], the On-axis spurious requirement for the “carrier-on” state shall be specified according to the following:

Proposal4: For CEPT countries, specify the On-axis spurious requirements for the “carrier-on” state according to the following tables:

- Outside a bandwidth of 5 times the occupied bandwidth:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 34 - K (K=10log10(N) with N the number of terminals) | 100 |

- Inside a bandwidth of 5 times the occupied bandwidth and outside the transmission bandwidth:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 48 - K (K=10log10(N) with N the number of terminals) | 100 |

### Off-axis spurious requirements

In last RAN4#114bis meeting, the following issue was captured ([4]):

As we mentioned in [5], the off-axis spurious emissions requirement was specified differently in the relevant ETSI Harmonized Standards, as shown in the following.

EN 303 980 / EN 303 981					EN 302 977

To remain consistent with 3GPP approach, when the same requirement is specified in different way in the ETSI Harmonized Standards, 3GPP shall consider the most stringent option to make sure 3GPP specification is compliant with all ENs. We propose then the following.

Proposal5: For CEPT countries, specify the Off-axis spurious requirements for the “Emission disabled” state according to the following table (Note that we kept the “EIRP limit” column with limits in dBpW for an easier understanding of the proposal but it should be removed when specifying the requirement):

Table 3: Limits of off-axis spurious EIRP - "Transmission disabled" state

| Frequency band | EIRP limit | EIRP limit (to be removed) | Measurement bandwidth |
| --- | --- | --- | --- |
| 1,0 GHz to 2,0 GHz | -38dBm | 42 dBpW | 100 kHz |
| 1,0 GHz to 10,7 GHz | -42 dBm | 48 dBpW | 100 kHz |
| 10,7 GHz to 21,2 GHz | -36 dBm | 54 dBpW | 100 kHz |
| 21,2 GHz to 40,0 GHz | -30 dBm | 60 dBpW | 100 kHz |

Proposal6: For CEPT countries, specify the Off-axis spurious requirements for the “carrier-on” and “carrier-off” states according to the following table (Note that we kept the “EIRP limit” column with limits in dBpW for an easier understanding of the proposal but it should be removed when specifying the requirement):

Table 4: Limits of off-axis spurious EIRP "Transmission Enabled" state

| Frequency band | EIRP limit | EIRP limit (to be removed) | Measurement bandwidth |
| --- | --- | --- | --- |
| 1,0 GHz to 2,0 GHz | -47 dBm | 43 dBpW | 100 kHz |
| 2,0 GHz to 3,4 GHz | -41 dBm | 49 dBpW | 100 kHz |
| 3,4 GHz to 10,7 GHz | -35 dBm | 55 dBpW | 100 kHz |
| 10,7 GHz to 13,75 GHz | -29 dBm | 61 dBpW | 100 kHz |
| 13,75 GHz to 14,00 GHz | 5 dBm (see note) | 95 dBpW (see note) | 10 MHz |
| 14,50 GHz to 14,75 GHz | 5 dBm (see note) | 95 dBpW (see note) | 10 MHz |
| 14,75 GHz to 21,2 GHz | -29 dBm | 61 dBpW | 100 kHz |
| 21,2 GHz to 27,35 GHz | -23 dBm | 67 dBpW | 100 kHz |
| 27,35 GHz to 31,15 GHz | -15 dBm | 75 dBpW | 100 kHz |
| 31,15 GHz to 60,00 GHz | -23 dBm | 67 dBpW | 100 kHz |
| NOTE: This limit may be exceeded in a frequency band which shall not exceed 50 MHz, centred on the carrier frequency, provided that the on-axis EIRP density at the considered frequency is 50 dB below the maximum on-axis EIRP density of the signal (within the nominated bandwidth) expressed in dBW/100 kHz. |  |  |  |

### Antenna related requirements

For the antenna pointing accuracy and performance requirements and for the receiver antenna off-axis performance, EN 303 980 and EN 302 977 have also specified different requirements, which are also different from TS 38.101-5 for EN 301 980/EN303 981.

Further study and discussion would be needed to finalize those requirements in RAN4.

## FCC and ISED requirements comparison

FCC and ISED requirements are almost aligned. Nevertheless, there are some small differences, RAN4 should discuss how to capture those differences. There are 2 options:

- RAN4 specifies requirements corresponding to the most stringent ones of the 2 Regulations.

- RAN4 specifies 2 additional NSs (one for FCC and one for ISED) when requirements are different.

Proposal7: RAN4 shall decide on how to handle FCC and ISED requirements when they are different:

- Capture the most stringent requirement of the 2 Regulations in TS 38.101-5.

- Introduce 2 new NSs (one for FCC and one for ISED) when requirements are different.

In the following table, we quickly compare the FCC and ISED requirements.

|  | FCC | ISED |
| --- | --- | --- |
| Power limitation | Same | Same |
| Power stability | None ? | 1dB |
| Frequency tolerance | 0.001% | 0.005% |
| OOBE | Same | Same |
| Off-axis EIRP density envelope | Meas. Bandwidth: 4kHz Slightly different range of Off-axis angle | Meas. Bandwidth: 40kHz Slightly different range of Off-axis angle |
| Antenna performance | Requirements for plane tangent to GSO arc and plane perpendicular to GSO arc | Only one set of requirements One additional range of off-axis angle |
| PFD | To protect NASA and RAS | None ? |

## Other Regulations

As captured in the previous Way Forward, RAN4 agreed so far to specify the NTN Ku-bands for countries following FCC, ISED, CEPT, Japan, Taiwan and Australia.

Nevertheless, only FCC, ISED and CEPT Regulations have been studied so far.

We’d like to strongly encourage the interested companies to contribute and share their analysis of the other Regulations.

Proposal8: Only FCC, ISED and CEPT Regulations have been studied until now. Other Regulations shall be further studied as well to justify the n247, 248, n508 and n509 bands’ applicability.

# Conclusion

In this contribution, we analyzed the NTN VSAT RF specification TS 38.101-5 for the new NTN Ku-bands considering the last update of the WID. We made the following observations and proposals:

Proposal1: Specify at least the following new NSs:

- In FR1-NTN:

  - NS_06N: to handle FCC and ISED Regulation.

  - NS_07N: to handle CEPT Regulation.

- In FR2-NTN:

  - NS_203N: to handle FCC and ISED Regulation.

  - NS_204N: to handle CEPT Regulation.

Proposal2: Capture PFD limits for NTN VSAT when specified by the Regulation.

Proposal3: For CEPT countries, specify the On-axis spurious requirements for the “emissions disabled” and “carrier-off” states according to the following table:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 9 | 100 |

Proposal4: For CEPT countries, specify the On-axis spurious requirements for the “carrier-on” state according to the following tables:

- Outside a bandwidth of 5 times the occupied bandwidth:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 34 - K (K=10log10(N) with N the number of terminals) | 100 |

- Inside a bandwidth of 5 times the occupied bandwidth and outside the transmission bandwidth:

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 48 - K (K=10log10(N) with N the number of terminals) | 100 |

Proposal5: For CEPT countries, specify the Off-axis spurious requirements for the “Emission disabled” state according to the following table (Note that we kept the “EIRP limit” column with limits in dBpW for an easier understanding of the proposal but it should be removed when specifying the requirement):

Table 3: Limits of off-axis spurious EIRP - "Transmission disabled" state

| Frequency band | EIRP limit | EIRP limit (to be removed) | Measurement bandwidth |
| --- | --- | --- | --- |
| 1,0 GHz to 2,0 GHz | -38dBm | 42 dBpW | 100 kHz |
| 1,0 GHz to 10,7 GHz | -42 dBm | 48 dBpW | 100 kHz |
| 10,7 GHz to 21,2 GHz | -36 dBm | 54 dBpW | 100 kHz |
| 21,2 GHz to 40,0 GHz | -30 dBm | 60 dBpW | 100 kHz |

Proposal6: For CEPT countries, specify the Off-axis spurious requirements for the “carrier-on” and “carrier-off” states according to the following table (Note that we kept the “EIRP limit” column with limits in dBpW for an easier understanding of the proposal but it should be removed when specifying the requirement):

Table 4: Limits of off-axis spurious EIRP "Transmission Enabled" state

| Frequency band | EIRP limit | EIRP limit (to be removed) | Measurement bandwidth |
| --- | --- | --- | --- |
| 1,0 GHz to 2,0 GHz | -47 dBm | 43 dBpW | 100 kHz |
| 2,0 GHz to 3,4 GHz | -41 dBm | 49 dBpW | 100 kHz |
| 3,4 GHz to 10,7 GHz | -35 dBm | 55 dBpW | 100 kHz |
| 10,7 GHz to 13,75 GHz | -29 dBm | 61 dBpW | 100 kHz |
| 13,75 GHz to 14,00 GHz | 5 dBm (see note) | 95 dBpW (see note) | 10 MHz |
| 14,50 GHz to 14,75 GHz | 5 dBm (see note) | 95 dBpW (see note) | 10 MHz |
| 14,75 GHz to 21,2 GHz | -29 dBm | 61 dBpW | 100 kHz |
| 21,2 GHz to 27,35 GHz | -23 dBm | 67 dBpW | 100 kHz |
| 27,35 GHz to 31,15 GHz | -15 dBm | 75 dBpW | 100 kHz |
| 31,15 GHz to 60,00 GHz | -23 dBm | 67 dBpW | 100 kHz |
| NOTE: This limit may be exceeded in a frequency band which shall not exceed 50 MHz, centred on the carrier frequency, provided that the on-axis EIRP density at the considered frequency is 50 dB below the maximum on-axis EIRP density of the signal (within the nominated bandwidth) expressed in dBW/100 kHz. |  |  |  |

Proposal7: RAN4 shall decide on how to handle FCC and ISED requirements when they are different:

- Capture the most stringent requirement of the 2 Regulations in TS 38.101-5.

- Introduce 2 new NSs (one for FCC and one for ISED) when requirements are different.

Proposal8: Only FCC, ISED and CEPT Regulations have been studied until now. Other Regulations shall be further studied as well to justify the n247, 248, n508 and n509 bands’ applicability.

# References

- RP-250799, Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT

- R4-2502290, Way Forward for [114][313] NR_NTN_Ku_Band_General, Eutelsat

- R4-2503944, Draft CR to TS 38.101-5 – Needed updates to support NTN VSAT in FR1-NTN, Ericsson

- R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

- R4-2501356, NTN Ku bands –WI update impacts on NTN VSAT RF, Ericsson