# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503960

# Wuhan, Hubei, China, 07th – 11th April, 2025

# Source: 	ZTE Corporation

# Title: 	Discussion on the ATG BS CA conformance test

# Agenda item:	7.12.3

# Document for:	Approval

# 1 Introduction

The RF requirements for ATG BS supporting CA were extensively discussed in the past RAN4 meetings, and the draft CR to TS38.104 to introduce ATG BS CA requirements was endorsed in [1]. Meanwhile, there are two draft CRs to TS38.141-1 and TS38.141-2 in last meeting, but these two draft CR were not endorsed due to there are no discussion for ATG BS CA conformance test.

In this contributions, we give discussions on the test tolerance for ATG BS CA conformance test.

# 2	Discussion

Actually, for the two draft CRs [2,3] for TS38.141-1/2 specification, they just reflect the same changes as TS38.104 such as multi-band operation, TAE requirements and so on. For the other conformance testing issue such as measurement uncertainty in the test requirement, there are no discussions/agreements so far.

In terms of the previous discussion, all of the NR BS CA RF requirements are reused by ATG BS CA, there are no new RF requirements introduced for ATG BS CA.

For conformance testing, the test tolerance/uncertainty should be considered for test requirements, which equal to minimum requirements + TT(test tolerance), different RF minimum requirements for either single carrier or CA operation has different test tolerance. For example, the test tolerance for TAE is 25ns in table C.1-1.

Table C.1-1: Derivation of test requirements (Transmitter tests)

| Test | Minimum requirement in TS 38.104 [2] | Test Tolerance (TT) | Test requirement in the present document |
| --- | --- | --- | --- |
| 6.2 Base station output power | See TS 38.104 [2], clause 6.2 | Normal and extreme conditions: 0.7 dB, f ≤ 3.0 GHz 1.0 dB, 3.0 GHz < f ≤ 7.125GHz (Note 1) 1.5 dB, for bands n46, n96 and n102 | Formula: Upper limit + TT, Lower limit - TT |
| 6.3 Output power dynamics | See TS 38.104 [2], clause 6.3 | 0.4 dB | Formula: Total power dynamic range – TT (dB) |
| 6.4.1 Transmitter OFF power | See TS 38.104 [2], clause 6.4.1 | 2.0 dB, f ≤ 3.0 GHz 2.5 dB, 3.0 GHz < f ≤ 7.125 GHz (Note 1) 3.0 dB, for bands n46, n96 and n102 | Formula: Minimum Requirement + TT |
| 6.4.2 Transient period | See TS 38.104 [2], clause 6.4.2 | N/A |  |
| 6.5.2 Frequency error | See TS 38.104 [2], clause 6.5.1 | 12 Hz | Formula: Frequency Error limit + TT |
| 6.5.4 Time alignment error | See TS 38.104 [2], clause 6.5.3 | 25ns | Formula: Time alignment error limit + TT+ TT |
| 6.5.3 Modulation quality (EVM) | See TS 38.104 [2], clause 6.5.2 | 1% | Formula: EVM limit + TT |
| 6.6.2 Occupied bandwidth | See TS 38.104 [2], clause 6.6.2 | 0 Hz | Formula: Minimum Requirement + TT |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | See TS 38.104 [2], clause 6.6.3 | ACLR/CACLR: BW ≤ 20MHz: 0.8dB BW > 20MHz: 1.2 dB Absolute ACLR/CACLR: 0 dB | Formula: ACLR Minimum Requirement - TT Absolute limit +TT |

The Test Tolerances are derived from Test System uncertainties, regulatory requirements and criticality to system performance. In our understanding, the measurement uncertainty and test uncertainty are largely dependent on operating frequency/band, channel bandwidths and the carrier numerology.

For ATG BS CA conformance testing, we think the same test equipment as TN BS would likely be used for these conformance tests, and considering the CA band combination supported by ATG BS are the same as TN BS, we think the current measurement uncertainty and test tolerance for TN BS CA conductive and OTA conformance test can be reused for ATG BS conductive and OTA conformance test, respectively.

Therefore, we have following proposal.

Proposal 1: For ATG BS conductive and OTA conformance CA RF requirements test, to adopt the same test tolerance for TN BS conductive and OTA CA RF requirements conformance test, respectively.

For the test configurations for ATG BS supporting CA, we think there is no need to introduce new test configurations. In other words the existing test configurations can be reused. Which test configuration is used for testing relay on the feature declared by manufacturer.

Proposal 2: For ATG BS conductive and OTA conformance CA RF requirements test, to adopt the existing test configurations for TN BS conductive and OTA CA RF requirements conformance test, respectively.

# 3 Conclusion

In this contribution, we give some discussions on test tolerance issue for ATG BS conformance test.

Proposal 1: For ATG BS conductive and OTA conformance CA RF requirements test, to adopt the same test tolerance for TN BS conductive and OTA CA RF requirements conformance test, respectively.

Proposal 2: For ATG BS conductive and OTA conformance CA RF requirements test, to adopt the existing test configurations for TN BS conductive and OTA CA RF requirements conformance test, respectively.

# 4	Reference

[1] R4-2502280, draft CR to TS 38.104: the introduction of Rel-19 ATG BS supporting CA, ZTE Corporation, CMCC, CATT, Huawei, Ericsson.

[2] R4-2500997, Draft CR for TS 38.141-1, Introduction of R19 ATG enhancement BS RF requirements, CATT

[3] R4-2500998, Draft CR for TS 38.141-2, Introduction of R19 ATG enhancement BS RF requirements, CATT