# 3GPP TSG-RAN WG4 Meeting # 114bis                                 R4-2503999

# Wuhan, China, 7th – 11th April, 2025

# Title: 	Discussion on UE demodulation performance requirements for 6Rx

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.1.3

# Document for:	Approval

# 1 Introduction

In RAN #103 meeting, the new WID of UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4 was approved in [1], wherein the objectives related to 6Rx for handheld UE and FWA UE are listed as below:

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In the following section, we will provide the detailed discussions for 6Rx UE demodulation requirements.

# 2 Discussion

In the past few meeting, RAN4 had an intense debate on whether handle UE should support 6 MIMO layers functionality. At he last meeting, RAN4 achieved the following agreement[2].

- 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices.

Considering RAN4 has been defined 8Rx demodulation requirements for FWA, we believe that some requirements definition could follow the Rel-18 discussion. In general, we think that PDSCH and CQI reporting requirements shall be defined for 6Rx UE at least. In the following section, we would like to provide more detailed discussion for demodulation requirements.

Since the above agreement is very clearly pointed that 6 layer is only supported for FWA device. We would like to introduce 4layer and 6 layers for FWA device, and 4 layer for handle UE device. Other parameters will be discussed in the following.

PDSCH requirements

Regarding PDSCH requirements, the important parameters, like MCS, Rank, antenna correlation, and propagation conditions shall be considered at first.

Firstly, we would like to consider different modulation orders for different Rank. In feasibility evaluation phase, RAN4 agreed to use MCS 13 and 17 to verified performance gain, which is always used in UE side testing. Therefore, we believe that MCS 13 and 17 could as a staring point to evaluation performance in link level simulations.

Proposal 1. Propose to consider MCS 17 and MCS 13 as starting point for 4 layer and 6layer evaluation.

Secondly, in the past meetings, some companies provided the actual antenna correlation data for 6Rx handle UE. Meanwhile, other companies mentioned that RAN4 shall use ECC values as antenna correlation assumption. We understand that those actual assumption are very sufficient and closely to actual product design. However, from RAN4 requirements definition perspective, we think that legacy correlation is more suitable for testing, since RAN4 defines minimum requirements.

Proposal 2. Propose to consider low correlation for 6layer and medium B correlation for 4layer.

Thirdly, the propagation conditions TDLA30-10 and TDLC300-100 shall be considered for different requirements definition.

Proposal 3. Propose to consider TDLA30-10 and TDLC300-100 for evaluation.

SDR requirements

The purpose of the SDR requirements is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. According to TS38.101-4, the existing MCS index table has already covered the configuration applicable for UEs up to rank 8 for QPSK/16QAM/64QAM/256QAM, while up to rank 2 for 1024QAM.

Proposal 4. Propose to define SDR requirements applicable to FR1 CC, considering to support Maximum number of PDSCH MIMO layers = 6.

# 3 Conclusion

In this contribution, we give some discussions on the MIMO layers related to 6Rx, The conclusions are:

Proposal 1. Propose to consider MCS 17 and MCS 13 as starting point for 4 layer and 6layer evaluation.

Proposal 2. Propose to consider low correlation for 6layer and medium B correlation for 4layer.

Proposal 3. Propose to consider TDLA30-10 and TDLC300-100 for evaluation.

Proposal 4. Propose to define SDR requirements applicable to FR1 CC, considering to support Maximum number of PDSCH MIMO layers = 6.

# 4 References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN4 chair (Huawei).

- R4-2503014, WF on 6Rx requirements, AT&T.