# 3GPP TSG-RAN WG4 Meeting #114bis                                 R4-2504001

# Wuhan, China, 7th – 11th April, 2025

# Title: 	Discussion on NTN_testing_NGSO_channel_model

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.10.4.1

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the revised WID of Enhanced requirements and conductive test methodology for NR NTN and IoT NTN was approved in [1], wherein the objectives related to channel model and demodulation requirements are copied below:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model Less than 5MHz for NTN - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

In this contribution, we provide some of our viewpoint on the NGSO channel model.

- Discussion

- Satellite position/velocity estimation based on equation of motion

- WF - RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay based on the Kepler’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test. Specify both the satellite position/velocity calculation methods in TS 38.101-5/TS 36.102 - Note: the details models for Kepler’s equation and equation of motion can be referred in Appendix - RAN4 will introduce the accuracy requirement applied for two models compared with GMAT - Companies are encouraged to provide the calibration and comparison results with two models before RAN4#114bis meeting, based on calibration and comparison results, further discuss how to define the related accuracy requirement in the next meeting. - FFS how to capture the TE-emulated time varying doppler shift and propagation delay model in the specification - Option 1: capture into the normative part of Appendix, details need to be further discussed

The first issue is how to capture the TE emulated time varying Doppler shift and propagation delay model in the specification. The initial purpose is to inform RAN5 or TE vendor how to calculate the Doppler and delay at the current moment by modeling the satellite’s orbit. And then TE vendor can add the calculated Doppler and delay to the simulated channel. For the DUT vendor, the Doppler and delay can be calculated and compensated based on ephemeris information. In general, to ensure consistency between TE vendor implementations, specific details need to be clearly stated.

About where to capture the details in specification, we don’ have strong view, may be appendix is a good choice.

Proposal 1. Propose to consider capture the approaches into the normative part of Appendix.

Correction of Propagation Delay Calculation

- WF - RAN4 discuss whether the Earth's rotation is considered during signal flight time for more accurate estimations of propagation delay - Considering the earth rotation, the propagation delay, DelayERtR can be derived as DelayERtR=ρtR/c1−1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR> Where tR is the signal’s received time, rtRECEFand vtRECEF are the satellite position and velocity vectors at time tR , ρtRis the difference in satellite and UE position vectors at time tR, K is the unit vector in standard z-axis, (a×b) is the cross-product and <a, b> is the dot product between a and b - The proponent is encouraged to provide the detail information, e.g., reference, if possible, about the propagation delay calculation with considering the signal flight time before RAN4#114bis meeting - Participating companies are encouraged to examine this issue from their perspective.

In the last meeting, some companies mentioned that the error caused by Earth’s rotation should be considered. As shown in the figure below, we suppose that at time t0 , the satellite’s coordinate under ECEF0 is X0S,Y0S,Z0S, and it transmits a signal. The signal passes through layers of atmosphere and finally reaches the earth at time t1, and is received by the receiver. At this time, the receiver’s coordinates under ECEF1 is XR1,YR1,ZR1.All calculations are carried out under ECEF1.

From our understanding, the receiver is fixed to the earth and rotates with the earth (in a static state). For the satellite, the receiver rotates during the signal propagation process. However, from a digital perspective, the coordinates of the receiver do not change (because the coordinate system is fixed to the earth). In other words, the satellite considers the coordinates of the receiver to be the same when the signal is received and when it is transmitted. The distance from the satellite to the receiver calculated by the data is ρ in the figure, but the actual distance from the satellite to the receiver is ρ+Δρ. Therefore, we only need to calculate Δρ.

Figure 1. The Sagnac effect

Firstly, rotate ECEF0 by an angle to get ECEF1. The angle of rotation is α,

α=ω(t1−t0)=ωτ

Where, ω=7292115.1467×10−11rad/s is the angular velocity of the Earth’s rotation. τ is the signal propagation time, which is generally obtained by dividing the distance from the satellite to the receiver by the speed of light, that is τ=ρc . Hence we need to convert the satellite’s coordinates from X0S,Y0S,Z0S to X1S,Y1S,Z1S. Now, α is known.

X1SY1SZ1S=cosαsinα0−sinαcosα0001X0SY0SZ0S

Generally speaking, α is very small, and approximately cosα=1, sinα=α, so

X1SY1SZ1S≈1α0−α10001X0SY0SZ0S

According to the above formula, the change in stellite coordinates caused by the Earth’s rotation is approximately,

ΔXSΔYSΔZS=αY0S−αX0S0=ωτ∙Y0S−ωτ∙X0S0

The change in distance from the satellite to the receiver caused by the rotation of the earth is

∆ρ=∂ρ∂X0S∙ΔXS+∂ρ∂Y0S∙ΔYS+∂ρ∂Z0S∙ΔZS

The distance from the satellite to he receiver is

ρ=X0S−XR02+Y0S−YR02+Z0S−ZR02

According to the linearization of the ball equation and considering ΔZS=0,

∆ρ=X0S−XR0ρ0∙ΔXS+Y0S−YR0ρ0∙ΔYS

Thus

∆ρ=ωcY0SX0S−XR0−X0SY0S−YR0

After the above series of derivations, we can use the above formula to calculate actual position to correct the error caused by the Earth’s rotation.

Observation 1. The propagation delay caused by the Earth’s rotation can be called the Sagnac effect.

Observation 2. In GNSS system, the effect of the Earth’s rotation must be taken into account when calculating the satellite position or the geometric distance form the satellite to the receiver.

Proposal 2. Propose to consider the above approach to calculate the delay error cause by Earth’s rotation.

Interpolation method to obtain the propagation delay and Doppler shift trajectory

- Proposals - Option 1: Cubic interpolation - Option 2: linear interpolation - Option 3: Open to not restrict the implementation on the TE side. However, RAN4 should discuss and determine an acceptable error value introduced by interpolation - Option 4: With the assumption of UE updates the satellite location/velocity at least every 1 second. The TE may update Doppler Shift and Propagation delay every slot linear interpolation or cubic spline interpolation. It is up to TE implementation which interpolation method to be used.

The next issue is about the interpolation method, we did some simulation to evaluate the performance between cubic and linear interpolation. The following figures shown the specific difference for two approaches.

Figure 2. Comparison of 2 interpolation methods and the per 1ms calculate values for Doppler shift

Figure 3. Comparison of 2 interpolation methods and the per 1ms calculate values for Delay

From the above 2 Figures we can find that the Doppler and Delay are very close for linear and spline interpolation.

Furthermore, Figure 4 and Figure 5 compare the differences between different interpolation method and the per TTI calculated values.

Figure 4. Difference of 2 interpolation methods for Doppler shift

Figure 5. Difference of 2 interpolation methods for Delay

From Figure 4 and Figure 5 we can see that spline interpolation method can get smaller errors compared to the linear interpolation method. In general, linear and cubic interpolation have different complexity, the cubic interpolation has high complexity, and the result is more accurate. Therefore, we think that RAN4 should discuss and determine the range of the error at first. If the interpolation error shows a little difference for the final results, we believer that they are negligible. Otherwise, RAN4 needs to consider the different approaches.

However, compared to the above Figures, we think that errors are within the margin. Even if we take those errors into account, we believe that they are negligible.

Observation 3. Spline interpolation method can get smaller errors compared to linear interpolation method.

Proposal 3. RAN4 should discuss and determine an acceptable error value introduced by interpolation.

# 3 Conclusion

In this contribution, we give some discussions on NTN_testing_NGSO_channel_model, The conclusions are:

Observation 1. The propagation delay caused by the Earth’s rotation can be called the Sagnac effect.

Observation 2. In GNSS system, the effect of the Earth’s rotation must be taken into account when calculating the satellite position or the geometric distance form the satellite to the receiver.

Observation 3. Spline interpolation method can get smaller errors compared to linear interpolation method.

Proposal 1. Propose to consider capture the approaches into the normative part of Appendix.

Proposal 2. Propose to consider the above approach to calculate the delay error cause by Earth’s rotation.

Proposal 3. RAN4 should discuss and determine an acceptable error value introduced by interpolation.

# 4	Reference

- RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN.