# 3GPP TSG-RAN WG4 Meeting # 114bis                                 R4-2504002

# Wuhan, China, 7th – 11th April, 2025

# Title: 	Discussion on spatial channel model for demodulation performance requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.2

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, the new WID of study on spatial channel model for demodulation performance was approved in [1], wherein the study item includes the following objectives :

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In this contribution, we give some initial discussion on the practical spatial channel modelling methodology and other analysis based on the last meeting conclusion [2].

# 2 Discussion

TDL and CDL Methodologies

CDL from TR 38.753 will include randomness reduction for the purpose of the SI - CDL (TR 38.753) - Option A1 : with Table 7.2.1-8 in TR 38.827 - Option A2: based on Table 7.7.1-3 in TR 38.901 without truncation - Option A3 : based on Table 7.7.1-3 in TR 38.901 with truncation According to the updated truncation procedure described in Annex B, truncated table to be shared before RAN4#114-bis. - Option B1: TDL - Option B2: TDL

During the last meeting, RAN4 discussed how to construct the SCM channel model and some simulation cases. Regarding the construction of the SCM channel model, there are essentially three approaches, CDL channel model based on the TR 38.827, extending current TDL channel model and TX-RX beam steering approach, and the simulation scenarios predominantly revolve whether MU-MIMO test case needs to be considered and the selection of current TDL channel model.

In the last meeting, RAN4 further discussed the CDL and enhanced TDL modeling approaches, we have three candidate options for CDL channel model and two options for TDL channel model. Compared to legacy DL approach, which captured in TR38.827 and 38.901, some companies still mentioned a truncation method based on TR 38.901. Regarding the truncation approach, the biggest advantage is that it easy to align caused by reducing the number of clusters, which is proposed by other companies.

In general, after so many meeting and discussions, from testing perspective, without no doubt that the CDL channel is hard to align compared with TDL channel model, due to the generation procedure, AOA/AODs, too many random methods, and AAV, etc, which become difficulties that hinder the alignment results. Nevertheless, RAN4 is still working on finding a channel model that can define the requirements. For CDL channel model, RAN4 has discussed this in detail. Next, we would like to tale about the TDL based channel modeling approach. As mentioned in [1][2], two different TDL based approaches was illustrated, one is based on TX-RX beam steering channel with two spatial clusters. Another is that each beam corresponds to some taps and each beam is steering of other beams. The first approach is to extent existing two cluster TDL channel model to multi-cluster channel model, where each cluster corresponds to one TX-RX beam. The second approach implement multi beams in one TDL channel, where each beam corresponds to some taps and each beam is steering of other beams with steering matrices. In general, both methods are based on the beam steering method to generate the angle of beam, so as to achieve the required beam angle in the TDL channel model.

Compared to CDL channel model, the TDL based approach is more easier to implement, and the angle information can be added according to the required angle during testing or simulation. Compared to TDL channel model, only beam information is considered in modeling procedure, which is also followed the same approach of beam steering to generate beams for different clusters or taps. From our understanding, we believe that the above two approaches are easy to converge and will not bring additional burden to the test, even TE vendor can also implement CDL channel model.

Observation 1. From our point of view, we believe that the TDL based approaches are easy to converge, and will not bring additional burden to the test.

Proposal 1. Propose to consider both CDL and TDL based approaches for spatial channel model.

PDSCH PMI Selection

Way Forward: Random PMI already agreed. Regarding potential fixed PMI: - Option 1: Random PMI from sub-set - Option 2: Collectively chosen fixed PMI - Option 3: Independently chosen fixed PMI Note: Companies to propose detailed method description for interested options for RAN4#114-bis.

Regarding PMI selection, random PMI has already agreed as one choice, and some companies wanted to consider fixed PMI, which is based on different implementations for different companies. Its advantage is that can reflect channel changes and ensure that the selected PMI is more in line with the current channel characteristics. From the perspective of existing network, fixed PMI is more suitable. However, from the perspective of testing, fixed PMI can’t guarantee that all companies choose the same PMI. For this purpose, the above options have somewhat relaxed.

For the option1, random PMI from sub-set, the most import thing is how to choose the sub-set, which is not explained in detail in the above options. From our understanding, the subset could from the fixed PMI provided by various companies or select from a codebook. Hence, we think this point needs to be clarified. Regarding option 2, collectively chosen fixed PMI, we understand that fixed PMI is more suitable for scenarios with relatively stable channel conditions. And if all companies could agree this fixed PMI to do simulation and testing, we think this choice of PMI is acceptable. For the last one, independently chosen fixed PMI, we don’t think this is a good approach. As we discussed before, PMI selection is based on different implementations for different companies, which is a challenge for consistency of results.

Observation 2. The sub-set is not explained on detail in the above options, RAN4 needs to further discuss how to chose sub set.

Proposal 2. Propose to consider creating sub set by collecting fixed PMI from interesting companies.

Antenna Array Virtualisation (for CDL options)

Way Forward: Downselection to be based upon (1. Best performance, 2. Ease of alignment, 3. Practicality) Options: - Option 1Y: with Antenna Array Virtualisation (AAV) using subarray configuration - [(M,N,P,Ms,Ns)= (16,8,2,8,1) for SU-MIMO eTypeII, for 32 TX, for comparison only] - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports - Option 3: with AAV with the following configuration: - (M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

Regarding antenna array virtualization, there still have some open issues on the above table. RAN4 may needs to consider downselection based on best performance, ease alignment, and practicality implementation. We evaluated AAV using sub array configuration and without AAV configuration. From our simulation results[5], the performance of AAV with sub array is better than without AAV configuration, since AAV can bring the gain of beamforming. At the same time, from the perspective of BS side implementation, vertical mergers are easier to implement and utilize in practical network. Therefore, from the point of performance and implementation, we would like to consider sub array configuration for 4Tx and 8Tx CSI-RS Ports.

Observation 3. From our simulation results, the performance of AAV with sub array is better than without AAV configuration.

Proposal 3. Propose to only consider AAV with sub array configuration for 4Tx and 8Tx CSI-RS Ports .

# 3 Conclusion

In this contribution, we give some discussions on spatial channel model for demodulation performance requirements, The conclusions are:

Observation 1. From our point of view, we believe that the TDL based approaches are easy to converge, and will not bring additional burden to the test.

Observation 3. From our simulation results, the performance of AAV with sub array is better than without AAV configuration.

Observation 2. The sub-set is not explained on detail in the above options, RAN4 needs to further discuss how to chose sub set.

Proposal 1. Propose to consider both CDL and TDL based approaches for spatial channel model.

Proposal 2. Propose to consider creating sub set by collecting fixed PMI from interesting companies.

Proposal 3. Propose to only consider AAV with sub array configuration for 4Tx and 8Tx CSI-RS Ports.

# 4 References

- RP-241610, New WID: Study on spatial channel model for demodulation performance requirements, Nokia.

- R4-2502378, Way Forward [114][322] NR_SCM. Nokia.

- R4-2500337, TP to TR38.753: TDL approaches and related Annex. MediaTek Inc.

- R4-2501738, Discussions on spatial channel models. Huawei, HiSilicon.

- R4-2504003, Simulation results on spatial channel model for demodulation performance requirements, ZTE Corporation, Sanechips.