# 3GPP TSG-RAN WG4 Meeting # 114bis                               R4-2504004

# Wuhan, China, 7th – 11th April, 2025

# Title: 	Draft TP introduction of comparison of spatial channel models for SU-MIMO PMI cases

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.2

# Document for:	Approval

# 1 Background

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.

# 2 Draft TP

# 6  Comparison of Spatial Channel Models

6.1  PDSCH performance under SU-MIMO scenario(PMI Aspects)

This section provides comparison for different spatial channel modes with simulation assumptions captured in Table 6.1-2.

Table 6.1-2: Simulation assumptions for PMI test cases

| Parameter |  |  | Unit | 8TX | 16TX (for information) | 32TX (for information) |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |  |
| Subcarrier spacing |  |  | kHz | 30 |  |  |
| Duplex Mode |  |  |  | TDD |  |  |
| TDD DL-UL configurations |  |  |  | FR1.30-1 as specified in Annex A |  |  |
| Antenna configuration |  |  |  | 8 x 4 (N1,N2) = (4,1) | 16 x 4 (N1,N2) = (4,2) | 32 x 4 (N1,N2) = (8,2) |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |
|  | Number of CSI-RS ports (X) |  |  | 8 | 16 | 32 |
|  | CDM Type |  |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  |  | 1 | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  |  | Row 8,(4,6) | Row 12,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | Row 8,(5) | Row 12,(5) | Row 17,(5, 12) |
|  | CSI-RS interval and offset |  | slot | 10, 0 |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |
| pmi-FormatIndicator |  |  |  | Not configured for eType II FFS Subband or Wideband for Type I |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |
| csi-ReportingBand |  |  |  | 11111111111111 |  |  |
| Codebook configuration | Codebook Type |  |  | - typeII-r16 - typeI-SP |  |  |
|  | eType II CB config | paramCombination-r16 |  | 6 (L =4, pν =1/2, β=1/2 ) |  |  |
|  |  | R(numberOfPMISubbandsPerCQISubband-r16) |  | 1 |  |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  |  | (4,1) | (4,2) | (8,2) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  |  | (4,1) | (4,4) | (4,4) |
|  | CodebookSubsetRestriction |  |  | 0x FFFF | 0x 7FF FFFF FFFF FFFF FFFF | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |
|  | RI Restriction (typeII-RI-Restriction-r16) |  |  | Rank 2: 0010 Rank 4: 1000 |  |  |
| Physical channel for CSI report |  |  |  | PUSCH |  |  |
| CQI/RI/PMI delay |  |  | ms | 7 |  |  |
| Maximum number of HARQ transmission |  |  |  | 4 |  |  |
| Measurement channel |  |  |  | R.PDSCH.2-8.3 TDD |  |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with i1 wideband granularity and i2 subband granularity |  |  |
| Note: Use DM-RS based FOE and compensation. |  |  |  |  |  |  |

The summary of simulation results are captured in the following.

TBD

Observation: TBD

# 3 Conclusion

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.