# 3GPP TSG-RAN WG4 Meeting #114bis                                R4-2504005

# Wuhan, China, 7th – 11th April, 2025

# Title: 	Discussion on CSI reporting requirement and testing framework for CSI prediction

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.19.2

# Document for:	Approval

# 1 Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

During the last meeting, some conclusions has been reached for CSI compression and CSI prediction[2]. In the following section, we will provide the detailed discussions for CSI prediction.

# 2 Discussion

Reference model for CSI prediction

Agreement: - Assume no reference AI model for CSI prediction as baseline - If it is difficult to align the results corresponding to test metric, RAN4 will discuss the reference model.

In the last meeting, RAN4 discussed whether it needs to assume a reference model for CSI prediction. In general, since CSI prediction is one side model, RAN4 can assume that the AI based CSI prediction as a black box, compared to legacy prediction algorithm, such as Wiener filter, Kalman filter, which is based on the UE vendor implementation. From baseband testing perspective, TE vendor only needs to count the throughput meets the requirements. Hence, we don’t think that RAN4 needs to spend more time on this.

On the other side, some companies mentioned that it is hard to align the results under AI based CSI prediction. Hence RAN4 needs to consider the reference model. From perspective of AI/ML, it is hard to say that the results is mainly affected by the model, but we cannot deny the impact of model selection on the results. There is no doubt that the result of simple model prediction is poor. Even if the complexity of the model is the same, it is difficult to judge whether the predicted results will be the same. Since different types of AI models excel at handling different types of features. In general, channel characteristics, model capabilities, data quality, and training methods all affect the convergence of results. As we discussed in CSI compression, RAN4 not only aligned the reference model but also the dataset. From our understanding, Whether it is necessary to consider the reference model, it is better to be determined based on the results.

Observation 1. CSI prediction is one side model, RAN4 can assume that the AI based CSI prediction as a black box, compared to legacy prediction algorithm, like Wiener filter, Kalman filter, which is based on the UE vendor implementation.

Observation 2. Channel characteristics, model capabilities, data quality, and training methods all affect the convergence of results.

Same as traditional method, RAN4 aligned the simulation assumptions for CSI prediction[3], focusing mainly on generating the training dataset, and did not restrict model assumptions. We followed the assumptions, and our results of SGCS can be found in the following Table 1. Compared to the legacy approach, RAN4 needs to discuss how to infer convergence based on the results, whether to follow the legacy method, if most companies results are within a certain range. For legacy approach, without no doubt, that the results could be aligned, which only needs to remove the results of the span. Therefore, if RAN4 wants to align the results, a potential criteria may be needed.

Table 1. Simulation results of SGCS for CSI prediction

| Duplex Mode and SCS | CHBW | MIMO | Channel Model | Antenna correlation | SGCS | Nearest historical CSI |
| --- | --- | --- | --- | --- | --- | --- |
| FDD 15KHz SCS | 10 MHz | 16T4R | TDLA30-20 | XP medium | 0.96 | 0.81 |
|  |  |  | TDLA30-100 |  | 0.94 | 0.81 |

Observation 3. From our simulation results, we can observe that the SGCS predicted by AI is 0.96, and non predicted is 0.82 under TDLA30-20 XP medium condition. At the same time, under the TDLA30-100 XP medium condition, the SGCS predicted by AI is 0.94 , and non predicted is 0.81.

Proposal 1. Propose to discuss the criteria for determining whether the results have reached alignment.

Training and test data consistency

Agreement: - further discuss the following options - Option 1: Consistency between training data and test data should be ensured - RAN4 to study how to ensure consistency between training data(simulation) and test data (conformance test) and whether anything special should be done during the test - Option 2: Consistency will be implicit as test emulated channel will be very similar to the one used in RAN4 simulations

Regarding the consistency of training data and test data, some companies believe that there is a difference between simulation data and conformance data. However, from our understanding, when UE performs conformance testing, TE vendor generates test data based on the scenarios defined by RAN4, including frequency, power, modulation order, channel conditions, physical waveform, signaling process,etc. In addition, training data is also generated based on the above parameters. In general, during the test, TE loaded parameters completely based on RAN4’s simulation assumptions. So we believe that the generated data by simulation could be similar in form and statistical characteristics to the data generated by the consistency test equipment.

Observation 4. During the conformance testing, TE vendor generates test data based on the scenarios defined by RAN4, including frequency, power, modulation order, channel conditions, physical waveform, signaling process, etc.

Proposal 2. Proposed to consider that consistency will be implicit, as the emulated channel used for testing will be highly similar to the one employed in RAN4 simulations.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for CSI prediction , The conclusions are:

Observation 1. CSI prediction is one side model, RAN4 can assume that the AI based CSI prediction as a black box, compared to legacy prediction algorithm, like Wiener filter, Kalman filter, which is based on the UE vendor implementation.

Observation 2. Channel characteristics, model capabilities, data quality, and training methods all affect the convergence of results.

Observation 3. From our simulation results, we can observe that the SGCS predicted by AI is 0.96, and non predicted is 0.82 under TDLA30-20 XP medium condition. At the same time, under the TDLA30-100 XP medium condition, the SGCS predicted by AI is 0.94 , and non predicted is 0.81.

Observation 4. During the conformance testing, TE vendor generates test data based on the scenarios defined by RAN4, including frequency, power, modulation order, channel conditions, physical waveform, signaling process, etc.

Proposal 1. Propose to discuss the criteria for determining whether the results have reached alignment.

Proposal 2. Proposed to consider that consistency will be implicit, as the emulated channel used for testing will be highly similar to the one employed in RAN4 simulations.

# 4 References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- R4-2502856, WF on the requirements for AI/ML air interfance. Qualcomm.

- R4-2502970, Way forward on simulation assumptions for CSI prediction. OPPO, CAICT, vivo, Nokia, Ericsson, Qualcomm, Huawei, Hisilicon, Mediatek, CATT, Samsung, ZTE Corporation, Apple.

# 5 Appendix

Table 1: Baseline Simulation assumptions for AI/ML based CSI prediction

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| Subcarrier spacing | 15kHz for 2GHz, 30kHz for 4GHz |
| gNB TX antennas | Option1: 32 (N1,N2)=(4,4) as defined in Rel-15 type1 Option2: 16 (N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 as baseline with XP medium correlation |
| Doppler spread | 20Hz, 50Hz and 100Hz |
| Delay spread | 30ns |
| Channel estimation | Start with Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Further consider practical channel estimation in the next step - Use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| Rank per UE | Rank 1 and 2 |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |
| CSI feedback assumption | Step1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account Step2: take CSI feedback into account (wait more related agreements in RAN4) - Option 1: Use Rel-18 eTypeII-Doppler - Companies are encouraged to check RAN1 agreements |
| CSI feedback overhead | Option1: Parameter combination = 2 (PC2) Option2: PC4 Option3: PC6 Note: this part relates to step2, may be down selected later |
| Baseline | Random PMI with Rel-15 Type I single panel codebook (for Step 2 or performance requirements) |
| KPI | Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), - comparing the SVD of model output with SVD of the ground truth Raw channel Note: - Step1: check SGCS before CSI feedback (between the raw channel matrix and ground-truth at UE) - FFS Step2: check SGCS (or other metrics, e.g. TPs) after CSI feedback |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |