TSG-RAN Working Group 4 (Radio) meeting #114-bis	R4-2504013

Wuhan, China, 7th – 11th April 2025

Source:	Ericsson

Title:	Simulation of BS-to-BS out-of-band isolation for edge-to-edge co-location isolation between 3.5 and 7 GHz

Agenda item:	7.13.3

Document for:	Approval

# 1	Introduction

At the last RAN4 meeting (RAN4#114 in Athens) simulation results for the coupling loss between two sideways mounted Urban Macro AAS BSs at 7.0 GHz were reported [2].

The BS-to-BS isolation is used as a background for BS RF requirements defined to guarantee co-location of two base station operating in different bands at the same site. In the current specification, two co-location requirements are defined: transmitter spurious emission requirement and receiver out-of-band blocking. In addition, TDD OFF power and Transmitter intermodulation is also relying on BS-to-BS isolation and corresponding test method used for co-location requirements. It is therefore essential, to evaluate relevance of the traditional assumptions for co-location requirements.

With the correct assumption on BS-to-BS coupling, the test concept can be evolved such that the need for a specific co-location reference antenna can be avoided as indicated in [2]. Considering the work to be started for transmitter spurious emissions and BS RF specification improvements [3] it would be reasonable to also investigate the details regarding the technical background for co-location requirements. In this contribution we present results relevant for colocation as three distinct topics:

- co-location in the case of well separated bands, illustrated with a case of one AAS system at 3.5 GHz and another at 7 GHz.

- Comparing coupling loss statistics for different beam sets.

- Effect of probe width when measuring near field pointwise or as average.

# 2	Discussion

For higher bands such as the band n104, a working assumption is that the physical antenna area shall be the same as in the 3.5 GHz case to maintain a decent link budget and coverage. Moreover, to maintain the spatial performance of the beams, the antenna element separations must also be kept the same in relation to the frequency (wavelength) used. This leads to much larger array sizes.

Some preliminary observations, of colocation coupling loss in the case of well separated frequency bands are presented. To model the out-of-band behavior of an AAS different levels of antenna weight correlation is used. The effect on the co-location coupling loss is presented and discussed.

To evolve the co-location testing method it has been observed that the beam directions will have a large impact on the coupling loss. In a test setup, a reasonable way of handling this is to prescribe which beams to test. To verify that this a reasonable way forward we present coupling loss statistics for different beam sets.

The co-location side-by-side scenario involves antenna to antenna coupling in the nearfield. In order to allow for efficient testing using generic test antennas such as horns, open-ended wave guides the co-location requirements need to reformulated using nearfield field quantities, e.g. electric field strength. In order to measure electric field strength correctly an electrically small antenna should be used. However, a more wideband horn antenna provides more time-efficient testing. The effect of probe size is illustrated with a simple example as well as a discussion on how to capture the average field strength, which seems to be a more relevant metric for co-location coupling loss.

## 2.1	Simulation methodology

In the simulations, the antenna element scattering matrix is calculated using PLM CST full-wave simulation. This data is then used in MathWorks MATLAB and the elements are coupled together using S-matrix algebra.

First, the subarray ports are created using a distribution network, and the coupling matrix between all subarray ports is calculated. Next, array beamforming weights are calculated. This is done either through a pre-defined Grid of Beams (GoB) or taking into account the deployment parameters, i.e., antenna height, mechanical tilt, pointing direction, etc. and a distribution of UEs. The beamforming weights are then applied to the subarray ports, in order to obtain the coupling loss in either an array-subarray or an array-array scenario. The resulting CDF is an aggregate of all beam directions and subarray ports.

The simulation methodology is visualized in Figure 2.1-1.

Figure 2.1-1: Overview of the simulation methodology

## 2.2	Coupling loss between different bands

### 2.2.1	Assumptions

The assumption to have a fixed physical size compared to the 3.5 GHz case results in a large numerical problem. In order to have a reasonable total simulation time and effort, the element model is simplified. Here, a pair of crossed dipoles over, but not connected to, a ground plane is used.

The simulation assumptions are provided in Table 2.2.1-1 and Table 2.2.1-2.

Table 2.2.1-1: Network assumptions

| Carrier Frequencies | 3500 and 7000 MHz |
| --- | --- |
| BS height | 25 m |
| UE height | 1.5 m |
| Minimum 2D distance BS to UE | 35 m |
| Number of UEs per sector | 1500 |
| ISD | 0.5 km |
| Network layout | Hexagon |

Table 2.2.1-2: BS antenna assumptions

| Parameter | 3500MHz | 7000MHz |
| --- | --- | --- |
| Sub-array configuration | 3x1 | 3x1 |
| Array configuration | 4x8 | 8x16 |
| Pre-set sub-array down-tilt | 3 degrees | 3 degrees |
| Mechanical down-tilt | 6 degrees | 6 degrees |
| Vertical element separation | 0.7 λ | 0.7 λ |
| Horizontal element separation | 0.5 λ | 0.5 λ |
| Design frequency | 3.5 GHz ( λ = 85.6550 mm) | 7 GHz (λ=42.8275 mm) |
| Evaluation frequencies | 3500 MHz | 6425, 6775 and 7125 MHz |
| Relative bandwidth | Not applicable | 10% |

The placement of the victim and aggressor base stations and the individual base station array antenna geometry is visualized in Figure 2.2.1-1.

The actual beam weights for out-of-band emissions are modelled as partly uncorrelated. The actual value of the correlation is unknown and is dependent on the specific implementation. Specifically, the digital predistortion will influence the out-of-band beam weights significantly.

Assuming a polynomial model, the beam weights for even harmonic frequencies equals the square of the beam weights of the carrier frequency, wc, while for odd harmonics the weights are equal to the beam weights of the carrier.

wnom=wcfor odd harmonicswc2wcfor even harmonics		Equation 1

where wnom, are the nominal beam weights.

All out-of-band beam weights, including the harmonics, are modelled as partly correlated with the correlation as a parameter. Partly correlated beam weights are modelled as:

w=r wnom+1-r wuncorr 		Equation 2

where wuncorr are random weights:

wuncorr=x+jy2N 		Equation 3

Here, x and y are normally distribute random variables with expectation value 0, and N is the number of subarray ports.

Figure 2.2.1-1: Deployment of the two AASs, one operating at 3.5 GHz and the other at band n104, separated 16 cm edge to edge, see Table 2.2.1-2.

### 2.2.2	Results

Two systems serving the same cell are considered and the AASs are mounted side by side in the same site, see Tables 2.2.1-1 and 2.2.1-2. The AAS array geometry and relative placement are shown in Fig. 2.2.1-1. The separation between the arrays is 16 cm, which assumes 3 cm distance between the chassis and the arrays of the AAS.

#### 2.2.2.a	Array to array coupling

The scenario of the array to array (A-to-A) coupling is depicted in Fig. 2.2.2.a-1. 1500 random users per system are used, and at each time instance a pair of randomly selected users are considered for the calculation of A-to-A coupling loss.

Figure 2.2.2.a-1: User positions for the A-to-A coupling loss case

The empirical CDFs of the A-A coupling results are shown in Fig. 2.2.2.a-2 for different levels of antenna correlation as explained in Section 2.2.1. Note that only the isolation between the beam (= array) ports are shown. Filters are not included in the analysis.

| - 6425 MHz | - 6775 MHz | - 7125 MHz |
| --- | --- | --- |

Figure 2.2.2.a-2: A-to-A coupling loss, aggressor at 3.5 GHz at different correlation levels and victim at three frequencies in band n104.

Observation 1: The A-to-A coupling loss is lower for higher correlation.

Observation 2: Considering AAS base station characteristics, choosing the minimum coupling loss represents an outlier case and is not fair. Maybe the 90, 95, or 99 percentile is more relevant.

Proposal 1: Co-location of two systems at different bands should be further studied.

Proposal 2: When deriving co-location requirements, we need to agree on what percentile to use in the coupling loss statistics.

Proposal 3: Zero correlation of antenna weights can be used to derive worst case requirements for co-location between different bands well separated in frequency.

For each beam direction of the left (or right) array, the coupling loss depends on frequency. In Fig. 2.2.2.a-3 the minimum coupling loss over frequency is depicted versus beam pointing direction for the left array (#1) and for 0 and 100% correlation levels (r = 0 or 1).

Figure 2.2.2.a-3: Minimum A-to-A coupling loss versus beam direction for 0% and 100% correlation.

#### 2.2.2.b	Array to subarray coupling

In the Array to subarray (A-to-SA) coupling case, beams are directed to the users of the left system as depicted below. The coupling loss is then calculated from the corresponding beam port (including beamforming weights applied to all sub-arrays of the left array which have a fixed subarray down tilt) to all the subarray ports of the right system. Note that only the isolation between the beam (= array) and subarray ports are shown. Filters are not included in the analysis.

| - 6425 MHz | - 6775 MHz | - 7125 MHz |
| --- | --- | --- |

Figure 2.2.2.b-1: A-to-SA coupling loss

Compared to the A-A case the spread is significantly lower, but again the trend that the lower correlation gives the highest coupling is still clear.

Observation 3: The spread in the coupling is lower in the A-SA compared to the A-A case.

Minimum coupling loss for A-to-SA case is depicted in Fig. 2.2.2.b-2, for the beams pointing to 1500 random users in the Urban Macro scenario, and for the subarrays of the right array. For the beam directions of the left array, the minimum coupling loss over frequency and subarrays of the right array, is used. For the subarrays of the right array, the minimum coupling loss over frequency and beam directions of the left array, are presented.

# Observation 4: The non-uniform coupling loss with respect to subarray position in the Array-to-subarray will impact requirements.

Figure 2.2.2.b-2: Minimum A-to-SA coupling loss

## 2.3	Beam set effects

The purpose of the investigation of this section is to explore how the choice of beam directions can influence the isolation. In a test setup some beam set should be used, since the coupling depends on the beam direction. Here, four cases are studied:

- The Urban macro case described in Table 2.2.1-1.

- A 2D Grid of Beam (2DGoB) using downtilt angles 0,1,…,12 degs and azimuth angles -60,-56,…,60 degs.

- A 1D Grid of Beam (1DGoB) using a fixed downtilt of 6 degs and azimuth angles -60,-56,…,60 degs.

- Conformance testing directions (ConfBeams), see Fig. 2.3-1.

Figure 2.3-1 Conformance testing beam directions. Transmit (left) and receive case (right).

Figure 2.3-2 A-A Coupling loss comparison between different beam sets.

Figure 2.3-3 A-SA coupling loss comparison between different beam sets

The difference between using different beam sets is large for the A-A coupling case as compared to the A-AS case. This trend has been observed also earlier for different comparisons between A-A and A-SA coupling. For the A-SA case the choice of beam set and reduction of beam set could be utilized in a test method where the difference related to beam direction is captured.

Observation 5: For the array to subarray coupling the difference between Urban Macro user-directed beams and 2D and 1D grids of beams and the 5 conformance Rx and Tx beams sets is small.

Proposal 4: For conformance testing a reduced set of beams can be used in co-location testing, especially for the antenna-subarray coupling.

## 2.4	Probe size effects

In order to allow for generic test antennas for co-location effects, the choice of probe is important. To correctly measure field strength in the near-field it is well known that a small antenna (aperture dimension less than half a wavelength) with small scattering should be used. Important effects that could be studied are impact of probe size, standing waves between Equipment under test (EUT) and test antenna. In this clause the effect of the size of the probe is illustrated with a simple example.

A hypothetical E-field (See Fig. 2.4-2, blue curve, E(x)) is convolved with a probe w(x) of uniform or cosine shape (Fig. 2.4-1), i.e., at each measurement point an average of nearby E-values is calculated (Eq. 4), the wider the probe, the larger interval is used in the average value. The uniform and cosine probe are normalized to unit area to make sure energy is conserved. The green horizontal line is the average E-field over the entire interval -0.25m to 0.25m. The highest frequency component in the field is sin⁡(2πxλ) corresponding 3.7GHz freqeuncy, implying that a probe of width λ2 should sample the field correctly. A calibration is also applied to set the peak of E(x) and the probed field equal. This could be thought of as a substitution method calibration.

Ex,w,B=-B/2+B/2wx'E(x+x')dx'	Equation 4

Figure 2.4-1 Probe shapes “uniform” and “cosine” .

Figure 2.4-2 Effects of probe shape and width. Dashed curves depict continuous sliding averages, circles depict the probed E-field using steps B, i.e., the distance between two circles is the probe width. Finally the average field strength, calculated using values at the circles, are depicted as crosses.

The resulting E-field flattens out when a wider probe is used, as is expected when measuring in the near-field with a large antenna such as a horn antenna or a CLTA. However, in a co-location context it is not directly the E-field strength that is important. It is rather the power transfer from the aggressor to the victim system. In such a case it is the average field strength over some “reaction surface” that is relevant. In the above Fig. 2.4-2 it is seen that if the field is measured with a uniform probe (left picture) of width B, stepped with increment B (circles), and then the average of these measurements is taken (indicated by crosses), then the average value is independent of probe width. If the probe field w(x) is cosine shaped this is not perfectly true, but the error is small in the example shown (right figure).

Note that, in the far-field, choosing a probe with too narrow beam leads to partial illumination of the EUT [3]. In the near-field, a large probe will also have this effect, which implies incorrect measurement of the E-field. However, taking the correct average will capture the average E-field which can be related to the transferred power.

Observation 6: The probe width has an impact on the measurement result in the near-field.

Observation 7: Correct averaging can provide a method that gives the correct average E-field.

Further work is needed to understand how to define a field strength metric. The above results show that it is the average field strength that is the relevant metric.

# Proposal 5: The capability of larger probes (wide band horns) to correctly capture an average field over a surface should be further studied.

Proposal 6: The choice of test area needed to sample the average E-field should be further studied. The needed sampling grid also needs to be described. The dependent parameters could be EUT height and depth, test antenna aperture dimensions and/or beamwidths, distance between test antenna and EUT.

# Conclusion

The information related to the coupling loss characteristics is needed when requirement levels for transmitter co-location spurious emission requirement levels and receiver co-location out-of-band blocking requirement levels are derived. The information presented in this contribution can be used as input for later re-evaluation of co-location requirement levels for bands in the upper region of FR1.

Co-location coupling loss calculations for a 10 cm side-by-side Urban macro scenario and between bands have been presented as well as the impact of choice of beam set and probe width. The following observations have been made.

Observation 1: The A-to-A coupling loss is lower for higher correlation.

Observation 2: Considering AAS base station characteristics, choosing the minimum coupling loss represents an outlier case and is not fair. Maybe the 90, 95, or 99 percentile is more relevant.

Observation 3: The spread in the coupling is lower in the A-SA compared to the A-A case.

# Observation 4: The non-uniform coupling loss with respect to subarray position in the Array-to-subarray will impact requirements.

Observation 5: For the array to subarray coupling the difference between Urban Macro user-directed beams and 2D and 1D grids of beams and the 5 conformance Rx and Tx beams sets is small.

Observation 6: The probe width has an impact on the measurement result in the near-field

Observation 7: Correct averaging can provide a method that gives the correct average E-field.

Based on these observations the following proposals are made:

Proposal 1: Co-location of two systems at different bands should be further studied.

Proposal 2: When deriving co-location requirements, we need to agree on what percentile to use in the coupling loss statistics.

Proposal 3: Zero correlation of antenna weights can be used to derive worst case requirements for co-location between different bands well separated in frequency.

Proposal 4: For conformance testing a reduced set of beams can be used in co-location testing, especially for the antenna-subarray coupling.

# Proposal 5: The capability of larger probes (wide band horns) to correctly capture an average field over a surface should be further studied.

Proposal 6: The choice of test area needed to sample the average E-field should be further studied. The needed sampling grid also needs to be described. The dependent parameters could be EUT height and depth, test antenna aperture dimensions and/or beamwidths, distance between test antenna and EUT.

# References

[1]	R4-2418398, “Simulation of BS-to-BS isolation for edge-to-edge co-location scenario”, Ericsson

[2]	R4-2502160, “Simulation of BS-to-BS isolation for edge-to-edge co-location isolation at 7 GHz”, Ericsson

[3]	 J. Fridén, A. Razavi, and A. Stjernman, “Angular sampling, Test Signal, and Near-Field Aspects for Over-the-Air Total Radiated Power Assessment in Anechoic Chambers,” IEEE Access, p. 1, 2018, doi: 10.1109/ACCESS.2018.2871708.