3GPP TSG-RAN WG4 Meeting # 114bis	R4-2504017

# Wuhan, China, 7th – 11th April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | <CR#> | rev | <Rev#> | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core maintenance draftCR on less than 5MHz RRM requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW_Ph2-Core |  |  |  |  |  | Date: |  |  | 2025-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Handover with PSCell - PSCell change delay requirements – same notation of T∆ is used in the same clause to refer to two different parameters which are for PCell and for PSCell respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Differentiate the PSCell T∆ by changing T∆ to T∆_PSCell in this clause 6.1.5.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Check further in the WG that whether this change should be also implemented in the previous release of specification. This draftCR can be technically endorsed in this meeting and formal CR should be agreed from an earlier release where the requirement is introduced, if the WG agrees so. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <<Start of Change1>>

#### 6.1.5.4	HO with PSCell from NR-DC to NR-DC

The requirements in this clause are applicable to handover with PSCell from NR-DC to NR-DC. The requirements in this clause are applicable to

-	FR1-FR1 NR-DC to FR1-FR1 NR-DC,

-	FR1-FR1 NR-DC to FR1-FR2 NR-DC,

-	FR1-FR2 NR-DC to FR1-FR1 NR-DC,

-	FR1-FR2 NR-DC to FR1-FR2 NR-DC.

This clause defines requirements for the delay within which the UE shall be able to handover from NR cell to NR cell and change NR PSCell in the meantime.

When the UE receives a RRC message implying handover with PSCell,

-	The UE shall be ready to start the transmission of the new uplink PRACH channel of the target PCell within DHOwithPSCell_PCell ms from the end of the last TTI containing the RRC command, and

-	The UE shall be capable of transmitting PRACH preamble towards the target PSCell no later than DHOwithPSCell_PSCell ms from the end of the last TTI containing the RRC command.

Where:

-	DHOwithPSCell_PCell equals the applicable RRC procedure delay defined in clause 12 in TS 38.331 [2] plus the interruption time stated in clause 6.1.5.4.1.

-	DHOwithPSCell_PSCell is the PSCell change delay stated in clause 6.1.5.4.2.

##### 6.1.5.4.1	HO with PSCell – PCell Interruption time

The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay.

When handover with PSCell from NR-DC to NR-DC is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

Where:

-	Tsearch, TIU, T∆ and Tmargin are the same as defined in clause 6.1.1.2.2.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period.

For FR1-FR1 NR-DC to FR1-FR1 NR-DC or FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

For FR1-FR1 NR-DC to FR1-FR2 NR-DC or FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 45 ms.

In the interruption requirement a cell is known if it has been meeting the relevant cell identification requirement during the last 5 seconds otherwise it is unknown. Relevant cell identification requirements are described in clause 9.2.5 for intra-frequency handover and clause 9.3.4 for inter-frequency handover.

##### 6.1.5.4.2	HO with PSCell – PSCell change delay

The requirements in this section shall apply for PSCell change during handover with PSCell from NR-DC to NR-DC.

When handover with PSCell from NR-DC to NR-DC is commanded, the PSCell change time shall be less than DHOwithPSCell_PSCell:

-	DHOwithPSCell_PSCell = TRRC_delay + Tprocessing + Tsearch_PCell + Tsearch_PSCell + T∆_PSCell + TPSCell_ DU + 2 ms

Where:

-	TRRC_delay, Tsearch_PSCell, T∆_PSCell and TPSCell_ DU are the same as defined in clause 8.9.2.

-	Tprocessing is the SW processing time needed by UE, including RF warm up period.

For FR1-FR1 NR-DC to FR1-FR1 NR-DC or FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms.

For FR1-FR1 NR-DC to FR1-FR2 NR-DC or FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 45 ms.

-	Tsearch_PCell is the time for obtaining the timing reference of target PCell. If SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync, Tsearch_PCell = Tsearch + TΔ + Tmargin, where Tsearch, TΔ and Tmargin are specified in clause 6.1.5.4.1. Otherwise, Tsearch_PCell = 0 ms.

The Trs definition from clause 8.9.2 is modified as following for requirements in this section:

-	Trs is the SMTC periodicity of the target NR cell if target PSCell is unknown and SMTC configuration of target unknown PSCell is present in either targetcellSMTC-SCG-r16 or reconfigurationWithSync, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the measObjectNRs having the same SSB frequency and subcarrier spacing configured by MN and SN have different SMTC, Trs is the periodicity of one of the SMTC which is up to UE implementation. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this section is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

PSCell known and unknown condition is as defined in clause 8.9.2.

<<End of Change1>>