3GPP TSG-RAN WG4 Meeting #114bis									                     R4- 2504037

Wuhan, China, Apr. 07 – 11, 2025

Agenda item:			7.5.3

Source:	Ericsson

Title:	Discussion on the DL PCell REFSENS degradation of fragmented carriers

Document for:	Approval

# Introduction

Fragmented carriers in the same band requiring separate Rx chains per CC may limit the UE CA capabilities if part of complex band combinations with many DL CCs combined with DL MIMO in fragmented operator spectrum holdings. A new RAN4 study item was approved in the RAN #104 meeting to investigate solutions to address this.

The objectives of the SI are as follows [1]:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

NOTE 1:	No RAN1 impact is foreseen

NOTE 2:	RAN2 work, if necessary, will be triggered by RAN4 LS

NOTE 3:	This study starts from single DL band. Sharing RF chain is not considered among inter-band DL carriers.

- Whether and how to apply the conclusions to intra-band component of inter-band CA will be decided after the study on 2CC in a single DL band is completed.

The following sections will discuss the impact of relaxed REFSENS requirements for intra-band non-contiguous CCs supported by a shared Rx chain.

# Discussions

## REFSENS degradation in PCell DL

The contributions [2] and [3] both contain a comprehensive analysis of the UE REFSENS performance for intra-band non-contiguous CA configuration supported by a shared Rx chain for the case in which one of the CCs is the PCell. It is shown that the degradation of the REFSENS for the PCell could exceed 10 dB for some carrier separations due to insufficient rejection of the UE Tx blocker by the common Rx chain. This could in turn result in radio link failure (RLF) when an SCell is added for this CA configuration.

REFSENS is a conducted noise factor test carried out with up to full allocation in the UL and always full allocation in the DL. Nevertheless, REFSENS performance affects the DL link budget and thus DL coverage.  For REFSENS, the key difference between the single Rx chain and the separate Rx chain lies in their filter capabilities to reject the Tx blocker as detailed in [2] and [3]. Insufficient rejection of the Tx blocker would require a lower Rx gain regardless of the UL allocation (by adjusting the AGC setting), thereby increasing the overall Rx noise figure, at least a 10 dB Rx noise increase due to the Tx blocker according to [2].

First, we discuss the difference between a REFSENS degradation by blocking of the own Tx for any channel bandwidth and by Tx noise for wider bandwidth to explain why REFSENS degradation in PCell DL is not feasible. To this end, we compare a band n26 case with a single CC of the same channel bandwidth as the frequency separation of two fragmented intra-band carriers of smaller channel bandwidth.

### Single carrier with 20/30 MHz UL/DL allocation

Consider a scenario with a max 20/30 MHz bandwidth allocation in the band n26, within a 35 MHz UL/DL passband, as shown in Figure 1. The REFSENS test is carried out with 25 PRB at the upper edge of the 20 MHz UL BW, minimizing the Tx-Rx spacing. The DL is fully allocated. The requirements are based on a specific attenuation of the Tx noise and Rx rejection of the Tx blocker, the figure below only shows the analog Rx filter, excluding the duplexer.

Figure 1. REFSENS test illustration with a single carrier and full allocation in the DL.

Assume next that the cell grid of a 20/30 MHz network is dimensioned to support these bandwidths. It is worth noting that the REFSENS test case does not necessarily limit the cell size. Instead, smaller UL transmission bandwidths at larger Tx-Rx separation (providing better isolation between the Tx and Rx branches and thus reduced Tx noise) can be scheduled based on UE location in the cell as part of link adaptation as shown in Figure 2. The rejection of the Tx blocker may also be improved. At the cell edge, smaller UL allocations are more common.

Figure 2. An example of an allocation with larger Tx-Rx spacing for the 20/30 MHz single carrier.

### Fragmented spectrum with SCell added

Next, assuming a 10 MHz UL/DL scenario shown in Figure 3, the REFSENS is improved by about 13 dB as compared to the 20/30 MHz case (with the same UL allocation) [3]. The increased Tx-Rx separation reduces the Tx noise impact, while the Rx filter provides better rejection of the Tx blocker.

Figure 3. An example of a 10 MHz UL/DL scenario.

Consider now a cell grid of an NW dimensioned for the 10 MHz carrier, where an operator has a fragmented spectrum allocation and can add a second collocated 10 MHz cell with a 10 MHz gap, the total frequency separation is 30 MHz between the lowest and highest carrier edges. Suppose that the UE is implemented with a single Rx chain and the NW configures the SCell next to the duplex gap. Consequently, the UE’s analog filter covers both the PCell and SCell (same total DL BW as in the 20/30 MHz case for comparison) but provides considerably less rejection of the Tx blocker. The example is shown in Figure 4.

Figure 4. An example of a fragmented spectrum dimensioning with SCell and PCell.

If adding the SCell in this fragmented spectrum causes a 13 dB REFSENS degradation in PCell due to the Tx blocking as discussed in [2] and [3], then the PCell coverage may be lost when this band combination is configured, at least when the Tx is active. The impact occurs regardless of the UL allocation and could lead to RFL since the blocker power is what matters (blocker power depends on the Tx signal strength). The AGC gain has to be reduced which increases the noise floor for all DL allocations. However, the AGC behavior and gain tables are completely up to UE implementation as noted in [2].

A desense of 13 dB due to blocking by the own Tx when the SCell is added (at CA configuration) is therefore not the same as the REFSENS difference of 13 dB between the 10 MHz BW and 20/30 MHz cases.

The comparison assumes that the degradation of the REFSENS for 20/30 MHz case as compared to a single 10 MHz carrier is predominately due to Tx noise aside from the bandwidth scaling. Limited rejection of the Tx blocker may also be a factor for the UL allocation used for the REFSENS test of the wider 30 MHz carrier, the analogue filter is the same as that in the case with the two 10 MHz carriers.

In practice, the blocker rejection is larger, the 13 dB discussed is an RF minimum requirement based on the in-band blocking minimum performance, the actual performance may be anything from 0 to 13 dB. Hence the NW cannot use this RF minimum requirement. RF requirements should make sure that the UE is functional in most of the scenarios in which it is intended to operate for the feature configured (non-contiguous CA in this case).

- Adding an SCell in an FDD band may cause significant REFSENS degradation in PCell DL due to its own Tx blocker, potentially leading to PCell coverage loss after CA configuration, at least when the Tx is active.

- A band combination supported by a single Rx chain with significant REFSENS degradation in PCell DL is not feasible.

The PCell REFSENS should be maintained for non-contiguous CA regardless of the UE implementation. In the case of the fragmented carriers in TDD bands or FDD bands where the PCell (Tx) operates in another band of a composite band combination (BC), a slight SCell degradation of a few dB in the absence of an Tx blocker in the same band may be acceptable and the ACS and in-band blocking requirements can be maintained.

- In the case of fragmented carriers in the TDD band or FDD band with PCell operating in another band, a slight REFSENS degradation of a few dB may be acceptable in SCell but not in PCell.

- The PCell REFSENS should be maintained for non-contiguous CA regardless of the UE implementation.

# Proposal

In the previous sections, we made the following observations:

Observation 1	Adding an SCell in an FDD band may cause significant REFSENS degradation in PCell DL due to its own Tx blocker, potentially leading to PCell coverage loss after CA configuration, at least when the Tx is active.

Observation 2	In the case of fragmented carriers in the TDD band or FDD band with PCell operating in another band, a slight REFSENS degradation of a few dB may be acceptable in SCell but not in PCell.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	A band combination supported by a single Rx chain with significant REFSENS degradation in PCell DL is not feasible.

Proposal 2	The PCell REFSENS should be maintained for non-contiguous CA regardless of the UE implementation.

# References

- RP-241669, Study on NR FR1 DL Fragmented Carriers, MediaTek Inc.

- R4-2500219, In-Gap Blocking for Fragmented Carrier, Murata Manufacturing Co. Ltd.

- R4-2501397, Discussion of impacts on UE RF requirements and DL performance impacts, MediaTek Inc.