3GPP TSG-RAN WG4 Meeting #114bis	R4-2504109

Wuhan, China, 7th – 11th, April 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements of NTN with channel BW less than 5MHz

Agenda Item:	7.10.3.4

Document for:	Discussion

# Introduction

In the WI NR Enhanced requirements and test methodology for NR and IoT NTN, the objective: RRM of NTN with channel BW less than 5MHz was discussed in the last meeting, the consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on the leftover as follows.

# Discussion

#### Issue 1-1-1: Measurement(SSB index detection requirements):

Agreements: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by [3 or 4] and [3] for intra-frequency and inter-frequency cases respectively. Notes: the final value can be confirmed in the next meeting and the corresponding value will be updated in the draftCR

In BW less than 5MHz in TN, extensions of 4 and 3 samples are applied. However, during the last meeting, some companies expressed concerns that these extensions could be shortened considering the characteristics of NTN channels, though formal simulations could not be provided definitive numbers. Given that, 3 and 3 samples is likely to be the midground.

Proposal 1:  We are agreeable to using [3] and [3] if it does not present a major issue.

# Conclusion

Proposal 1:  We are agreeable to using [3] and [3] if it does not present a major issue.

# References

- R4-2502589, WF for RRM of NTN with channel BW less than 5MHz, Xiaomi