3GPP TSG-RAN WG4 Meeting #114	R4-2504111

Wuhan, China, 7th – 11th, April 2025

Source: 	Ericsson

Title:  	Discussion on other RRM requirements for NTN for NR Phase 3

Agenda Item:	7.29.4.2

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: SSB periodicity larger than 20 ms

## Issue 1-1: Cell Reselection

Further discussion: - In cell reselection requirements, discuss whether any update is needed for case where SSB periodicity is larger than 20 ms: - Option 1 (Samsung): FFS - Option 2 (vivo): No update is needed

Proposal 1:  Support Option 2, No update is needed.

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

Agreement (online): - In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided. Further discussion: - In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided. - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined. - FFS: whether the above is for only inter-frequency or for both intra-frequency and inter-frequency

## Issue 1-2-B: RRC Re-establishment (SSB Ês/Iot < -8dB and SSB periodicity >20 ms)

Further discussion: - In RRC re-establishment requirements, discuss whether any update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided: - Option 1: Yes, an update is needed - Note: With the understanding that the by-default SSB periodicity for initial access is 160ms for Rel-19, from RRM requirement perspective. - Companies are encouraged to provide details on how to update the specification. - Other option is not precluded.

Below is the req. relevant to initial access SSB periodicity in TS38.133.

6.2 RRC Connection Mobility Control 6.2.1 SA: RRC Re-establishment … TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signaling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell. TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms. Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); Kmulti_SMTC * MAX (800 ms, 10 x TSMTC). < -8; FR1; N/A; k * 800 ms Note1, 3. NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: Kmulti_SMTC is defined in clause 9.2C.5.1. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.. Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); K_satellite * MAX (800 ms, 13 x TSMTC, i). < -8; FR1; N/A; k * 800 ms Note1, 3. NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: K_satellite is defined in clause 9.3C.4. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt..

When Serving cell SSB Ês/Iot < -8dB, which is based on initial access SSB periodicity, 800ms is derived by 20ms *40 (SSB samples). When additional assumption of SSB period for initial access is introduced, UE may expect that the maximal 160ms SSB periodicity is provided by the NW in case of NW deployment compatible of 3GPP Rel-19.

When Serving cell SSB Ês/Iot ≥-8dB, when source cell does not provide SMTC information, UE can only do blind search. For the same reason as above, this assumption should be updated as well.

For example, with an SSB periodicity of 160 ms and 40 SSB samples, the total identification time could reach 6400 ms per cell. This raises the question of whether such a worst-case scenario is practically necessary.

However, in our opinion, the requirements explicitly refer to a UE in the RRC_CONNECTED state. This implies that the UE has already completed the initial cell selection process, successfully detecting and decoding the SS/PBCH block. Consequently, if the UE is required to perform an RRC connection re-establishment procedure, the expectation is that the configured periodicity should align with the SS/PBCH block periodicity used during initial cell selection.

Moreover, from test perspective, it’s not reasonable if 20ms is the SSB periodicity, but the UE can pass the requirements based on 160ms.

Observation 1: Depending on network implementation, 20ms or 160ms both may be implemented in practice.

Proposal 1: At the least for RRC connection re-establishment to NR intra-frequency cell, the assumption of SSB periodicity is assumed same as the SSB periodicity used during initial cell selection.

Below are updates to the existing note and table:

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms or 160 ms depending on the SSB periodicity used during initial cell selection.

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | Kmulti_SMTC * MAX (800 ms or 6400ms, 10 x TSMTC) |
| < -8 | FR1 | N/A | k * 800 or 6400ms ms Note1, 3 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: Kmulti_SMTC is defined in clause 9.2C.5.1. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. Note: 800 or or 6400ms depending on the SSB periodicity used during initial cell selection. |  |  |  |

Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 or 6400ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | k * 800 or 6400ms ms Note1, 3 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: K_satellite is defined in clause 9.3C.4. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. Note: 800 or or 6400ms depending on the SSB periodicity used during initial cell selection. |  |  |  |

# Conclusion

Observation 1: Depending on network implementation, 20ms or 160ms both may be implemented in practice.

Proposal 1: At the least for RRC connection re-establishment to NR intra-frequency cell, the assumption of SSB periodicity is assumed same as the SSB periodicity used during initial cell selection.

Below are updates to the existing note and table:

TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms or 160 ms depending on the SSB periodicity used during initial cell selection.

Table 6.2C.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 5 x TSMTC) | Kmulti_SMTC * MAX (800 ms or 6400ms, 10 x TSMTC) |
| < -8 | FR1 | N/A | k * 800 or 6400ms ms Note1, 3 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: Kmulti_SMTC is defined in clause 9.2C.5.1. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. Note: 800 or or 6400ms depending on the SSB periodicity used during initial cell selection. |  |  |  |

Table 6.2C.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell

| Serving cell SSB Ês/Iot (dB) | FR of target NR cell | Tidentify_inter_NR, i [ms] |  |
| --- | --- | --- | --- |
|  |  | Known NR cell | Unknown NR cell |
| ≥ -8 | FR1 | MAX (200 ms, 6 x TSMTC, i) | K_satellite * MAX (800 or 6400ms, 13 x TSMTC, i) |
| < -8 | FR1 | N/A | k * 800 or 6400ms ms Note1, 3 |
| NOTE 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB. NOTE 2: K_satellite is defined in clause 9.3C.4. NOTE 3: k = 1 if the cells on the target frequency are served by GEO. k = (N+1) if the cells on the target frequency are served by LEO, where N is the number of different satellites associated to the list of configured neighbour cells in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt. Note: 800 or or 6400ms depending on the SSB periodicity used during initial cell selection. |  |  |  |

# References

- R4-2502605, The WF on RRM requirements for Rel-19 NR NTN phase3, QC