3GPP TSG-RAN WG4#114-bis	R4-2504121

Wuhan, China, April 7th – 11th, 2025

Agenda item:	7.1.1.3.4

Source: 	Qualcomm France

Title: 	SRS IL imbalance indication

Document for:	Approval

# Introduction

SRS IL imbalance indication considered in this contribution.

- Discussion

# WF for SRS IL imbalance indication is as follows [1]

## Sub-topic 3-1:	General considerations for SRS IL imbalance issue

Issue 3-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way Forward: RAN4 to further discuss the following options.

-	Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

-	Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on the candidate solution framework with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 3-2-1.

Issue 3-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: RAN4 to focus on refining the compromised solution framework in Issue 3-2-1.

## Sub-topic 3-2:	SRS IL imbalance issue solutions

Issue 3-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: RAN4 to take the candidate solution framework below as baseline for further discussion.

Candidate solution framework:

-	Rel-19 UE performs self-compensation of SRS IL up to its configured maximum output power.

-	FFS on RAN4 specification impacts.

-	No RAN1 specification impact.

-	FFS if UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g.

-	Per SRS resource power headroom reporting

-	Configured maximum output power per SRS resource reporting

-	SRS insertion loss value reporting (per SRS resource or per UE)

-	Other

-	ΔPPowerClass reporting is proposed as a package with per SRS resource power headroom and configured maximum output power per SRS resource reporting.

-	Others

RAN4 to further discuss the solution framework and consider the additional proposals presented at RAN4#114 below.

Proposals:

- Proposal 1: The self-compensation should be triggered only if the PCMAX,f,c with IL influence is smaller than the necessary SRS power.

- Proposal 2: UE should report whether supporting SRS IL self-compensation or not, and the compensation as well as reporting should all be activated by configuration.

- Proposal 3: Single digit optional capability to indicate self-compensation of SRS IL up to its maximum Tx power capabilities.

- Proposal 4: If UE reports statically, report the actual SRS insertion loss with no UE self-compensation.

- Proposal 5: If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time.

- Proposal 6: Reporting power threshold needs to be considered for static reporting and dynamic reporting.

- Proposal 7: The granularity of reporting could be per SRS resource or per band.

- Proposal 8: If dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern is considered, several thresholds associated with capability class for the actual SRS IL reporting can be considered.

- Proposal 9: To keep the current UE implementation untouched and avoid RAN1&RAN4 specification impact to most extent, following solution can be considered in Rel-19:

-	UE is allowed to indicate whether it enables self-compensation on the SRS IL once the network requests such information, which would benefit the network by adjusting expectation on the antenna switching SRS based PMI estimation.

-	Proposal 10: Adjust the existing equations on PCMAX_L,f,c so that UE applies compensation up to UE-specific SRS IL.

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS,UE, P-MPRc) }

- Proposal 11: Introduce UE assistance signalling on the information on the UE SRS IL (∆TRxSRS,UE). Consider at least static reporting.

Proposal 12: UE provides dynamic assistance to the network on SRS IL compensation.

- Discussion

As known, this topic has been in stalemate situation for the past several years. After more thorough look, we don’t see that any of the proposed solutions would improve overall situation, even though they would cause a lot of extra complexity.

Any reporting per SRS resource would only work when SRS resource consists of one SRS port. For SRS resource consisting of 2 ports, if would not help because the IL can be (and often is) different. Hence reporting one shared delta value would be inaccurate e.g. in case when power available on SRS port#3 is 21dB, and power available in SRS port#4 is 17dB.

Static reporting, e.g. reporting IL value delta for each RX path would not work either, because then basically UE would be forced not to compensate in any conditions, because otherwise the delta would be useless. Also, this would preclude dynamic mapping between antenna ports and SRS ports.

In previous meeting, there was discussion on the premise that UE does compensate the SRS IL when possible by the power reserves. Proponent companies were eager to formally agree on that, which anyhow would be likely RAN1 decision if formal decision is targeted. We re-iterate our comments from previous meetings that is when the UE compensates SRS IL when possible by power reserves, any extra indication would not help as the said delta would be zero as long as UE is able to compensate. There have been no simulation results to show system-level gains for the cases where UE compensates when it has power to do so and does not compensate when there is no power remaining.

It is obvious that the system-level benefits, if any, would be very small especially for UE’s compensating the SRS IL. In far-cell scenario where compensation may not be possible, SRS may not be that beneficial anyhow independent of indication or no indication. In any kind of indication there would be quite a bit of additional complexity, and no guarantee that NW would use any of the additional information.

Observation 1: Proposed SRS IL indication schemes would not work for all cases.

Observation 2: Proposed SRS IL indication schemes would introduce quite a bit of extra complexity.

Observation 3: There have been no simulation results to show system level gains for the cases where UE compensates when it has power to do so and does not compensate when there is no power remaining.

After numerous discussions during the past years, RAN4 should stop discussions on this topic.

Proposal 1: RAN4 should not continue the discussion on SRS IL imbalance indication.

- Conclusion

SRS IL compensation was discussed with the following observations and proposal.

Observation 1: Proposed SRS IL indication schemes would not work for all cases.

Observation 2: Proposed SRS IL indication schemes would introduce quite a bit of extra complexity.

Observation 3: There have been no simulation results to show system level gains for the cases where UE compensates when it has power to do so and does not compensate when there is no power remaining.

Proposal 1: RAN4 should not continue the discussion on SRS IL imbalance indication.

- Reference

[1] R4-2503014, “WF on 6Rx requirements”, AT&T