3GPP TSG-RAN WG4 Meeting # 114bis					R4-2504129

Wuhan, China, Apr. 7th ‒ Apr. 11th, 2025

Source:	Nokia

Title:	General aspects of Fragmented CA

Agenda item:	7.5.1

Document for:	Discussion

Introduction

Agreements at previous meetings

The latest WF for NR FR1 DL fragmented carriers study [1] presents the open issues as found below:

#### Issue 3-3: Discussion on in-gap interference requirements

- For image of in-gap interference not overlap with wanted CC: Evaluate whether relaxation on ACS/IBB is needed based on evaluated ΔRIBNC on PCC and SCC.

- For image of in-gap interference (ACS/IBB) overlap with wanted CC: companies are encouraged to provide the impact of interference to wanted signal power ratio of [15-26]dB to the ΔRIBNC for PCC and SCC.

# Open issues

### How to indicate UE capability for supporting ”one Rx RF chain” for DL fragmented carriers

Option 1: Legacy band combination reporting scheme + Rx sharing capability indication

- FFS on whether the capability is specified as ”per-band per-BC” or “per CC per band per BC”.

Other options are not precluded

### Discussion on semi-statically switch hardware resources procedure and fallback considerations

### Triggering condition to enable ”One RF Rx chain”

FFS:

- FFS on whether the UE indication solution is needed or not, with understanding that enabling one Rx chain is under the control of network.

- Triggering condition for UE working in one Rx RF chain is needed

- Triggering condition for UE working in legacy Rx RF chains is needed

- The switching should be not be dynamic and be under network control with UE indication of condition.

### Discussion on applicability for PCell and Scell

### Discussion on the number of Rx and DL MIMO layer configuration for the UE supporting ”one Rx RF chain” mode

Discussion

Indicate” One Rx RF chain” for fragmented carriers

We find it necessary to clearly distinguish the radio configuration of the non-contiguous intra-band carrier aggregation between the modes of operation related to using the partially shared RF architecture and the fully shared RF chain at the UE side. It must be agreed that configuring the UE using the fully shared RF chain includes the operation at the UE as identified in this study including a risk of performance degradation.

To allow both the UE and the network separating fragmented carrier operation from other types of CA configurations a different indication is needed.

The currently undefined NR CA bandwidth class (BCS) ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

TS 38.101-1: Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34, 4 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |  |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |  |
| F | BWChannel ≤ 100 MHz | 1, NOTE 5 | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |  |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 5: For fragmented carrier operation with a single contiguous frequency range configured for the UE with multiple fragmented carriers as indicated by IE [FragmentedCarrier]. |  |  |  |

In the proposal above, where the newly added parts are highlighted with yellow, F is inserted with a corresponding number of contiguous CCs as ‘1’, as the intention is to treat the multiple fragmented carriers as a single CC regarding the number of occupied Rx RF chains. Adding Note 5 clarifies that this single contiguous CC can consist of multiple fragmented carriers assumed occupying 1 Rx chain. The new BCS, F, is then added with fallback group 4, which is added correspondingly for the single CC BCS, A, as the fallback from fragmented carrier operation would be single CC operation. However, if the UE can fallback to 2 Rx RF chains from single Rx RF chain (due to high in-gap interference) this should also be possible.

RAN shall consider the currently undefined NR CA bandwidth class ‘F’ to be introduced for indicating a CA configuration with fragmented carrier operation.

By adding the new NR CA bandwidth class, this can be used to identify fragmented carrier operation and the number of fragments. Examples using band n25 with two and four (should the concept evolve beyond the 2CC in scope) fragments are shown in the figures below.

Fig.  2: Example of CA_n25(2F)

Fig.  3: Example of CA_n25(4F)

The combination of non-contiguous intra-band carriers and fragmented carriers are also supported with syntax as shown in Fig. 4, but there’s currently no scenarios of the study that are obvious to project the syntax onto.

Fig.  4: Example of CA_n78(2A-2F)

TP to TR 38.755

--- > Start of TP < ---

# 8	Study of means for a UE to inform the network of appropriate CA configuration it can support

## 8.1	Indication of fragmented CA

To allow both the UE and the network separating fragmented carrier operation from other types of CA configurations a different indication is suggested during the study phase.

A proposal is to use the currently undefined NR CA bandwidth class (BCS) ‘F’ should be introduced for indicating a CA configuration with fragmented carrier operation.

TS 38.101-1: Table 5.3A.5-1: NR CA bandwidth classes

| NR CA bandwidth class | Aggregated channel bandwidth | Number of contiguous CC | Fallback group |
| --- | --- | --- | --- |
| A | BWChannel ≤ BWChannel,max | 1 | 1, 2, 34, 4 |
| B | 20 MHz ≤ BWChannel_CA ≤ 100 MHz | 2 | 2, 34 |
| C | 100 MHz < BWChannel_CA ≤ 2 x BWChannel,max | 2 | 1, 34 |  |
| D | 200 MHz < BWChannel_CA ≤ 3 x BWChannel,max | 3 |  |  |
| E | 300 MHz < BWChannel_CA ≤ 4 x BWChannel,max | 4 |  |  |
| F | BWChannel ≤ 100 MHz | 1, NOTE 5 | 4 |  |
| G | 100 MHz < BWChannel_CA ≤ 150 MHz | 3 | 2 |  |
| H | 150 MHz < BWChannel_CA ≤ 200 MHz | 4 |  |  |
| I | 200 MHz < BWChannel_CA ≤ 250 MHz | 5 |  |  |
| J | 250 MHz < BWChannel_CA ≤ 300 MHz | 6 |  |  |
| K | 300 MHz < BWChannel_CA ≤ 350 MHz | 7 |  |  |
| L | 350 MHz < BWChannel_CA ≤ 400 MHz | 8 |  |  |
| M3 | 50 MHz ≤ BWChannel_CA ≤ 200 MHz | 3 | 34 |  |
| N3 | 80 MHz ≤ BWChannel_CA ≤ 300 MHz | 4 |  |  |
| O3 | 100 MHz ≤ BWChannel_CA ≤ 400 MHz | 5 |  |  |
| NOTE 1: BWChannel, max is maximum channel bandwidth supported among all bands in a release NOTE 2: It is mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration within a fallback group. It is not mandatory for a UE to be able to fallback to lower order NR CA bandwidth class configuration that belong to a different fallback group. NOTE 3: This bandwidth class is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 4: Fallback group 3 is only applicable to bands identified for use with shared spectrum channel access in Table 5.2-1. NOTE 5: For fragmented carrier operation with a single contiguous frequency range configured for the UE with multiple fragmented carriers as indicated by IE [FragmentedCarrier]. |  |  |  |

In the proposal above, where the newly added parts are highlighted with yellow, F is inserted with a corresponding number of contiguous CCs as ‘1’, as the intention is to treat the multiple fragmented carriers as a single CC regarding the number of occupied Rx RF chains. Adding Note 5 clarifies that this single contiguous CC can consist of multiple fragmented carriers assumed occupying 1 Rx chain. The new BCS, F, is then added with fallback group 4, which is added correspondingly for the single CC BCS, A, as the fallback from fragmented carrier operation would be single CC operation. However, if the UE can fallback to 2 Rx RF chains from single Rx RF chain (due to high in-gap interference) this should also be possible.

By adding the new NR CA bandwidth class, this can be used to identify fragmented carrier operation and the number of fragments. Examples using band n25 with two fragments are shown in Fig 1.

Fig.  1: Example of CA_n25(2F)

--- > End of TP < ---

Shall agree the TP to TR presented in Clause 3.

Conclusion

In this contribution, we discuss the study item on fragmented carriers. The following Observations and Proposals were made:

Observation 1: To allow both the UE and the network separating fragmented carrier operation from other types of CA configurations a different indication is needed.

Proposal 1: RAN shall consider the currently undefined NR CA bandwidth class ‘F’ to be introduced for indicating a CA configuration with fragmented carrier operation.

Proposal 2: Shall agree the TP to TR presented in Clause 3.

- References

- R4-2502873, WF on NR FR1 DL fragmented carriers study, RAN4#114

- R4-2504173, Discussion on methods for reducing the number of UE Rx chains, Nokia

- R4-2501662, Discussion on requirements for Fragmented CA, Nokia