3GPP TSG-RAN WG4 Meeting#114bis		R4-2504179

Wuhan, China, Apr 7th -11th, 2025

Agenda item:	7.5.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

Introduction

In RAN4#114 meeting, a WF [1] was approved. In this contribution, we continue to provide some initial views on methods for reducing the number of UE RX chains.

- Main session online agreements #### Issue 2-1: Discussion on baseband assumptions Agreement: - Consider only same SCS for both CCs for one Rx RF chain architecture in this SI - 15KHz SCS is used as the starting point - Single FFT or separate FFT can be left to UE implementation, and however the condition of single FFT compared to separate FFT can be captured in the TR. - Ad hoc meeting agreements #### Issue 2-5: Triggering condition to enable ”One RF Rx chain” RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled - Further study whether the absolute power level of in-gap interferer and/or the PSD difference between in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled - Other determining factors are not precluded to be studied - Study feasibility to measure total aggregated power of all interferences in the gap between CC1 and CC2 #### Issue 3-3: Discussion of impacts on DL performance for UE enabling fragmented carrier mode - Companies are encouraged to bring analysis with their own assumptions following the table format - A separate table would be needed for Tx leakage + blocker and individual CA channel bandwidth combination in Table 7.3A.2.2-1 of TS 38.101-1 Table 1. Link budget of Self-interference for 1st configuration on band n3 Tx leakage; Wanted Carrier (PCC); Wanted Carrier (SCC); Noise floor; ADC noise (11 bit): -68.0dBFs (6.02N+1.76). Tx leakage mapping to 0dBFs (PAPR is not considered); Tx leakage mapping to -12dBFS (PAPR is considered). PA out; 27; -; -; -; -; -. Duplexer isolation; 50; -; -; -; -; -. Rx Antenna input; -; -97; -92.3; -; -; -. FE loss; -; 4; 4; -; -; -. Duplexer output; -23; -101; -96.3; -; -; -. LPF attenuation; 13.4; 1; 1; -; -; -. LPF output (dBm); -36.4; -102; -97.3; -; -; -. Noise floor for basic REFSENS; -; -; -; -101; -; -. NF increase due to AGC adjustment; -; -; -; 10; -; -. Noise floor; -; -; -; -91; -; -. In-band noise (Derived based on MSD and SNR = -1dB); -; -; -96.3; -; -; -. ADC output (dBm); -; -; -; -; -104.4; -92.4. SNR after ADC (dB); -; -; -; -; -11.2(PCC); -13.4(PCC). -7.6(SCC); -9.4(SCC). #### Issue 3-4: Discussion on in-gap requirements RAN4 evaluate performance impact on the cases: - image of in-gap interference overlaps with wanted carrier - For the cases, image of ACS/IBB1/IBB2/NBB overlaps with wanted CC - image of in-gap blocking interference does not overlap with wanted carrier #### Issue 2-4: Signaling related WF: Companies are encouraged to provide analysis with pros and cons on preferred signalling solutions so that they can be discussed in next meeting. #### Issue 3-1: Discussion on evaluation priority of example bands WF: It would be up to companies’ preference and contribution driven on the example bands evaluation. The study needs to cover evaluation on both FDD and TDD bands and focus on general requirements. Band specific requirements can be left for WI phase. - Open issues (for information) Open issues are to be discussed in the meetings later on. Other issues are not precluded to be further discussed #### Issue 2-7: Others RAN4 to clarify whether either of the two fragmented carriers shall be a PCELL or if fragmented carriers can be defined purely as DL SCELL’s #### Issue 3-2: Discussion on maximum power level of in-gap interference WF: For one Rx RF chain, the maximum acceptable absolute power of in-gap interferer should be less than or equal to -25dBm, specific value is FFS #### Issue 3-5: Discussion on DL fragmented carriers in TDD bands WF:Background: To consider transition time if there’s one LO for the TDD band and study the feasibility Discuss following options for the number LO’s needed for fragmented carrier operation in TDD with one of the carriers having UL allocation: - Option 1: One LO will be enough for both UL and DL. - Option 2: Separate LOs will be needed for UL and DL

Discussion

Triggering condition to enable “One RF Rx chain”

According to the last meeting agreement, RAN4 to identify the mechanisms that surround the UE implementation, i.e. whether one Rx RF chain operation can be enabled. Further study whether the absolute power level of in-gap interferer and/or the PSD difference in-gap interferer and target CC could be used to determine whether one Rx RF chain is enabled. In our understanding, in-gap interferer just comes from other operators. The other factors, such as IMD5 and IMD7, image of LO and TX leakage all need to be considered. Hence, we need to consider total aggregated power of interferences in the gap between CC1 and CC2. In addition, how to measure the total aggregated power of all interferences in the gap between CC1 and CC2. We can depend on gNB triggering the UE or UE autonomously to measure the total aggregated power of all interferences in the gap between CC1 and CC2, there is no need to consider a new reference signal.

Proposal 1: The total aggregated power of all interferences in the gap between CC1 and CC2 needs to be studied.

Proposal 2: GNB triggers UE or UE autonomously to measure the total aggregated power of all interferences in the gap between CC1 and CC2, there is no need to consider a new reference signal.

Regarding whether the UE indication solution is needed or not, we have already discussed how to indicate UE capability for supporting “one Rx RF chain” for DL fragmented carriers, which means that Rx sharing capability indication is able to report to NW. In order to simplify UE implementation, the UE indication for one RF Rx chain is not needed. Enabling or disabling one Rx RF chain can be done under the network.

Proposal 3: The UE indication for enabling one RX chain solution is not needed, which can be done under the control of network.

About whether the triggering condition for UE working in one Rx RF chain is needed or not, based on the above analysis, UE can measure the total aggregated power of all interferences in the gap between CC1 and CC2, whether gNB trigger the UE or UE autonomously to measure. UE reports the results to NW, then NW judges whether the UE can satisfy the trigger condition for enabling one Rx RF chain or not according to the threshold. When the NW judge that the UE cannot satisfy the triggering condition in one Rx RF chain, then the UE will autonomously fall back to legacy Rx RF chains. In our understanding, a triggering condition for UE working in one Rx RF chain is needed, triggering condition for UE working in legacy Rx RF chains is not needed.

Proposal 4: Triggering condition for UE working in one Rx RF chain is needed, and triggering condition for UE working in legacy Rx RF chains is not needed.

Discussion on 2Rx applicability

According to the current spec, there is no clear specification about how many Rx should be implemented for DL NCCA. In our understanding, there is no need to limit the UE need to support 4Rx when UE is configured DL NCCA in the mandatory when configured in fragmented carrier operation. It is up to UE implementation. However, in order to simplify the workload in SI, RAN4 doesn’t need to discuss 2Rx. We can assume UE can support 4Rx when UE is configured DL NCCA in the mandatory 4Rx when configured in fragmented carrier operation, RAN4 doesn’t need to discuss 2Rx.

Proposal 5: Whether 4Rx or 2Rx is applicable to DL NCCA up to UE implementation. In SI stage, UE can support 4Rx when UE is configured DL NCCA in the mandatory 4RX when configured in fragmented carrier operation.

How to indicate UE capability for supporting “one Rx RF chain” for DL fragmented carriers

In terms of indicating UE capability, in order to reduce the signalling overhead, the existing legacy band combination reporting scheme plus Rx sharing capability indication is preferred. When the UE reports this Rx sharing capability, the NW will trigger UE or UE will automatically measure the total aggregated interference between CC1 and CC2. The detailed indication solution could be discussed in the WI stage.

Proposal 6: Existing legacy band combination reporting scheme plus Rx sharing capability indication is preferred. Detailed solutions could be discussed in the WI stage.

Conclusions

In this paper, we provide our views on methods for reducing the number of UE RX chains, we have made the following proposals.

Proposal 1: The total aggregated power of all interferences in the gap between CC1 and CC2 needs to be studied.

Proposal 2: GNB triggers UE or UE autonomously to measure the total aggregated power of all interferences in the gap between CC1 and CC2, there is no need to consider a new reference signal.

Proposal 3: The UE indication for enabling one RX chain solution is not needed, which can be done under the control of network.

Proposal 4: Triggering condition for UE working in one Rx RF chain is needed, and triggering condition for UE working in legacy Rx RF chains is not needed.

Proposal 5: Whether 4Rx or 2Rx is applicable to DL NCCA up to UE implementation. In SI stage, UE can support 4Rx when UE is configured DL NCCA in the mandatory 4RX when configured in fragmented carrier operation.

Proposal 6: Existing legacy band combination reporting scheme plus Rx sharing capability indication is preferred. Detailed solutions could be discussed in the WI stage.

- Reference

- R4-2420519,WF on fragmented DL carriers study, Mediatek Inc. TSG-RAN WG4 113 meeting