3GPP TSG-RAN WG4 Meeting #114bis	R4-2504180

# Wuhan, China, 7th - 11th, April 2025

Source: 	Huawei, Hisilicon

Title: 	Discussion on introducing Tx switching for 4Tx FWA

Agenda Item:	5.3.1

Document for:	Discussion

1	Introduction

Based on 2TX UE, TX switching feature was introduced in Rel-16 NR_RF_FR1 WI [1] and has been further enhanced in Rel-17 NR_RF_FR1_enh WI [2] and Rel-18 NR_MC_enh WI [3]. By dynamically selecting the best available uplink carrier based on channel conditions and U slot duty cycle, UE can achieve higher uplink throughputs.

Driven by strong commercial demand, Tx switching for 3Tx UEs co-sourced by 12 operators was proposed and discussed in the last RAN4 meeting [4]. Since 4Tx FWA has already been introduced in Rel-18 NR_ENDC_RF_FR1_enh2 WI [5], Tx switching for 4Tx FWA would be also important and beneficial to boost the uplink peak data rate and throughput for NR CA band combinations with TDD band. Besides, supporting the switchedUL between one SUL/FDD/TDD band and one TDD band would be with less specification impacts, because switchedUL is basically UL carrier(s) switching between two bands no matter how many Tx ports are supported on each band.

In this paper, we propose to introduce switchedUL for 4Tx FWA in order to minimize spec effort.

2	Discussions

Rel-16/17 specified Tx switching based on SUL or uplink CA band combinations. For this, a 2Tx UE is configured with uplinkTxSwitchingOption set to 'switchedUL' in the following cases, which is a simpler configuration. The 'switchedUL' is an ideal match for 4Tx FWA. Furthermore, 'switchedUL' also can help minimize the impact on the specification.

| [2] | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

A 4Tx FWA configured with uplinkTxSwitchingOption set to ' switchedUL ' can achieve benefits at least as good as, and potentially exceeding, those of the original 2Tx UEs with ' switchedUL '. Further, the scenario can be considered as SUL or uplink CA band combinations.

| uplinkTxSwitchingOption set to 'switchedUL' | Scenario 1: (TDD+SUL) | Scenario 2: (TDD+TDD) | Scenario 3: (TDD+FDD) |
| --- | --- | --- | --- |
| Case1 | 4Tx+0Tx | 4Tx+0Tx | 4Tx+0Tx |
| Case2 | 0Tx+2/4Tx | 0Tx+2/4Tx | 0Tx+2/4Tx |

Scenario 1: (TDD+SUL)

Scenario 2: (TDD+TDD)

Scenario 4: (TDD+FDD)

As can be seen from the simplified results presented above, there is a significant increase in uplink throughput. Regarding Scenario 1 and Scenario 3, the gains are primarily attributed to the continuous availability of uplink slots, whereas for Scenario 2, the gains mainly stem from the configuration of different uplink slot ratios in TDD.

Observation1: Allowing Tx switching of 4Tx FWA can bring significant UL throughput by switchedUL for the following three scenarios.

| uplinkTxSwitchingOption set to 'switchedUL' | Scenario 1: (TDD+SUL) | Scenario 2: (TDD+TDD) | Scenario 3: (TDD+FDD) |
| --- | --- | --- | --- |
| Case1 | 4Tx+0Tx | 4Tx+0Tx | 4Tx+0Tx |
| Case2 | 0Tx+2/4Tx | 0Tx+2/4Tx | 0Tx+2/4Tx |

To minimize the specification impact of this TEI, only switchedUL is considered. This includes the following changes: RAN1 extends Clause 6.1.6.2.0 of TS38.214 from 2 to 4 ports by adding the new clause with the simplified definition of switchedUL; RAN2 introduces new UE capabilities for 4T support; and RAN4 introduces 4T-based switching scenarios.

Proposal 1: Introduce Tx switching for 4Tx FWA configured with uplinkTxSwitchingOption set to 'switchedUL' in Rel-19 TEI.

3	Summary

In this paper, we illustrate that 4Tx FWA configured with uplinkTxSwitchingOption set to 'switchedUL' offers substantial advantages, and these benefits can be extended to 4Tx FWA through the support of Tx switching feature with minimal specification impacts.

Observation1: Allowing Tx switching of 4Tx FWA can bring significant UL throughput by switchedUL for the following three scenarios.

| uplinkTxSwitchingOption set to 'switchedUL' | Scenario 1: (TDD+SUL) | Scenario 2: (TDD+TDD) | Scenario 3: (TDD+FDD) |
| --- | --- | --- | --- |
| Case1 | 4Tx+0Tx | 4Tx+0Tx | 4Tx+0Tx |
| Case2 | 0Tx+2/4Tx | 0Tx+2/4Tx | 0Tx+2/4Tx |

Proposal 1: Introduce Tx switching for 4Tx FWA configured with uplinkTxSwitchingOption set to 'switchedUL' in Rel-19 TEI.

7	References

- [1] RP-193200, WID revision: RF requirements for NR frequency range 1 (FR1), Huawei, HiSilicon, 3GPP TSG-RAN Meeting #85.

- [2] RP-220681, Revised WID: RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon, 3GPP TSG-RAN Meeting #95-e.

- [3] RP-234012, Revised WID on Multi-carrier enhancements for NR, NTT DOCOMO, INC, 3GPP TSG RAN Meeting #102

- [4] R4-2501624, Introducing Tx switching for 3Tx UEs in Rel-19 TEI, MediaTek Inc., Orange, T-Mobile USA, Vodafone, Verizon, 3GPP TSG-RAN WG4 Meeting #114.

- [5] RP-233186, Revised WID on Further RF requirements enhancement for NR and EN-DC in frequency range 1 (FR1) , Huawei, HiSilicon, 3GPP TSG-RAN Meeting #102.