3GPP TSG RAN WG4 #114bis	R4-2504195

Wuhan, China, April 7 – 11, 2025

Agenda item:		7.23.2.2

Source:	Nokia

Title:	On existing BS RF TX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112bis [1], and updates were made in RAN4#113 [2] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the existing BS RF TX requirements for SBFD operation.

# Discussion

## Unwanted emissions

The following was agreed in RAN4#113:

Agreement: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

Also regarding this issue, there was a following way forward in RAN4#114:

WF: - For FR2-1 WA SBFD BS, FFS re-use the current FR2-1 WA BS in-band blocking requirements. - For FR1 WA BS, define the in-band blocking requirement, FFS on following options. Other options are not precluded. - Option 1: Define the requirement by selecting one of the following grid-shift values. Additional emission issue is not included in this release. If regulators make a decision to require SBFD on adjacent operators in a band, a future WI would address necessary emissions and blocking requirements for the SBFD-SBFD adjacent operator scenario. - Option A: 20% - Option B: 50% - Option 2: Define general requirements for SBFD-TDD adjacent operator scenario. Define an additional in-band blocking requirement and additional emission to address SBFD-SBFD adjacent operator scenario. - Option 3: Define in-band blocking requirements based on co-existence simulation results with grid-shift 5%. - Option 4: Define in-band blocking requirements based on co-existence simulation results with grid-shift 100%. - For FR1 MR SBFD BS, FFS.

We do not see the necessity of additional emission would be needed from 3GPP to define. The described scenario where two operators SBFD capable gNB’s would coexist, is a new scenario and a rare one. It has not been discussed in the study item phase and should not be included here. We find it very problematic regarding the progress on the RX in-band blocking issue to bundle this additional item together with the RX in-band blocking. There have been several meetings without any progress. Also, one thing to notice here is that if two SBFD networks would coexist, this would mean that TDD networks would not be possible to deploy. For FR1 frequencies, only the current n104 band would then be suitable. This in effect would mean reserving n104 only to SBFD use. In our view the customer specific proprietary requirement can be used in such case where two SBFD systems are co-existing and reasonable co-location is needed. If there is a requirement from regulators for SBFD on adjacent operators, RAN4 can have an enhancement to SBFD on a dedicated WI to define additional emissions and blocking requirements for this scenario.

Therefore, due to reasons mentioned above, we support option 1 with modified grid-shift value of 10%. The reasoning for this grid shift value is described in [2].

No need for RAN4 to specify additional emission requirements.

## Joint measurement for normal DL symbols/slots and SBFD symbols/slots

The following was agreed in RAN4#114:

WF: - Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity.

After discussion in the last meeting RAN4#114, it can be agreed that slot-level test granularity should be kept. This would be captured well in the TP from Ericsson (R4-2502157) that states the following:

- ---------------------------------------------------------Start of Text Proposal -------------------------------------------------------

# B.7	Averaged EVM

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval. For SBFD, only resource blocks allocated to downlink subbands shall be measured.

For FDD the averaging in the time domain equals the Ndl slot duration of the 10 ms measurement interval from the equalizer estimation step.

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVM requirements shall be tested against the maximum of the RMS average at the window W extremities of the EVM measurements:

-	Thus   is calculated using in the expressions above and  is calculated using  in the  calculation.

-	Thus we get:

EVM=maxEVMframe,l,EVMframe,h

For TDD or SBFD, let NdlTDD be the number of slots with downlink symbols or SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

- ---------------------------------------------------------End of Text Proposal---------------------------------------------------------

EVM calculation in Annex B.7 can be updated as proposed in R4-2502157.

# Conclusion

In this contribution, our views on the existing SBFD BS RF Tx requirements are presented. Our proposals are listed below.

- No need for RAN4 to specify additional emission requirements.

EVM calculation in Annex B.7 can be updated as proposed in R4-2502157.

# References

- R4-2502380 “Way Forward for [114][307] NR_duplex_evo_BSRF”, RAN4#114, Athens, Greece, 17th-21st February, 2025

- R4-2504196 “On existing BS RF RX requirements for SBFD operation”, RAN4#114bis, Wuhan, China, 7th-11th April, 2025