3GPP TSG-RAN4 Meeting #114bis	R4-2504212

# Wuhan, China, 7th – 11th April, 2025

Source:	LG Electronics

Title: 	On Ku-band NTN VSAT requirements

Agenda Item:	7.11.4

Document for:	Discussion and Approval

# Introduction

RAN4 is discussing requirements for Ku-band NTN VSAT. Way forward [1] was agreed in RAN WG4 Meeting #114 in Athens in February 2025.

This document further discusses and makes proposals on the following sub-topics captured to the Way Forward.

Issue 1-1-1: VSAT types to be considered for NTN Ku band after RAN#106 update

- VSAT type 1-3 will not be specified for Ku bands based on the revised WID

- The mobile VSAT support of the Ku bands will be added based on the existing mobile VSAT type 4 & 5

  - The following changes can be the starting point

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52,3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: LEO support is only defined for Ku bands (TBD) |  |  |

Issue 1-1-5: General RF requirements impact on numerologies dependency

- Only radiated requirements shall be updated with the new channel bandwidths to be supported.

- It is confirmed the requirements in section 9.2 Transmitted power, 9.4 Transmitter signal quality, 10.8 Receiver antenna off-axis are numerology independent.

- The requirements in section 9.5 Output RF spectrum emission, 10.3 OTA reference sensitivity level, 10.4 Maximum input level, 10.5 ACS & 10.6 blocking needs to be updated due to the new channel bandwidth at least.

Issue 1-2-3: Minimum EIRP requirements

- Companies are encouraged to provide views on the minimum peak EIRP requirement for the NTN Ku band in the next meeting.

# Discussion

## VSAT types to be considered for NTN Ku band after RAN#106 update

RAN4 agreed to use the following as starting point, where LEO support is added for Ku-bands for NTN VSAT types 4 and 5.

- VSAT type 1-3 will not be specified for Ku bands based on the revised WID

- The mobile VSAT support of the Ku bands will be added based on the existing mobile VSAT type 4 & 5

  - The following changes can be the starting point

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Mobile VSAT | 43 | Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 52,3 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: LEO support is only defined for Ku bands (TBD) |  |  |

Observation 1: Based on the agreed starting point and the discussions in the RAN4 #114 meeting these type 4 and 5 devices MUST be able to communicate with BOTH GSO and LEO satellites, which is indicated with word and “with GSO and LEO” in type description.

RAN4 should keep this kind of device ability to communicate with satellites on two different orbits at very different distance from the earth in mind, when defining the corresponding VSAT RF requirements. In order to define clear minimum performance requirements for VSAT RAN4 should decide whether to define set of RF requirements as per satellite orbit, i.e. different RF requirements depending on SAN orbit or set of requirements that are agnostic of the SAN orbit. This choice can be heavily impacted by the architecture choices for the VSAT manufacturers. The RF requirements should ensure that devices meet the minimum requirements in all modes of operation but at the same time do not introduce testing overhead. Testability aspect is especially important, as RAN4 will define only radiated RF requirements.

For transmitter EIRP requirements it is relatively easy conclude that communication with GSO needs higher minimum value and this value is enough to ensure the communication with LEO also. On receiver side, on the other hand, communication with LEO on the other hand may needs higher dynamic range than communication with GSO.

Proposal 1: In order to define clear minimum performance requirements for Ku-band VSAT RAN4 should decide whether to define set of RF requirements as per satellite orbit, i.e. different RF requirements depending on SAN orbit or set of requirements that are agnostic of the SAN orbit.

Observation 2: The existing requirements for Ka-band NTN VSAT device types 1 and 2 (Fixed VSAT communicating with GSO and LEO) defines the orbit agnostic requirements. The orbit is only considered for additional regional requirements when required by regulation.

Proposal 2: Follow the approach selected for Ka-band and define set of requirements that are agnostic of the SAN orbit. Consider the orbit for additional regional requirements only if/when required by regulation.

## General RF requirements impact on numerologies dependency

The WF agreed the following:

- Only radiated requirements shall be updated with the new channel bandwidths to be supported.

- It is confirmed the requirements in section 9.2 Transmitted power, 9.4 Transmitter signal quality, 10.8 Receiver antenna off-axis are numerology independent.

- The requirements in section 9.5 Output RF spectrum emission, 10.3 OTA reference sensitivity level, 10.4 Maximum input level, 10.5 ACS & 10.6 blocking needs to be updated due to the new channel bandwidth at least.

We re-confirm the above and also make the following remarks and proposal for required changes in [2].

Proposal 3: Shown in Table 1.

Table 1. Remarks and proposal for required changes in 38.101-5

| 9 | Radiated transmitter characteristics |  |
| --- | --- | --- |
| 9.1 | General | Should be numerology independent |
| 9.2 | Transmitter power | Is numerology independent as agreed in WF |
| 9.3 | Output power dynamics | Reuse the time masks and transient period times at least for FR2 numerology - Study if it also applies to FR1 numerology to maintain this section of the specification generic. Focus especially for narrower CBWs. Transient times and durations are not a function of SCS, but bandwidth of the transmitted signal. - Include the 15kHz and 30kHz PRACH preamble formats and measurement period to Table 9.3.3.4-1: PRACH ON power measurement period |
| 9.4 | Transmitter signal quality | Is numerology independent, already part of WF |
| 9.5 | Output RF spectrum emission | Needs to be updated due to the new channel bandwidth at least as agreed in WF In addition: - 9.5.2.2.1 General NR spectrum emission mask and 9.5.2.2.2 Additional spectrum emission mask should be numerology agnostic - 9.5.3.2 On-axis spurious requirement and 9.5.3.3 Off-axis spurious requirement should be numerology agnostic |
| 9.6 | Antenna pointing accuracy and performance | Should be numerology independent |
| 9.7 | Additional regional requirements indicated by NS | Should be numerology independent |
| 10 | Radiated receiver characteristics |  |
| 10.1 | General | Should be numerology independent |
| 10.2 | Polarization characteristics | Should be numerology independent |
| 10.3 | OTA reference sensitivity level | Needs to be updated due to the new channel bandwidth at least as agreed in WF In addition: - Define the applicable reference measurement channels - New bands to be introduced to Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT - Use of vendor declared EISREFSENS_50MHz parameter might cause confusion for FR1 numerology bandwidths that are narrower than 50MHz although the actual requirement still works. - Clarification and new “factor” parameter for Table 10.3.2-1: OTA reference sensitivity requirement for NTN VSAT to be considered - Define REFSENS for Ku-bands. There should not be differences between the requirements originating from numerology apart from different bandwidth(s) and SNR delta(s) of used reference measurement channel. |
| 10.4 | Maximum input level | Needs to be updated due to the new channel bandwidth at least as agreed in WF In addition: - Define the applicable reference measurement channels for Table 10.4.2-1: Maximum input level (Mobile VSAT) - Define Maximum input level for Ku-bands. There should not be differences between the requirements originating from numerology apart from different bandwidth(s) and SNR delta(s) of used reference measurement channel. |
| 10.5 | ACS | Needs to be updated due to the new channel bandwidth at least as agreed in WF In addition: - Define the applicable reference measurement channels - New bands and ACS requirement to be introduced to Table 10.5.1-1: Adjacent channel selectivity (Mobile VSAT) - There should not be differences between the requirements originating from numerology apart from different bandwidth(s) and SNR delta(s) of used reference measurement channel. - Use of vendor declared EISREFSENS_50MHz parameter might cause confusion for FR1 numerology bandwidths that are narrower than 50MHz although the actual requirement still works. - Clarification and new “factor” parameter for Table 10.5.1-2: Adjacent channel selectivity test parameters for NTN VSAT to be considered |
| 10.6 | blocking | Needs to be updated due to the new channel bandwidth at least as agreed in WF In addition: - Define the applicable reference measurement channels - New bands and ACS requirement to be introduced to Table 10.6.2-1: In band blocking requirements (Mobile VSAT) - There should not be differences between the requirements originating from numerology apart from different bandwidth(s) and SNR delta(s) of used reference measurement channel. - Use of vendor declared EISREFSENS_50MHz parameter might cause confusion for FR1 numerology bandwidths that are narrower than 50MHz although the actual requirement still works. Clarification and new “factor” parameter for Table 10.6.2-1: In band blocking requirements for NTN VSAT to be considered |
| 10.7 | Spurious emissions | Should be numerology independent |
| 10.8 | Receiver antenna off-axis performance | Should be numerology independent |

## Minimum EIRP requirements

The WF agreed the following:

- Companies are encouraged to provide views on the minimum peak EIRP requirement for the NTN Ku band in the next meeting.

The link budget calculation for 30- and 90-degrees elevation for Ku-band GSO orbit is shown in Table 1.

Table 2. Link budget calculation (Ku-GSO)

|  | Ku-band |  |
| --- | --- | --- |
| Satellite type | GSO | GSO |
| Elevation angle (VSAT) (deg) | 30 | 90 |
| Distance (VSAT-SAN) (km) | 38612 | 35786 |
| UL frequency (MHz) | 14500 | 14500 |
| FSPL (dB) | 207.4 | 206.7 |
| Atmosphere loss (dB) | 2 | 2 |
| BW (MHz) | 100 | 100 |
| target SNR | -1 | -1 |
| Noise figure (dB) (SAN) | 3 | 3 |
| SAN Rx gain (dBi) | 55.5 | 58.5 |
| Implementation margin | 2 | 2 |
| minimum EIRP (dBm) | 63.9 | 60.2 |

Proposal 4: Consider minimum peak EIRP of [64] dBm for Ku-band NTN VSAT type 4 and 5.

# Conclusions

Observation 1: Based on the agreed starting point and the discussions in the RAN4 #114 meeting these type 4 and 5 devices MUST be able to communicate with BOTH GSO and LEO satellites, which is indicated with word and “with GSO and LEO” in type description.

Proposal 1: In order to define clear minimum performance requirements for Ku-band VSAT RAN4 should decide whether to define set of RF requirements as per satellite orbit, i.e. different RF requirements depending on SAN orbit or set of requirements that are agnostic of the SAN orbit.

Observation 2: The existing requirements for Ka-band NTN VSAT device types 1 and 2 (Fixed VSAT communicating with GSO and LEO) defines the orbit agnostic requirements. The orbit is only considered for additional regional requirements when required by regulation.

Proposal 2: Follow the approach selected for Ka-band and define set of requirements that are agnostic of the SAN orbit. Consider the orbit for additional regional requirements only if/when required by regulation.

Proposal 3: Shown in Table 1.

Proposal 4: Consider minimum peak EIRP of [64] dBm for Ku-band NTN VSAT type 4 and 5.

# References

[1] R4-2502288, WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL), TSG-RAN WG4 Meeting #114, Feb 2025

[2] 3GPP TS 38.101-5, User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 18)