3GPP TSG-RAN WG4 Meeting #114bis                                                           R4-2504220

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:	On the modification of existing TX requirements for SBFD-capable BS

Agenda item:			7.23.2.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1.

Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”, and this work shall be based on the good outcome from the study, i.e., Section 10.1, “Impact on BS RF requirements” in TR 38.858. Furthermore, the way forward encourages companies to continue contributing to ongoing discussions on (1) the modification of existing requirements for SBFD-capable BS and (2) potentially new requirement for SBFD operation, and address issues that have been raised during meetings [4]. In the RAN4#111, #112 and #112bis, further agreements are achieved on the modification of existing requirements for SBFD-capable BS [6] [8] [10] and [12]. In this contribution, accordingly, we would continue to provide our analysis and viewpoints on the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS.

# 2. Modification on Existing TX requirement for SBFD

## 2.1 Base Station output power and radiated transmit power

For the requirement of BS output power and radiated transmit power, the following agreements is achieved in the study phase [2]:

Since configuration (e.g. antenna, power configuration etc) between SBFD and non-SBFD symbols/slots might be different, RAN4 reached the following consensus for the BS RF requirement of BS output power for both conducted and OTA output power: - It is allowed to have the different conducted declaration for normal DL symbols/slots and SBFD DL symbols/slots. - It is allowed to have different EIRP/TRP declaration (for level and direction) for normal DL symbols/slots and SBFD DL symbols/slots. - Accuracy requirement for TRP/EIRP and conducted power shall be the same for normal DL symbols/slots and SBFD DL symbols/slots.

And the following agreement is achieved in RAN4#111 [6]:

Issue 2-1-1: PSD scaling for normal and SBFD slots/symbols - Agreement: - PSD scaling for normal and SBFD slots/symbols: - No need to introduce the restriction on PSD scaling for normal and SBFD slots/symbols - Vendors can declare different TX power values for normal and SBFD slots/symbols

Based on the above agreements achieved in study item and RAN4#111, we expect the manufacturer declaration-based method will be used for base station output power, i.e., radiated transmit power and OTA base station output power, and further discussion could be expected on the specification implementation by allowing different declarations of output power for normal DL symbols and SBFD DL symbols. Based on our accompanying contribution on general aspects for SBFD BS RF requirement, RAN4 take the new chapter for SBFD-capable BS in TS38.104.

Taking the conducted requirement 6.2B “Base station output power for SBFD” as example, we see the following text proposal can be given for the corresponding requirement for SBFD operation. Different from last meeting’s proposal, we have updated the text proposal by removing the BS type 1-C part, according to the agreement from RAN4#113, i.e., “For SBFD, BS type 1-C is precluded.”

Proposal 1: The following text proposal is endorsed for BS output power requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

## 12.3.2 Base station output power for SBFD

### 12.3.2.1	General

The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1.

### 12.3.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation.

=================== End of Text Proposal ===================

## 2.2 Output power dynamics

In the Rel-18 study item, the following conclusion has been reached for output power dynamics:

#### 10.1.2.2 Output power dynamics Regarding the output power dynamic requirement, which mainly consists of RE power control dynamic range requirement and total dynamic range requirement, RAN4 reached the following consensus: - To reuse the existing RE power control dynamic range requirement for SBFD BS; - The total dynamic range requirement is applicable for SBFD-capable BS during normal DL symbols/slots, it is agreed to define the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Based on the agreement from RAN4#113, the requirement of total dynamic range can be defined as [10], by considering the different definition for DU/UD and DUD case respectively:

Agreement: - The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

Accordingly, we further propose to setting up the new sub-clauses 6.3.3B (Total power dynamic range for SBFD) and 9.4.3B (OTA total power dynamic range for SBFD) by defining the output power dynamic range requirement for SBFD as the ratio of the declared rated output power with all DL RBs active for SBFD (maximum) and the same single RB power as non-SBFD (minimum).

Proposal 2: The following text proposal is endorsed for BS output power dynamics requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.3.3	Total power dynamic range for SBFD

#### 12.3.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

#### 12.3.3.2	Minimum requirement for BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

=================== End of Text Proposal ===================

## 2.3 Transmit OFF power

For the requirement of transmit ON/OFF power, the following agreement is provided in the study item:

Regarding the transmitter ON/OFF power requirement, RAN4 mainly focus on the ON-OFF time mask and concluded that transmit ON/OFF power requirement is not applicable within SBFD time slot.

For the study item conclusion that “transmit ON/OFF power requirement is not applicable within SBFD time slot”, we expect there are two options for specification implementation: (1) adding Notes in both clause 6.4.1.1 and 9.5.2.1; (2) adding new sub-clauses 6.4.1B and 9.5.2B for SBFD operation. Among these two options, “adding notes in both clause 6.4.1.1 and 9.5.2.1” is preferred by us for its simplicity.

Proposal 3: To implement agreements, the following Note can be adopted to clarify OFF power requirement shall not be applied for SBFD symbols, i.e.,

- “For SBFD operation, transmitter OFF power requirements shall not be applied in SBFD symbols.”

Proposal 4: The following text proposal is endorsed for transmit OFF power (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.3.4	Transmit ON/OFF power

### 12.3.4.1	Transmitter OFF power

#### 12.3.4.1.1	General

For SBFD operation, transmitter OFF power requirements shall not be applied in SBFD symbols.

=================== End of Text Proposal ===================

## 2.4 Transmitted signal quality

For the requirement transmitted signal quality requirement, particularly for EVM measurement, the following bullet is agreed for separate measurement over SBFD and non-SBFD symbols:

Agreement: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots.

For the issue of EVM measurement method update for the Annex, symbols/slots, the following agreement is achieved:

Agreement: - FFS on the Proposal 3 from R4-2419221.

Furthermore, in RAN4#114, it is agreed that

WF: - Separate testing for SBFD slot and non-SBFD slot, with the consideration to try to adopt slot-level for the test granularity.

As provided in R4-2419221, the detailed averaging method in Annex B.7 shall be updated as below:

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl_symbol symbols where Ndl_symbol is the number of dl symbols in a 10 ms measurement interval. For TDD, let Ndl_symbolsTDD be the number of downlink SBFD symbols or non-SBFD symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from Ndl_symbolsTDD symbols of different 10 ms measurement intervals and should have a minimum of Ndl_symbol symbols averaging length where Ndl_symbol is the number of symbols in a 10 ms measurement interval. - EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS). EVMframe=1i=1Ndl_symbolTDDNii=1Ndl_symbolTDDj=1NiEVMi,j2 - Where Ni is the number of resource blocks with the considered modulation scheme in symbol i. - The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2). EVMframe=maxEVMframe,l,EVMframe,h - In order to unite at least Ndl_symbol slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by. Nframe=10×Nsymbol Ndl_symbolsTDD and Nsymbol=14 for 15 kHz SCS, Nsymbol=28 for 30 kHz SCS and Nsymbol=56 for 60 kHz SCS normal CP. - Unite by RMS. EVM=1Nframek=1NframeEVMframe,k2

Proposal 5: Text proposal from R4-2419221 could be used as baseline, while considering the following aspects:

- No impact on existing FDD/TDD BS EVM measurement, in which it is still slot-based;

- Ni should be the number of resource blocks in DL subband with the considered modulation scheme in symbol i if the symbol is SBFD symbol.

- Editorial errors should be corrected such as “Ndl_symbol slots”,

## 2.5 Unwanted emissions

For the ACLR requirement of unwanted emissions, the following agreements is achieved in the study phase [2]:

- For ACLR requirement, it shall be defined outside of the whole carrier instead of sub-band for SBFD DL symbols/slots and ACLR requirement is still defined as the ratio of sum of TX power within the whole carrier to the adjacent carrier.

In RAN4#111, the following agreement is achieved:

- Agreement: - RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement. - Agreement: - Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

By separating the discussion on ACLR requirement by (1) coexisting with legacy TDD system in adjacent channel and (2) coexisting with new SBFD system in adjacent channel, the following agreement and way forward is achieved in RAN4#112 and #113:

## <from RAN4#112> Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS. ## <from RAN4#113> Agreement: - FFS whether/how to introduce the additional ACLR and/or OBUE requirements for FR1 WA BS.

Especially for the scenario in which SBFD BS coexisting with new SBFD BS in the adjacent channel, RAN4 has not yet achieved further progress in RAN4#112bis and #113.

Based on our understanding, for the scenario of SBFD BS coexisting with new SBFD BS in the adjacent channel, although it is not explicitly analyzed in the Rel-18 study phase, the situation could be the same as Case 3 (the aggressor as NR TDD DL and victim as SBFD DU). The Case 3 results can be applied for the SBFD-SBFD coexistence scenario due to the assumption of flat adjacent channel interference modelling.

Observation 1: For the scenario of SBFD-SBFD co-existence in the adjacent channel, the Case 3 results from Rel-18 co-existence study can be applied due to the assumption of flat adjacent channel interference modelling.

For the unwanted emission requirement for BS coexisting with new SBFD BS in the adjacent channel, it is FFS that RAN4 shall define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. Based on the technical analysis in [R4-2413283], it is proposed that the new OBUE requirement can be defined with tightened emission mask. Similarly, the ACLR requirement when coexisting with new SBFD BS in the adjacent channel is another option. Introducing either of new tightened requirements can be regarded as the requirement of implementing TX filtering before band filter. By investigating the victim BS DESENS, we propose to reuse the 1dB DESENS assumption used in Rel-18.

Proposal 6: RAN4 shall restrict the discussion of the additional ACLR and/or OBUE requirements for FR1 macro BS only.

Proposal 7: The support of additional ACLR and/or OBUE requirements for FR1 macro BS is manufacture declaration based, for certain gNB.

If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

Proposal 8: If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

To ensure feasible coexistence with SBFD-capable gNB, the assumption of 1dB DESENS in the victim SBFD receiver and certain level MCL (e.g., 67dB MCL) can be used as the starting point.

Proposal 9: If the method of new OBUE requirement is considered, the assumption of 1dB DESENS in the victim SBFD receiver and certain level MCL (e.g., 67dB MCL) can be used as the starting point.

## 2.6 Transmitter intermodulation

Based on last meeting’s agreement, there will be no receiver requirement specified for transmitter intermodulation requirement applicable in SBFD symbols, i.e.,

Issue 2-2-1: TX intermodulation requirement - Agreement: - The transmitter intermodulation requirement is applicable in SBFD slots/symbols: - Follow the existing requirement - No receiver requirement is specified.

To implement the above agreement, we have the following simple proposals:

Proposal 10: For transmitter intermodulation requirement, RAN4 can specify a new sub-clause 12.3.7 (Transmitter intermodulation) and a new sub-clause 12.6.8 (OTA transmitter intermodulation), by simply explain that the existing requirement shall be applied while no receiver requirement is specified for SBFD symbols.

Proposal 11: The following text proposal is adopted for transmitter intermodulation requirement (taking the conducted requirement as example)):

=================== Start of Text Proposal ===================

## 12.3.7	Transmitter intermodulation

The transmitter intermodulation requirement for SBFD is specified over SBFD symbols. For the transmitter intermodulation requirement for SBFD, the same requirement as specified in clause 6.7 shall be applied.

=================== End of Text Proposal ===================

# 3. Conclusion

In this contribution, we provided the viewpoints on the modification of existing TX requirements for FR1 and FR2-1 SBFD-capable BS, accordingly with the following observations and proposals obtained:

Proposal 1: The following text proposal is endorsed for BS output power requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

## 12.3.2 Base station output power for SBFD

### 12.3.2.1	General

The BS conducted output power requirement for SBFD operation is specified over SBFD symbols, in which the rated carrier output power for SBFD could be different from the rated carrier output powers for normal symbols as specified in clause 6.2.1.

### 12.3.2.2	Minimum requirement for BS type 1-H

The same requirement as clause 6.2.3 is applicable to BS output power requirement for SBFD operation.

=================== End of Text Proposal ===================

Proposal 2: The following text proposal is endorsed for BS output power dynamics requirement (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.3.3	Total power dynamic range for SBFD

#### 12.3.3.1	General

The BS total power dynamic range for SBFD is the difference between the maximum and the minimum transmit power of an SBFD symbol for a specified reference condition.

For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band.

NOTE 1:	The upper limit of the total power dynamic range for SBFD is the OFDM symbol power for a BS when transmitting on all RBs in transmission bandwidth configuration for SBFD DL subband(s) at maximum output power. The lower limit of the total power dynamic range for SBFD is the average power for single RB transmission. The OFDM symbol shall carry PDSCH and not contain RS or SSB.

NOTE 2:	The requirement does not apply to operation with shared spectrum channel access.

#### 12.3.3.2	Minimum requirement for BS type 1-H

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level calculated as 10log(NRB,SBFD,DL) if one SBFD DL subband is configured, and as 10log(NRB,SBFD,DL,1 + NRB,SBFD,DL,2) if two SBFD DL subbands are configured.

=================== End of Text Proposal ===================

Proposal 3: To implement agreements, the following Note can be adopted to clarify OFF power requirement shall not be applied for SBFD symbols, i.e.,

- “For SBFD operation, transmitter OFF power requirements shall not be applied in SBFD symbols.”

Proposal 4: The following text proposal is endorsed for transmit OFF power (taking the conducted requirement as example):

=================== Start of Text Proposal ===================

### 12.3.4	Transmit ON/OFF power

### 12.3.4.1	Transmitter OFF power

#### 12.3.4.1.1	General

For SBFD operation, transmitter OFF power requirements shall not be applied in SBFD symbols.

=================== End of Text Proposal ===================

Proposal 5: Text proposal from R4-2419221 could be used as baseline, while considering the following aspects:

- No impact on existing FDD/TDD BS EVM measurement, in which it is still slot-based;

- Ni should be the number of resource blocks in DL subband with the considered modulation scheme in symbol i if the symbol is SBFD symbol.

- Editorial errors should be corrected such as “Ndl_symbol slots”,

Observation 1: For the scenario of SBFD-SBFD co-existence in the adjacent channel, the Case 3 results from Rel-18 co-existence study can be applied due to the assumption of flat adjacent channel interference modelling.

Proposal 6: RAN4 shall restrict the discussion of the additional ACLR and/or OBUE requirements for FR1 macro BS only.

Proposal 7: The support of additional ACLR and/or OBUE requirements for FR1 macro BS is manufacture declaration based, for certain gNB.

Proposal 8: If the method of new OBUE requirement is considered, the BS RF Bandwidth edge should be based on the declared SBFD DL subband and the frequency offset of measurement shall be determined by considering the practical co-existence scenario.

Proposal 9: If the method of new OBUE requirement is considered, the assumption of 1dB DESENS in the victim SBFD receiver and certain level MCL (e.g., 67dB MCL) can be used as the starting point.

Proposal 10: For transmitter intermodulation requirement, RAN4 can specify a new sub-clause 12.3.7 (Transmitter intermodulation) and a new sub-clause 12.6.8 (OTA transmitter intermodulation), by simply explain that the existing requirement shall be applied while no receiver requirement is specified for SBFD symbols.

Proposal 11: The following text proposal is adopted for transmitter intermodulation requirement (taking the conducted requirement as example)):

=================== Start of Text Proposal ===================

## 12.3.7	Transmitter intermodulation

The transmitter intermodulation requirement for SBFD is specified over SBFD symbols. For the transmitter intermodulation requirement for SBFD, the same requirement as specified in clause 6.7 shall be applied.

=================== End of Text Proposal ===================

# 4. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2405655, “Discussion on BS RF requirements for SBFD-capable BS”, Samsung, RAN4#110bis.

[4] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[5] R4-2409764, “On the modification of existing requirements for FR1 and FR2-1 SBFD-capable BS”, Samsung, RAN4#111.

[6] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[7] R4-2411638, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#112.

[8] R4-2413515, “Way Forward for [112][308] NR_duplex_evo_BSRF”, Huawei, RAN4#112.

[9] R4-2416499, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#112bis.

[10] R4-2416585, “Way Forward for [112bis][307] NR_duplex_evo_BSRF”, Huawei, RAN4#112bis.

[9] R4-2418099, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#113.

[10] R4-2419908, “Way Forward for [113][307] NR_duplex_evo_BSRF”, Huawei, RAN4#113.

[11] R4-2501969, “On the modification of existing TX requirements for SBFD-capable BS”, Samsung, RAN4#114.

[12] R4-2502380, “Way Forward for [114][307] NR_duplex_evo_BSRF”, Huawei, RAN4#114.