# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504223

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.26.4.1

Source: 	Ericsson

Title:	Simulation assumption on LP-WUS RRM measurement

Document for:	Discussion

- Simulation setting

In last meeting, companies proposed initial simulation results, but it seems the results are not aligned very well. We suggest companies to first align the understanding about the simulation chain design and how to achieve the ideal results before aligning the simulation results.

The signal flow to calculate the RSRP is as follow.

Figure 1. The RSRP estimation flow

In our understanding, ideal LP-RSRP shall be obtained from the serving signal passing the channel in each sample. Similar to NR, to evaluate the LP-RSRP or LP-RSRQ accuracy, we need to compare the practical LP-RSRQ value with ideal. For the calculation of RSRP or RSRQ in NR, the receiver could make channel estimation, then calculate practical and ideal RSRP or RSRQ value respectively to evaluate RRM performance. However, in OOK-based receiver, channel estimation is probably skiped since no phase information after envelop detector. To measure practical LP-RSRP or LP-RSRQ, the signal used needs to consider the impact of noise, interference, channel model, timing error and frequency error, etc. Regarding to the ideal LP-RSRP, the signal used can be received ideal without the impact of timing error, frequency error, noise and interference. Considering the RSRQ reflects the signal quality, such as SINR in NR scenario that RSRQ somehow equals to the ratio of signal power to noise power. For LP-RSRQ, it can be considered as the ratio of signal power to total received power since LP-RSSI has been agreed as total power of ON and OFF symbols.  For the practical LP-RSRQ calculation, the practical receiving signal is used to calculate LP-RSRP and LP-RSSI. To evaluate ideal LP-RSRQ, ideal receiving sigal can be used to calculate LP-RSRP as above, but it can not used to calculate ideal LP-RSSI since LP-RSSI should carry the power of noise and interference.

Proposal 1: RAN4 to align the ideal results in simulation setting before aligning the simulation results.

- Ideal RSRP is derived from the serving cell signal after the fading channel

- Ideal RSRQ is derived based on ideal RSRP divides the calculated RSSI.

- Summary

In this contribution, we have initially discussed the LP-WUS RRM measurement simulation assumption. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to align the ideal results in simulation setting before aligning the simulation results.

- Ideal RSRP is derived from the serving cell signal after the fading channel

- Ideal RSRQ is derived based on ideal RSRP divides the calculated RSSI.

- Reference

[1]. R4-2502600, “WF for RRM requirements for LP-WUS/WUR”, vivo