# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2504226

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.25.2.1

Source: 	Ericsson

Title:	Discussion on OD-SSB in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, the OD-SSB based deactivated SCell measurement were agreed.

- Sub-issue 1-1 – The length of FMW Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify). - Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs? Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW.

In this contribution, we continue to discuss on the topic on OD-SSB operation of Rel-19 NES WI from RAN4 perspective.

- OD-SSB based deactivated SCell measurement

## 2.1 Case 1: No always-on SSB on the cell

In last meeting, RAN4 made the following agreements about OD-SSB based deactivated SCell measurement in Case 1. The OD-SSB based fast L3 measurement framework has agreed, but some details are still FFS.

OD-SSB based deactivated SCell measurement requirement

In last meeting, RAN4 discussed the OD-SSB based deactivated SCell measurement requirement in fast mode as follow.

Sub-issue 3-1: Lower bound for measurement in FMW Wayforward: - Option 1: RAN4 not to define lower bound to measurement in FMW. - Option 2: Define a lower bound with OD-SSB periodicity as [10ms]. - The lower bound is only applied to measurement in FMW. Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers Agreement: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW - Option 1: The CSSF only counts OD-SSB layers - Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers

To maximize the NES gain, NW is expected to extend the muting duration before SCell activation as much as possible. When NW indicates the OD-SSB transmission for deactivated Scell measurement, UE shall follow the OD-SSB periodicity to perform measurement, thus no need to include a lower bound in the requirement. The OD-SSB based deactivated Scell measurement delay could be ceil(5 x Kp) x OD-SSB periodicity.

Proposal 1: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, the CSSF only counts the number of activated OD-SSB in the FMW.

Proposal 2: Lower bound is not needed for measurement within the FMW.

Proposal 3: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, the OD-SSB based deactivated Scell measurement delay could be ceil(5 x Kp) x OD-SSB periodicity.

Multiple Layers measurement

When NW configures different measurement objects for multiple frequency layers’ meaurements, the following scenarios need to be considered.

Scenario 1: One legacy measurement object with AO-SSB and one OD-SSB SCell measurement

As we mentioned before, to maximize both the NW and UE’s power saving gain, the duration of fast window shall be as short as possible. Thus, UE shall prioritize the OD-SSB based SCell’s measurement and deprioritize the legacy SCell’s measurement. The CSSF of the OD-SSB equals to 1. Longer delay of legacy SCell is expected.

Figure 1. Legacy measurement object with fast measurement window of OD-SSB based deactivated SCell

Observation 1: When OD-SSB based deactivated SCell measurement colliding with the frequency layer with AO-SSB, the CSSF of OD-SSB equals 1, longer delay is expected for the frequency layer being measured.

Scenario 2: Two OD-SSB SCells measurement with non-overlapping fast window

When considering multiple OD-SSB SCells, one of the typical scenarios is NW configures multiple OD-SSBs with non-overlapping. For example, NW indicates OD-SSB transmission for SCell 1 and no transmission for other SCells. After UE finishing the first SCell’s OD-SSB measurement, NW deactivates the OD-SSB for SCell 1 and activates the OD-SSB for SCell 2. In our understanding, the CSSF for each OD-SSB SCell should still be 1 if the OD-SSB fast windows are non-overlapping among the OD-SSB SCells.

Figure 2. Non-overlapping Fast Measurement Windows of OD-SSB based deactivated SCell

Observation 2: When two OD-SSB based deactivated SCell measurement fast windows are non-overlapping, the CSSF for each OD-SSB based SCell equals 1.

Scenario 3: Two OD-SSB SCells measurement with overlapping fast window

However, when the OD-SSBs’ fast window are overlapping, UE had to perform measurement for these OD-SSBs in parallel. For example, the CSSF equals 2 in the following figures.

Figure 3. Overlapping Fast Measurement Windows of OD-SSB based deactivated SCell

Observation 3: When two OD-SSB based deactivated SCell measurement fast windows are overlapping, the CSSF for each OD-SSB based SCell equals 2.

Proposal 4: Longer measurement delay of other frequency layers with AO-SSBs only are expected within the OD-SSBs FMW.

Proposal 5: The CSSF of OD-SSB based deactivated SCell shall only account the number of frequency layers with activated OD-SSBs within its fast measurement window.

- The CSSF equals 1 if non-overlapping with other OD-SSBs transmission during the fast measurement window.

- The CSSF equals N+1 if overlapping with N other layres’ OD-SSB fast measurement window.

T2 start point and duration

In last meeting, RAN4 spent lots of time to discuss UE’s behaviour when NW deactivated the OD-SSB and activates the OD-SSB again. The agreement is as follow.

Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs? Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. - Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW.

In our understanding, when UE finished the first deactivated Scell measurement and reporting, UE is allowed to fallback to the legacy deactivated measurement considering the UE power consumption. If NW decides to activate the Scell fast, NW can indicate the Scell activation command immediately when NW receives the deactivated measurement report.

Some companies proposed to define a minimum time interval T2 which means NW doesn’t need to re-activate the OD-SSB within T2 since UE can still believe the SCell as a known cell if UE has reported the measurement report before.

In our understanding, the intention of T2 is to protect UE to achieve power saving gain in deactivated SCell to avoid perform fast OD-SSB measurement frequenctly. To facilitate both NW and UE to understand when to trigger the OD-SSB reactivation, T2 should decouple with any measurement report to avoid dynamic change.

Observation 4: From NW perspective, OD-SSB deactivation time occasion aligns with the end of FMW(Tend) based on a reasonable NW design.

Observation 5: The intention of defining T2 is to guarantee UE can achieve power saving when NW enters power saving mode.

Proposal 6: The start point of T2 is defined as UE receiving the OD-SSB deactivation command.

Proposal 7: The duration of T2 equals to 5*measCycleSCell.

In our understanding, a reasonable NW behaviour is not to activate the OD-SSB again within T2 since it violates the NES princinple to introduce the OD-SSB. When NW re-activates the OD-SSB within T2, UE only needs to follow legacy deactivated SCell measurement requirement without cell identification. However, from NW’s perspective, it is expected UE to perform FMW once NW triggers the OD-SSB transmission. A possible compromise solution is to define a capability for UE’s behaviour within T2.

## 2.2 Case 2: With always-on SSB on the cell

In last meeting, RAN4 discussed OD-SSB based deactivated SCell measurement in Case 2. We proposed to follow the OD-SSB based deactivated SCell measurement procedure in Case 1.

Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

At the same time, there is another solution which is to apply the Case 1 approach only in unknown SCell condition.  UE should follow the legacy requirement if the serving cell is already known. However, if following such approach, it implies OD-SSB transmission before SCell activation is useless which is contradicted with RAN1 agreement to introduce such scenario 2 case 2. Thus, we propose to follow the same approach in case 1 to apply the fast mode when NW indicates the OD-SSB first time and after T2.

Proposal 8: In Case 2, the same FMW approach as Case 1 can be reused.

- Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within FMW. After the FMW, the UE shall only meet the legacy deactivated SCell measurement requirement.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW.

Scenario

In RAN4 #112b meeting, RAN4 sent LS to RAN1 to check the following scenarios.

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement.

In last RAN4 meeting, RAN4 agreed the following scenarios.

Agreement: RAN4 to start from the following scenarios: - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity - The scenarios can be updated based on other WGs’ progress.

In last RAN1 meeting, RAN1 further replied RAN4 LS to clarify the valid sceanarios between always-on SSB and OD-SSB as follow[2].

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted

Alt Time-C1

In last meeting, RAN4 agreed the measurement delay requirement for scenario 4 as follow.

Issue 2-3: Combine OD-SSB and AO-SSB in measurement Agreement: - In scenario 4, the measurement requirement is defined based on the effective measurement periodicity. - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and - The effective measurement periodicity is half of OD-SSB periodicity - AO-SSB and OD-SSB are on the same frequency with the same spatial relation

However, considering the scenario Alt Time-C1 in RAN1 LS, RAN4 needs to update the agreement to apply to consider all possible cases once the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern supported in legacy specification. In other words, RAN4 needs to merge the scenario 1 and 4 together to define the requirement.

Figure 4. OD-SSB and AO-SSB transmission in scenario Alt Time-C1

Proposal 9: In Case 2, RAN4 to update the agreement of effective measurement periodicity(EMP) to RAN1 Alt Time-C1 scenario as follow.

- The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification

Alt Time-C2

At the same time, RAN1 also introduced a new scenario Alt Time-C2 where the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern. In our understanding, some UEs may have issue to combine both OD-SSB and AO-SSB to perform measurement and SCell activation in this case. RAN4 can follow the basic princinple to only use OD-SSB to perform measurement in FMW. Furthermore, a capability can be introduced if some UEs can support to combine both OD-SSB and AO-SSB to perform the measurement in FMW.

Figure 5. OD-SSB and AO-SSB transmission in scenario Alt Time-C2

Proposal 10: In Case 2 Alt Time C2, UE follows the OD-SSB periodicity to perform measurement in FMW as baseline. Optionally, a new capability is introduced for the UEs which can support to perform measurement based on both AO-SSB and OD-SSB in FMW.

Scenario 3

In last RAN1 meeting, RAN1 also agreed to introduce a new scenario in which the frequency location of OD-SSB is different from the frequency location of AO-SSB.

Figure 6. OD-SSB and AO-SSB transmission in scenario 3

In our understanding, when NW indicates the OD-SSB transmission, UE shall at least monitor OD-SSB for serving cell measurement and/or SCell activation. However, it’s unclear how to handle the AO-SSB frequency. Whether UE needs to monitor both frequencies, whether needs to perform neighbour cell measurements in both frequencies, or even which frequency or both can be defined as intra-frequency is unclear. Therefore, we suggest to send LS to RAN2 to clarify the OD-SSB measurement configuration in this scenario. The detail questions can be discussed during the meeting.

Proposal 11: When the frequency location of OD-SSB is different with AO-SSB, UE shall perform serving cell measurement in FMW and/or SCell activation based on OD-SSB once NW indicates OD-SSB.

Proposal 12: RAN4 to send LS to RAN2 to ask the serving cell MO and OD-SSB measurement configuration’s design when the frequency location of OD-SSB is different with AO-SSB but in the same BWP.

- OD-SSB based SCell activation

In last meeting, RAN4 agreed to follow the legacy known/unknown Scell activation princinples. Based on current known condition in SCell activation, the UE needs to send a valid L3 measurement report (MR) for the SCell being activated. Thus, during the time period where the Scell is deactivated, UE needs to perform deactivated SCell measurement. After NW receiving the L3 MR, the SCell can be believed as a known SCell. When NW indicates the SCell activation command, UE is assumed to perform SCell activation based on known SCell requirement. RAN4 needs to discuss the impacts due to OD-SSB for these Scell activation requriements.

OD-SSB uncertainty

The SCell activation requirement is based on OD-SSB periodicity if OD-SSB activation indication is no later than SCell activation command.

Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

The remaining issue is how to update the uncertainty time in the SCell activation requirement. Currently, in legacy requirement, TFirstSSB, TFirstSSB_MAX are introduced to define the uncertainty to receive the first complete SSB burst after HARQ processing and MAC-CE parsing time. We think a similar uncertainty shall also be introduced in OD-SSB based SCell activation. UE needs to wait for the first complete OD-SSB burst after HARQ processing and MAC-CE parsing time. RAN4 to introduce TFirstOD-SSB, TFirstOD-SSB_MAX  to the end of the first actual transmitted complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX. TFirstOD-SSB_MAX  shall be based on max SSB periodicity among active serving cells other than SMTC.

Proposal 13: RAN4 to introduce the SSB transmission uncertainty duration TFirstOD-SSB, TFirstOD-SSB_MAX which is the time to the end of the first actual transmitted complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX.

- TFirstOD-SSB_MAX is based on max SSB periodicity other than SMTC.

known cell condition

In RAN4 #113 meeting, there is an agreement about the known cell condition as follow.

Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

Currently, the known cell condition is as follow.

FR1 SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. FR2 For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Currently, the known cell condition is assumed the same SSB is used to perform measurement and Scell activation. Considering OD-SSB, the known cell condition shall be updated based on different OD-SSB and AO-SSB relations.

- Case 1: Deactivated SCell Measurement – AO-SSB; SCell activation-OD-SSB

- Case 2: Deactivated SCell Measurement – AO-SSB+OD-SSB; SCell activation-OD-SSB

- Case 3: Deactivated SCell Measurement – OD-SSB; SCell activation-AO-SSB

Observation 6: The measured SSB and the SSB used for SCell activation can be different.

In RAN1 reply LS, the spatial relation is the same between AO-SSB and OD-SSB.

Thus, after UE performs measurement based on any type of SSB, the cell shall be believed as a known cell even if the SSB used in SCell activation is different with the SSB in measurement.

Proposal 14: The OD-SSB based known cell condition shall be updated to clarify the measured SSB and the SSB used in SCell activation can be different.

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation

- The SSB used in SCell activation have the same SSB index as the SSB measured which can be detectable

SSB-less SCell scenario

RAN4 also introduced the SSB-less SCell activation requirement for intra-band and inter-band collocated scenario. UE follows the SCell activation command to activate the SCell without SSB. In our understanding, UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation phase. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 15: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

- Others

L1 measurement after Scell activation

In last meeting, RAN4 made the agreement on L1 measurement before and during SCell activation procedure. There is an open issue about OD-SSB based L1 measurement after SCell activation.

Issue 4-1: L1 measurement before SCell activation Agreement: - RAN4 not to support OD-SSB based L1 measurements for deactivated Scell. Issue 4-2: L1 measurement after SCell activation Agreement: - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. - Based on OD-SSB periodicity Wayforward: - RAN4 to support OD-SSB L1 measurement after SCell activation. - FFS the requirement when DRX is configured. - Option 1: Follow legacy, based on DRX cycle (vivo, Apple, OPPO, Xiaomi) - Option 2: Based on OD-SSB periodicity (Nokia, ZTE, Samsung) - Option 3: FFS (HW, QC, Nokia, CMCC, Xiaomi, CATT) - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

In our understanding, after SCell activation, OD-SSB shall be always transmitted as a normal AO-SSB in Case 1. In case 2, NW can only transmit the AO-SSB and deactivate the OD-SSB to achieve the power saving gain. When NW wants UE to speed up the measurement, NW doesn’t need to configure the DRX at the same time. Thus, when DRX is configured, RAN4 to support OD-SSB L1 measurement based on DRX cycle in current agreed RAN1 scenarios 2/2a. There is one remaining issue in RAN1 about the scenario 3b, RAN4 can wait RAN1’s further progress.

Proposal 16: When NW configures DRX cycle, OD-SSB L1 measurement after SCell activation shall base on DRX cycle in scenario 2/2a.

Scheduling restriction for L1 measurement after SCell activation

As mentioned in section 3.2 case 2, UE may only perform measurement on OD-SSB in some scenarios, such as Alt Time C-2. In such scenario, no scheduling restriction and measurement restriction is expected for L1 measurement by AO-SSB.

Proposal 17: In Case 2 L1-RSRP measurement, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

Neighbour cell measurement

In last meeting, RAN4 discussed the neighbour cell measurement requirement. The agreements are as follow.

Issue 7-2: OD-SSB based Neighbour cell measurement Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell Agreement: - For Scenario 1a: No RAN4 spec. change. Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2.

Some companies proposed to further discuss UE’s measurement behaviour on neighbour cells in FR2.

Proposal 18: UE shall follow legacy FR2 measurement requirement based on MeasCycleSCell for the neighbour cell measurements regardless of OD-SSB based neighbour cell exists or not.

Cell identification component

In last meetings, there is an open issue about whether UE needs to always perform cell identification to obtain coarse timing before deactivated Scell activation. The motivation to always require additional cell indentification is due to OD-SSB is not always transmitted.

Issue 2-1-6: OD-SSB based deactivated SCell measurement components - Proposals - Option 1: vivo - RAN4 to clarify the deactivated SCell measurement components includes both cell identification and measurement.

However, we think at least for the following cases UE doesn’t need to always perform cell identification. Firstly, for always-on SSB transmitted in case 2 means UE has performed the cell identification based on always-on SSB and doesn’t need to perform cell identification when OD-SSB is indicated. Secondly, in Case 1, if the OD-SSB SCell being measured is contiguous to an active serving cell in the same band, UE can refer on the active serving cell’s timing as legacy. No cell identification is needed. Thirdly, if UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is less than Xs and then the OD-SSB is transmitted again, cell identification is also unnecessary.

Proposal 19: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [X]s and then the OD-SSB is activated again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

Interruption

In last meeting, companies proposed to check the possible interruption requirements. We checked the interruption spec. and think two sections may need to further clarify the OD-SSB impact.

Issue 6-6: Interruption Open Issue: - RAN4 to investigate whether there is impact due to OD-SSB on the allowed interruptions during measurements on deactivated Scell.

Proposal 20: RAN4 to update the following interruption requirement based on OD-SSB periodicity.

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- Summary

In this contribution, we have discussed the OD-SSB requirements. Based on the discussions, we have made following proposals and observations:

Observation 1: When OD-SSB based deactivated SCell measurement colliding with the frequency layer with AO-SSB, the CSSF of OD-SSB equals 1, longer delay is expected for the frequency layer being measured.

Observation 2: When two OD-SSB based deactivated SCell measurement fast windows are non-overlapping, the CSSF for each OD-SSB based SCell equals 1.

Observation 3: When two OD-SSB based deactivated SCell measurement fast windows are overlapping, the CSSF for each OD-SSB based SCell equals 2.

Observation 4: From NW perspective, OD-SSB deactivation time occasion aligns with the end of FMW(Tend) based on a reasonable NW design.

Observation 5: The intention of defining T2 is to guarantee UE can achieve power saving when NW enters power saving mode.

Observation 6: The measured SSB and the SSB used for SCell activation can be different.

Proposal 1: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, the CSSF only counts the number of activated OD-SSB in the FMW.

Proposal 2: Lower bound is not needed for measurement within the FMW.

Proposal 3: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, the OD-SSB based deactivated Scell measurement delay could be ceil(5 x Kp) x OD-SSB periodicity.

Proposal 4: Longer measurement delay of other frequency layers with AO-SSBs only are expected within the OD-SSBs FMW.

Proposal 5: The CSSF of OD-SSB based deactivated SCell shall only account the number of frequency layers with activated OD-SSBs within its fast measurement window.

- The CSSF equals 1 if non-overlapping with other OD-SSBs transmission during the FMW.

- The CSSF equals N+1 if overlapping with N other layres’ OD-SSB FMW.

Proposal 6: The start point of T2 is defined as UE receiving the OD-SSB deactivation command.

Proposal 7: The duration of T2 equals to 5*measCycleSCell.

Proposal 8: In Case 2, the same FMW approach as Case 1 can be reused.

- Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within FMW. After the FMW, the UE shall only meet the legacy deactivated SCell measurement requirement.

- When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW.

Proposal 9: In Case 2, RAN4 to update the agreement of effective measurement periodicity(EMP) to RAN1 Alt Time-C1 scenario as follow.

- The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals legacy SSB periodicity defined in the specification

Proposal 10: In Case 2 Alt Time C2, UE follows the OD-SSB periodicity to perform measurement in FMW as baseline. Optionally, a new capability is introduced for the UEs which can support to perform measurement based on both AO-SSB and OD-SSB in FMW.

Proposal 11: When the frequency location of OD-SSB is different with AO-SSB, UE shall perform serving cell measurement in FMW and/or SCell activation based on OD-SSB once NW indicates OD-SSB.

Proposal 12: RAN4 to send LS to RAN2 to ask the serving cell MO and OD-SSB measurement configuration’s design when the frequency location of OD-SSB is different with AO-SSB but in the same BWP.

Proposal 13: RAN4 to introduce the SSB transmission uncertainty duration TFirstOD-SSB, TFirstOD-SSB_MAX which is the time to the end of the first actual transmitted complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX.

- TFirstOD-SSB_MAX is based on max SSB periodicity other than SMTC.

Proposal 14: The OD-SSB based known cell condition shall be updated to clarify the measured SSB and the SSB used in SCell activation can be different.

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation

- The SSB used in SCell activation have the same SSB index as the SSB measured which can be detectable

Proposal 15: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 16: When NW configures DRX cycle, OD-SSB L1 measurement after SCell activation shall base on DRX cycle in scenario 2/2a.

Proposal 17: In Case 2 L1-RSRP measurement, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement.

Proposal 18: UE shall follow legacy FR2 measurement requirement based on MeasCycleSCell for the neighbour cell measurements regardless of OD-SSB based neighbour cell exists or not.

Proposal 19: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [X]s and then the OD-SSB is activated again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

Proposal 20: RAN4 to update the following interruption requirement based on OD-SSB periodicity.

- Interruptions at SCell addition/release

- Interruptions at SCell activation/deactivation

- References

- R4-2502598, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

- R1-2501633, “LS on time location of on-demand SSB for SCell”, LG