3GPP TSG-RAN WG4 Meeting #114bis	R4-2504230

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	RRM core requirements and testing framework for AI/ML based Beam Management

Agenda item:	17.9.3

Document for:	Discussion

Introduction

. The discussions on AI/ML enabled BM use case were continued in RAN4#114 in the scope of Rel-19 WI based. Agreements achieved during this meeting are summarized in the WF ‎[1].

The objectives of WI NR_AIML_air ‎[2] were updated in RAN#107 and following scope was agreed for RAN4:

- For Beam Management, Positioning Accuracy enhancement and CSI prediction use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring. Note: the following aspects may be considered: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g. CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

Some of the issues related to AI/ML enabled BM-Case1 and BM-Case2 require further discussion still in the scope of ‎[2], In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement error impact and the ground truth

- KPIs for beam prediction

- Indicated/activated TCI state requirements

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Applicable functionality reporting

- Testing framework for AI/ML based Beam Management

Discussion

Measurement error impact and the ground truth

During RAN4#112 and RAN4112bis meetings, RAN4 agreed to study the impact of measurement errors to AI/ML BM functionality on the performance of prediction of AI/ML BM. The agreements of RAN4 #113 are provided below:

Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. - impact on different prediction accuracy metrics should be taken into account - studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

Agreement: ​ RAN4 to perform study to assess the impact of measurement errors on prediction accuracy​ Perform LLS to evaluate the RSRP prediction accuracy and other KPIs​ - UE Model trained using ideal measurements​ Simulation assumptions in R4-2417211 can be used as starting point.

In this section, we evaluate the impact of measurement errors for an AI/ML model in inference operation. We consider spatial domain prediction with 32 beams in Set A, Set B is subset of Set A and contains 8 beams for measurement as a baseline. The AI/ML model’s output is the Top-K predicted beam IDs.

Methodology

We followed the procedure described in Section 6 of R4-2420480, where we trained an AI/ML model with a subset of samples from an ideal L1-RSRP dataset generated from the system level simulation (SLS) following the assumptions defined in TR 38.843 Table 6.3.1-1.

Next, we followed the “Approach A” of R4-2502969, and we used the LLS simulation assumptions defined in Table 3 of Annex I to generate the Baseband (BB) errors. We also modelled the RF errors, and we added both BB and RF errors to the subset of ideal L1-RSRP data to derive the dataset with measurement errors as follows,

RSRP with measurement errors = Ideal RSRP + Baseband (BB) error + Radio frequency (RF) error

Given the data obtained by this approach, we evaluate the performance metrics considering as the input of the AI/ML model the RSRP with measurement errors and we use as ground-truth for evaluating the predictions using the ideal RSRP.

The results are compared to the performance metrics without considering measurement errors, for which we used another subset of samples from the ideal L1-RSRP dataset generated from the SLS and without measurement errors.

We evaluated the performance of the AI/ML model with KPI1: Top-K/1 without considering some margin for K =1, and 3. In Addition to KPI1, we evaluated the KPI2: 90%-tile L1-RSRP difference between the maximum RSRP of the Top-1 predicted beam and the RSRP of the genie aided strongest beam .

For BB error modeling, we consider following options:

- No error: This is the baseline case where BB errors are not considered in AI/ML model input.

- LLS (# samples = 3, SINR = -4 dB): The BB error distribution obtained by LLS simulations based on the LLS assumption in Table 3 for side conditions where number of samples in L1 averaging=3 and SINR = -4 dB.

- Gaussian distribution: 95% of users are distributed within ±2.5 dB error margin with zero mean.

For RF error modeling, we consider following options:

- No error: This is the baseline case where RF errors are not considered as input of the AI/ML model.

- Gaussian (Independent for each TX beam): 95% of users are distributed within ±4.5 dB or ±3 dB error margin with mean zero. The RF error is generated independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

- Uniform distribution (Independent for each TX beam): All users are distributed within ±4.5 dB error margin with mean zero. The RF error is added independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF errors.

- Gaussian distribution (Same for all TX beams): 95% of users are distributed within ±4.5 dB error margin with mean zero. The same RF error is added to all the RSRP measurements of the different TX beams for a single inference instance.

In addition to the above-mentioned measurement error configurations, we separate low SNR UEs and high SNR UEs among the subset of ideal L1-RSRP dataset as follows:

- Low SNR UEs: 5% of UEs who are experiencing lowest SNR among all UEs, and SNR of UE is calculated for the highest quality beam using SLS results.

- High SNR UEs: 5% of UEs who are experiencing highest SNR among all UEs, and SNR of UE is calculated for the highest quality beam using SLS results.

Then the impact of measurement error is considered for low SNR UEs, high SNR UEs and all UEs (including high SNR and low SNR UEs) as shown in Table 1.

Table 1. Evaluation of BB and RF measurement error for UEs in high and SNR range.

| Measurement error |  | KPI2:L1-RSRP difference (Low SNR UEs) | KPI2:L1-RSRP difference (All UEs) | KPI2:L1-RSRP difference (high SNR UEs) | KPI1:Top-1/1 (Low SNR UEs) | KPI1:Top-1/1 (All UEs) | KPI:1Top-1/1 (High SNR UEs) | KPI:1Top-3/1 (Low SNR UEs) | KPI1:Top-3/1 (All UEs) | KPI1:Top-3/1 (High SNR UEs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BB | RF |  |  |  |  |  |  |  |  |  |
| No BB error | No RF error | 7.84 | 5.63 | 0.55 | 66.35 | 83.78 | 92.53 | 94.17 | 96.48 | 99.04 |
| LLS (# samples = 3, SINR = -4 dB). | Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 10.93 | 13.04 | 8.85 | 55.46 | 70.55 | 75.71 | 88.47 | 92.36 | 95.26 |
|  |  | 3.09 | 7.41 | 8.30 | 10.89 | 13.23 | 16.82 | 5.7 | 4.12 | 3.78 |
|  | Gaussian with ±3 dB error margin (Independent for each TX beam) | 9.76 | 10.35 | 3.19 | 59.47 | 76.05 | 82.71 | 90.91 | 94.48 | 97.29 |
|  |  | 1.92 | 4.7 | 2.64 | 6.88 | 7.73 | 9.8 | 3.26 | 2.00 | 1.75 |
|  | Uniform with ±4.5 dB error margin (Independent for each TX beam) | 10.68 | 13.99 | 12.62 | 52.93 | 67.75 | 71.88 | 86.42 | 90.97 | 93.69 |
|  |  | 2.84 | 8.36 | 12.07 | 13.42 | 16.03 | 20.65 | 7.75 | 5.51 | 5.35 |
|  | Gaussian with ±4.5 dB error margin (Same for all TX beams) | 8.51 | 5.84 | 0.54 | 67.19 | 83.63 | 92.98 | 93.76 | 96.51 | 99.02 |
|  |  | 0.67 | 0.21 | 0 | 0.84 | 0.15 | 0.45 | 0.41 | 0.03 | 0.02 |
| Gaussian (95% of UEs in ±2.5 dB). | Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 10.94 | 13.84 | 12.44 | 53.09 | 68.77 | 73.16 | 86.74 | 91.36 | 94.44 |
|  |  | 3.10 | 8.21 | 11.89 | 13.26 | 15.01 | 19.37 | 7.43 | 5.12 | 4.60 |
|  | Gaussian with ±3 dB error margin (Independent for each TX beam) | 9.65 | 11.86 | 5.17 | 58.21 | 73.74 | 80.54 | 90 | 93.59 | 96.61 |
|  |  | 1.81 | 6.23 | 4.62 | 8.14 | 10.04 | 11.99 | 4.17 | 2.89 | 2.43 |
|  | Uniform with ±4.5 dB error margin (Independent for each TX beam) | 10.61 | 14.60 | 13.85 | 53.41 | 66.30 | 69.67 | 86.47 | 90.26 | 93.16 |
|  |  | 2.77 | 8.97 | 13.30 | 12.94 | 17.48 | 22.86 | 7.70 | 6.22 | 5.88 |
|  | Gaussian with ±4.5 dB error margin (Same for all TX beams) | 8.86 | 5.72 | 0.63 | 68.15 | 83.66 | 91.8 | 94.24 | 96.54 | 98.95 |
|  |  | 1.02 | 0.09 | 0.08 | 1.80 | 0.12 | 0.73 | 0.07 | 0.06 | 0.09 |

Note: L1-RSRP difference = max(abs(95%-tile L1-RSRP difference), abs(5%-tile L1-RSRP difference)) and gray colored field highlight the difference of KPI with respect to the scenario with no errors (row 1).

Impact of SNR (without considering measurement error):

Results show that the 90%-tile L1-RSRP difference for low SNR UEs is 7.84 dB in contrast to 0.55 dB of high SNR UEs in the case where no measurement error is considered. The same trend is observed in terms of Top-K/1 accuracy. The main reason for this observation is that the AI/ML model is trained with a mixed dataset containing low and high SNR UEs to get better generalization, and as a result, the low SNR UEs show low prediction accuracy even before considering measurement errors.

When no measurement error, low SNR UEs experience a high 90%-tile L1-RSRP difference and high SNR UEs experience a low 90%-tile L1-RSRP difference. The same trend is observed in Top-K/1 accuracy.

Impact of SNR (considering measurement error):

When the measurement error is added to RSRP measurements, both low SNR and high SNR UEs are experiencing performance degradation in beam prediction related KPIs. However, the impact of measurement error is higher for low SNR UEs compared to high SNR UEs based on the performance degradation which is observed comparatively to the AI/ML model performance without measurement error. For high SNR UEs, there can be a strong beam and rest of the beams may have relatively low strength in terms of L1-RSRP. Addition of measurement error tends to make higher number of prediction errors. Hence, RSRP error drops significantly. Contrary, low SNR UEs may have a set of weak beams in terms of L1-RSRP and relative L1-RSRP difference between beams could be very close to each other beam which cause beam prediction challenging even without measurement error. Addition of measurement error does not decrease the beam prediction drastically as in high SNR UEs because already UEs with low SNR suffering from low prediction accuracy.

We considered the same BB error derived based on LLS (# samples = 3, SINR = -4 dB) conditions for all UEs regardless of the SNR of the UE. Similarly, Table 3 in Annex 1 considers BB error derived based on LLS (# samples = 5, SINR = 0 dB) with a higher SINR target, and the result verifies the afore-mentioned trend for a BB error model derived using a higher SINR target. Hence, we show that regardless of the BB error model, high SNR UEs experience a higher performance degradation compared to low SNR UEs in the presence of measurement error.

With measurement error, all UEs experience degradation in beam prediction-related KPIs. However, a severe impact is observed with high SNR UEs compared to low SNR UEs.

Evaluation of Gaussian with ±4.5 dB error margin vs. Uniform with ±4.5 dB error margin RF error:

If we consider the Gaussian BB error model, results shows a Top-1/1 accuracy reduction of 15.01% with Gaussian RF error (Independent for each TX beam) compared to the scenario without measurement error.  For the same BB error model, the Uniform distribution (Independent for each TX beam) shows the highest degradation of 17.48% in terms of Top-1/1 accuracy compared to the scenario with no measurement error. The Top-1/1 accuracy degradation of Gaussian is lower than the Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

The performance degradation of AI/ML model in presence of Gaussian RF error is lower than that of Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

Evaluation of independent RF errors vs. the same RF error for different TX beams:

For Gaussian RF error independent for each TX beam, results show that with RF+BB errors for both LLS and Gaussian, the impact on performance KPIs is significant. On the other hand, when using the same RF error, the Top-K/1 accuracy reduction is less than 2% for all BB error models compared to no error case. Similarly, the 90%-tile L1-RSRP difference shows less than 1 dB degradation when the same RF error is applied for all TX beams. Additionally, same trend is observed for high SNR UEs as well as low SNR UEs. Therefore, using the same RF error for all TX beams does not significantly impact the beam prediction accuracy. This is similar to apply a RSRP offset for AI/ML model’s input and therefore, since normalization is applied, the AI/ML model is able to predict the strongest beam IDs without impacting the accuracy.

RAN4 to clarify the impact of measurement error is higher for low SNR UEs or high SNR UEs, given the significant degradation in prediction performance observed with high SNR UEs.

Based on evaluation assumption, the observed performance degradation of AI/ML model for Set B=8 and Set A = 32 when Set B is subset of Set A is as follows, 
If the BB error model with LLS (# samples = 3, SINR = -4 dB) and RF error model with Gaussian with ±4.5 dB error margin (Same for all TX beams) are considered,

- Top-1/1 without considering some margin is 83%.

- L1-RSRP difference is 6 dB.

Due to the nature of analog RF design, the RF power will vary every time it is used.  The main contribution of RF error is the power amplifier (PA) on the RF circuit.  Each single PA is unique in the world, it has its own parameters that are different from any other PA’s even from the same factory and produced at the same time. The PA’s performance is also significantly impacted by the temperature of PA, battery levels, the human body and other surroundings of UE, and etc. Therefore, each single usage of RF circuit will generate different results. This is the reason that we still keep some tolerance in the standard to limit the PA power variations within a certain range in 38.101 series, and the close loop between UE and gNB can still see the uncertainty of UE power. Based on the reciprocity and the complexity of designing analog circuit for Rx chain, we should expect similar RF uncertainty presenting all the time for Rx, and RF error will be presenting as independent error for every time RF circuit is used for receiving signals, do L1-meansurements. The error will however follow statistics distribution. There are some examples of using independent RF error on each beam in the studies of previous WI, R4-1904820, R4-1904760.

The performance degradation of AI/ML model is high when RF error is added independently. 
For example, if the BB error model with LLS (# samples = 3, SINR = -4 dB) and RF error model with Gaussian with ±4.5 dB error margin (Independent for each TX beam) are considered,

- Top-1/1 without considering some margin is 70%

- L1-RSRP difference is 13 dB.

RAN4 to discuss the RF error model since a large difference of the prediction accuracy was observed with different RF models. RAN4 should consider also RF error model with independent error on each TX beam.

KPIs for beam prediction

- The following details on the KPIs for AI/ML beam prediction were agreed in RAN4 #114 meeting:

- Agreement: - If RSRP and beam ID are reported: - RSRP accuracy will be one of the KPIs. - How option 1 is applied is FFS - Beam ID prediction accuracy will be one of the KPIs - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options - If only beam ID is reported: - Beam ID prediction accuracy will be used as the KPI - FFS whether beam ID prediction accuracy refers to a combination of Option 2 and/or Option 3 or modified Option 2/3 or other options

RSRP accuracy

RSRP accuracy verifies whether the top-K predicted RSRP values of set A beams lie within an acceptable error margin in comparison to their corresponding ground truth RSRPs. For this, two major KPIs that include Absolute RSRP accuracy and Relative RSRP accuracy are under discussion.

      - Absolute RSRP accuracy

Absolute accuracy follows a straight-forward way to verify the RSRP prediction performance, by directly comparing the predicted RSRP of beam i with its ground truth value. And a following agreement was reached on the definition of absolute accuracy for AI/ML BM in RAN4#112bis meeting:

Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

Since the RSRP measurements of set B beams are used as input to train and test the beam prediction model, the predicted RSRPs of set A beams will have measurement errors apart from the prediction errors from the AI/ML model. Note that it has been agreed by RAN4 that the ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam. So, comparing the predicted RSRPs with their measured values through the absolute accuracy metric might only capture relative prediction error, as shown in Figure 1. And this relative prediction error does not lie within the acceptable RSRP margin of the legacy. Hence, acceptable absolute accuracy requirement for AI/ML BM prediction is relaxed with respect to the legacy measurements. Considering the performance loss due to this, as depicted in terms of performance gap w.r.t legacy in Figure 1, the requirements for the RSRP measurements that are used as input to the RSRP prediction model have to be improved such that it results the RSRP prediction performance meets the acceptable absolute RSRP requirement.

Acceptable absolute RSRP accuracy for AI/ML BM should meet the existing legacy requirements to avoid performance loss

In order to meet existing legacy requirements, the measured RSRP accuracy for the input to the AI/ML model should be improved. Alternatively, the acceptable predicted RSRP accuracy for AI/ML BM can be tightened such that it is close to legacy requirement

Figure 1: L1-RSRP measurement accuracy - Legacy vs AI/ML

Relative RSRP accuracy

Relative RSRP accuracy is expected to identify the errors in the test cases that go undetected with absolute accuracy tests alone, thereby reducing false negatives arising due to the absolute metric. Hence, it must be used in conjunction with absolute accuracy to verify the performance of RAN4 measurements.

RAN4 should consider both absolute and relative L1-RSRP accuracies for the verification of AI/ML BM functionality in case of L1-RSRP prediction.

Following the general principle from the legacy definition for relative accuracy along with the agreed description of the metric in RAN4 AI/ML Mobility SI ‎[3], we propose the following definition of it for AI/ML Beam Management

Relative RSRP accuracy for AI/ML BM can be defined as:

- relative RSRP accuracy = (reported predicted RSRP of beam i – reported RSRP of beam j) – (reported RSRP of beam i – reported RSRP of beam j) where beam i is one of the top-K predicted beams and beam j is the beam with maximum measured RSRP

- RAN4 to further discuss whether the reported RSRP of beam j can be predicted or measured or a combination of both predicted and measured samples.

Beam IDs prediction accuracy

Beam IDs prediction accuracy verifies whether the top-K beams predicted by the AI/ML BM model are indeed the strongest/top-K genie-aided beams. In this section, we discuss the significance of modified_option3 which is one of the KPIs, under discussion, that evaluate Beam IDs prediction accuracy. For this, we start by explaining the first introduced success rate metric called option 3 (Soption3) and present its drawbacks. The discussion is followed by the details of its proposed variant, modified_option3/weighted success metric (Sw) and how it addresses drawbacks of option3 by still retaining the merits. The simulation results at the end confirm that unlike option3, modified_option3 is robust in terms of identifying the correct prediction events with a clear drop in the success rate in presence of noise for all the evaluated cases.

Option 3, evaluates the successful rate of correct prediction by verifying whether the condition that the maximum L1-RSRP among top-K predicted beams is larger than the L1-RSRP of the strongest beam - X dB. This KPI, here referred as Soption3 is expressed as follows:

Soption3=1Nt n=1Ntlmaxbk ∈ set ARSRPbk≥maxbk ∈ set ARSRPbk-X

Where

set A: set containing top-K predicted beams, A=K

set A: set containing all SSB/CSI-RS beams

X: allowed RSRP margin

RSRPb: ground-truth RSRP of beam b

l(C): Indicator function. It equals 1 when the condition C is true, else 0

Nt: Number of inference instances

Note that here, the threshold used for comparison in condition C of option 3 is maxbk ∈ set ARSRPbk-X

Even though it is logical for the network to use the highest RSRP beam among the top-K predicted beams for transmission to the UE, load-balancing and scheduling requirements can constrain the network to choose one of the other top-K predicted beams. Also, the beam prediction model accuracy should be verified in terms of identifying the best K-cardinality beam set rather than any K-cardinality set containing the strongest beam. In this regard, if the prediction accuracy of other top-K beams is not verified then it may cause a degraded in-field performance even though a UE passes the success rate requirement test with option 3.

At this, we identify that the success rate of correct prediction instead of evaluating only a single condition that checks for the highest RSRP beam amongst the top-K predicted beams meeting certain threshold, the KPI should also verify whether other beams within the predicted beam set indeed satisfy such threshold requirements.

For a fair evaluation, the threshold for ith ranked predicted beam, bi could be set as RSRP of the corresponding ith ranked genie-aided beam bi – X. Hence, the condition to be checked for the ith ranked beam is Ci=lRSRPbi≥RSRPbi-X, where X is a fixed value for all i.

The threshold for ith ranked predicted beam, bi can be considered as the RSRP of the corresponding ith ranked genie-aided beam, bi – X

The success rate of correct prediction can be computed as a weighted average that considers all top-K predicted beams

Weighted success rate, Sw=1Nt n=1Nti=1Kwi,n lRSRPbi,n≥RSRPbi,n-X

Where the weights can be implementation specific. For example, we use

wi,n=RSRPbi,nRSRPbi,ni=1KRSRPbi,nRSRPbi,n with RSRPs here in linear scale

NOTE: All the RSRPs and their ranking in the above are determined from the ground-truth values

Below table compares the success rate metrics Sw and Soption3 for different X and K values.

Table 2: Sw and Soption3 for different X and K

| K X(dB) | 2 |  | 3 |  | 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 0.88 | 0.92 | 0.90 | 0.97 | 0.90 | 0.99 |
|  | 0.96 | 0.98 | 0.95 | 0.99 | 0.93 | 0.99 |
| 4 | 0.94 | 0.97 | 0.95 | 0.99 | 0.95 | 0.99 |
|  | 0.98 | 0.99 | 0.97 | 0.99 | 0.96 | 0.99 |
| 6 | 0.96 | 0.98 | 0.97 | 0.99 | 0.97 | 0.99 |
|  | 0.99 | 0.99 | 0.99 | 0.99 | 0.98 | 0.99 |

Legend for Table 1 cell layout

| Sw, with error | Soption3 , with error |
| --- | --- |
| Sw, no error | Soption3, no error |

We observe from Table 1 that, in all the evaluated cases the weighted success metric differentiates the simulation instances with and without RF/BB errors better than its unweighted version/option 3.

The simulation results confirm that the weighted success metric is robust in identifying the performance impact due to errors in comparison to option 3

Following measurement error setting is used for above simulations

        - RF error is obtained from a Gaussian distribution where 95% of users are distributed within ±4 dB error margin with mean zero.

        - BB error is generated with a link level simulation with SINR = 0 dB, number of samples considered for L1-averaging = 3.

        - The RF and BB error is generated independently for each RSRP measurement of TX beam in a single inference instance so that different TX beams are measured with different RF and BB errors.

RAN4 to discuss adaptation of Option 3 with following changes:

- All top-K predicted beams have to be considered in the success rate evaluation

- RSRP of each top-K predicted beam is to be compared with the RSRP of its equal ranked genie-aided beam - X

Relative difference in prediction due to change in conditions/inputs:

We introduced Option 6 as another option, which is to define the requirements not based on UE measurement as a reference, but to verify the expected behaviour of AI/ML-enabled functionality based on the change in the configurations/inputs of the functionality. For example, the best predicted beam should not change, if the power of all measured beams is scaled in the same way or when one of the measured beams cannot be received anymore, etc. In other words, the AI/ML functionality can be tested without explicit definition of the ground truth.

The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Relative difference in prediction due to change in controlled conditions/input should be considered as an additional metric to test the robustness of prediction in combination of other approximated ground truth-based options.

Indicated/activated TCI state requirements

Agreement from RAN4#114 meeting:

Agreement: - wait for RAN1 discussion to conclude without sending any LS

In Rel-19 AI/ML BM, only Top-1 or Top-K/1 DL Tx beams out of Set A is considered. The DL Rx beam paired with the predicted DL Tx beam using Set B DL Tx beams as input to the AI/ML functionality is out of scope of Rel-19 AI/ML BM following the approval in RAN#102 Rel-19 WI on AI/ML for NR Air Interface and RAN #105 meeting which provided the revised WID [RP-242399].  However, the UE has the information about complete set of beams (i.e. Set A), it is reasonable to assume that UE may know already the Rx beam to be used for receiving the predicted Tx beam.

Another option for UE to know the Rx beam corresponding to predicted DL Tx beam is to consider specific UE implementation especially in the chamber and the legacy assumption. For example, whenever the UE reports L1-RSRP, the UE would know the Tx-Rx beam pair, this legacy assumption should remain applicable for AI/ML BM.

Regarding TCI state associated with a predicted DL Tx beam, the NW can determine whether any changes are required in the set of active TCI states, and if yes, the NW can adapt the set of active TCI states based on the prediction report received from the UE. Here, the NW needs to ensure that the RS resource corresponding to the activated/indicated beams which can be measured by the UE prior any beam indication takes place corresponding to the predicted RS resource.

In the case of UE uses subset of beam measurements (Set B) to predict beam(s) from Set A, the UE may not have measured the RS resource included in the indicated/activated TCI state. Since the predicted beam(s) is not measured by the UE for longer period, the predicted beam(s) will be corresponding to unknown TCI state. The delay for switching the beam to an unknown TCI state is higher than switching to a known TCI state. Therefore, in our view, for a UE that supports beam prediction, there is a need to relax the known conditions for TCI state, specified in Clause 8.10.2 of TS 38.133:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

If this condition is relaxed, then the predicated beam (which may not be part of Set B beams) corresponding to a Set A/subset of set A, may be measured with a longer delay than 1280 ms and still fulfil TCI known state condition. In other words, a higher value of the delay requirements should be used for known TCI state conditions in Clause 8.10.2 of TS 38.133 for predicted beams.

In order to relax the known conditions of TCI state such that the predicted beam could be measured, e.g.,with a delay longer than 1280 ms, we need to further specify on how long the delay could be possible. Further analysis of this delay requirements or simulation results might be needed to define the exact delay requirements of known TCI states.

RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2. Two possible approaches for RAN4 to relax known TCI states conditions:

- The TCI state switch command is received within a delay X*1280 ms upon the last transmission of the RS resource for beam reporting or measurement or prediction.

- FFS: Further analysis on defining X values.

Measurement delay requirements

Following agreement was reached during RAN4#113:

Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS

The delay component related to CSI-RS measurement period could be addressed for measurement delay. This delay component can also be mapped into the delay due to CSI-RS measurement period component in Table 5, Section 2.6.

For NW-sided model, the UE would need to report the measured L1-RSRP for both BM-Case1 (spatial domain beam prediction) and BM-Case2 (temporal domain beam prediction) based on measurement delay condition. For UE-sided model, the UE also needs to follow the measurement delay condition when performing beam prediction.

In BM-Case2, multiple time instances of measurement reporting need to be reported as observation window. The CSI report configuration will associate to time requirement defined with measurement period, where the corresponding delay related to measurement should be within measurement period (TL1-RSRP_Measurement_Period_CSI-RS) defined in Clause 9.5.4.2 TS 38.133. However, the current measurement period for L1-RSRP measurement, TL1-RSRP_Measurement_Period_CSI-RS,  in TS 38.133 Clause 9.5.4.2 could not be applied for AI/ML beam prediction especially BM-Case2. Since the current measurement period in Clause 9.5.4.2 does not include when there are multiple time instances measurements corresponding to observation window for BM-Case2. The new measurement reporting requirements will require to be addressed with inference delay to allow multiple time instances measurements for BM-Case2.

The below table shows the measurement period for L1-RSRP reporting in legacy.

Table 9.5.4.2-2: Measurement period TL1-RSRP_Measurement_Period_CSI-RS for FR2 Configuration; TL1-RSRP_Measurement_Period_CSI-RS (ms). non-DRX; max( TReport, ceilM*P*N*TCSI-RS ). DRX cycle ≤ 320ms; max( TReport, ceil 1.5*M*P*N)*max⁡( TDRX, TCSI-RS). DRX cycle > 320ms; ceilM*P*N* TDRX. Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-RSRP measurement. TDRX is the DRX cycle length. Treport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for L1-RSRP measurement is transmitted with Density = 3..

RAN4 to further discuss the impact of inference delay on the prediction delay requirement for BM-Case1.

RAN4 to discuss whether the legacy measurement delay requirement should be increased for BM-Case2.

Impacts on requirements related to candidate beam detection

In current RAN4 requirements, as mentioned in Clause 8.5.6.2 of TS 38.133, the UE is supposed to evaluate during a certain time period the L1-RSRP measured on the configured CSI-RS resource in the candidate beam set (called q1) containing CSI-RS resources configured for a serving cell. The objective is to verify if the L1-RSRP measured on the configured CSI-RS resource has not been degraded too much and is still better than a certain threshold (Qin_LR). The certain period used to evaluate L1-RSRP is known as TEvaluate_CBD_CSI-RS and is specified in Table 8.5.6.2-2 of clause 8.5.6.2 in TS 38.133.

Table 8.5.6.2-2: Evaluation period TEvaluate_CBD_CSI-RS for FR2 Configuration; TEvaluate_CBD_CSI-RS (ms). non-DRX, DRX cycle ≤ 320ms; Max(25, Ceil(MCBD ´ P ´ N ´ PCBD) ´ TCSI-RS). DRX cycle > 320ms; Ceil(MCBD ´ P ´ N ´ PCBD) ´ TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length..

- However, with the addition of AI/ML BM feature, the UE is supposed to predict best beams from the Set A beams and the UE is configured to measure only a few beams (i.e. Set B). Therefore, the network will often configure only CSI-RS of the Set B beams, therefore the CSI-RS periodicity of Set A beams may be increased. If the NW uses same CSI-RS periodicity also for candidate RS resources, it may lead to a higher value of TCSI-RS used for the calculation of TEvaluate_CBD_CSI-RS. This will eventually lead to a higher value of  TEvaluate_CBD_CSI-RS, which consequently may cause a delayed beam recovery operation and may cause frequent beam failures A higher value of TCSI-RS in Table 8.5.6.2-2 may lead to a higher value of TEvaluate_CBD_CSI-RS, which consequently may be the cause of frequent beam failures.

RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

LCM and performance monitoring related requirements

LCM related requirements

Agreement:

- Whether transition from AI to non-AI would be UE initiated or network initiated

  - This will be a decision for other groups

  - RAN4 to assess what RRM requirements are needed for each case.

Agreement:

- The discussion should consider use-cases specific aspects

Agreements from RAN4#113 and agreements from RAN2#127 meetings:

Agreements on definitions from RAN2#127:

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

The LCM operations for AI/ML BM, e.g., activation, deactivation, and fall-back (e.g., fall-back from AI/ML beam prediction to legacy mode) could be supported by CSI report configuration. These operations can be triggered, for example, based on the performance monitoring output. Reusing the existing capability framework, NW can configure CSI report configurations to enable BM-Case1 or BM-Case2 beam prediction. The CSI reports for both BM-Case1 and BM-Case2 can be based on periodic, semi-persistent, or aperiodic CSI-reporting. The latency of these operations will depend on multiple factors such as,

- Whether CSI configuration (ResourceSet) to be used is already available in the device or not,

- The type of the CSI reporting (periodic, semi-persistent, aperiodic),

- The availability of (previous) measurements that shall be used as an input to the model/functionality inference/prediction,

- Availability of the necessary model(s) in the device,

- The time it takes to substitute one model/functionality with another (the UE may have specialized AI-hardware accelerators to run the inference, which may require loading of the model),

However, it should be clearly specified how much time it takes for the UE to report the new predictions/inference results following the changes indicated by the LCM operation. Without such information NW will not be able to decide when to schedule grants for the reporting of measurements/predictions or data and taking too much time in these operations may also degrade the performance.

In LCM operation, the functionality activation operation is the first step when the UE receives from NW the necessary inference configuration needed for running the inference with the AI/ML functionality available at the UE. Then, other operations, e.g., deactivation, switching and fallback, come after functionality activation operation.

For AI/ML beam prediction reporting enabled by periodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

- delay related to RRC Reconfiguration,

- delay from UE processing time: This may be related to the size of the inference report, model-related pre-processing time or memory limitations on storing models (associated with beam prediction),

- delay related to associated ID: This delay will depend on if the associated ID is used or not used in the past, there might be delay corresponding to configure/indicate the associate ID to UE,

- delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related to RRC Reconfiguration,

  - delay from UE processing time

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by semi-persistent CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via semi-persistent CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from activating semi-persistent CSI resource set and delay for selecting of semipersistent CSI resource set.

  - delay from UE processing time

  - delay related to associated ID (Optional)

  - delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay from NW selecting of semi-persistent CSI resource set.

  - delay from UE processing time related

  - delay due to CSI-RS measurement period.

For AI/ML beam prediction reporting enabled by aperiodic CSI-report, the LCM-related latency requirements can be defined as below:

- For activating or switching inference operation of AI/ML-enabled beam prediction via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related activating aperiodic CSI resource set,

  - delay from UE processing time

  - delay related to associated ID (Optional),

  - delay due to CSI-RS measurement period.

- For deactivating or switch to fallback to non-ML operation (beam measurement) via periodic CSI reporting, a total delay can be defined by considering the following components as part of the delay requirement:

  - delay related NW selecting of aperiodic CSI resource set,

  - delay from UE processing time related

  - delay due to CSI-RS measurement.

For UE-side model in beam management use case, the LCM operations are based on CSI report configuration, in our view, the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

RAN4 should consider Table 3 as a baseline for further discussion on LCM operations related RRM core requirements.

Table 3: Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Switching | Fall-back (From AI/ML BM to legacy) |
| --- | --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Aperiodic CSI reporting | Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

Performance monitoring related requirements

In RAN1#118 bis and RAN1#119, there are ongoing discussions [R1-2408544, #RAN1 112bis] on a need to determine the monitoring window for performance monitoring instances, where the total number of valid performance monitoring instances should be considered when UE calculates performance monitoring metric/KPI.

For UE-sided model, the delay requirements related to performance monitoring of AI/ML-enabled functionality (CSI-ReportConfig) depending on performance monitoring mechanism especially for UE-assisted performance monitoring option. The UE could calculate one of the performance metrics and report it to the NW or UE could report event to NW based on the performance metric.

Accordingly, RAN4 will need to define new delay requirements on the reporting of the performance metric(s) related to the duration of the monitoring window when the UE calculates performance monitoring metric/KPI or performs event-based performance monitoring. When the UE calculates and reports the performance monitoring metric, then NW would interpret those reports and, if necessary, makes a decision and indicates to UE about functionality (CSI-ReportConfig) selection/activation/ deactivation/switching/ fallback operation.

Following the RAN1 discussions, the performance information is calculated per sample (one-shot), or per set of samples (monitoring window). In the case, when a monitoring window is used, RAN 4 requirements will need to be defined.

RAN4 should consider the impact in RAN4 requirements for performance monitoring window when UE reports performance metric or event to the NW.

Applicable functionality reporting

RAN2 agreed as working assumption on five steps procedure, intended to cover the main signalling steps from UE capability enquire up to NW provisioning and activation of the ML functionality.

Following RAN2 agreements, the applicability of the inference related parameters should be checked by the UE and/or by the NW before the AI/ML capability/feature is enabled/activated. A configuration for ML-enabled CSI reporting, e.g., a configuration for periodic beam prediction reporting, may become inapplicable. In RAN2#128 agreement, it was agreed that during the inapplicability of functionality period, the output of beam prediction could be unreliable, the UE doesn’t autonomously deactivate the functionality. The NW is expected to deactivate an activate functionality when it receives a report from UE that the functionality is inapplicable. For periodic beam prediction reporting, the UE can report the inapplicability of inference output using UAI, while periodic beam prediction reporting is still activated and reporting. This leads to the transmission of Set A beam prediction become suspended due to inapplicable of functionality, a new UE reporting behaviour, e.g., UE could be configured to report Set B beam measurements is proposed in RAN2 after the UE reports inapplicability of beam prediction reporting configuration. In this case, beam failure would happen regarding the ambiguity of inference reports. This will also cause a delay for beam failure recovery operation if the NW uses the same CSI-RS periodicity of Set A beams for candidate RS resources.

- According to RAN2, when an active beam prediction configuration becomes inapplicable, the UE should report Set B measurements corresponding to the newly inapplicable. RAN4 should further study     (i) delay conditions related inapplicability reporting and                                                                                     (ii) beam failure evaluation period due to inapplicable functionality.

Testing framework for AI/ML based Beam Management

The discussions on the testing framework of one-sided models, which is also applicable for the testing of AI/ML enabled beam management (BM) use case, progressed in RAN4#114 meeting and the following agreements were achieved during the meeting under the agenda “Testing for one sided models” (see WF ‎[1]).

Issue 4-1: Baseline test setup Agreement: - Discuss both single AoA based and multiple AoA based set up - Continue to study the two multiple AoA candidate setups - One option to continue the discussion proposed as following: - Multi AoA setup – evaluation of already feasible systems for AI/ML BM testing suitability - AI/ML BM performance centric, not CDL channel emulation centric. - Define test system candidates which are already feasible - Perform system level simulations including the probe layout of candidate systems to address the impact on the UE AI ML BM performance. - Start with the set up with Set A = 32 beams, Set B = 8 beams, and develop the test procedures with single AoA and multiple AoAs Issue 4-2: Channel model Agreement: - Use CDL-based channel model as starting point

In the above agreement(s), there are several open issues that need to be addressed, e.g.,

- Further analysis on the test setups with single AoA and multiple AoAs,

- Evaluation of already feasible multi-AoA test systems for AI/ML BM testing suitability,

- How to define candidates test system which are already feasible,

- System level simulations results explaining the impact on the UE AI/ML BM performance.

In this contribution, we provide our views on these open issues as well as on the generalization and test coverage related aspects.

Test setup based on single or multiple AoA(s)

- For a while, RAN4 is actively discussing whether the set up for the conformance testing of AI/ML based beam management use case should be based on single AoA or multiple AoAs. Undoubtedly, each of these test setups have their own pros and cons, which were discussed by the several companies during the past RAN4 meetings.

- The main challenge is the emulation of Set A beams during the conformance testing to extract the ground truth which depends on the UE architecture and implementation. Also, RAN4 has recently agreed that the Set A should have 32 beams for the conformance testing. So, the question is how to generate 32 beams inside the OTA test chamber during the conformance testing?

Ideally, all the beams in Set A and/or Set B should be generated without the UE rotation as the similar scenario will be observed in the real deployments. However, based on the TE vendors’ input, only a limited number of beams with different AoAs (RX-PAS profiles) can be emulated inside the test chamber due to which multiple beams in Set A and/or Set B will arrive at the DUT from the same/similar direction(s). This implies that all Set A and Set B beams, with different AoAs, can’t be generated during the conformance testing without using the UE rotation unless the test chamber is equipped with sufficiently large number of probes. Therefore, the emulation of agreed 32 beams for Set A and 8 beams for Set B will be challenging.

We below provide two alternatives to emulate the Set A and Set B beams during the conformance testing, one of which is based on the concept of UE rotation while the other solution does not assume any kind of UE rotation.

      - Emulation of Set A/Set B beams without using UE rotation

- This solution provides a method to emulate the Set A and/or Set B beams without using the UE rotation. This solution is applicable only for the multiple AoAs based test systems.

Each Tx beam from the Set A and/or Set B can be seen as a particular Tx antenna pattern creating a unique constructive or destructive interference pattern at the UE receive antenna(s). Considering a test chamber (e.g., anechoic chamber) equipped with ‘M’ test probes where each test probe is contributing to form a definite Tx antenna pattern (beam), the formation of overall Tx antenna pattern is also affected by the amount of Tx power of each test probe. As there are ‘M’ test probes in the test chamber, a particular Tx antenna pattern (containing at least a beam) can be formed by selecting/activation any of the ‘N’ test probes out of the available ‘M’ probes, where 1≤N≤M. This means that ‘PN’ Tx antenna patterns can be formed if ‘N’ test probes are activated where PN = MCN (M Choose N), i.e.,

- MC1 Tx antenna patterns can be formed if only 1 probe is selected/activated.

- MC2 Tx antenna patterns can be formed if 2 probes are selected/activated.

- MC3 Tx antenna patterns can be formed if 3 probes are selected/activated.

- …

- MCM-1 Tx antenna patterns can be formed if ‘M-1’ probes are selected/activated.

- MCM Tx antenna patterns can be formed if ‘M’ probes are selected/activated.

Hence, a test system with ‘M’ test probes in the test chamber can emulate (MC1 + MC2 + MC3 +…+ MCM-1 +MCM) Tx antenna radiation patterns (or beams).

For example, a practical test system equipped with

- 6 test probes can emulate upto (6C1 + 6C2 + 6C3 + 6C4 + 6C5 + 6C6) = (6 + 15 + 20 + 15 + 6 + 1) = 63 Tx antenna radiation patterns (or beams).

- 8 test probes can emulate upto (8C1 + 8C2 + 8C3 + 8C4 + 8C5 + 8C6 + 8C7 + 8C8) = (8 + 28 + 56 + 70 + 56 + 28 + 8 + 1) = 255 Tx antenna radiation patterns (or beams).

This also implies (as an example) that:

- For a test system, having 6 test probes, selecting at least 3 test probes will result in the creation of (20+15+6+1) = 42 Tx antenna patterns.

- For a test system having 8 test probes, selecting at least 4 test probes will result in the creation of (70+56+28+8+1) = 163 Tx antenna patterns.

- For a test system having 8 test probes, selecting at least 5 test probes will result in the creation of (56+28+8+1) = 93 Tx antenna patterns.

Therefore, with this concept, the existing MIMO-OTA and/or multi-Rx test system(s) which are equipped with 6 test probes can emulate 32 or more Tx Beams (or Tx antenna patterns) with at least 3 simultaneously activated probes. This is also explained below with the help of few examples.

Figure 2: Example 1 showing emulation of Antenna Pattern ‘1’ or Beam ‘1’ using 3 test probes.

In example 1 shown in Figure 2 above, 3 test probes (Probe 1, Probe 2 and Probe 6) are used to emulate a unique antenna pattern by exploiting the constructive/destructive interference created by the multi-antenna/probe signals transmitted by the Probe 1, Probe 2 and Probe 6 simultaneously. The resulting beam/antenna pattern also depends on the P_11, P_21 and P_61, which are the transmit powers of the Probe 1, Probe 2, and Probe 6, respectively.

Figure 3: Example 2 showing emulation of Antenna Pattern 2 or Beam ‘2’ using 5 test probes.

- In example 2 shown in Figure 3 above, 5 test probes (Probe 1, Probe 2, Probe 3, Probe 4 and Probe 5) are used to emulate a unique antenna pattern by exploiting the constructive/destructive interference created by the multi-antenna signals simultaneously transmitted by the Probe 1, Probe 2, Probe 3, Probe 4 and Probe 5. The resulting beam/antenna pattern also depends on the P_12, P_22, P_32, P_42, and P_52, which are the transmit powers of the Probe 1, Probe 2, Probe 3, Probe 4 and Probe 5, respectively.

- In this solution, the selection of ‘N’ probes out of ‘M’ available probes can be seen in one way as the selection of ‘N’ dominant/significant/important clusters from the CDL or simplified CDL channel model. On the other hand, this selection of probes can also be viewed as selecting ‘N’ probes out of the 6 available probes in the multi-Rx test setup which has 6 probes installed in the same two-dimensional plane spanning an 150o wide angular area. But, how to decide or select the required Tx power of the selected probes is FFS.

RAN4 to discuss and explore the feasibility of generating Set A and Set B beams, during the conformance testing of the AI/ML-BM use case, by selecting the number of probes and controlling the transmit power of each probe. How to decide or select the required Tx power of the selected probes is FFS.

It is also important to note that this proposed solution may need a special beam codebook for the generation of Set A and Set B beams during the conformance testing and the UE needs to be (pre)-trained for such a codebook. To ensure that the UE is (pre)-trained for this type of Set A and Set B beam(s) generation during the conformance testing, a pre-alignment check may be needed before the exact start of the conformance test where the objective of the pre-alignment check is to ensure that the TE provided parameters (e.g., for the beam codebook) and the DUT’s model/functionality/configuration’s current/preferred parameter (e.g., the parameters using which the training was performed) are in agreement with each other. This is needed to ensure that the test does not fail because of the (AI/ML) parameter(s) mismatch between the DUT (pre)-training and conformance testing.

RAN4 to discuss and decide the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

      - Emulation of Set A/Set B beams with UE rotation

In this solution, an alternative method to emulate the Set A and Set B beams in an iterative manner by using the concept of UE rotation is explained. This solution is applicable for both single AoA based test systems as well as for the multiple AoA based test systems.

To enable the beam sweeping of Set A and/or Set B beams in an iterative manner, a combination of the available test probes and different possible UE rotations is exploited where single or multiple test probe(s) are available. One benefit of using the UE rotation is that it enables the test system to simulate the AoA angles outside the angular range of test probes (from UE’s point of view) and hence the test system can generate any possible AoA beam angles.

The proposed solution assumes: (1) OTA test chamber is equipped with single or multiple test probe(s), (2) any of the CDL/TDL/AWGN channel model is used during the test, (3) One beam is generated by using only one of the available test probe(s) during the conformance test, (4) or one beam is generated by using a probe array for any possible AoA angles. The solution proposes the DUT to perform beam sweeping in multiple iterations and to measure the RSRP of the corresponding beams in Set A and/or Set B during the conformance test for which a new test command is to be defined and to be sent to the DUT, and we assume that the origin of the 3D coordinate system to define the AoA(s) is in the center of the UE positioner inside the test chamber. After the DUT receives this test command from the TE and configures itself to perform beam sweeping in multiple iterations, one of the possible UE orientations is selected and TE selects the power level (typically different) for each of the available test probes. It is worth mentioning that in this solution, only one test probe is active at any given point in time and represents one Tx beam. This implies that with ‘P’ test probes in the OTA testing (anechoic) chamber, using one probe at a time for one beam will implicitly mean emulating ‘P’ different Tx beams (one at a time) for a given UE orientation. Thereafter, if more than ‘P’ Tx beams are required, TE can change the UE orientation but uses the same set of available test probes (with the Tx power of each test probe to be held constant as previously selected or to be set differently) to emulate another set of ‘P’ Tx beams. In this fashion, multiple UE orientations can be selected depending on the number of Tx beams required. The above solution is valid for emulating all Set A and/or Set B beams in an iterative manner to perform beam sweeping during the conformance testing.

Figure : An example of the proposed test system with UE rotation.

Given the Set A and/or Set B beams can be generated iteratively during the conformance test as proposed above, DUT can measure the L1-RSRP of each beam in Set B and provides it as input to its AI/ML model for Set A beam prediction.

RAN4 to discuss and explore the feasibility of performing beam sweeping in multiple iterations during the conformance testing of AI/ML based BM use case.

Generalization and test coverage related aspects

In the RAN4#110 and RAN4#111 meetings, it was agreed to discuss generalization aspects for each AI feature in case-by-case manner. The overfitting of UE functionalities to the test environment was also raised by some companies. In our view, testing of generalization aspect is one of the main challenges for RAN4 testing. RAN4 may define multiple tests with different conditions.

In each of the test, TE configures the same specified UE configuration for AI/ML BM, the same UE configuration is tested under different scenarios to verify the UE’s model capability to generalize. In the tables below we attempt to capture different parameters that may be needed to define different generalization scenarios for AI/ML enabled BM use case based on set of parameters/configurations to both DUT / TE. For identified scenarios in AI/ML enabled BM-Case1 and BM-Case2, the minimum level of performance may need to be defined. Similarly acceptable tolerance margin for other scenarios may also need to be agreed, potentially based on simulation results.

Table 1: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR (Deployment Scenario) | Good / Bad Radio conditions |
| Propagation Model | TDL/CDL |

For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in Table 1.

In order to address the testability issue of generalization aspects of AI/ML BM use case, we propose a new metric to indicate the generalization capabilities of an AI/ML functionality, which may be referred as “generalization score”. This may be used to verify/check whether the BM functionality generalize well or not.

As an example, if there are 3 scenarios of interest, any one scenario can be the baseline setup and the remaining 2 scenarios will then be used as other set ups to test the generalization aspects. The generalization score can be calculated as a function of individual scenario verdicts, e.g., as simple average or weighted average of the individual generalization verdict of each Scenario ‘j’ (i.e., Scenario ‘1’ to Scenario ‘N), where the verdict of Scenario ‘j’ = 1 if the functionality generalizes to Scenario ‘j’ else it is equal to 0.

Overall Generalization Score= (VerdictScenario1+…+VerdictScenarioN)N×100%

RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML BM functionality, to verify the generalization performance of the functionality in different scenarios.

Conclusion

In this paper we share our views on RAN4 impacts from issues related to RRM core requirements and testability and interoperability for AI/ML enabled BM-Case1 and BM-Case2. Specifically, we cover following aspects:

In this paper, we provide some additional views on issues related to RRM core requirements, as listed below:

- Measurement error impact and the ground truth

- KPIs for beam prediction

- Indicated/activated TCI state requirements

- Measurement delay requirements

- Impacts on requirements related to candidate beam detection

- LCM and performance monitoring related requirements

- Applicable functionality reporting

- Testing framework for AI/ML based Beam Management

In the paper, the following Observations and Proposals were made:

. Observation 1: When no measurement error, low SNR UEs experience a high 90%-tile L1-RSRP difference and high SNR UEs experience a low 90%-tile L1-RSRP difference. The same trend is observed in Top-K/1 accuracy.

Observation 2: With measurement error, all UEs experience degradation in beam prediction-related KPIs. However, a severe impact is observed with high SNR UEs compared to low SNR UEs.

Observation 3: The performance degradation of AI/ML model in presence of Gaussian RF error is lower than that of Uniform distribution as most of the RF errors are concentrated near the mean value zero, and few RF errors appear further away from the mean.

Proposal 1: RAN4 to clarify the impact of measurement error is higher for low SNR UEs or high SNR UEs, given the significant degradation in prediction performance observed with high SNR UEs.

Observation 4: Based on evaluation assumption, the observed performance degradation of AI/ML model for Set B=8 and Set A = 32 when Set B is subset of Set A is as follows,

If the BB error model with LLS (# samples = 3, SINR = -4 dB) and RF error model with Gaussian with ±4.5 dB error margin (Same for all TX beams) are considered,

- Top-1/1 without considering some margin is 83%.

- L1-RSRP difference is 6 dB.

Observation 5: The performance degradation of AI/ML model is high when RF error is added independently.

For example, if the BB error model with LLS (# samples = 3, SINR = -4 dB) and RF error model with Gaussian with ±4.5 dB error margin (Independent for each TX beam) are considered,

- Top-1/1 without considering some margin is 70%

- L1-RSRP difference is 13 dB.

Proposal 2: RAN4 to discuss the RF error model since a large difference of the prediction accuracy was observed with different RF models. RAN4 should consider also RF error model with independent error on each TX beam.

Observation 6: Acceptable absolute RSRP accuracy for AI/ML BM should meet the existing legacy requirements to avoid performance loss

Proposal 3:  In order to meet existing legacy requirements, the measured RSRP accuracy for the input to the AI/ML model should be improved. Alternatively, the acceptable predicted RSRP accuracy for AI/ML BM can be tightened such that it is close to legacy requirement

Proposal 4: RAN4 should consider both absolute and relative L1-RSRP accuracies for the verification of AI/ML BM functionality in case of L1-RSRP prediction.

Proposal 5: Relative RSRP accuracy for AI/ML BM can be defined as:

- relative RSRP accuracy = (reported predicted RSRP of beam i – reported RSRP of beam j) – (reported RSRP of beam i – reported RSRP of beam j) where beam i is one of the top-K predicted beams and beam j is the beam with maximum measured RSRP

- RAN4 to further discuss whether the reported RSRP of beam j can be predicted or measured or a combination of both predicted and measured samples.

Observation 7: At this, we identify that the success rate of correct prediction instead of evaluating only a single condition that checks for the highest RSRP beam amongst the top-K predicted beams meeting certain threshold, the KPI should also verify whether other beams within the predicted beam set indeed satisfy such threshold requirements.

Observation 8: For a fair evaluation, the threshold for ith ranked predicted beam, bi could be set as RSRP of the corresponding ith ranked genie-aided beam bi – X. Hence, the condition to be checked for the ith ranked beam is Ci=lRSRPbi≥RSRPbi-X, where X is a fixed value for all i.

Proposal 6: The threshold for ith ranked predicted beam, bi can be considered as the RSRP of the corresponding ith ranked genie-aided beam, bi – X

Proposal 7: The success rate of correct prediction can be computed as a weighted average that considers all top-K predicted beams

Weighted success rate, Sw=1Nt n=1Nti=1Kwi,n lRSRPbi,n≥RSRPbi,n-X

Where the weights can be implementation specific. For example, we use

wi,n=RSRPbi,nRSRPbi,ni=1KRSRPbi,nRSRPbi,n with RSRPs here in linear scale

NOTE: All the RSRPs and their ranking in the above are determined from the ground-truth values

Observation 9: The simulation results confirm that the weighted success metric is robust in identifying the performance impact due to errors in comparison to option 3

Proposal 8: RAN4 to discuss adaptation of Option 3 with following changes:

- All top-K predicted beams have to be considered in the success rate evaluation

- RSRP of each top-K predicted beam is to be compared with the RSRP of its equal ranked genie-aided beam - X

Observation 10: The behaviour of beam prediction functionalities can be tested without explicit definition of the ground truth by comparing the predictions to each other.

Proposal 9: RAN4 to introduce additional requirements based on relative difference in between the predictions due to change in the conditions/inputs, i.e., without dependency on the measurement accuracy errors and uncertainties.

Proposal 10: Relative difference in prediction due to change in controlled conditions/input should be considered as an additional metric to test the robustness of prediction in combination of other approximated ground truth-based options.

Proposal 11: RAN4 should consider relaxing known conditions for TCI states (Clause 8.10.2 of TS 38.133) for BM-Case1 and BM-Case2. Two possible approaches for RAN4 to relax known TCI states conditions:

- The TCI state switch command is received within a delay X*1280 ms upon the last transmission of the RS resource for beam reporting or measurement or prediction.

- FFS: Further analysis on defining X values.

Proposal 12: RAN4 to further discuss the impact of inference delay on the prediction delay requirement for BM-Case1.

Proposal 13: RAN4 to discuss whether the legacy measurement delay requirement should be increased for BM-Case2.

Proposal 14: RAN4 should consider the impact in RAN4 requirements for the Evaluation period TEvaluate_CBD_CSI-RS due to the presence of predicted beams.

Proposal 15: For UE-side model in beam management use case, the LCM operations are based on CSI report configuration, in our view, the delay components of LCM operations should be considered for different CSI-report configurations (i) periodic CSI report (ii) semi-persistent CSI report and (iii) aperiodic CSI report for both BM-Case1 and BM-Case2.

Proposal 16: RAN4 should consider the Table below as a baseline for further discussion on LCM operations related RRM core requirements.

Table : Delay components related LCM operations for different CSI-report configurations

| CSI-Report Configuration | Activation | Switching | Fall-back (From AI/ML BM to legacy) |
| --- | --- | --- | --- |
| Periodic CSI-reporting | Delay from RRC Reconfiguration Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay from RRC Reconfiguration Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Semi-persistent CSI-reporting | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay for activating semi-persistent CSI resource set Delay for selecting of semi-persistent CSI resource set Delay from UE processing related to capability report Delay due to CSI-RS measurement period |
| Aperiodic CSI reporting | Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report (Optional) Delay related to associated ID Delay due to CSI-RS measurement period | Delay from UE processing related to capability report Delay due to CSI-RS measurement period |

Proposal 17: RAN4 should consider the impact in RAN4 requirements for performance monitoring window when UE reports performance metric or event to the NW.

Proposal 18: According to RAN2, when an active beam prediction configuration becomes inapplicable, the UE should report Set B measurements corresponding to the newly inapplicable. RAN4 should further study

- delay conditions related inapplicability reporting and

- beam failure evaluation period due to inapplicable functionality.

Proposal 19: RAN4 to discuss and explore the feasibility of generating Set A and Set B beams, during the conformance testing of the AI/ML-BM use case, by selecting the number of probes and controlling the transmit power of each probe. How to decide or select the required Tx power of the selected probes is FFS.

Proposal 20: RAN4 to discuss and decide the contents of pre-alignment check which needs to be performed between the TE and DUT before the exact start of the conformance test.

Proposal 21: RAN4 to discuss and explore the feasibility of performing beam sweeping in multiple iterations during the conformance testing of AI/ML based BM use case.

Proposal 22: For the verification/testing of generalization related aspects in RAN4 for AI/ML BM, RAN4 should define different scenarios based on parameters listed in Table below:

Table: Parameters for Generalization Scenarios

| Parameters | Description |
| --- | --- |
| UE Speed | Slow / Medium / Fast |
| SINR (Deployment Scenario) | Good / Bad Radio conditions |
| Propagation Model | TDL/CDL |

Proposal 23: RAN4 needs to design a new metric, indicative of generalization capabilities of AI/ML BM functionality, to verify the generalization performance of the functionality in different scenarios.

- References

- R4-2502856, “WF on the requirements for AI/ML air interface”, Qualcomm Incorporated, RAN4#114 meeting, 17th – 21st February 2025, Athens, Greece.

- RP-250792, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm Incorporated, RAN#107 meeting, 12 – 14 March 2025, Incheon, S. Korea.

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- Annex I (LLS assumptions for BB error modeling)

Table 3. Baseline LLS assumptions for AI/ML in beam management evaluations.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30GHz. |
| Subcarrier spacing | 120kHz |
| Channel model | AWGN channel |
| BS antenna configurations | Option 2: 1 Tx antenna |
| BS antenna element radiation pattern | Option 2: N/A |
| BS antenna height and antenna array down-tilt angle. | Option 2: N/A |
| Beamforming characteristic of the BS pattern | Option 2: N/A |
| UE antenna configurations | Option 2: 2 Rx |
| UE antenna element radiation pattern | Option 2: N/A |
| UE moving speed | 3km/h |
| Reference signal | CSI-RS |
| DRX | No |
| Number of samples in L1 averaging | [3, 5] |

Table 4. Evaluation of BB and RF measurement error for UEs in high and SNR range with BB error model: LLS (# samples = 5, SINR = 0 dB).

| Measurement error |  | KPI2: L1-RSRP difference (Low SNR UEs) | KPI2: L1-RSRP difference (All UEs) | KPI2: L1-RSRP difference (high SNR UEs) | KPI1: Top1/1 (Low SNR UEs) | KPI1: Top1/1 (All UEs) | KPI1: Top1/1 (High SNR UEs) | KPI1: Top3/1 (Low SNR UEs) | KPI1: Top3/1 (All UEs) | KPI1: Top3/1 (High SNR UEs) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BB | RF |  |  |  |  |  |  |  |  |  |
| No BB error | No RF error | 7.84 | 5.63 | 0.55 | 66.35 | 83.78 | 92.53 | 94.17 | 96.48 | 99.04 |
| LLS (# samples = 5, SINR = 0 dB). | Gaussian with ±4.5 dB error margin (Independent for each TX beam) | 10.71 | 12.81 | 8.11 | 56.55 | 71.23 | 75.8 | 88.49 | 92.47 | 95.63 |
|  |  | 2.28 | 7.18 | 7.56 | 9.8 | 12.55 | 16.73 | 5.68 | 4.01 | 3.41 |
|  | Gaussian with ±3 dB error margin (Independent for each TX beam) | 9.97 | 9.98 | 2.68 | 60.47 | 76.94 | 83.85 | 91.48 | 94.78 | 97.65 |
|  |  | 2.13 | 4.35 | 2.13 | 5.88 | 6.84 | 8.68 | 2.69 | 1.7 | 1.39 |
|  | Uniform with ±4.5 dB error margin (Independent for each TX beam) | 10.53 | 13.73 | 12.18 | 52.4 | 68.34 | 72.82 | 86.08 | 91.21 | 94.49 |
|  |  | 2.69 | 8.10 | 11.63 | 13.95 | 15.44 | 19.71 | 8.09 | 5.27 | 4.55 |
|  | Gaussian with ±4.5 dB error margin (Same for all TX beams) | 8.68 | 5.79 | 0.54 | 67.53 | 83.71 | 92.69 | 94.14 | 96.6 | 98.95 |
|  |  | 0.84 | 0.16 | 0.01 | 1.18 | 0.07 | 0.16 | 0.03 | 0.12 | 0.09 |