3GPP TSG-RAN WG4 Meeting #114bis	R4-2504232

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On RAN4 Impacts for RRM measurement prediction in AIML Mobility

Agenda item:	7.21.2.1

Document for:	Discussion

Introduction

The discussions on AI/ML Mobility continued in RAN4#114 in the scope of Rel-19 SI ‎[1] and ‎[2]. Agreements achieved during this meeting are summarized in the WF ‎[3].

Some of the issues related to AI/ML Mobility RRM measurement prediction use case require further discussion. In this paper, we provide some additional views on issues related to RRM requirements, as listed below:

- Performance metrics

- Ground truth definition for RSRP accuracy for FR1

- Impacts on Core requirements

- Impacts on beam/cell level measurements

- Impacts of measurement errors

- Impacts from observation and prediction windows

- Inter-frequency prediction scenario

Measurement event prediction and Testability related aspects are discussed in our accompanying papers ‎[4] and ‎[5].

Discussion

Performance Metrics

The following scenarios have been prioritized as high priority in RAN2:

| Scenario | Prioritization | Evaluation scenario combination | Objective | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | Measurement reduction | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | Measurement reduction | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | Mobility optimization | Intra-cell |

Relative Accuracy

In the RAN4#114 meeting, the following agreement was made on relative RSRP accuracy.

Issue 2-1-1a: Relative RSRP Prediction Accuracy Agreement: - Predicted relative RSRP accuracy = (reported predicted RSRP of cell 1/beam1 – reported RSRP of cell 2/beam2) – (ground truth of RSRP of cell 1/beam1 – ground truth of RSRP of cell 2/beam2) - FFS: Beam 1 and Beam 2 can be from same cell or different cells. - FFS: whether ground truth can be reported or ideal - FFS: whether the reported RSRP of cell2/beam2 can be predicted or measured or a combination of both predicted and measured samples.

In the following, we shed light on relative RSRP agreement from the perspective of each scenario (2, 3, 4).

Scenario-3: Inter-cell Frequency Domain

In a cluster-based approach, input of ML model/functionality can be coming from different cells. Hence beam measurements can be coming from different cells as well. On the other hand, non-clustered case with, e.g., 2 cells, input of ML model can be of one cell and output could be of second cell.

Beam 1 and Beam 2 are to be preferred from different cells to align with legacy schemes.

Compatibility with legacy scheme can help smooth transition, e.g., it can use predefined parameters (CCSF, measurement gap configurations). In addition, a better relative RSRP accuracy can be calculated by utilizing the beams of different cells, which ultimately helps in better decision making (for instance HO decisions etc.).

In the following part, we provide our views on relative RSRP accuracy for two scenarios, i.e., time-domain case A and time-domain case B.

Scenario-2: Time domain case B

An example of Case B is depicted in Figure 1. Briefly, in this case, there is measurement reduction as specific measurements periods are skipped by the UE and the L3 cell-level measurements of skipped measurement periods are predicted using an ML by exploiting OW, or derived from predictions. For example, based on the past two observations, future two RSRP value are predicted.

Figure 1: Scenario 2 - Time domain case B RRM measurement prediction

The measurement reporting accuracy requirements are evaluated over several reported measurements. In time domain case B, this means that the reported measurements evaluated in the requirement may include both actual and predicted measurements, as shown in Figure 1. The ratio of actual measurements and predictions depends on the configured Measurement Reduction Ratio in Time (MRRT). Different filters, for example, L1 and L3, can also make the distinction between actual and predicted measurements less clearly defined. Therefore, in case B, the measurement accuracy metric should not need to distinguish between measured and predicted measurement periods; they must be within a threshold requirement.

The existing measurement reporting accuracy requirement metrics are evaluated over several measurements; in time domain case B, the evaluation may include both actual and predicted measurements.

In case of evaluating measurement reporting accuracy, both measured and predicted measurements periods, or a mix of the two, are evaluated the same way.

In case of relative accuracy metric, one or both measured cells may be configured with time domain case B measurement reduction. In either case, the measurements of the two cells for a given measurement period may contain two actual measurements, a prediction and a measurement, or two predictions (in case reduction was configured in both cells). Furthermore, since the relative accuracy requirement is also evaluated over several measurements, several and potentially different prediction and/or actual measurement pairs are evaluated to compute the metric.

In case of relative accuracy metric, several and potentially different prediction and/or actual measurement pairs are evaluated to compute the metric.

The existing relative accuracy metric can be applied to the relative accuracy of reported measurements with temporal domain case B RRM measurement prediction.

Figure 2: 3GPP TS 38.300 Measurement Model.

In relative RSRP accuracy, the reported predictions could be filtered L3 RSRP values. Hence, ML output could be a filtered L3 value, that is, “Point C” indicated in Figure 2. Besides, it can also be “Point D” shown in Figure 2. Considering this possibility, the inputs and outputs of ML model are depicted in Figure 3. However, training of ML model could be challenging due to exploiting filtered data for training. This may raise various questions. For example, what filtering must be applied on the training data? Besides, there is additional training effort required on the UE, which depends on UE capabilities. This may also restrict configuration options at the UE for filtering.

Figure 3: Input and outputs for unfiltered and filtered prediction. Details of B/C/D are provided in Figure 2.

L3 filtering prediction can limit UE configuration and correspondingly additional efforts required to train ML model.

RAN4 should wait for RAN2's agreement on the progress of predicting filtered RSRP values C or D, before proceeding further.

On the other hand, prediction of unfiltered RSRP values, that is, “B” in Figure 2 can be a lot easier due to many reasons. First, UE can be configured freely for desired legacy L3 filtering once the unfiltered prediction is done. Also, UE can be configured to send values without filter coefficients. Second, unfiltered prediction supports current test setup. Due to these reasons, unfiltered prediction of RSRP values is a preferred option. The progress in RAN4 can be developed once agreement in RAN2 is done.

Unfiltered prediction can be beneficial as UE can be configured freely for any kind of filtering coefficients. Also, unfiltered prediction supports current test setup.

Unfiltered prediction is a preferable option due to flexible filtering configuration.

RAN4 to wait for RAN2 agreement on how filtering is implemented.

Scenario-4: Time domain case A

An example of time domain Case A is depicted in Figure 4. In summary, the objective is to optimize mobility performance instead of measurement reduction. In this use case, no measurement periods are skipped and predicted future measurements are used in mobility optimization decisions. The required accuracy of predicted measurements depends on the optimization actions they are used in.

Figure 4: Scenario 4 - Time domain case A RRM measurement prediction

Motivated by relative RSRP accuracy, in time domain case A, RSRP value of first cell (serving cell) could be predicted, RSRP value of second cell (target cell) could be measurement. Hence, relative RSRP accuracy can be calculated accordingly. Besides, RSRP values of both cells could be predicted RSRP; thereby, relative accuracy could be between two predictions.

In time-domain case A, predictions for two cells can also be compared for relative RSRP accuracy. Besides, cell 1 could be predicted and cell 2 could be measurement and vice versa.

In time domain case A, we might be comparing predictions of two cells for relative RSRP accuracy.

  - Ground truth definition for RSRP accuracy for FR1

Ground Truth

In the RAN4#114 meeting, the following agreements were made for FR1 Ground truth definition for RSRP accuracy:

Issue 2-1-2: Ground Truth Definition for RSRP accuracy Agreement: - The way the ground truth is extracted in the testing may differentiate between FR1 and FR2.

Issue 2-1-2: Ground Truth Definition for RSRP accuracy - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list

Alternative 1, the ground truth for FR1 can be derived by TE based on the transmitted or received power:

This is aligned with the legacy requirement for L3 RSRP accuracy. However, the legacy test is based on AWGN channel model. The AWGN channel is a deterministic channel, where signal power will be constant for both spatial and time domain.

For AIML Mobility, the fading channel is the preferred. The fading channel is a random channel, where variation in both spatial and time domain is included. The calculation of transmitted/ received power i.e. ideal L3-RSRP will be challenging with variations in time domain.

To support TE to calculate the ideal L3-RSRP, it will require additional considerations like L1/ L3 filtering configuration and L1 filtering algorithm is a UE implementation aspect. Hence it will be challenging to define the requirements where the UE has to provide the implementation specific information to facilitate TE to derive the ideal L3-RSRP.

Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE.

Alternative 2, the ground truth for FR1 is derived based on the measurements reported by UE based on certain conditions:

With this approach, there is no complexity of considering the channel variations and the parameters that are UE implementation specific since these would be considered at UE while reporting the measurements. However, there are few points to consider.

For temporal domain case A, measurement is always performed (i.e. there is no measurement reduction), hence UE can always report the measured values that will serve as ground truth.

For temporal domain case B, few measurements are skipped in the observation window. Hence the continuous measurement values will not be available at UE to report to TE. However, there are mechanisms to overcome this issue during testing where UE can perform legacy measurements and AIML predictions parallelly and report to TE depending upon the UE capability. Another method is to perform the test with iteration 1 configured for legacy method of measurement reporting (serves as ground truth) and next iteration of test can be configured for AIML predictions under same channel conditions in both the iterations.

Alternative 2 where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered while reporting the measurements from UE.

Other aspects that may impact the FR1 ground truth derivation is listed in the below table:

Different issues that may impact the FR1 ground truth derivation are listed in the table below:

|  | Issues | Impact to Alternative 1 | Impact to Alternative 2 |
| --- | --- | --- | --- |
| 1. | The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction | YES. TE may not be aware of the exact time period of the prediction performed at UE. This may lead to inaccuracies of ground truth derivation at TE due to dynamic channel conditions | NO |
| 2. | Impact of L3 filtering | L3 filtering methods, filtering co-efficient are configurable parameters, hence known to TE that can be utilized for deriving the ground truth. However, this may increase the complexity at TE when compared to legacy test setup. | NO |
| 3. | Number of samples of L1 filtering | The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of ground truth at TE. | NO |
| 4. | Timing window, if it exists, where UE can’t measure the ground truth in Alt2 | NO | Depending upon the UE capabilities, some UEs may not be able to measure and predict at the same time. However, this can be handled during testing as described above under Alternative 2 description. |
| 5. | Channel condition | With fading channel, the dynamic channel variations in addition to L1/L3 filtering will have to be considered at TE for deriving the ground truth which may not be feasible due to proprietary algorithms used at UE. | NO |
| 6. | Avoid UE cheat in the test | NO | UE cheating can be an issue in this alternative |
| 7. | S(I)NR variations in the test at different time instances (for Alt2) | NO | One option is to measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR. |

Alternative 2 is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

Impacts on Core requirements

RRM Measurement prediction use case, due to prediction in time or frequency domain (high priority scenarios), the impact on measurement period requirements needs to be further analyzed.

The prediction delay may include:

- Measurement for prediction, for this delay the legacy measurement delay can be reused.

- Inference delay: this may include UE processing delay based on UE capabilities.

The above factors may be particularly impacting the measurement delay for the following cases:

- Temporal domain Case B, where some measurements are skipped, and the predictions are made instead of those skipped measurements. Therefore, delays for predictions need to be further studied.

- Frequency domain prediction, in this case, predictions on one frequency will depend on the measurements from another frequency. Therefore, delays for predictions need to be further studied.

Temporal domain Case A doesn’t need new prediction delay requirements since legacy measurement delay requirements should be sufficient.

RAN4 to further study the impact on legacy measurement delay requirements for Temporal domain Case B and Frequency domain cases.

Impacts on beam/cell level measurements

In addition, as outlined in below from RAN4#112bis, further investigation is needed on the performance metrics used to evaluate the effectiveness of AI/ML measurement predictions, as discussed in the previous meeting.

Issue 2-1-1: Performance Metrics/KPIs for Measurement Prediction use case Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

Accordingly, RAN4 has agreed to focus on studying L3 cell-level RSRP (Point C) for evaluating measurement prediction. Additionally, two other options—L1 beam-level (Point A1) and L3 beam-level (Point E)—remain open for further discussion. For L1 measurements, as outlined in sub-use cases 1 and 3, the UE can either derive L3 cell-level measurements from predicted L1 measurements or directly predict cell-level measurements based on actual L1 measurements. Since L3 cell-level measurements and predictions are expected to be reported by the UE or used by the network for RRM and handover decisions, it is logical to consider these as the primary evaluation point for performance metrics. As L3 beam-level measurements could further optimize mobility, they can be examined after the study of cell-level measurements or undergo additional evaluation by RAN2.

Additionally, considering both L3 beam-level and L3 cell-level measurements, RAN2#127 and RAN2#127bis have reached agreements prioritizing L3 cell-level measurements for simulation and performance evaluation.

RAN2#127 FS_NR_AIML_Mob - Release 19 Agreements - Companies can consider to do L3 filtered beam level results for any of this cases. L3 filtered beam level prediction cases are lower priority. - Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. - Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. - Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. - If companies do L3 filtered beam level prediction simulations, they should focus on FR2-to-FR2 intra-frequency temporal domain prediction case A

RAN2#127bis FS_NR_AIML_Mob - Release 19 Agreements - To keep two filtering options on the table and up to company to report. - One fixed sampling period of FR2 is introduced for L1/L3 filtering option 1 to replace existing one i.e., 20ms. The detail value is FFS. - In the definition of 3 RRM sub-cases, all cell level measurement result(s) refers to L3 filtered cell level measurement

In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM.

RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

For L3 cell-level RSRP (Point C) prediction, as agreed in RAN4, measurement prediction can be based on three sub-use cases: (i) predicted L1 beam-level measurement results derived from actual L1 measurements (Sub-use case 1), (ii) actual L3 cell-level measurement results (Sub-use case 2), or (iii) actual L1 beam-level measurement results (Sub-use case 3). Since the accuracy of AI/ML model outputs is highly sensitive to the choice of sub-use case as well as the quality, quantity, and characteristics of the inputs, it is essential to study and potentially prioritize one of the agreed sub-use cases. Based on system-level simulation evaluations conducted in RAN2, the majority of companies have indicated that cell-level prediction based on actual L3 cell-level measurements provides the highest accuracy in Mean Absolute Error (MAE) performance.

The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement.

According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy in many companies’ simulations.

RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2).

Impacts of measurement errors

The RAN4#113 meeting has discussed the impact of measurement errors for AI/ML Mobility evaluation and identified a number of options on measurement errors.

Issue 2-1-3: Impacts of measurement error on prediction accuracy - Candidate options: - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase - Option 2: RAN4 to not study the impact of measurement error on the prediction accuracy during SI phase - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4) - Option 4: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. Way Forward: Many companies are supporting Option 4. Companies are encouraged to bring proposals on what input RAN4 should provide to RAN2 to consider impact of error on measurements used for prediction on the accuracy of the predicted measurement/event

Also, in RAN4#114, it has been agreed that RF and baseband errors on the measurements would have an impact on the measurement prediction accuracy.

Issue 2-1-4: Impacts of measurement error on prediction accuracy Agreement: For AI based RRM measurement prediction in the SI, while RAN4 has not agreed yet on the exact error model(s) in the SI phase, RAN4 think RF and baseband errors on the measurements would impact the prediction accuracy.

In mobility scenarios, it is unavoidable that measurement errors will occur when using data as input or ground truth for AI/ML models. These errors can significantly impact network performance by causing inaccurate predictions, which consequently can affect mobility management, handover optimization, and RRM.

In the SI phase, the potential performance gain using AIML-based mobility has been evaluated based on RAN2 metrics such as a mean square error without having an assumption on measurement error. When initiating the WI phase, RAN4 could evaluate the performance of the AIML mobility with having an assumption on the measurement error based on RAN2 and/or RAN4 metrics, thus measuring and understanding the impact on the prediction accuracy. Therefore, it is essential for RAN4 to have a common understanding on the cause and modeling of the measurement error.

When conducting simulations, it is imperative to consider measurement errors to define the inputs of AI/ML models. One option involves utilizing a Gaussian distribution, similar to the RAN4 measurement error requirements for both RF and baseband. The parameters of an error distribution can be selected to capture the characteristics of normal or extreme conditions for the simulation assumption. Alternatively, the different distributions for baseband and RF can be assumed, as discussed in the RAN4 AI/ML for Beam Management. In this scenario, baseband errors may be generated using a Gaussian distribution, or they can be modeled through link-level simulations. For RF errors, generation can occur via a uniform or Gaussian distribution, or they can be assigned a constant value. Moreover, when incorporating RF errors in simulations, RAN4 can also consider whether each TX beam has the error independently.

RAN4 should study the measurement error modeling by using the tables below as a baseline.

Table 1 RF and BB error modelling principle

| RF and BB modeling principle | Supporting companies |
| --- | --- |
| Both RF and baseband errors follow a Gaussian distribution. | X, Y, Z, U ... |
| Baseband and RF errors follow different distributions. | A, B, C, D … |

Table 2 BB error modeling

| Baseband error modeling | Supporting companies |
| --- | --- |
| Gaussian distribution | A, B … |
| Link-level simulation results | C, D … |

Table 3 RF error modeling

| RF error modeling | Supporting companies |
| --- | --- |
| Uniform distribution | A … |
| Gaussian distribution | B … |
| Truncated Gaussian distribution | C … |
| Fixed value | D ... |

Table 4 RF error application

| How RF error is applied | Supporting companies |
| --- | --- |
| Independent for each Tx beam | A … |
| same for each Tx beam | B … |
| Independent for each module/panel | C … |
| same for each module/panel | D ... |

Impacts from observation and prediction windows

Issue 2-1-9: Impacts from observation and prediction windows Agreement: - RAN4 to consider the impact of UE speed, MRRT and OW and PW lengths on prediction accuracy requirements for temporal predictions. - RAN4 to also consider number of measurements performed during OW and its impacts on the prediction accuracy.

For temporal domain prediction case B, the UE is not required to measure all the time and may use instead predicted measurements. The degree of measurement reduction can be selected through the Measurement reduction rate in temporal domain (MRRT) parameter. However, for a given MRRT value, different possible ways can be employed to achieve the targeted measurement reduction level. As shown in the example below, 50% MRRT can be reached with two different skipping patterns: either completely replacing consecutive measurement slots with predictions in the PW (example 1) or alternating measured and skipped slots within the OW (example 2).

Figure 5: Example of MRRT 50% scenario 2 with different skipping patterns

For the testing, the MRRT can be set to [50, 60, 66, 75, 80] % and UE speed can be set to [30, 60, 90, 120] km/h, resulting in a total number of 20 combinations. In addition, for each MRRT value, different skipping patterns may be also considered, which may enable to particularly test the impact of reduction slots placement within the OW on the performance accuracy. Therefore, for a given MRRT value, the skipping pattern should be clearly specified by the TE to clearly indicate to the DUT when to measure and when to skip and ensure collecting he required number of measurement samples to use as input for the ML model inference.

Therefore, to test a given scenario with selected OW and PW, the number of possible combinations with MRRT and related skipping patterns as well as UE speeds, should be considered and eventually reduced to optimize the testing process.

The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT, accounting for multiple possible skipping patterns for a given MRRT value.

For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as selected skipping patterns for a given MRRT value.

Inter-frequency prediction scenario

One of the high-priority study items for AI/ML in NR inter-cell mobility is inter-cell inter-frequency measurement prediction. Inter-frequency measurements require configuring measurement gaps, during which the UE must temporarily stop communicating with the serving cell to measure other frequencies. This process can lead to service interruptions and increased power consumption. AI/ML techniques can help reduce the number of inter-frequency measurements a UE needs to perform, thereby minimizing the need for continuous measurement and enhancing UE energy efficiency.

By utilizing measurement data from two frequency layers, an ML model can be trained to understand the correlation between radio measurements across these layers. Specifically, the model can either predict first-layer measurements based on second-layer measurements or determine if a UE connected to a macro layer can connect to a micro layer. This may involve a clustering approach, where measurements from multiple cells serve as inputs for prediction. A time series of measurements, along with additional inputs, may also be considered. Consequently, for inter-frequency measurement prediction, both the input data and model performance metrics (e.g., accuracy) differ from those used in temporal domain predictions

The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions.

RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

Conclusion

In this paper we provide our further view on the RRM Measurement prediction use case and RRM requirements to be studied in RAN4 working group for this use case.

The following Observations and Proposals were made:

Proposal 1: Beam 1 and Beam 2 are to be preferred from different cells to align with legacy schemes.

Observation 1: The existing measurement reporting accuracy requirement metrics are evaluated over several measurements; in time domain case B, the evaluation may include both actual and predicted measurements.

Proposal 2: In case of evaluating measurement reporting accuracy, both measured and predicted measurements periods, or a mix of the two, are evaluated the same way.

Observation 2: In case of relative accuracy metric, several and potentially different prediction and/or actual measurement pairs are evaluated to compute the metric.

Proposal 3: The existing relative accuracy metric can be applied to the relative accuracy of reported measurements with temporal domain case B RRM measurement prediction.

Observation 3: L3 filtering prediction can limit UE configuration and correspondingly additional efforts required to train ML model.

Proposal 4: RAN4 should wait for RAN2's agreement on the progress of predicting filtered RSRP values C or D, before proceeding further.

Observation 4: Unfiltered prediction can be beneficial as UE can be configured freely for any kind of filtering coefficients. Also, unfiltered prediction supports current test setup.

Proposal 5: Unfiltered prediction is a preferable option due to flexible filtering configuration.

Proposal 6: RAN4 to wait for RAN2 agreement on how filtering is implemented.

Observation 5: In time-domain case A, predictions for two cells can also be compared for relative RSRP accuracy. Besides, cell 1 could be predicted and cell 2 could be measurement and vice versa.

Proposal 7: In time domain case A, we might be comparing predictions of two cells for relative RSRP accuracy.

Observation 6: Alternative 1 where the FR1 ground truth can be derived by TE based on the transmitted or received power (legacy approach) is not suitable for AIML Mobility due to the fading channel and due to the L1 filtering which is UE implementation specific and not known to the TE.

Observation 7: Alternative 2 where the FR1 ground truth is derived based on the measurements reported by UE based on certain conditions, is more suitable for AIML Mobility since the channel variations and L1 filtering are already considered while reporting the measurements from UE.

Proposal 8: Different issues that may impact the FR1 ground truth derivation are listed in the table below:

|  | Issues | Impact to Alternative 1 | Impact to Alternative 2 |
| --- | --- | --- | --- |
| 1. | The information at TE side, on the timing UE performs and/ or finishes the measurement and/or prediction | YES. TE may not be aware of the exact time period of the prediction performed at UE. This may lead to inaccuracies of ground truth derivation at TE due to dynamic channel conditions | NO |
| 2. | Impact of L3 filtering | L3 filtering methods, filtering co-efficient are configurable parameters, hence known to TE that can be utilized for deriving the ground truth. However, this may increase the complexity at TE when compared to legacy test setup. | NO |
| 3. | Number of samples of L1 filtering | The L1 filtering method, filtering co-efficient and the number of samples for filtering are not known by the TE since it is UE proprietary implementation, hence it will impact the derivation of ground truth at TE. | NO |
| 4. | Timing window, if it exists, where UE can’t measure the ground truth in Alt2 | NO | Depending upon the UE capabilities, some UEs may not be able to measure and predict at the same time. However, this can be handled during testing as described above under Alternative 2 description. |
| 5. | Channel condition | With fading channel, the dynamic channel variations in addition to L1/L3 filtering will have to be considered at TE for deriving the ground truth which may not be feasible due to proprietary algorithms used at UE. | NO |
| 6. | Avoid UE cheat in the test | NO | UE cheating can be an issue in this alternative |
| 7. | S(I)NR variations in the test at different time instances (for Alt2) | NO | One option is to measure at higher SINR level for more accurate ground truth and the SINR may be lower during the prediction with reference to the average SINR. |

Proposal 9: Alternative 2 is the preferred option for ground truth definition for RSRP accuracy even though there are inaccuracies in the UE measurements, since it is more feasible option for the dynamic channel conditions.

Observation 8: Temporal domain Case A doesn’t need new prediction delay requirements since legacy measurement delay requirements should be sufficient.

Proposal 10: RAN4 to further study the impact on legacy measurement delay requirements for Temporal domain Case B and Frequency domain cases.

Observation 9: In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM.

Proposal 11: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and after that L3 beam-level prediction (Point E).

Observation 10: The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement.

Observation 11: According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy in many companies’ simulations.

Proposal 12: RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2).

Proposal 13: RAN4 should study the measurement error modeling by using the tables below as a baseline.

Table 1 RF and BB error modelling principle

| RF and BB modeling principle | Supporting companies |
| --- | --- |
| Both RF and baseband errors follow a Gaussian distribution. | X, Y, Z, U ... |
| Baseband and RF errors follow different distributions. | A, B, C, D … |

Table 2 BB error modeling

| Baseband error modeling | Supporting companies |
| --- | --- |
| Gaussian distribution | A, B … |
| Link-level simulation results | C, D … |

Table 3 RF error modeling

| RF error modeling | Supporting companies |
| --- | --- |
| Uniform distribution | A … |
| Gaussian distribution | B … |
| Truncated Gaussian distribution | C … |
| Fixed value | D ... |

Table 4 RF error application

| How RF error is applied | Supporting companies |
| --- | --- |
| Independent for each Tx beam | A … |
| same for each Tx beam | B … |
| Independent for each module/panel | C … |
| same for each module/panel | D ... |

Observation 12: The number of possible test cases can increase considerably for a set of selected OW, PW and MRRT, accounting for multiple possible skipping patterns for a given MRRT value.

Proposal 14: For temporal domain prediction case B, RAN4 to agree on pre-selected reference testing scenarios relating to OW, PW and MRRT as well as selected skipping patterns for a given MRRT value.

Observation 13: The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions.

Proposal 15: RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

- References

- RP-242393, “Revised SID on AIML for mobility in NR”, OPPO, RAN#105 meeting, 9 – 12 September 2024, Melbourne, Australia.

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR; (Rel. 19)

- R4-2502594, WF on AI/ML Mobility, Nokia, RAN4#114, Athens, Greece, 17th – 21st February, 2025.

- R4-2504233, “Study of RAN4 impacts for measurement event prediction”, Nokia, RAN4#114bis meeting, 7-11 april 2025, Wuhan, China.

- R4-2504234, “Study of testability and interoperability on AI/ML Mobility”, Nokia, RAN4#114bis meeting, 7-11 april 2025, Wuhan, China.