3GPP TSG-RAN WG4 Meeting #114bis	R4-2504234

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	Testability and Interoperability Issues for AIML Mobility

Agenda item:	7.21.3

Document for:	Discussion

Introduction

In RAN4#114bis meeting, following aspects are to be discussed during this meeting.

- RAN4 impacts for RRM measurement prediction

- RAN4 impacts for measurement event prediction

- Testability and Interoperability Issues in AI/ML Mobility

In this contribution we provide our views and discuss different aspects of testability and interoperability issues in RAN4 for AI/ML Mobility.

Discussion

This document addresses the Testability and interoperability issues related to AI/ML mobility SI in RAN4 based on the agreements and the recommended way forward of the issues from the RAN4#113 meeting.

Following topics are covered:

- Testing setup

- Prediction consistency in time domain

- Testing aspects of inter-carrier scenarios

- Testing of generalization aspects

- Principle of FR2 testing

Testing setup

Agreement from RAN4#113 meeting ‎[1]:

- Agreement - RAN4 to further discuss: - Whether and how legacy testing setup for L3 measurements can be reused for FR1 and FR2

Considering measurement prediction use case within AI/ML mobility, the testing will involve either L1 or L3 measurements or both, either measured or predicted, on a preselected set of cells / beams (either intra or inter frequency). The considered trained ML model outputs predicted measurements on a preselected time horizon. It is therefore necessary to map the measured values to the predicted value with the selected prediction horizon, accounting for the standardized filtering procedure when estimating L3 measurements based on L1 or L3 measurements.

RAN4 should define test setups considering L1 or L3 measurements from serving and neighbouring cells with related configuration (inputs, prediction horizon).

RAN4 should consider the following additional aspects for AI/ML mobility use case testing compared to the legacy testing:

- L3 filtering

- channel model (CDL/ TDL etc.,)

- number of AoAs may be different from BM use case due to higher number of cells and beams requirements in AI/ML mobility use case.

- maximum number of cells

- maximum number of Tx beams

RAN4 should consider the additional aspects for testing (compared to the legacy test) in AI/ML mobility such as L3 filtering, channel model, maximum number of cells, Tx beams and the number of AoAs.

In the RAN4#112bis meeting, the following agreements are captured in the way forward ‎[2] as

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Agreement: - Scenario 2, 3 and 4, the 3 cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

In case of intra cell (i.e. scenarios 2 and 4) measurement and prediction considering sub-use-case 2, the legacy test setup can be used as baseline. However, for the case where the measurement is performed over one cell and the predictions are required for multiple cells/ clusters and for the inter-frequency inter-cell scenario (i.e. scenario 3), there is a need for higher number of cells in the test setup. Hence RAN4 should consider further study on the maximum number of cells required for the test.

RAN4 should consider higher number of cells for inter-frequency inter-cell scenario (i.e. scenario 3).

Channel conditions may vary frequently/ randomly in real time environments. Test setup should be able to emulate the variations of the radio channel for evaluating the prediction accuracy of the models/ functionalities. Hence RAN4 should further study on identifying the suitable fading channel models like CDL/ TDL for FR1 testing.

For FR1, emulation of the radio channel variations is important for evaluating the prediction accuracy of the AIML models/ functionalities. Hence RAN4 should consider fading channels (like CDL/ TDL) for testing.

One of the considerations for RAN4 in establishing RRM requirements and defining test cases for measurement prediction is the effect of L3 filtering and the number of beams utilized within test environments. Currently, a majority of legacy test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero). Additionally, to streamline test configurations, only a limited number of beams are typically simulated in the test setup.

Figure 1: 3GPP TS 38.300 Measurement Model

As illustrated in Figure 1, when L3 filtering is absent, the L1 and L3 beam measurement levels at point A1 align. Furthermore, for L3 cell-level RSRP predictions at Point C, the predicted values may correspond to the L1 and L3 beam-level predictions when a single beam is employed for prediction purposes.

- Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup.

- Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E.

RAN4 to consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases.

Test verification procedure:

The purpose of the test is the verification of mobility accuracy KPIs (Absolute RSRP accuracy and Relative RSRP accuracy) within specified limits (preselected within the testing framework). It is important to mention that the TE can use either both metrics or one of them for the evaluation.

Test parameters: For the temporal domain measurement prediction, single cell is required in the test (however at least two cells may be required for relative RSRP calculation considering beams from different cells). The test parameters are to be selected accounting for (not limited to) the frequency, Bandwidth, transmission power and antenna configuration.

In addition, parameters related to AI/ML model should be specified, namely:

- Observation Window (OW)

- Prediction Window (PW)

- Delay (𝛿)

- Measurement reduction rate in temporal domain (MRRT)

Ground truth extraction

This testing procedure targets AI/ML mobility use case with a focus on FR2-to-FR2 intra-frequency temporal domain case A and FR1-to-FR1 temporal domain case B. Therefore, it is needed to configure the UE channel variations during a predefined time duration, accounting for a given UE speed value.

At each time iteration, the UE radio conditions with regards to the serving cell is set up. This allows to assess RSRP value at each iteration. The TE should configure the DUT to report Predicted and Measured (Ground Truth) RSRP.

Impacts on Testability aspects for Time domain Case A

An example of Case A is depicted in Figure 2. Briefly, in this case, there is no measurement reduction, and the focus is instead to optimize mobility performance based on predictions.

Figure 2: Time domain case A RRM measurement prediction.

The delay parameter, δ, must be configured, which is based on the DUT (UE) capability to predict and measure concurrently or sequentially. However, when the UE does the measurement and prediction sequentially then there will be delay between the time measurement and prediction are reported. Hence, the gNB must account for those delays. In contrast, when the UE can do prediction and measurement concurrently then delay might be small to gather the values at the TE. However, in the latter case, there can be UE processing delays, which we leave to UE implementation details.

- UE can perform prediction and Ground Truth measurements sequentially or concurrently depending on its capabilities.

TE must account for the delays to acquire prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently.

Impacts on Testability aspects for Time domain Case B

In temporal domain case B, a measurement reduction is realized following the parameter MRRT (Measurement reduction rate in temporal domain). However, for a given selected MRRT value (e.g. 50%), different possible skipping patterns can be applied as shown in the example below.

Figure 3.  Example of intra-frequency temporal domain case B with MRRT of 50%

For the testing, the DUT needs to match a skipping pattern for a given configured MRRT value. However, so far, RAN2 does not have yet any agreement related to skipping pattern indication (if it is part of the ML model configuration or not).

- At each iteration, the DUT should measure only at the identified slots following the skipping pattern within the current OW.

- The collected measurement samples are then input to AIML model to output predicted samples in the PW, reported to the TE.

- Additionally, DUT can be configured to measure and report the ground truth measurements within PW.

- At the TE, error (E) is calculated based on the selected evaluation metric (EM).

  - Generally, when E≤τ then it is assumed that test has passed. However, there can be a requirement that error for first EM, E1 (for absolute RSRP accuracy), and error for second EM, E2 (fore relative RSRP accuracy), both must satisfy the condition,  τ, or one of them at least. Further, there can be separate threshold values, τ1, τ2 for E1 and E2, resp.

  - Test procedure can also be looped over multiple iterations following the selected skipping pattern, where ‘Max’ value can be predefined at the TE.

    - When both E1 and E2 are configured by TE, TE calculates E, and then conditions E1 and E2 can be evaluated independently. The test results with respect to E1 and E2 are stored in the TE. The same test can be repeated until the iteration reaches to Max.

    - In addition to the aforementioned testing procedure, if the ratio of tests passed to tests failed has reached to a certain threshold, the testing can be deemed ended. For example, the ratio can be 0.9 or 90% of the tests passed out of the total number of tests.

As RAN2 does not have agreement on the way the skipping pattern information is identified by the UE, two options may be considered:

- Skipping pattern to be part of the ML inference configuration which means that this information is available along with OW and PW parameters

- Skipping pattern is not part of the ML inference configuration: UE may select autonomously the skipping pattern for a given configured MRRT value. However, in this case, RAN4 should agree on test configuration to explicitly indicate to DUT on the skipping pattern to use and identify the measurement slots.

For testing time domain prediction case B, RAN4 to follow RAN2 agreement regarding skipping pattern indication either part of inference configuration or test configuration.

Prediction consistency in time domain

Candidate Options for prediction consistency in time domain captured in the moderator’s summary in RAN4#113 meeting is mentioned below.

Issue 3-1-2: Prediction consistency in time domain - Candidate Options: - Option 1: Model different time-varying characteristics per cell/site due to moving UE trajectories - Option 2: how to introduce controlled randomness and time-domain variation/correlation - Option 3: TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes - Option 4: Consider additional side conditions - Recommended WF - To discuss above listed candidate options.

For inter-cell inter-frequency scenario (i.e., scenario 3) in measurement prediction use case, network assistant information can be different between the BM use case of AI/ML NR for air interface and AI/ML mobility use case. In contrast to the BM use case, the AI/ML mobility use case may have the multiple target cells/sites in the same or different frequency layers.

The moving UE trajectory in the case of AI/ML mobility will cause the time-varying received power and SINR. RAN4 will need to study how to model the different time-varying or non-static characteristics per cell/site. It is also needed to study how to model the different variation per test iteration, Hence, Option 1 should be considered for further study.

Furthermore, the test setup should be able to emulate the randomness and variations of the radio channel for evaluating the prediction accuracy of the models or functionalities. Hence, Option 2 may also be considered for further study.

RAN4 should consider how to model different non-static channel characteristics per cell/site due to moving UE trajectories and to emulate channel variations during the test.

Testing aspects of inter-carrier scenarios

In AI/ML Mobility, in order to train and infer, AI/ML functionalities need a dataset/environment that guarantees certain consistency between primary and secondary carrier components. This constraint is not present in the AI/ML BM use case and therefore is not considered in the scope of ongoing WI. Thus, in the scope of AI/ML mobility SI, RAN4 should consider a method to guarantee the consistency between different carriers.

RAN4 should study how to ensure consistency between different carriers in the context of AI/ML mobility SI.

Testing of Generalization aspects

In the scope of AIML_NR_air WI, the concern regarding overfitting of UE models of a functionality to the test environment, was raised by multiple companies, and companies were encouraged to bring proposals to avoid overfitting issue.  In our view, testing generalization aspects of a functionality provide a rather realistic and feasible solution to the overfitting issue. A UE functionality, if tested in different generalization scenarios, would not be overfitted to a certain test environment, and, therefore, this is an important topic to be discussed. However, RAN2 has not progressed much on the generalization related discussion, and it may be too early to start the discussion on this topic in RAN4.

RAN4 should wait for further progress on generalization discussions in RAN2 before starting any discussion on generalization related topics in RAN4.

Principle of FR2 Testing

In the scope of AI/ML air interface WI, RAN4 is currently studying a test setup for AI/ML BM FR2 OTA testing. The study is considering multiple candidate options for FR2 testing including both single AoA and multiple AoA testing systems. In our view, it is judicious to wait for the conclusion from that study instead of starting a new study in the scope of AI/ML mobility SI. Furthermore, the test setup and the channel model, which are currently under study for AI/ML BM use case in the WI on AI/ML NR for air interface, should be considered as a baseline for the FR2 AIML mobility testing. This would also imply that while selecting a testing setup for AI/ML BM, RAN4 should select a testing setup that could be extended to AI/ML scenarios.

For FR2, the testing setup and channel model in the AI/ML BM use case which is under study should be reused as a baseline for measurement prediction use case testing.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to testability and interoperability of AI/ML Mobility use cases. Specifically, we cover following aspects:

- Testing setup

- Prediction consistency in time domain

- Testing aspects of inter-carrier scenarios

- Testing of generalization aspects

- Principle of FR2 testing

In the paper, the following Observations and Proposals were made:

Proposal 1: RAN4 should define test setups considering L1 or L3 measurements from serving and neighbouring cells with related configuration (inputs, prediction horizon).

Observation 1: RAN4 should consider the additional aspects for testing (compared to the legacy test) in AI/ML mobility such as L3 filtering, channel model, maximum number of cells, Tx beams and the number of AoAs.

Proposal 2: RAN4 should consider higher number of cells for inter-frequency inter-cell scenario (i.e. scenario 3).

Proposal 3: For FR1, emulation of the radio channel variations is important for evaluating the prediction accuracy of the AIML models/ functionalities. Hence RAN4 should consider fading channels (like CDL/ TDL) for testing.

Observation 2: Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup.

Observation 3: Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E.

Proposal 4: RAN4 to consider L3 filtering and multi-beam scenarios while studying the test cases for AI/ML mobility uses cases.

Observation 4: UE can perform prediction and Ground Truth measurements sequentially or concurrently depending on its capabilities.

Proposal 5: TE must account for the delays to acquire prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently.

Proposal 6: For testing time domain prediction case B, RAN4 to follow RAN2 agreement regarding skipping pattern indication either part of inference configuration or test configuration.

Proposal 7: RAN4 should consider how to model different non-static channel characteristics per cell/site due to moving UE trajectories and to emulate channel variations during the test.

Proposal 8: RAN4 should study how to ensure consistency between different carriers in the context of AI/ML mobility SI.

Proposal 9: RAN4 should wait for further progress on generalization discussions in RAN2 before starting any discussion on generalization related topics in RAN4.

Proposal 10: For FR2, the testing setup and channel model in the AI/ML BM use case which is under study should be reused as a baseline for measurement prediction use case testing.

- References

- R4-2420095, WF on AI/ML Mobility, Nokia, 18th – 22nd November 2024, Orlando, USA.

- R4-2416855, WF on AI/ML Mobility, Nokia, Hefei, Anhui, China, 14th – 18th October 2024.