3GPP TSG-RAN WG4 Meeting #114bis	R4-2504246

# Wuhan, China, 7th – 11th April, 2025

Source:	ZTE Corporation, Sanechips

Title:	Discussion on On-demand SSB SCell operation of R19 NES

Agenda Item:	7.25.2.1

Document for:	Discussion

- Introduction

In last RAN4 meeting, around the application of OD-SSB and the requirements impact, warm discussion was held. The following progress was achieved and summarized in [1].

- Sub-issue 1-1 – The length of FMW Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify). - Sub-issue 1-2: What’s the expected minimum interval(T2, sage 4) between multiple FMWs? Agreement: RAN4 to define measurement requirement based on T2. - When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. Option 3: UE is required to perform FMW after T2. - When NW re-activates/re-configures the OD-SSB transmission after T2, UE performs the OD-SSB based fast L3 measurement within FMW - The UE is assumed to meet fast cell identification and measurement. - The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3. - The duration is - Option 1: 5*max(measCycleSCell, DRX cycles) [– OD-SSB periodicity] - Option 2: 5* measCycle when measCycle =160 ms - Other options are not precluded - The activation and deactivation of OD-SSB is fully up to NW. Issue 2-1: (Case 2) OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered Agreement: When NW indicates the OD-SSB transmission for deactivated SCell measurement(Case 2 Scenario 2), - Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation. Issue 2-2: Always-on SSB and OD-SSB time/frequency domain relation Agreement: RAN4 to start from the following scenarios: - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity - The scenarios can be updated based on other WGs’ progress. Issue 2-3: Combine OD-SSB and AO-SSB in measurement Agreement: - In scenario 4, the measurement requirement is defined based on the effective measurement periodicity. - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and - The effective measurement periodicity is half of OD-SSB periodicity - AO-SSB and OD-SSB are on the same frequency with the same spatial relation Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers Agreement: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW - Option 1: The CSSF only counts OD-SSB layers - Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers Agreement: RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement. - FFS multiple OD-SSB carriers Issue 4-1: L1 measurement before SCell activation Agreement: - RAN4 not to support OD-SSB based L1 measurements for deactivated Scell. Issue 4-2: L1 measurement after SCell activation Agreement: - RAN4 to support OD-SSB L1 measurement during and after SCell activation. - When non-DRX is configured, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB - RAN4 to support OD-SSB L1 measurement during SCell activation. - The requirement when DRX is configured. - Based on OD-SSB periodicity Issue 4-3: OD-SSB accuracy Agreement: - The legacy measurement accuracy is applied for OD-SSB based measurement. Issue 6-1: OD-SSB based direct SCell activation Agreement: - RAN4 to define OD-SSB based direct SCell activation at SCell addition requirements. - The details can be discussed in the CR directly. - It doesn’t impact the WI’s completion. Issue 7-2: OD-SSB based Neighbour cell measurement - Scenario 1a: There is OD-SSB neighbor cell in the same MO with legacy serving cell Agreement: - For Scenario 1a: No RAN4 spec. change. - Scenario 1b: Neighbour cell with legacy SSB in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement with legacy SSB. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2. - Scenario 1c: There is OD-SSB neighbor cell in the same MO with OD-SSB serving cell Agreement: - For FR1, follow legacy requirement for neighbour cell measurement. - The agreement can be revisited if not aligned with the further progress in other WGs. - Further discuss for FR2.

In this paper, we provide our further views around the on-demand SSB SCell operation.

- Discussion

- The R19 NES aims to further realize NW energy saving through other solutions differing from R18. Furthermore, the applicable scenarios between R18 and R19 are more or less different. After several meetings’ constructive discussion, around how to apply on-demand SSB, the deactivated SCell measurement and SCell activation procedure were identified as the applicable scenarios. Besides the specific discussion on the two applicable scenarios, some general issues should be determined.

2.1 Consideration on the deactivated SCell measurement for Case 1

- The length of the FMW was determined in last meeting. Besides the length of FMW, RAN4 still discussed the expected minimum interval(T2) between multiple FMWs. The intention of to define the minimum interval between multiple FMWs is to further reduce the length of the subsequent FWM when the OD-SSB is re-activated. if it is assumed that UE could hold the coarse T/F syncronization before enter into the re-activated FMW, then the cell detection could be ignored, as a result, the re-actviated FMW could be shorter than the 1st FWM. Around this case and intention, the minimum interval T2 and the exact UE behavior should be determined.

Figure 1. The example of multiple OD-SSB activcation during the deactivated SCell measurement

When NW re-activated OD-SSB within T2, regarding the UE behavior, the following options were raised in last meeting:

- When NW re-activates the OD-SSB within the T2, - Option 1: UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement. - Option 2: UE is required to perform “fast measurement” without cell search. - The UE is assumed to skip the PSS/SSS detection and/or SSB index acquisition. - Option 3: UE is required to perform FMW after T2.

- It is straightforward that Option 1 is not consistent with the intention of discussion on T2. Just due to NW hopes the UE to perform the fast measurement, NW re-activated OD-SSB after the 1st FMW. Option 1 is against with the NW desire. Between Option 2 and 3, Option 2 is aligned with the intention of further reduction of the re-activated FWM. Option 3 is also accepted, which is to follow the normal UE behavior in 1st FWM. If the whole discussion on T2 is too complicated, we are fine to Option 3, i.e. not introduce any further reduction on the re-activated FWM.

- Another controvercial issue is how to define the starting point of T2. The following options were touched in last meeting:

- The start point of T2 is based on - Option 1: the time point Tend - Option 2: the latter point of L3 measurement reporting for FMW and Tend - Option 3: the latter point of latest L3 measurement reporting before NW re-indicating OD-SSB transmission and Tend - Option 4: the time point of OD-SSB deactivation - Other options are not precluded Where, Tend equals the time point of OD-SSB indication plus Tuncertain +Tidentify applies to option 1,2,3.

- Technically, all the Option 1, 2, and 3 make sense to some extent. But in our opinion, Option 1 is the moset straightforward, why we define Tend as min(Treport, Tidentify), that is just because no matter the UE finished cell identification or the UE reported a L3 result, both of them implies the UE has already known the channel quality of the deactivated SCell. So directly count the T2 from this point, which is already sufficient.

Proposal 1: When multiple OD-SSB activation happens during the deactivated SCell measurement, we propose the following two solutions:

- Solution 1: To simplify the requirements, not to define further enhancement to reduce the re-activated FWM. UE always follow single principle to determine the length of FWM.

- Solution 2: To further reduce the re-activated FWM. The details are as below:

- When NW re-activates the OD-SSB within the T2, UE is required to perform “fast measurement” without cell search, i.e. skip the PSS/SSS detection and/or SSB index acquisition.

- The starting point of T2 is based on the time point Tend, i.e. the end of last FWM.

- Besides the whole picture of UE measurement under Case 1, another hot part is around the CSSF assumption. When both the OD-SSB carrier(s) and legacy SSB carrier(s) co-exist, what is the UE behavior guided by the CSSF assumption.

Sub-issue 3-2: OD-SSB measurement in FMW(Case 1-1): single OD-SSB carrier with multiple legacy SSB carriers Agreement: When NW configures single OD-SSB carrier with multiple legacy SSB carriers, in FMW - Option 1: The CSSF only counts OD-SSB layers - Option 2: The CSSF of OD-SSB carrier counts OD-SSB layers and other layers Sub-issue 3-3: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers Agreement: RAN4 to prioritize single OD-SSB carrier with multiple legacy SSB carriers in deactivated SCell measurement. - FFS multiple OD-SSB carriers

- For the single OD-SSB carrier case, during last meeting, the majority prefer Option 1 so that the 2nd searcher can be exclusively for the single OD-SSB carrier. But one conern is whether there is requirements for all other frequency layer. If no requirements, UE may ignore these measurement within FMW, is it what NW expect? If no, maybe the two options should be indicated by NW, if NW indicates to follow Option 2, which means all carriers(OD-SSB based and legacy SSB based) should be counted equally. From our perspective, we believe such concern could be addressed through NW implementation. If NW expects both the OD-SSB layer and other legacy SSB carriers are measured within the FWM, actually NW could configure the OD-SSB non-overlapping with other legacy SSBs, it is feasible since RAN1 has already approved the enhancement of half frame index in latest RAN1#120 meeting. Therefore, we prefer Option 1.

Agreement(RAN1#120 meeting) - For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC, - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e., - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) - By defining reference point as SFN where AO-SSB burst exists

- For the multiple OD-SSB carrier case, to simplify the case, we can only focus on the non-overlapping multiple OD-SSB in this release, and follow Option 1 for single OD-SSB case to address the non-overlapping multiple OD-SSB case.

Proposal 2: When activated OD-SSB layer and legacy SSB layers co-existence within the FMW,  the CSSF only counts OD-SSB layers to speed up the OD-SSB based measurement. This principle applies to both single OD-SSB layer and multiple but non-overlapping OD-SSB layers.

Another relevant issue is whether to restrict the lower bound of the periodicity of OD-SSB. From our side, we can not see the necessity. Furthermore, if we prioritize the OD-SSB measurement within FMW, to weaken the impact on other legacy SSB layers, it is better to speed up the OD-SSB measurement as much as possible. We prefer not to restrict the lower bound of the periodicity of the OD-SSB.

Proposal 3: Not to restrict the lower bound of the periodicity of the OD-SSB, faster OD-SSB based measurement within FMW could reduce the impact on other legacy SSB layers.

2.2 Consideration on the deactivated SCell measurement for Case 2

After the on-demand SSB be activated, both always-on SSB and on-demand SSB are transmitted. The relationship between the two SSBs in terms of periodicity, time location, frequency and QCL are significant to determine the requirements. Based on RAN1 LS reply [2] , RAN4 initially approved two scenarios as below in last meeting:

Agreement: RAN4 to start from the following scenarios: - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities. - Scenario 4: OD-SSB and always-on SSB are with same frequency, different offset but same periodicities. - At least the offset between OD-SSB and AO-SSB equals the half of AO-SSB periodicity

It should be noted that in the latest RAN1#120 meeting, regarding the frequency relation between OD-SSB and AO-SSB, RAN1 approved some further progress:

Agreement Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, - For the case when the center frequency locations of always-on SSB and on-demand SSB are same, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - From RAN1 perspective, - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification). - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported. - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2 - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted Send an LS to RAN4 to inform them of the above agreement. Final LS in R1-2501633. Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

For the even interval case, RAN4 has already decided to define requirements with the consideration of half periodicity. RAN1 has confirmed the non-even interval case, whether to define requirements for non-even interval case is up to RAN4. If UE performs measurement at each OD-SSB and AO-SSB occasions under such non-even interval case, we are wondering what is the challenge for UE implementation. If no additional challenge, RAN4 could define requirements for this case to benefit from short measurement period.

In addition, RAN1 also confirm the case of different center frequency location between AO-SSB and OD-SSB, RAN4 could define requirements for this case as well. For the OD-SSB based measurement, reuse the requirements for Case 1 is preferred by us.

Proposal 4: For Scenario 4(OD-SSB and always-on SSB are with same frequency, different offset but same periodicities), RAN1 confirm both even interval and non-even interval cases. If no challenge from UE implementation on the non-even interval case, RAN4 could define requirements for both.

The UE behavior within FMW and outside FMW under Case 2 should be identified. Since the approved Scenario 1 and Scenario 4 show different feature, we discuss the two scenarios respectively. Companies show a bit diverse and the two options were discussed in last meeting:

- Option 1: The same approach in Case 1 can be reused. - UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window. After the time window, UE fallbacks to a slow mode measurement(with DRX and measCycleSCell). - Option 2: - If the serving cell is unknown, the same approach in Case 1 can be reused. - If the serving cell is known, UE follows legacy requirements based on measCycleSCell. - The known condition reuses the principle of legacy known condition. - This scenario applies at least when UE sends the meas. reporting. - FFS the case if the L3 meas. report cannot be triggered. - The above requirement is applied at least when the OD-SSB and AO-SSB are on the same frequency and spatial relation.

The Option 2 tries to split the UE behavior into known and unknown cases and for known case, it is none of the OD-SSB. We have concern on such distinguish between known and unknown. Actually when RAN4 defines the known/unknown conditions for the SCell activation, one reference time point is needed for UE to check whether UE meet some conditions during a while before this reference time point. In the known condition of SCell activation, the reference time point is when UE receives the activation command. But here, what is the reference time point, no conclusion. Furthermore, even RAN4 would define a reference time point to evable the known/unknown condition workable, the conclusio of known/unknown detection is only aware by UE itself, NW does not know. So what is the NW behavior, whether to send the OD-SSB or not? If NW sends the OD-SSB but UE only follow legacy measCycleSCell to perform measurement since UE decide known case, not only no NW power saving gain could be expected, but also more NW power consumption with no benefit.

As a result, we prefer Option 1, i.e. apply same principle as what we approved for Case 1. For details,

- For Scenario 1, obviously the OD-SSB is more dense than AO-SSB, so it is perferred to follow the OD-SSB periodicity within the FMW, same as what happened to Case 1. Outside the FMW, UE follows legacy measurement behavior, i.e. follow measCycleSCell to perform measurement.

- For Scenario 4, the OD-SSB and AO-SSB are non-overlapped due to different offset. So only the combination of them could speed up the measurement. So within the FMW, UE performs fast measurement based on the half of the OD-SSB periodicity.

Proposal 5: For Case 2, same approach in Case 1 is reused. UE performs the OD-SSB/AO-SSB based fast L3 measurement within a time window. After the time window, UE fallbacks to a slow mode measurement. For details:

- For Scenario 1, UE follows the OD-SSB periodicity within the time window, and fallbacks to slow mode to follow the measCycleSCell after the time window.

- For Scenario 4, UE follows the half of OD-SSB periodicity within the time window, and fallbacks to slow mode to follow the measCycleSCell after the time window.

Based on the above UE bahavir analysis, the CSSF under Case 2 should be clarified as well. Since Scenario 1 and Scenario 4 show different feature, we discuss the CSSF for the two scenarios respectively.

- For Scenario 1, obviously the OD-SSB is more dense than AO-SSB, so it is perferred to prioritize OD-SSB within the FMW, same as what happened to Case 1. Outside the FMW, reuse the legacy searcher allocation assumption and either OD-SSB or AO-SSB is counted on for this SCell since actually the AO-SSB is fully covered by OD-SSB before the OD-SSB been deactivated.

- For Scenario 4, the OD-SSB and AO-SSB are non-overlapped due to different offset. So only the combination of them could speed up the measurement. So within the FMW, both OD-SSB and AO-SSB are prioritized over all other measurements. Outside the FWM, only the AO-SSB is counted on for CSSF assumption.

Proposal 6: For Case 2, the CSSF assumption should be clarified as below:

- For Scenario 1, prioritize OD-SSB within the FMW, same as what happened to Case 1. Outside the FMW, reuse the legacy searcher assumption and either OD-SSB or AO-SSB is counted on for this SCell since actually the AO-SSB is fully covered by OD-SSB before the OD-SSB is deactivated.

- For Scenario 4, both OD-SSB and AO-SSB are prioritized over all other measurements within the time window. Outside the FWM, AO-SSB is counted on for this SCell and legacy searcher assumption is applied.

2.3 Consideration on the phase of SCell activation

In previous meeting, some general principle of known condition was determined, but some clarification on the relation of SSB measured for known condition and the SSB used in SCell activation procedure is needed. For the case of same frequency and same QCL, no need to clarify. But for the case of different frequency and/or different QCL, it is necessary to clarify that the SSB measured for known condition should be aligned with the SSB used in SCell activation procedure.

Proposal 7: For Case #2, to specify known condition, under the case of different frequency and/or different QCL, it should be clarified that the SSB measured for known condition should be aligned with the SSB used in SCell activation procedure.

Besides the basic single SCell activation, the application of on-demand SSB could be extended to other SCell activation procedure, we list them as below:

- Direct SCell activation

- PUCCH SCell activation

- Multiple SCells activation

- R18 SCell activation with L3 reporting

The extention to direct SCell activation has been approved in last meeting. Regarding other scenarios, we are fine to deprioritize the multiple SCell activation from the perspective of workload. For R18 SCell activation with L3 reporting, the typical use case is that UE has already prepared L3 report when receiving the SCell activation command. So there is not enough room to improve the SCell activation procedure itself via on-demand SSB, instead, the L3 report could be derived based on on-demand SSB to facilitate the whole procedure. For the PUCCH SCell activation, it could be considered.

Proposal 8: Besides the direct SCell activation, the application of on-demand SSB could also be extended to PUCCH SCell activation and R18 SCell activation with L3 reporting.

- For R18 SCell activation with L3 reporting, focus on obtaining the L3 report based on on-demand SSB.

2.4 Others

L1 measurement after SCell activation

The case of non-DRX configured has been confirmed and the requirements have been determined in last meeting. For the OD-SSB based L1 measurement when DRX is configured, companies are diverse during last meeting discussion.

Wayforward: - RAN4 to support OD-SSB L1 measurement after SCell activation. - FFS the requirement when DRX is configured. - Option 1: Follow legacy, based on DRX cycle - Option 2: Based on OD-SSB periodicity - Option 3: FFS - Note: the OD-SSB is triggered before or together with the Scell activation command, the OD-SSB transmission kept after the SCell activation until the OD-SSB deactivation command received.

For the period after SCell activation, if the OD-SSB is not deactivated by NW, which implies NW still hopes UE to perform OD-SSB based measurement to speed up the measurement. But if DRX is configured, actually the DRX cycle should also be considered to determine the measurement occasion to achieve some UE power saving gain. The intention of NW power saving and UE power saving is more or less controdictory each other. How to trade-off, companies could consider. From our perspective, if no urgent channel quality acquisition demand or beam management demand, NW would not expect UE to perform fast L1 measurement after the SCell activation. The mechanisms of the deactivation of OD-SSB includes counter N and deactivation command, NW could deactivated OD-SSB via any of them. So we prefer Option 2, if NW does not deactivate OD-SSB, then the fast L1 measurement is expected for the sake of RSRP/SINR acquisition.

Proposal 9: For the L1 measurement after SCell activation, OD-SSB based L1 measurement with the periodicity of OD-SSB is applicable regardless DRX configured or not.

OD-SSB based neighbour cell measurement

The related issues were discussed warmly in last meeting and the approvement for FR1 have been achieved. That is, UE measures the SCell with OD-SSB but follows legacy behavior for the intra-frequency neighbour cells.

For FR2, it is still suspending during last meeting. To our understanding, no matter for FR1 or FR2, this principle is both applicable. Even for OD-SSB based SCell measurement for FR2, Rx beam sweeping is still expected to seek the adaptive Rx beam to measure the configured OD-SSB indexs at least for Case 1. For the neighbour cell measurement, UE follows the legacy requirements with legacy SSB. For Case 2, the neighbour cell measurement is same as that for Case 1, the only difference lies in the OD-SSB based SCell measurement, whether UE to perform Rx beam sweeping for the OD-SSB based SCell measurement, we are open to discuss.

Proposal 10: For the neighbour cell measurement, apply same principle for both FR1 and FR2, i.e. UE follows legacy requirements with legacy SSB. For the SCell measurement in which the OD-SSB is activated, UE has to perform Rx beam sweeping at least for Case 1 in FR2.

- Conclusion

In this contribution, we have the following proposals on the RRM enhancements around on-demand SSB:

Proposal 1: When multiple OD-SSB activation happens during the deactivated SCell measurement, we propose the following two solutions:

- Solution 1: To simplify the requirements, not to define further enhancement to reduce the re-activated FWM. UE always follow single principle to determine the length of FWM.

- Solution 2: To further reduce the re-activated FWM. The details are as below:

- When NW re-activates the OD-SSB within the T2, UE is required to perform “fast measurement” without cell search, i.e. skip the PSS/SSS detection and/or SSB index acquisition.

- The starting point of T2 is based on the time point Tend, i.e. the end of last FWM.

Proposal 2: When activated OD-SSB layer and legacy SSB layers co-existence within the FMW, the CSSF only counts OD-SSB layers to speed up the OD-SSB based measurement. This principle applies to both single OD-SSB layer and multiple but non-overlapping OD-SSB layers.

Proposal 3: Not to restrict the lower bound of the periodicity of the OD-SSB, faster OD-SSB based measurement within FMW could reduce the impact on other legacy SSB layers.

Proposal 4: For Scenario 4(OD-SSB and always-on SSB are with same frequency, different offset but same periodicities), RAN1 confirm both even interval and non-even interval cases. If no challenge from UE implementation on the non-even interval case, RAN4 could define requirements for both.

Proposal 5: For Case 2, same approach in Case 1 is reused. UE performs the OD-SSB/AO-SSB based fast L3 measurement within a time window. After the time window, UE fallbacks to a slow mode measurement. For details:

- For Scenario 1, UE follows the OD-SSB periodicity within the time window, and fallbacks to slow mode to follow the measCycleSCell after the time window.

- For Scenario 4, UE follows the half of OD-SSB periodicity within the time window, and fallbacks to slow mode to follow the measCycleSCell after the time window.

Proposal 6: For Case 2, the CSSF assumption should be clarified as below:

- For Scenario 1, prioritize OD-SSB within the FMW, same as what happened to Case 1. Outside the FMW, reuse the legacy searcher assumption and either OD-SSB or AO-SSB is counted on for this SCell since actually the AO-SSB is fully covered by OD-SSB before the OD-SSB is deactivated.

- For Scenario 4, both OD-SSB and AO-SSB are prioritized over all other measurements within the time window. Outside the FWM, AO-SSB is counted on for this SCell and legacy searcher assumption is applied.

Proposal 7: For Case #2, to specify known condition, under the case of different frequency and/or different QCL, it should be clarified that the SSB measured for known condition should be aligned with the SSB used in SCell activation procedure.

Proposal 8: Besides the direct SCell activation, the application of on-demand SSB could also be extended to PUCCH SCell activation and R18 SCell activation with L3 reporting.

- For R18 SCell activation with L3 reporting, focus on obtaining the L3 report based on on-demand SSB.

Proposal 9: For the L1 measurement after SCell activation, OD-SSB based L1 measurement with the periodicity of OD-SSB is applicable regardless DRX configured or not.

Proposal 10: For the neighbour cell measurement, apply same principle for both FR1 and FR2, i.e. UE follows legacy requirements with legacy SSB. For the SCell measurement in which the OD-SSB is activated, UE has to perform Rx beam sweeping at least for Case 1 in FR2.

- Reference

- R4-2502598 WF on Netw_Energy_NR_enh_part 1, Ericsson, RAN4#114.

- R1-2410917 Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell, Ericsson, RAN1#119.