3GPP TSG-RAN4 Meeting #114bis			R4-2504277

Wuhan, China, April 7th-11th 2025

Agenda item:	7.10.4

Title:	Discussion on NGSO channel modelling

Source:	Nordic Semiconductor ASA

Document for:	Discussion

- Introduction

This contribution is related to topics discussed in Athens RAN4#114 in [1].

- Discussion

Issue 1-1-2 Correction of Propagation Delay Calculation

- WF

- RAN4 discuss whether the Earth's rotation is considered during signal flight time for more accurate estimations of propagation delay

- Considering the earth rotation, the propagation delay, DelayERtR can be derived as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where tR is the signal’s received time, rtRECEFand vtRECEF are the satellite position and velocity vectors at time tR , ρtRis the difference in satellite and UE position vectors at time tR,  K  is the unit vector in standard z-axis, (a×b) is the cross-product and <a, b> is the dot product between a and b

- The proponent is encouraged to provide the detail information, e.g., reference, if possible, about the propagation delay calculation with considering the signal flight time before RAN4#114bis meeting

- Participating companies are encouraged to examine this issue from their perspective.

Issue 1-1-2 Correction of Propagation Delay Calculation

- WF

- RAN4 discuss whether the Earth's rotation is considered during signal flight time for more accurate estimations of propagation delay

- Considering the earth rotation, the propagation delay, DelayERtR can be derived as

DelayERtR=ρtR/c1-1c<vtRECEF+ωE(K×rtRECEF),ρtRρtR>

Where tR is the signal’s received time, rtRECEFand vtRECEF are the satellite position and velocity vectors at time tR , ρtRis the difference in satellite and UE position vectors at time tR,  K  is the unit vector in standard z-axis, (a×b) is the cross-product and <a, b> is the dot product between a and b

- The proponent is encouraged to provide the detail information, e.g., reference, if possible, about the propagation delay calculation with considering the signal flight time before RAN4#114bis meeting

- Participating companies are encouraged to examine this issue from their perspective.

Issue 1-1-2

Earth rotation should not be considered. The Earth rotation is taken into account in the conversion from ECI frame to ECEF frame. TE should keep orbit running in ECI, and convert to ECEF when filling SIB31s.

Proposal-1:  Earth's rotation is already accounted for in ECI to ECEF conversion. Do not support additional Earth’s rotation correction considered in Issue 1-1-2.

Issue 1-1-4-3

Issue 1-1-4-3 Interpolation method to obtain the propagation delay and Doppler shift trajectory

- Proposals

- Option 1: Cubic interpolation

- Option 2: linear interpolation

- Option 3: Open to not restrict the implementation on the TE side. However, RAN4 should discuss and determine an acceptable error value introduced by interpolation

- Option 4: With the assumption of UE updates the satellite location/velocity at least every 1 second. The TE may update Doppler Shift and Propagation delay every slot linear interpolation or cubic spline interpolation. It is up to TE implementation which interpolation method to be used.

Issue 1-1-4-3 Interpolation method to obtain the propagation delay and Doppler shift trajectory

- Proposals

- Option 1: Cubic interpolation

- Option 2: linear interpolation

- Option 3: Open to not restrict the implementation on the TE side. However, RAN4 should discuss and determine an acceptable error value introduced by interpolation

- Option 4: With the assumption of UE updates the satellite location/velocity at least every 1 second. The TE may update Doppler Shift and Propagation delay every slot linear interpolation or cubic spline interpolation. It is up to TE implementation which interpolation method to be used.

Our preference is clearly Option 3. It does not matter how TE implement interpolation of doppler and TA between consecutive [1] sec states, what matters is that the error requirement with respect to Kepler is less than a given error requirement, FFS error requirement.  Option3 should be complemented with error requirement with respect to Kepler.

Proposal-2: Support Option 3. FFS: maximum error requirement with respect to Kepler.

Issue 1-2-1

Issue 1-2-1 UE assumption on propagator model for satellite motion for RAN4 simulation purpose

- WF

  - Consider Keplerian model and Equation of Motion models in UE side as baseline assumption for RAN4 requirement

    - Note: There is no limitation for UE implementation to estimate the doppler shift and propagation delay based on the signalled ephemeris information

Issue 1-2-1 UE assumption on propagator model for satellite motion for RAN4 simulation purpose

- WF

  - Consider Keplerian model and Equation of Motion models in UE side as baseline assumption for RAN4 requirement

    - Note: There is no limitation for UE implementation to estimate the doppler shift and propagation delay based on the signalled ephemeris information

It is acceptable to assume Keplerian model in UE. However, some testing assumptions/error-margins should be introduced in order to allow implementations which are capable to track live satellite orbits better than Kepler propagator.

Figure 1: Deviation from true orbit for Keplerian propagation for 600km LEO orbit.

Figure 1 shows the difference between real GNSS location and speed of a 600km orbit vehicle and Kepler propagator, which takes the first GNSS state vector and propagates it over a time.

From Figure 1 it can be noticed that after 2minutes, ppm error is 0.01ppm. However, TA error after 2minutes is already 0.5µs. For about 30s the difference between real orbit and the Kepler orbit seems to be insignificant.

Therefore, we suggest two alternatives for Way forward:

- Alt-1: All test cases will include sufficiently short validity timer set to be <[30s]

- Alt-2: Error margin on top of legacy NTN error margin is introduced for Timing Error and CFO

From Nordic point of view Alt-1 is preferred, because it can ensure that link in Test equipment will be running without impairments irrespective of propagator implemented in the UE.

Proposal-3: All test cases for NGSO will include sufficiently short validity timer set to <[30sec].

- Conclusion

Based on discussion in this contribution, we have the following proposals:

Proposal-1:  Earth's rotation is already accounted for in ECI to ECEF conversion. Do not support additional Earth’s rotation correction considered in Issue 1-1-2.

Proposal-2: Support Option 3. FFS: maximum error requirement with respect to Kepler.

Proposal-3: All test cases for NGSO will include sufficiently short validity timer set to <[30sec].

- References

[1] R4-2502314 Way Forward for [114][325] NTN_testing_NGSO_channel_model, Samsung