3GPP TSG-RAN WG4 Meeting #114                                   R4-2504279

Wuhan, Hubei, China, 07th – 11th April, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on NTN Ku band UE RF requirements

Agenda Item:	7.11.4

Document for:	Approval

- Introduction

In RAN4#114, a WF for VSAT types [1] was agreed upon, where NTN VSAT types 4 and 5 could communicate with both GSO and LEO satellites. However, there is an unresolved issue [2] regarding the minimum EIRP for the VSAT, as shown below. In this paper, we will provide our views on the minimum EIRP for the VSAT.

- Issue 1-1-1: VSAT types to be considered for NTN Ku band after RAN#106 update - VSAT type 1-3 will not be specified for Ku bands based on the revised WID - The mobile VSAT support of the Ku bands will be added based on the existing mobile VSAT type 4 & 5 - The following changes can be the starting point NTN VSAT class; NTN VSAT type; Type description. Mobile VSAT; 43; Mobile VSAT communicating with GSO and LEO with mechanical steering antenna.. 52,3; Mobile VSAT communicating with GSO and LEO with electronic steering antenna.. NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. Note 3: LEO support is only defined for Ku bands (TBD). - Issue 1-2-3: Minimum EIRP requirements - Companies are encouraged to provide views on the minimum peak EIRP requirement for the NTN Ku band in the next meeting.

- Discussion

2.1  UE minimum peak EIRP

From the revised WID [3] shown below, we need to consider both FR1 numerology and FR2 numerology to specify the UE RF requirements. Link budget analysis is required to derive the UE minimum peak EIRP.

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz channel bandwidths with 15kHz and 30kHz subcarrier spacing, and 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 Hz subcarrier spacing.

Table 1 and Table 2 show the NTN Ku-band link budgets for FR1 numerology and FR2 numerology. From the EIRP results, it can be seen that the required minimum peak EIRP for GSO is the largest. In order to support both GSO and LEO, the worst case (GSO) needs to be considered for the minimum peak EIRP. Therefore, for FR1 numerology, the minimum peak EIRP is at least 64 dBm to support 100 MHz CBW. For FR2 numerology, the minimum peak EIRP is at least 67 dBm to support 200 MHz CBW.

Table 1: NTN Ku-band link budget analysis for FR1-NTN

|  | Ku band (FR1-NTN) |  |  |
| --- | --- | --- | --- |
| Satellite type | GSO | LEO 1200 | LEO 600 |
| Elevation angle (VSAT) | 30 deg | 30 deg | 30 deg |
| Distance (VSAT-SAN) | 38612 km | 1998.88 km | 1075.09 km |
| UL frequency | 14500MHz | 14500MHz | 14500MHz |
| Free space path loss (dB) | 207.41 | 181.69 | 176.31 |
| Atmosphere loss (dB) | 2 | 2 | 2 |
| BW (MHz) | 100 | 100 | 100 |
| target SNR | -1 | -1 | -1 |
| SAN noise figure (dB) | 3 | 3 | 3 |
| SAN Rx gain (dBi) | 58.5 | 38.5 | 38.5 |
| Implementation margin | 2 | 2 | 2 |
| minimum EIRP | 63.9 | 58.2 | 52.8 |

Table 2: NTN Ku-band link budget analysis for FR2-NTN

|  | Ku band (FR2-NTN) |  |  |
| --- | --- | --- | --- |
| Satellite type | GSO | LEO 1200 | LEO 600 |
| Elevation angle (VSAT) | 30 deg | 30 deg | 30 deg |
| Distance (VSAT-SAN) | 38612 km | 1998.88 km | 1075.09 km |
| UL frequency | 14500MHz | 14500MHz | 14500MHz |
| Free space path loss (dB) | 207.41 | 181.69 | 176.31 |
| Atmosphere loss (dB) | 2 | 2 | 2 |
| BW (MHz) | 200 | 200 | 200 |
| target SNR | -1 | -1 | -1 |
| SAN noise figure (dB) | 3 | 3 | 3 |
| SAN Rx gain (dBi) | 58.5 | 38.5 | 38.5 |
| Implementation margin | 2 | 2 | 2 |
| minimum EIRP | 66.9 | 61.2 | 55.8 |

Proposal 1: RAN4 to consider a minimum peak EIRP of at least 64 dBm to support 100 MHz CBW for FR1 numerology.

Proposal 2: RAN4 to consider a minimum peak EIRP of at least 67 dBm to support 200 MHz CBW for FR2 numerology.

- 3	Conclusion

In this contribution, we provide our views on minimum peak EIRP for NTN Ku-band. The proposals are summarized as follows:

Proposal 1: RAN4 to consider a minimum peak EIRP of at least 64 dBm to support 100 MHz CBW for FR1 numerology.

Proposal 2: RAN4 to consider a minimum peak EIRP of at least 67 dBm to support 200 MHz CBW for FR2 numerology.

- 4	References

- R4-2502290, “Way Forward for [114][313] NR_NTN_Ku_Band_General”, Eutelsat, RAN4#114.

- R4-2502288, “WF for [114][314] NR_NTN_Ku_Band_UE_SAN_RF”, CHTTL, RAN4#114.

- RP-250467, ” Revised WID: Introduction of Ku bands for NR NTN”, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT, RAN#107