3GPP TSG-RAN WG4 Meeting #114bis                                    R4-2504286

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:              General discussion on AI mobility regarding testability and interoperability

Agenda item:			7.21.3

Document for:	Discussion

Introduction

Some AI mobility regarding testability and interoperability were discussed extensively in last meeting. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 Considering multiple cells in the testing setup

- For mobility, neighbouring cells are of the same-level significance as the serving cell since handover is a multi-cell related behaviour. The cross-cell RRM measurement contains inter-frequency (Inter_F_C) and intra-frequency should be studied with certain spatial consistency.

- For scenarios FR1 to FR1 intra-frequency temporal domain case B and FR1 to FR1 inter-frequency inter-cell (frequency domain), cell specific and cluster approaches are considered. Based on RAN2 evaluation results, cluster approach outperforms the cell-specific approach not only in temporal domain prediction but also in frequency domain prediction. While, cluster approach refers to the prediction method where the number of cells for measurements and/or predictions is more than one. Besides, the cluster approach can implicitly learn some correlation information between UE location and SS-RSRP results/channel correlation across different collocated cells, and the prediction performance for non-collocated cells without channel correlation can be tested accordingly.

- With the input data from two frequencies (e.g., serving cell X at frequency f1 and neighbour cell Y at frequency f2), the AI/ML model can be trained to learn the correlation between the measurements in the two frequencies. Based on RAN2 simulations, if the cell to be measured and the cell to be predicted are collocated (located in the same sector/site with the same antenna configuration), the correlation is high, the RSRP difference between the two cells is relatively stable and a good prediction accuracy is obtained. While, non-collocated inter-frequency prediction is under discussion

- However, since in reality, cell deployment is irregular, different cells may have different configurations and different properties, if we to consider multiple cells in the testing, the generalization discussions across cells for intra-frequency and/or generalization discussions across frequencies might be discussed along with. And accordingly, cell configurations should be also accounted carefully.

Proposal 1: RAN4 to consider the following testability aspects for AI mobility

- Consideration of serving and neighbour cells with robust cell configurations

- Generalization across cells for intra-frequency and/or across frequencies should be guaranteed

- Multi-cell setup and number of cells to measure

  - The legacy number of cells per carrier that the UE is able to monitor for intra-frequency and inter-frequency can be considered as the baseline

2.2 Ground Truth Definition for RSRP accuracy for FR1

One key issue for RRM test methodology is the procedure of RSRP prediction accuracy testing. There are two directions to define ideal RSRP based on the agreements achieved in last meeting

- Direction 1/Alt 1: Ground truth for FR1 will be based on the transmitted or reception power (i.e., settled by TE). Since TE is able to control the RSRP level at the centre of quiet zone, the ideal RSRP can be derived from TE for noise-free or very high SNR conditions. And in this sense, the ideal RSRP is of the transmit power.

- Direction 2/Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions (Settled by measurement report).

Putting aside the Alts, from our understanding, some side conditions should to define to enable the test.

- The information at the TE side.

- The most important information should know at TE—Prediction related time including absolute and relative prediction time. Irrelevant to when UE to report the prediction result, the TE should know the prediction related time, the time can be the absolute time UE performs the prediction and the relative prediction time (prediction window), otherwise the TE does not know the accurate timestamp the maximum available Tx power to transmit, and consequently, the predicted RSRP accuracy requirements cannot be verified under fast fading channel accurately.

Fig. 1 Illustration of conducted test

- The other information should to know at TE—Fast fading channel condition. Unlike AWGN, the rapid variation in channel condition contains too much randomness. And the effects of channel models are very different. The multi-path faded DL channel model information should to be known at TE in case the un-testability caused by different kinds of randomness and channel characteristics in various complex real-world environment.

- Emulated SNR/Testable SNR range. From our understanding, emulation of noise-free/very high SNR test conditions are supported. However, due to the fast fading channel condition random fluctuation, time instant A (e.g., the highest peak) and time instant B (the lowest peak) have different understandings on high-SNR. In our opinion, there are two solutions:

  - The upper bound of signal power as the maximum available Tx power transmitted from TE, then TE controls the level of artificial noise for target SNRs at different time instances.

  - Control the signal and artificial noise dynamically with total power constraint for target SNRs at different time instances. However, in this case, the testable SNR range in the test system and the assessment of the range shall be discussed.

Observation 1: Due to the fast fading channel condition random fluctuation, the definition of high-SNR shall be different at each time instant.

Proposal 2: Regarding the emulated SNR, to align the definition of high-SNR at each time instant under the fast fading channel condition, the following two solutions of controlling the noise/SNR can be done at TE

- The upper bound of signal power as the maximum available Tx power transmitted from TE, then TE controls the level of artificial noise for target SNRs at different time instances

- Control the SNR and artificial noise dynamically with total power constraint for target SNRs at different time instances

  - The testable SNR range in the test system and the assessment of the range shall be discussed

- Besides, the information regarding how UE to measurement, from our understanding, it is not necessary. Speaking of the measured RSRPs, L1 filtering and L3 filtering are involved, different L1 filtering options/the value of L3 filtering parameters (i.e., FilterCoefficient) may influence the derivation of the ideal SS-RSRP conducted at TE. However, we prefer to keep the corresponding measurement information transparent to TE, since “deterministic testing” will inevitably occur if too many parameter values become known at TE, the generalization capability of AI model may be poorly verified and the test becomes meaningless. However, if all the uncertainties are considered, the test tolerance range of accuracy difference will become wide, it is so easy for UE to pass the testing.

Observation 2: If to consider all the uncertainties including filtering, channel fading condition, measurement samples, etc., an unnecessarily wide predicted accuracy gain range may make it easier for the UE to pass RSRP prediction accuracy test.

- Furthermore, from our understanding, if to consider Alt 2, cheating issue should be also solved. The ideal SS-RSRP will be measured by UE and reported to TE, since there is no limitation on UE implementation, it can report a fake value, e.g., the best previous predicted value based on AI prediction or even non-AI approach, since the later can also achieve comparable performance under some scenarios/sub-use-cases. In this sense, it might be so easy for UE to pass the testing, but predicted RSRP accuracy requirement can not be verified. Besides, recalling that for measurement accuracy requirement, AWGN is typically specified as a side condition, and SNR side condition, number of samples, and measurement accuracy are linked together as a packet. If Ground truth for FR1 will be settled by measurement report, the SNR side condition under the fast fading channel environment should be defined.

From this, we think Alt 1 is more suitable for FR1, and FR1 RRM testing is in conducted test environment and the input of useful signals to the UE baseband can be known.

Proposal 3: For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition.

Proposal 4: To make FR1 RSRP prediction accuracy test feasible, at least the following information should to know at TE：

- Prediction related time, the time can be the absolute time UE performs the prediction and the relative prediction time (prediction window)

- Fast fading channel condition

Proposal 5: Suggest to keep UE measurement related parameter information including L3 filtering, L1 filtering and the timing UE performs and/or finishes the measurement transparent to TE

- A suitable test tolerance range of predicted accuracy gain should to be defined to consider the uncertainties as much as possible and to ensure UE will not pass the testing easily in parallel

Proposal 6: If Ground truth for FR1 will be settled by measurement report, the SNR side condition requirement under the fast fading channel environment should be defined

2.3 Principle of FR2 testing in AI/ML mobility

Prior to the start of the FR2 RSRP accuracy prediction test, we’d better to study the methodology of FR2 testing in AI/ML mobility. From our understanding, at least the following aspects

- Emulation of the propagation conditions. In AI mobility L3 cell-level prediction, it is highly possible to emulate the multiple beam environment. In this sense, multi-path propagation conditions, i.e., the multi-path fading conditions between the gNB emulator and test chamber probe may need to be emulated to ensure the verification of MIMO algorithms, where TDL can be modelled. Besides, since RRM requirements are defined with AWGN as side condition, suitable AWGN implementation in an OTA environment is necessary.

- Emulation of channel models. The key parameters can contain: Maximum CBW in one carrier; Maximum CCs (if CA implementation is considered); Total number of faded paths; Number of taps for each faded path; Number of transmitters; Number of UE Rx receivers in different evaluation scenarios (temporal domain, frequency domain, cell-specific approach, cluster approach, intra-frequency cells, inter-frequency cells, different frequencies) for the proposed Sub-use-case (s).

- The number of emulated transmission reception points (TRxP). As a reference, NR cell corresponds to one or multiple TRxPs, which may be co-located or non-collocated.

- Emulation of noise and EPRE. How the emulated noise is generated/transmitted. And the SNR/SINR in various evaluation scenarios, the test will define the exact noise/SNR level per TRxP at the reference point.

- AoA setup. For OTA testing, one of the primarily part is AoA setup. The baseline RRM setup assumes that measurement equipment shall be able to emulate the signals from 2 AoAs. However, considering the aims of different tests, we have: verification of TX/RX beamforming is needed (2 AoAs/3 AoAs, 3AoAs is for multi-Rx simultaneous reception), verification of TX/RX beamforming is not needed (1AoA). While, if we focus on intermediate KPI, i.e., RSRP difference and/or measurement overhead reduction test, the assumptions of AoA setup should be further discussed.

- Rx beam direction assumption. RRM tests will consist of tests with signals transmitted from different directions that have to be identified and reported by UE. And Rx antenna gain is an important factor that to enable the search of signals from different directions by improving the link budget. Actually, the Rx antenna gain experienced by the UE will depend on the Rx beamforming codebook selected, and the exact orientation of the DUT is controlled by the TE turntable. That is to say, on the proposed AoA, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage. While, antenna gains corresponding to different directions are different, can be found in Annex B in TS 38.133. Besides, Noc level for signals coming from the two directions (beam peak and non-beam peak) are different also.

  - Considering the test uncertainties and complexities the non-beam peak direction may bring, we have concern that if our RRM tests at current stage are mainly to verify the RRM RSRP prediction performance without involving beamforming performance, whether test in non-beam peak direction is not necessary. And another reason is that since the cells to predict and train are co-located cells so far, UE has the peak beam direction for the predicted/inferred neighbor cell (s) can be testable

Observation 3: For FR2 testing, on the exact AoA to be used, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage

Proposal 7: For FR2 testing in AI/ML mobility, at least the following aspects can be studied:

- Emulation of the propagation conditions

    - AWGN and TDL

- Emulation of channel models

- The number of emulated transmission reception points (TRxP)

- Emulation of noise and EPRE

- AoA setup

- Rx beam direction assumption

Proposal 8: For FR2 testing in AI/ML mobility, multiple beam environment could be emulated

However, considering that there are some ongoing discussions on the testability and interoperability in AI/ML for NR air interface. Considering the commonality and difference of AI/ML based beam management and AI/ML mobility: FR2 VS FR1+FR2; Spatial and temporal beam prediction, serving cell (intra-cell) prediction VS temporal, spatial, and frequency domain predictions, cross-cell prediction; BM, predicted L1-RSRP VS Cell, predicted L3-RSRP; The Top-1/K (%) beam prediction accuracy KPI VS intermediate KPIs (Average L3 cell level RSRP difference) + system KPIs (HOF, RLF rate, throughput, etc) . From this, for FR2, to avoid the overlapping of similar discussions in the two WIs, especially intra-frequency intra-cell beam level prediction, we’d better to wait for more progress from AI/ML-BM

Proposal 9: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline

- RAN4 to discuss what test parameters/methodologies on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to L3 cell level measurement prediction testing to avoid the duplicate study

Proposal 10: For FR2 testing in AI mobility, if the test is to mostly verify the RRM RSRP prediction performance, RAN4 to study the feasibility of testing in beam peak direction on the exact AoA to be used

2.4 Testing of Generalization aspects

AI Model generalization is a very important and challenging issue in deep learning, which refers to the ability of the trained model to perform well on new inputs unseen during the training. From our understanding, compared to the PHY prediction, the generalization/adaptability of the AI models for mobility is more worth studying. The reasons are:

- For AI mobility, various deployment scenarios (e.g., UMa, UMi, etc.), channel environments, UE movement patterns and speeds, and frequencies exist. Meanwhile, different RRC configurations and conditions for measurement (events) need to be considered in practice. All of them will impact generalization performance of an AI/ML model.

- Multiple cells would be considered in L3 AI mobility WI. While, across different cells may capture variations in NW conditions and local radio conditions. In this sense, the risk of model overfitting to specific cell characteristics may arise, which may decrease the adaptability to different cells

- Since the prediction measurement results are L3 filtered. Different L3 filtering factors may incur different prediction accuracy. While, when training datasets collected with fixed L3 filtering factor, the corresponding AI models may have worse generalization capability in different cells, especially.

- The measurement event prediction may benefit of cell-, site- or area-specific models and the study on AI model generalization for measurement event prediction is needed

During the Rel-18 studies about AI/ML for PHY, the generalization evaluation performance of AI/ML models is verified by testing its performance with different training and test datasets (evaluation methodology in TR38.843 for generalization study), we can reuse such principle/methodologies.

Proposal 11: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

- Reuse the principle/methodologies/signallings on generalization achieved in AI/ML for NR air interface as much as possible

3 Conclusion

In this contribution, we provided our viewpoints on AI RRM requirements for testability and interoperability. The following observations and proposals are obtained:

Observation 1: Due to the fast fading channel condition random fluctuation, the definition of high-SNR shall be different at each time instant.

Observation 2: If to consider all the uncertainties including filtering, channel fading condition, measurement samples, etc., an unnecessarily wide predicted accuracy gain range may make it easier for the UE to pass RSRP prediction accuracy test.

Observation 3: For FR2 testing, on the exact AoA to be used, the test can be done in one Rx beam peak direction or in one or more directions from the UE spherical coverage

Proposal 1: RAN4 to consider the following testability aspects for AI mobility

- Consideration of serving and neighbour cells with robust cell configurations

- Generalization across cells for intra-frequency and/or across frequencies should be guaranteed

- Multi-cell setup and number of cells to measure

  - The legacy number of cells per carrier that the UE is able to monitor for intra-frequency and inter-frequency can be considered as the baseline

Proposal 2: Regarding the emulated SNR, to align the definition of high-SNR at each time instant under the fast fading channel condition, the following two solutions of controlling the noise/SNR can be done at TE

- The upper bound of signal power as the maximum available Tx power transmitted from TE, then TE controls the level of artificial noise for target SNRs at different time instances

- Control the SNR and artificial noise dynamically with total power constraint for target SNRs at different time instances

  - The testable SNR range in the test system and the assessment of the range shall be discussed

Proposal 3: For FR1 RSRP prediction accuracy testing, the ideal RSRP (i.e., the ground truth) can be of the transmit power settled by TE in noise-free/high SNR test condition.

Proposal 4: To make FR1 RSRP prediction accuracy test feasible, at least the following information should to know at TE：

- Prediction related time, the time can be the absolute time UE performs the prediction and the relative prediction time

- Fast fading channel condition

Proposal 5: Suggest to keep UE measurement related parameter information including L3 filtering, L1 filtering and the timing UE performs and/or finishes the measurement transparent to TE

- A suitable test tolerance range of predicted accuracy gain should to be defined to consider the uncertainties as much as possible and to ensure UE will not pass the testing easily in parallel

Proposal 6: If Ground truth for FR1 will be settled by measurement report, the SNR side condition requirement under the fast fading channel environment should be defined

Proposal 7: For FR2 testing in AI/ML mobility, at least the following aspects can be studied:

- Emulation of the propagation conditions

    - AWGN and TDL

- Emulation of channel models

- The number of emulated transmission reception points (TRxP)

- Emulation of noise and EPRE

- AoA setup

- Rx beam direction assumption

Proposal 8: For FR2 testing in AI/ML mobility, multiple beam environment could be emulated

Proposal 9: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline

- RAN4 to discuss what test parameters/methodologies on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to L3 cell level measurement prediction testing to avoid the duplicate study

Proposal 10: For FR2 testing in AI mobility, if the test is to mostly verify the RRM RSRP prediction performance, RAN4 to study the feasibility of testing in beam peak direction on the exact AoA to be used

Proposal 11: For Rel-19 AI/ML mobility, it is more necessary for RAN4 to study generalization related tests/requirements

- Reuse the principle/methodologies/signallings on generalization achieved in AI/ML for NR air interface as much as possible

4 References

[1] R4-2502594 “WF on AI/ML Mobility”, Nokia