3GPP TSG-RAN WG4 Meeting #114bis                                    R4-2504290

Wuhan, Hubei, China, 07th – 11th April, 2025

Source:	Samsung

Title:              Discussion on NES common signal adaptation

Agenda item:			7.25.2.2

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on NES common signal adaptation. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 RAN1 agreements update (SSB related) and analysis

RAN 1## 120 From Tuesday session Conclusion There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell. From Wednesday session Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used From Friday session Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Based on the above RAN1 conclusions/agreements, we have the following observations and proposals:

Observation 1:

- For SSB burst periodicity adaptation, the indication is signaled by group common DCI, DCI format 2_9

  - (Unified) MAC-CE details for SSB burst periodicity adaptation is up to RAN2, depending on the corresponding discussions in OD-SSB feature

Proposal 1:

- Time domain positions of SSBs within a burst is same before and after SSB adaptation

- RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements

- No RRM impacts for SSB adaptation requirement for SCell NCD-SSB.

Besides, regarding time domain location (SFN offset and half frame index) of the SSB bursts, based on the relationship between SSB occasions with larger periodicity and shorter periodicity, the following cases may occur:

Fig.1 Time domain location of the SSB bursts for SSB occasions with different periodicities

Case 1: Periodicity: LargeShort. HF index is same before and after SSB adaptation. SFN offset is same before and after SSB adaptation.

Case 2: Periodicity: ShortLarge. HF index is same before and after SSB adaptation. SFN offset is same or different before and after adaptation.

Different from OD-SSB has concrete agreements that the time domain location of OD-SSB can be explicitly provided by RRC, it is still kept open in RAN1 that whether the related indication for SSB adaptation will be introduced or just it is up to UE implementation to blind detect the cases.

Proposal 2: For SSB adaptation, RAN4 to define requirements considering the following cases

- HF index is same before and after SSB adaptation and SFN offset is same before and after SSB adaptation

- HF index is same before and after SSB adaptation and SFN offset is same or different before and after adaptation

  - Wait for RAN1 conclusions on whether to introduce explicit indication or up to UE implementation to perform blind SFN detection

3 RAN2 agreements update and observation

R2-2501484 Reply LS on SSB adaptation Agreements on SSB adaptation - RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. - For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. Agreements on SSB adaptation - RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

Observation 2: For L3 RRM measurement on SCell with SSB adaptation.

- Existing SMTC based L3 RRM framework can be inherited

- Additional SMTC configuration accommodating with SSB adaptation will be introduced and defined

Observation 3: For L3 measurement, the adapted SSB on neighbor cell is measured based on legacy SMTC

4 Discussion

4.1 L3 requirements for SSB adaptation – general

As per RAN2 agreement guidance, companies agree to consider the possibility of optimizing/adjusting the SMTC configuration for NES instead of using adapted SSB to perform L3 measurement directly. However, how to configure the additional SMTC is still suspending. From our understanding, the main ongoing solutions from RAN2 perspective are at least (combining with OD-SSB discussions) following:

Fig. 2. The Alts. to optimize SMTC configuration for NES discussed in RAN2

In summary, 1) Configuration is provided under MeasObjectNR, e.g., using smtcXlist; 2) Configuration is provided under SCellConfig., the SCell with SSB adaptation is configured with a MO, and the measObject is indicated by the servingCellMO, provided via the ServingCell Configuration

However, no matter based on which way to configure the “adapted SMTC”, the L3 requirements involving such novel changes shall be enhanced, and the corresponding clarifications should be added. And the activated SCell measurement requirements include 9.2 NR intra-frequency measurements (including cell identification, measurement);9.3 NR inter-frequency measurements (including cell identification, measurement), but how to enhance the corresponding requirement can wait RAN2’s concrete conclusions.

Proposal 3: For RRM requirement impact on activated SCell measurement/detection, RAN4 to discuss how to specify time periods for intra-frequency SCell identification and measurement, and inter-frequency SCell identification and measurement after receiving RAN2 agreements on the detailed SMTC configuration mechanism (s) for NES

For deactivated SCell measurement, RAN1 agreed to consider group-common DCI to indicate the adaptation of SSB to all the UEs in the cell, it does make sense, since 1) SSB transmission/SSB periodicity switches is cell-specific, group-common DCI is more suitable for indicating cell-level information; 2) Actually, based on the WID, the applicable scenario of adaptation SSB is not limited to SCell operation only, but if only DCI indication is confirmed, we have concerns on whether there is a need to reuse the same conclusion from OD-SSB case#2 (scenario#X) discussion to define the corresponding RRM requirements since no data transfer and no PDCCH monitoring are expected on such SCell without taking cross-carrier scheduling into consideration and active BWP is not available on such deactivated SCell.

However, considering the commonality between OD-SSB transmission and SSB adaptation transmission, RAN2 strives for the solutions on unified/common MAC-CE framework for OD-SSB and adaptation SSB for NW and UE implementation efficiency, and in this sense, if MAC-CE (UE-specific) indication can also be confirmed, the same principle on FMW-based fast measurement from OD-SSB case #2 can be reused.

Proposal 4:

- If group-common DCI is considered to indicate the adaptation of SSB, for deactivated SCell measurement requirement, same as legacy.

- If MAC-CE can also be considered to indicate the adaptation of SSB, for deactivated SCell measurement, impose the same principle from OD-SSB case#2 (same frequency but with different periodicities) discussion to define RRM requirements

  - Wait for RAN2 conclusions on whether MAC-CE indication for SSB adaptation can be supported

Proposal 5: Wait for RAN2 conclusions on whether additional SMTC configuration according to SSB adaptation for measurement on deactivated SCell with SSB adaptation can be supported

4.2 SCell activation requirement for SSB adaptation

The same logic as we point out in Section 4.2, based on the RAN1 agreement, group-common DCI is considered to indicate the adaptation of SSB to all the UEs. We are wondering whether it is possible that the SCell can be activated based on SSB resources for adaptation by using such DCI-based indication even if UE receive SCell activation command and SSB adaptation indication command at the same time

Proposal 6:

- If separate MAC CE to activate SSB adaptation and SCell, define the SCell activation requirements based on the SSB periodicity after SSB adaptation

  - For the case SSB adaptation command comes together with SCell activation command

- If MAC CE to activate SCell but group-common DCI to activate SSB adaptation, RAN4 to discuss the feasibility of defining the SCell activation requirements for the case SSB adaptation command comes together with SCell activation command

4.3 L1 requirements impact after/without transition

The following agreement was achieved in last meeting regarding L1 requirements, copied below

Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. Tfirst_SSB is the uncertainty time from the time point when SSB adaptation occurs till the first SSB after SSB adaptation Issue 1-2-1: Applicable L1 requirements for SSB adaptation - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD

And if we take a look at the measurement period for FR1/FR2, below (taking L1-RSRP measurement as example),

Table 9.5.4.1-2: Measurement period TL1-RSRP_Measurement_Period_SSB for FR2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

the SSB based L1-RSRP measurement is based on M SSB samples (M=1/M=3), sweeping factor N, SSB periodicity, DRX cycle length, configured periodicity for reporting, measurement gap related sharing factor P, relaxation of additional 1.5, and the computation rule is max(A, B*max (TDRX,TSSB)) for DRX cycle<=320 ms.

However, we have some concerns on DRX cycle <=320 ms configuration case. Since from our understanding, once the SSB adaptation activation has been triggered, one of the most important tasks for the UE is to finish the L1 measurement and to complete Tx beam selection as fast as possible (i.e., with minimum delay), in this sense, if we to consider L1-RSRP measurement under DRX mode, the measurement period will be always limited by DRX cycle, even Tadapted SSB<< TDRX, we cannot see the benefits of SSB adaptation with dense periodicity. From this, we proposal

- Proposal 7: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode

Furthermore, regarding how to guarantee the measurement accuracy at transition, from our understanding, one of the candidate options is that to prioritize adapted resources measurement (i.e., SSB sources after transition). After transition, the UE ready to receive adapted SSB resource, it is reasonable to measure adapted measurement resources with the highest priority and perform X samples L1 measurement filter. While, whether to release the measurement results based on legacy resources up to UE implementation, since due to the un-complete measurement, the measurement accuracy based on the SSB sources before transition would be sacrificed.

Proposal 8: To guarantee the measurement accuracy at transition

- UE can to perform measurement based on the SSB resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation

5 Conclusion

In this contribution, we provided our viewpoints on RRM requirements for NES common signal adaptation. The following observations and proposals are obtained:

Observation 1:

- For SSB burst periodicity adaptation, the indication is signaled by group common DCI, DCI format 2_9

  - (Unified) MAC-CE details for SSB burst periodicity adaptation is up to RAN2, depending on the corresponding discussions in OD-SSB feature

Observation 2: For L3 RRM measurement on SCell with SSB adaptation.

- Existing SMTC based L3 RRM framework can be inherited

- Additional SMTC configuration accommodating with SSB adaptation will be introduced and defined

Observation 3: For L3 measurement, the adapted SSB on neighbor cell is measured based on legacy SMTC

Proposal 1:

- Time domain positions of SSBs within a burst is same before and after SSB adaptation

- RAN4 to consider legacy SSB burst periodicity as starting point to define the requirements

- No RRM impacts for SSB adaptation requirement for SCell NCD-SSB.

Proposal 2: For SSB adaptation, RAN4 to define requirements considering the following cases

- HF index is same before and after SSB adaptation and SFN offset is same before and after SSB adaptation

- HF index is same before and after SSB adaptation and SFN offset is same or different before and after adaptation

  - Wait for RAN1 conclusions on whether to introduce explicit indication or up to UE implementation to perform blind SFN detection

Proposal 3: For RRM requirement impact on activated SCell measurement/detection, RAN4 to discuss how to specify time periods for intra-frequency SCell identification and measurement, and inter-frequency SCell identification and measurement after receiving RAN2 agreements on the detailed SMTC configuration mechanism (s) for NES

Proposal 4:

- If group-common DCI is considered to indicate the adaptation of SSB, for deactivated SCell measurement requirement, same as legacy.

- If MAC-CE can also be considered to indicate the adaptation of SSB, for deactivated SCell measurement, impose the same principle from OD-SSB case#2 (same frequency but with different periodicities) discussion to define RRM requirements

  - Wait for RAN2 conclusions on whether MAC-CE indication for SSB adaptation can be supported

Proposal 5: Wait for RAN2 conclusions on whether additional SMTC configuration according to SSB adaptation for measurement on deactivated SCell with SSB adaptation can be supported

Proposal 6:

- If separate MAC CE to activate SSB adaptation and SCell, define the SCell activation requirements based on the SSB periodicity after SSB adaptation

  - For the case SSB adaptation command comes together with SCell activation command

- If MAC CE to activate SCell but group-common DCI to activate SSB adaptation, RAN4 to discuss the feasibility of defining the SCell activation requirements for the case SSB adaptation command comes together with SCell activation command

Proposal 7: For L1-RSRP measurement based on the SSB periodicity after transition, if the adapted SSB periodicity is less than DRX cycle, UE to measure the SSB resource for adaptation using the measurement period corresponding to non-DRX mode

Proposal 8: To guarantee the measurement accuracy at transition

- UE can to perform measurement based on the SSB resources after transition after transition period, whether to release the measurement results from resources before transition up to UE implementation

6 References

[1] R4-2502689 “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei