3GPP TSG-RAN WG4 Meeting #114bis	R4-2504328

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.18.2

Source:	Ericsson

Title:	Simulation results for MMSE-IRC receiver for 8Rx reception

Document for:	Information

# 1	Introduction

In RAN4 #114 meeting, companies have agreed on the rank and MCS configurations for the PDSCH test cases for inter-cell interference scenario. In the meantime, companies are encouraged to bring simulation results for CQI reporting tests with both rank 1 and rank 2. For intra-cell interference scenario, there is one open issue left but no additional simulation cases are needed.

In this contribution, we mainly shared our simulation results for CQI reporting cases of inter-cell interference scenario, and resubmitted our results for other agreed cases. Corresponding observations and the analysis can be found in our companion discussion paper[2].

# 2	PDSCH requirements with Inter-cell interference

In this section, we first exhibited the simulation assumption that we used for evaluation, then we shared our simulation results comparing the performance with baseline receiver and MMSE-IRC receiver.

## 2.1	Simulation assumption

In our initial evaluations, for other parameters that are not included in the WF, we mostly reused the parameter assumption listed in the table 5.2.2.1.15-2 in TS38.101-4 with slightly extension for 8Rx reception:

Table 2.1-1: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for test 1-6, Disable for test 7-12 |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-6 N/A for Test 7-12 |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for Test 1-6 7.58 for Test 7-12 | 2.29 for Test 1-6 N/A for Test 2-1, 2-2 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | 1st SSB in Slot#0 for Test 1-6 2nd SSB in Slot #0 for Test 7-12 | 1st SSB in Slot#0 for Test 1-6 N/A for Test 7-12 |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 |  |  |  |  |  |

## 2.2	Simulation results

For the simulation results below, we have used the baseline receiver.

Table 2.2-1 PDSCH simulation results for 2 and 1 interference cell (baseline)

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 2 interference cells |  |  |  |  |  |  |  |  |
| 2 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 7.26 | 13.22 | 5.96 |
| 1 interference cell |  |  |  |  |  |  |  |  |
| 6 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 7.30 | 12.83 | 5.53 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |

For the simulation results below, we have used the simplified receiver.

Table 2.2-2 PDSCH simulation results for 2 and 1 interference cell (simplified)

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 2 interference cells |  |  |  |  |  |  |  |  |
| 2 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 9.07 | 13.25 | 4.18 |
| 1 interference cell |  |  |  |  |  |  |  |  |
| 6 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 8.6 | 12.82 | 4.2 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |

# 3	PDSCH requirements with Intra-cell inter-user interference

In this section, we first exhibited the simulation assumption that we used for evaluation, then we shared our simulation results comparing the performance with baseline receiver and MMSE-IRC receiver.

## 3.1	Simulation assumption

Table 3.1-1: Test parameters

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | 1000 | 1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 | 1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Orthogonal |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 |  |
| Number of HARQ Processes |  |  | 4 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

## 3.2	Simulation results

For the simulation results below, we have used the baseline receiver.

Table 3.2-1 PDSCH simulation results (baseline)

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 1 | 2+2 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 9.23 | N/A | INF |
| 2 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | 13.9 | N/A | INF |

For the simulation results below, we have used the simplified receiver.

Table 3.2-2 PDSCH simulation results (simplified)

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 1 | 2+2 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 9.23 | N/A | INF |
| 2 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | N/A | N/A | N/A |

# 4	CQI reporting requirements with Inter-cell interference

In this section, we first exhibited the simulation assumption that we used for evaluation, then we shared our simulation results comparing the performance with baseline receiver and MMSE-IRC receiver.

## 4.1	Simulation assumption

In our initial evaluations, for other parameters that are not included in the WF, we mostly reused the parameter assumption listed in the table 6.2.2.1.2.3-1.

Table 4.1-1 CQI reporting test with inter-cell interference

| Parameter |  |  | Unit | Test1 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |
| Bandwidth |  |  | MHz | 10 | 10 |
| Duplex Mode |  |  |  | FDD | FDD |
| Subcarrier spacing |  |  | kHz | 15 | 15 |
| SINR |  |  | dB | -2 | - |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| Interference Model |  |  |  |  | As specified in B.6.2 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 | 4 |
|  | CDM Type |  |  | FD-CDM2 | FD-CDM2 |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 | Row 5, (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 | 9 |
|  | CSI-RS periodicity and offset |  | slot | 10/1 | Same as serving cell |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 | 1 |
|  | CDM Type |  |  | FD-CDM2 | noCDM |
|  | Density (ρ) |  |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 3(6, -) | Row 2(6, -) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic | Periodic |
|  | CSI-IM RE pattern |  |  | 0 | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) | (6, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 | Same as serving cell |
| ReportConfigType |  |  |  | Periodic | Not configured |
| CQI-table |  |  |  | Table 2 | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI | Not configured |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband | Wideband |
| Sub-band Size |  |  | RB | 8 | - |
| Csi-ReportingBand |  |  |  | 1111111 | Not configured |
| CSI-Report periodicity and offset |  |  | slot | 5/0 | Not configured |
| aperiodicTriggeringOffset |  |  |  | Not configured | Not configured |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured | Not configured |
|  |  | CodebookSubsetRestriction |  | 000001 for rank 1 010000 for rank 2 | Not configured |
|  |  | RI Restriction |  | N/A | Not configured |
| Physical channel for CSI report |  |  |  | PUCCH | Not configured |
| CQI/RI/PMI delay |  |  | ms | 8 | Not configured |
| Maximum number of HARQ transmission |  |  |  | 1 | Not configured |
| Measurement channel |  |  |  | For Rank 1: As specified in Table A.4-2, TBS.2-1 For Rank 2: As specified in Table A.4-2, TBS.2-2 | - |
| INR (Note 6) |  |  | dB | N/A | 10.04 |
| Propagation condition |  |  |  | TDLA30-5 | AWGN |
| Antenna configuration |  |  |  | 2×8 | 1×8 |
| Correlation configuration |  |  |  | ULA Low | N/A |
| Note 1: The respective received power spectral density of each interfering cell relative to is defined by its associated INR value as specified in clause B.6.1. Note 2: Two cells are considered in which Cell 1 is the serving cell and Cell 2 is the interfering cell. Interfering cell is fully loaded. Note 3: Both cells are time-synchronous. Note 4: Static channel is used for the interference model. In case for white Gaussian noise model Cell 2 is not present. Note 5: SINR corresponds to of Cell 1 as defined in clause 4.4.5. Note 6: INR is defined in clause B.6.1. |  |  |  |  |  |

## 4.2	Simulation results

For the simulation results below, we can observe the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source.

Figure 4.2-1 Simulation results of CQI reporting with rank 1 (baseline receiver)

Figure 4.2-2 Simulation results of CQI reporting with rank 2 (baseline receiver)

# 5	Summary

In this contribution, we shared the simulation assumptions and results for PDSCH demodulation for both inter-cell and intra-cell interference scenarios, and shared CQI reporting simulation results for inter-cell interference scenario.

# References

[1]	R4-2502275: Way forward for [114][324] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #114, Athens, Greece