3GPP TSG-RAN WG4 Meeting #114bis	R4-2504410

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.10.3.2

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN 3MHz REFSENS requirements

Document for:	Approval

- Introduction

In RAN#103 plenary meeting, the new WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands.

In the contribution, the consideration for NR-NTN 3MHz REFSENS requirements is provided for discussion.

- Discussion

2.1		WI scope: Enhanced requirements and test methodology for NR and IoT NTN

First, regarding UE RF requirements, some objectives of the WI RF core part is shown below for reference.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

2.2		Previous Meeting Agreements

In RAN4#114, the approved WF [2] provided the guidance for following works. The partial contents are listed below for reference.

System parameters Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - Agreement: - Introduce 3MHz Channel bandwidth for NR-NTN in FR1-NTN bands as optional feature in Rel-19. - Further discuss whether 3MHz channel bandwidth is mandatory on certain bands in Rel-20 if there is a specific request from operators.

UE RF requirements Issue 2-2-1: Reference sensitivity - Agreement - For band n254, [2.2] dB improvement compared with 5 MHz CBW; - For band n256, [1.7]dB improvement compared with 5 MHz CBW; - FFS for band n255

2.3		UE RF requirements

2.3.1	Reference sensitivity

In TS 38.101-5, NR-NTN UE reference sensitivity is shown below for reference.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |

In TS 36.101[3], TN UE reference sensitivity is shown below for reference.

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 24 |  |  | -100 | -97 |  |  | FDD |
| 65 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |

The UL/DL frequency ranges for B24/B65/n255/n256 are shown below.

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| B24 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| N255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| B65 | 1920 MHz | – | 2010 MHz | 2110 MHz | – | 2200 MHz | FDD |
| N256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |

Proposal 1: Consider table before for discussion on bands n256 and n255 3MHz REFSENS.

| Band | REFSENS 3 MHz(dBm) | REFSENS 5 MHz(dBm) | FUL_low | FUL_high | FDL_low | FDL_high |
| --- | --- | --- | --- | --- | --- | --- |
| B24 |  | -100 | 1626.5 MH | 1660.5 MHz | 1525 MHz | 1559 MHz |
| N255 | [-101.7] | -100 | 1626.5 MH | 1660.5 MHz | 1525 MHz | 1559 MHz |
| B65 | -101.2 | -99.5 | 1920 MHz | 2010 MHz | 2110 MHz | 2200 MHz |
| N256 | [-101.2] | -99.5 | 1980 MHz | 2010 MHz | 2170 MHz | 2200 MHz |

3 Conclusion

In this paper, our view on NR-NTN 3MHz REFSENS UE RF requirements is provided.

Proposal 1: Consider table before for discussion on bands n256 and n255 3MHz REFSENS.

| Band | REFSENS 3 MHz(dBm) | REFSENS 5 MHz(dBm) | FUL_low | FUL_high | FDL_low | FDL_high |
| --- | --- | --- | --- | --- | --- | --- |
| B24 |  | -100 | 1626.5 MH | 1660.5 MHz | 1525 MHz | 1559 MHz |
| N255 | [-101.7] | -100 | 1626.5 MH | 1660.5 MHz | 1525 MHz | 1559 MHz |
| B65 | -101.2 | -99.5 | 1920 MHz | 2010 MHz | 2110 MHz | 2200 MHz |
| N256 | [-101.2] | -99.5 | 1980 MHz | 2010 MHz | 2170 MHz | 2200 MHz |

4 Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

- R4- 2502282, “WF for [113][311] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi

- TS 36.101, “Evolved Universal Terrestrial Radio Access (E-UTRA);User Equipment (UE) radio transmission and reception”