3GPP TSG-RAN WG4 Meeting #114bis	R4-2504413

Wuhan, Greece, Apr 7th – Apr 11th, 2025

Source:	Nokia

Title:	System parameters for LP-WUR

Agenda item:	7.26.2.1

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following agreements were made

# Topic #1: General and system parameters ### Sub-topic 1-1 General Issue 1-1-1: How to define two LP-WUR types Agreement: - Define two set of RF requirements as the baseline, considering the following aspects including - Noise figures - Interference performance - Modulation - Consider to merge two sets of requirements into one if the requirements are close to each other ### Sub-topic 1-2 System parameters Issue 1-2-1: Whether operation band clause for LP-WUS/WUR is needed (5.2M Operating bands for LP-WUS)? Agreement: - Operation band clause for LP-WUS/WUR is needed in Rel-19 - Keep the band with DL operating band defined, and SDL band is precluded Issue 1-2-2: Whether Channel bandwidth clause for LP-WUS/WUR is needed (5.3M Channel Bandwidth for LP-WUS)? Agreement: - [Transmission bandwidth configuration] for LP-WUS/WUR signal is needed to specify. Issue 1-2-3: Whether Channel raster clause for LP-WUS/WUR is needed (5.4M Channel raster for LP-WUS)? Way Forward: - No update of core spec for this part.

Discussion

System parameters in the TR

In the last meeting, it was agreed that only bands with a downlink range defined will be available for LP-WUS. Further, SDL bands were excluded from LP-WUS operation. This update is however not captured in the RF specific TR 38.774.

Update TR 38.774 to exclude SDL bands from the list of operating bands.

Transmission bandwidth configuration for LP-WUS

In previous meeting [1], it was agreed that transmission bandwidth configuration for LP-WUS will be specified under  section 5.3 with LP-WUS specific suffix of M. Below is a draft wording proposed for the section 5.3M

The transmission bandwidth configuration for LP-WUS shall be of length 11 RBs for both 15 KHz and 30 KHz SCS. LP-WUS RBs shall be surrounded by x blanked RBs on both sides. (Note: 11 RBs are for LP-WUS and x are for ASCS guard). LP-WUS RBs shall be flexibly allocated anywhere within the UE channel bandwidth by taking into account the guard RBs required for ACS.

Agree to the following wording for transmission bandwidth configuration specification for LP-WUS “The transmission bandwidth configuration for LP-WUS shall be of length 11 RBs for both 15 KHz and 30 KHz SCS. LP-WUS RBs shall be surrounded by x blanked RBs on both sides.  (Note: 11 RBs are for LP-WUS and x are for ASCS guard). LP-WUS RBs shall be flexibly allocated anywhere within the UE channel bandwidth by taking into account the guard RBs required for ACS.”.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Update TR 38.774 to exclude SDL bands from the list of operating bands.

Proposal 2: Agree to the following wording for transmission bandwidth configuration specification for LP-WUS “The transmission bandwidth configuration for LP-WUS shall be of length 11 RBs for both 15 KHz and 30 KHz SCS. LP-WUS RBs shall be surrounded by x blanked RBs on both sides.  (Note: 11 RBs are for LP-WUS and x are for ASCS guard). LP-WUS RBs shall be flexibly allocated anywhere within the UE channel bandwidth by taking into account the guard RBs required for ACS.”.

- References

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.