3GPP TSG-RAN WG4 Meeting #114bis	R4-2504417

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	TP for TR 38.774 on LP-WUS

Agenda item:	7.26.1

Document for:	Discussion

Introduction

In this text proposal for TR 38.774, we provide our inputs on section 7.1 and 8, as was agreed in RAN4#112bis. These inputs are based on the agreements achieved in the previous meetings.

- References

- R4-2503028, WF on UE RF requirements for LP-WUS, Vivo, RAN4#114, Athens, Greece, 17th – 21st Feb, 2025.

- R4-2420528, WF on UE RF requirements for LP-WUS, Vivo, RAN4#113, Orlando, US, 18th – 22nd Nov, 2024.

- R4-2417112, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112bis, Hefei, China, 14th – 18th Oct, 2024.

- R4-2414309, WF on UE RF requirements for LP-WUS, Vivo, RAN4#112, Maastricht, Netherlands, 19th – 23rd August, 2024.

****************************************** Start of TP ******************************************

### 7.1.3	Architecture and NF considerations

The basic architecture considered for the LP_WUR is a zero-IF architecture for both envelope and sequence based detectors. This assumption is common for both FR1 and FR2. This is just an assumption made to derive parameters which dictate the requirements. This assumption does not preclude any other RF implementations.

In case of FR1, only single RX is assumed. For FR2 requirement derivation, the baseline assumption is an OFDM based receiver with two receiver chains with mutually orthogonally polarized antennas.

In RAN4, there had been no consensus on what IM includes, and no agreement was achieved on individual values of NF and IM, so a joint number was agreed. There are two sets of NF + IM values agreed, and these will be used for REFSENS calculations. The values are,

- Set 1: 18 dB,

- Set 2: 13.5 dB.

The above mentioned numbers are for FR1.

### 7.1.4	REFSENS requirements

It was agreed to derive the REFSENS for LP-WUR in the typical RF manner with AWGN being the channel for which SNR is derived. Thus, the REFSENS for LR is defined as

REFSENSLR (dBm) = -174 + BWLR + NFLR + IMLR + SNRLR.

************************* UNCHANGED CLAUSES HAVE BEEN OMITTED *************************

****************************************** End of TP ******************************************