3GPP TSG-RAN WG4 Meeting #114bis	R4-2504443

Wuhan, China, April 7 – 11, 2025

Source:	Nokia

Title:	On Enhancements for Air-to-ground network for NR

Agenda item:	7.12.6

Document for:	Discussion

Introduction

The work Item Description (WID) of Rel-19 Enhancements for Air-to-ground (ATG) network for NR [1] describes the justification of the Work Item (WI) and the objectives of both the core and performance parts of the WI. Specifically, the Objective of the Performance part is as follows [1]:

### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The performance part of the work item includes: - Specify corresponding RRM performance and demodulation requirements for intra-band co-located and inter-band co-located DL CA. - Specify corresponding demodulation requirements for support of UL MIMO with 2TX for single CC for UE with dual-polarization antenna. - Specify demodulation requirements for support of DL MIMO for UE with limited test cases. - Specify conformance testing and conformance requirements for BS.

The highlighted objectives above are related to demodulation performance requirements and needed to be addressed in RAN4#114bis. In this contribution, we share our initial views on Rel-19 Enhancements for ATG network for NR for those objectives related to demodulation performance requirements.

Discussion

Each subsection, in the following, is dedicated to each of the Objectives of Performance part of the WI related to demodulation requirements.

Demodulation requirements for intra-band co-located and inter-band co-located DL CA

Referring to the Objective of Core part in [1], the examples of band combinations are CA_n79C for FR1 intra-band contiguous CA, and CA_n3A-n39A for FR1+FR1 inter-band CA, as highlighted below:

### 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]: - FR1 intra-band contiguous CA - Example band combination: CA_n79C - FR1+FR1 inter-band CA - Example band combination: CA_n3A-n39A - Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4] - Specify the TAE requirements for support of DL MIMO for BS. [RAN4].

The Objective of Core part of the WI provides examples of the considered band combinations, namely: CA_n79C for FR1 intra-band contiguous CA and CA_n3A_n39A for FR1+FR1 inter-band CA.

RAN4 to clarify the bandwidth combination sets for the FR1 intra-band contiguous CA in CA_n79C and for FR1+FR1 inter-band CA in CA_n3A-n39A. Single carrier requirements for each of the considered bandwidth sizes in the bandwidth combination sets shall then be specified.

Demodulation requirements for support of UL MIMO with 2TX

Referring to TS. 38.104 [2], the PUSCH requirements for ATG are defined in Table 8.2.14.2-1 and Table 8.2.14.2-2 for FDD and TDD, respectively. The Subcarrier Spacing (SCS) for FDD is 15 KHz with 5 MHz Bandwidth (BW), while for TDD 30 KHz SCS with 10 MHz BW. The requirements are considering 70% of maximum throughput, Type A, and assuming HARQ retransmission.

The ATG PUSCH requirements in TS 38.104 [2] are specified only for 1TX with 2RX case. As stated in one of the Objectives of Performance part, in Rel-19 Enhancement of ATG for NR, RAN4 need to specify the demodulation requirements for UL MIMO with 2TX.

The currently available PUSCH requirements for ATG in 38.104 are considering 1TX (with 2RX) for both FDD (15KHz SCS, 5MHz BW) and TDD (30KHz SCS, 10MHz BW), with Type A and HARQ retransmission.

The ATG PUSCH requirements for UL MIMO 2TX shall prioritize the same parameters as in 1TX case, namely, 2RX, FDD (15KHz SCS, 5MHz BW), TDD (30KHz SCS, 10MHz BW), Type A and HARQ retransmission, for the 70% of maximum throughput.

Demodulation requirements for support of DL MIMO for UE with limited test cases

In 38.101-4 [3], PDSCH minimum performance requirements for ATG have been defined for DL MIMO of 2x2 and 2x4 antenna configurations for both TDD and FDD.

TS 38.101-4 already specifies ATG PDSCH requirements for DL MIMO of 2x2 and 2x4 antenna configurations for both TDD and FDD.

RAN4 to discuss the considered DL MIMO configurations and the corresponding test cases for Rel-19 ATG PDSCH requirements.

Conclusion

In this contribution, we provide our views on Rel-19 Enhancements for ATG network for NR. The following Observations and Proposals were made:

Observation 1: The Objective of Core part of the WI provides examples of the considered band combinations, namely: CA_n79C for FR1 intra-band contiguous CA and CA_n3A_n39A for FR1+FR1 inter-band CA.

Proposal 1: RAN4 to clarify the bandwidth combination sets for the FR1 intra-band contiguous CA in CA_n79C and for FR1+FR1 inter-band CA in CA_n3A-n39A. Single carrier requirements for each of the considered bandwidth sizes in the bandwidth combination sets shall then be specified.

Observation 2: The currently available PUSCH requirements for ATG in 38.104 are considering 1TX (with 2RX) for both FDD (15KHz SCS, 5MHz BW) and TDD (30KHz SCS, 10MHz BW), with Type A and HARQ retransmission.

Proposal 2: The ATG PUSCH requirements for UL MIMO 2TX shall prioritize the same parameters as in 1TX case, namely, 2RX, FDD (15KHz SCS, 5MHz BW), TDD (30KHz SCS, 10MHz BW), Type A and HARQ retransmission, for the 70% of maximum throughput.

Observation 3: TS 38.101-4 already specifies ATG PDSCH requirements for DL MIMO of 2x2 and 2x4 antenna configurations for both TDD and FDD.

Proposal 3: RAN4 to discuss the considered DL MIMO configurations and the corresponding test cases for Rel-19 ATG PDSCH requirements.

- References

- RP-243285, Revised WID on Enhancements for Air-to-ground network for NR, 3GPP RAN#106, CMCC, et al., Madrid, Spain, December 9-12, 2024

- 3GPP TS 38.104, NR - Base Station (BS) radio transmission and reception, V18.8.0, 12-2024

- 3GPP TS 38.101-4, NR - User Equipment (UE) radio transmission and reception, Part 4: Performance requirements, V18.6.0, 12-2024