3GPP TSG-RAN WG4 Meeting #114bis														R4-2504456

Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda item:	7.17.2.1

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#113, which includes open issues for further discussion. In this contribution, we continue the discussion for FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor.

# Discussion

In the WF, the following agreements and open issues were captured.

## Sub-topic 1-1 Conditions to apply L3 measurement delay reduction by optimizing Rx BSF ### Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2. ### Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement in ad-hoc session: - UE is not required to perform FBS based fast L3 measurements when threshold is not configured to UE for scenario 1/2/3/4. Further discuss this issue. ### Issue 1-1-3: SSB processing delay/time for processing multiple beams received in a SMTC Agreement in RRM session (Monday): - No need to add additional processing time in the delay requirement for scenario 5. ## Sub-topic 1-2 Requirement of L3 measurement delay reduction by optimizing Rx BSF ### Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF Further discuss this issue. ### Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF Further discuss this issue. ### Issue 1-2-3: Requirement at transition between the normal L3 measurement and FBS L3 measurement Agreement: - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

FBS triggering condition

The first open issue is whether to limit the serving cell to FR2 serving cell. The general understanding is that FR2 network is deployed in some urban areas or hot spots and does not provide wide area coverage. Therefore, when the serving cell is an FR2 cell and the signal quality of such an FR2 serving cell is below some threshold, there is a need for a UE to activate FBS with the aim to find a target FR2 cell to hand over to. If a target FR2 cell cannot be identified, the UE may search for FR1 cells and consider handover to such FR1 cells.

If the serving cell is an FR1 cell, most likely the UE can identify another FR1 neighbor cell for handover if the serving cell quality is not good, considering that FR1 has good wide area coverage. Therefore, it is reasonable to limit the serving cell to FR2 serving cell.

Proposal 1: As part of FBS triggering condition, it is proposed to limit the serving cell to FR2 serving cell.

The second open issue is whether to add the hysteresis and timeToTrigger parameters. Usually for L3 measurement events where signal quality is considered, both Hysteresis and TimetoTrigger are configured to prevent ping-pong effect. For activation and deactivation of FBS based on signal quality, it is reasonable to consider both. That said, it is OK to leave it to RAN2 to decide.

Proposal 2: Both Hysteresis and TimetoTrigger can be considered to prevent ping-pong effect between activation and deactivation of FBS. It is OK to leave it to RAN2 to decide.

FBS exit condition

Here the exit condition is not the deactivation of FBS based on signal quality being above the threshold. Instead, it refers to the exceptional cases where FBS is not deactivated when considering the serving cell signal quality alone. The main reason is the need of UE power saving or overheating. For this purpose, UAI mechanism can be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed.

Proposal 3: To consider UE power consumption or overheating, UAI mechanism should be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed.

Furthermore, we want to discuss the issues and the proposed solution raised in [2].

Observation 1: Based on agreed triggering condition (serving cell’s signal quality), it is possible that UE will always activating multiple panels for L3 measurement, which is problematic for at least following case: - Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving. - Case 2: UE is still in the cell centre and the triggering conditions are met due to block. Observation 2: Rel-18 UAI indication is not sufficient as the decision is made by gNB, and gNB has no idea whether fast L3 measurement is urgent or not (e.g. case 1 and case 2 mentioned above). And gNB even don’t known whether fast measurement is triggered or not. Proposal 1: Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification.

First, we agree the two cases would lead to inefficient use of FBS and hence UE power consumption. However, we think the proposed solution that UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results is problematic since it is hard for the network/UE to predict with good confidence that the UE is currently in one of the two cases, i.e., Case 1 and Case 2. In addition, even after the UE reports qualified measurement results, the network may want to wait for more report to be sent in before making a handover/SCell change decision. Therefore, we propose to consider the two possible solutions:

- Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue.

- Option 2: After the UE sends a qualified measurement report, the UE can wait for a certain period. If no decision form the network is made, the UE can deactivate FBS. To implement it, a timer can be used. The timer starts when the UE sends a qualified measurement report (say A2 or A3), and when the timer expires, the UE exits from FBS operation. The value of the timer should be at least two or three times the L3 SSB-based measurement periods.

Proposal 4: To address the power consumption issues in the two cases (Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving; Case 2: UE is still in the cell centre and the triggering conditions are met due to block.), it is proposed to consider the following two options:

- Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue.

- Option 2: After the UE sends a qualified measurement report, the UE can start a timer, and when the timer expires, the UE exits from FBS operation.

Requirement of L3 measurement delay reduction by optimizing Rx BSF

For the two remaining open issues:

- Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

- Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

Our view is that we can directly discuss it when reviewing draft CRs, based on the previous agreement that Rx BSF can be reduced according to the UE capability.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: As part of FBS triggering condition, it is proposed to limit the serving cell to FR2 serving cell.

Proposal 2: Both Hysteresis and TimetoTrigger can be considered to prevent ping-pong effect between activation and deactivation of FBS. It is OK to leave it to RAN2 to decide.

Proposal 3: To consider UE power consumption or overheating, UAI mechanism should be used. Either the existing UAI IE “multiRx-PreferenceFR2-r18” can be re-purposed to indicate not only L1 multi-RX operation, but also L3 measurement delay reduction, or a new IE such as “FastL3Measurement-r19” can be designed.

Proposal 4: To address the power consumption issues in the two cases (Case 1: UE moves to the cell edge when the triggering conditions met and UE stop moving; Case 2: UE is still in the cell centre and the triggering conditions are met due to block.), it is proposed to consider the following two options:

- Option 1: No specific action is taken, assuming UE can use UAI to inform the network that it needs to fall back to Non-FBS mode (i.e., deactivate FBS operation) if power consumption/overheating becomes an issue.

- Option 2: After the UE sends a qualified measurement report, the UE can start a timer, and when the timer expires, the UE exits from FBS operation.

# References

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT

- R4-2501218, Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, Huawei, HiSilicon