3GPP TSG RAN WG4 Meeting #114-bis	R4-2504458

Wuhan, China 07 April 2025 – 11 April 2025

Agenda Item:	7.3.3

Source: 	Ericsson

Title:	Discussion on RRM Performance Requirement for NR Less Than 5 MHz CA/DC

Document for:	Discussion

# Introduction

The RRM core requirements related to the Rel-19 NR CA/DC for less than 5 MHz core requirement work item have been completed and will be introduced in Rel-19 for less than 5 MHz based on [1]. In the last meeting, TCs were listed and allocated to companies [2]. This paper elaborates on responsible test cases and requirements for CA/DC.

# Test Cases on SCell activation delay with less than 5 MHz

Scenarios

The NR less than 5 MHz SCell activation delay requirements for inter-band NR CA are defined in clause 8.3.2 of [1]. These requirements are applicable for the following SCell activation conditions

- FR1 known SCell with meas. period <= 2400ms

- FR1 known SCell with meas. period > 2400ms

- FR1 unknown SCell without active serving cell(s) on the same band

- FR1 unknown SCell without TCI indication in MAC-CE, and with semi-persistent CSI-RS is used for CSI reporting

- FR1 unknown SCell without TCI indication in MAC-CE, and with periodic CSI-RS is used for CSI reporting

It is noted that only co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band (example band n100) and 5 MHz or 10 MHz CBW in the other band (example band n100) should be considered in the test cases [1].

PCell should be on 5 MHz or 10 MHz CBW.

Scell should be on 3MHz CBW.

SCell Activation and deactivation of known and unknown SCell in FR1 is relevant for less than 5 MHz.

RAN 4 should agree on test cases for the following configurations for known and unknown SCell activation delay with less than 5 MHz

- PCC: 10 MHZ and SCC: 3 MHz in FR1.

Measurement Cycle

In Rel 15 SCell activation delay test case scenarios, in clauses A.6.5.3.1 and A.6.5.3.2, measCycleSCell is considered to be 160 ms and 640 ms, respectively, for known SCell in FR1. Since we are taking Rel-15 as a baseline for NR less than 5 MHz, it is reasonable to consider both 160 ms and 640 ms for measCycleSCell. Therefore, we propose to capture both measurement cycles for frequencies less than 5 MHz.

NR test cases for CA below 5 MHz should be defined for both 160 ms and 640 ms measurement cycles for a known SCell in FR1, under non-DRX conditions.

Test configurations

For SCell activation delay with less than 5 MHz, the following test configurations are needed for SCell activation in non DRX where PCell is configured with 5 or 10 MHz CBW and SCell is configured with 3 MHz. The test configurations for PCell and SCell is proposed in Table 1 and Table 2, respectively. RAN4 should discuss and agree on the test configurations.

Table 1: known FR1 SCell activation in non-DRX for 160 ms SCell measurement cycle supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥5 MHz bandwidth, FDD duplex mode |

‘Table 2: known FR1 SCell activation in non-DRX for 160 ms SCell measurement cycle supported test configurations for NR SCell with less than 5 MHz

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, < 5 MHz bandwidth, FDD duplex mode (12 PRB) |
| Note: The UE is required to pass the test with configuration 1 for every supported 3 MHz band in FR1. If the UE indicates support for band n100, the UE is also required to pass the test with configuration 2 in addition to the test with configuration 1. |  |

The SCell activation delay with less than 5 MHz test case is defined based on the test configurations shown in Tables 1 and 2.

Test requirements:

The NR less than 5 MHz test case extends clauses A.6.5.3.1, A.6.5.3.2, and A.6.5.3.3. The general test parameters for FR1 PCell with 10MHz CBW should follow the legacy requirements specified in Table A.6.5.3.1-2 for NR. For less than 5 MHz, the SCell should follow the general parameters stated in Table A.6.5.3.1-2 for NR, except the parameters stated in Table 3.

Table 3: Cell specific test parameters for NR SCell for known FR1 SCell activation case, 160 ms SCell measurement cycle with less than 5 MHz

| c |  | Unit | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| BWchannel | ConfigSCell 1, | MHz | 3 MHz |  |  |
| BWoccupied | ConfigSCell 1 | RB | 12 |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.X FDD |  |  |
| SSB Configuration | ConfigSCell 1 |  | SSB.13 FR1 |  |  |
| Io Note3 | ConfigSCell 1 | dBm/ 2.7 MHz | -60.31 | -58.58 | -58.58 |
| NOTE : On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |  |

The test parameters for activating known and unknown FR1 SCells with less than 5 MHz utilize the general parameters outlined in Table A.6.5.3.1-2, except for those specified in Table 3.

Time Window

Furthermore, it is specified that due to the puncturing UE takes 2 Trs to 3 Trs extra time to read the SSB. Hence, RAN4 should discuss and agree on the time window for SCell activation in known and unknown cases.

RAN4 should discuss and agree on the time window T1, T2, and T3 for SCell activation in known and unknown test cases.

For the unknown case, the T1 activation time is set to 100 ms, i.e., the SCell shall be configured within 100 ms. Due to the reduced BW, the SSB reading gets extended; hence, the activation time for SCell follows the equation below.

Tactivation_time = TFirstSSB_MAX + TSMTC_MAX + 2*Trs + T∆ + 5 ms, where, T∆ = 2*Trs [1]

If we assume,

TFirstSSB_MAX =20ms

TSMTC_MAX = 10ms

Trs =10ms

Then, Tactivation_time =105ms.

To ensure the test accurately reflects the activation delay measurement, T1 should be at least equal to the full activation time window, so the SCell is detected just at or after the start of measurement. To avoid premature detection and provide a valid test interval, T1 should be set larger than Tactivation_time. A common practice is to set a ~5–10 ms buffer. Hence, T1 should be set to 110ms.

RAN4 should discuss and decide the T1 value for the unknown FR1 SCell activation case

- Option 1: T1=110ms

- Option 2: T1=115ms.

Based on the agreed test cases and the above proposal, a change request containing the above-discussed types of SCell activation delay is presented in [4].

# Test Case on Addition and release delay of PSCell with less than 5 MHz

Scenarios

The NR less than 5 MHz PSCell activation delay requirements for inter-band NR CA are defined in clause 8.9A.2 of [1]. In the last meeting, we agreed to introduce a new test case under Clause A.6.5.11.X to evaluate the PSCell addition and release delay performance for NR less than 5 MHz in FR1. PSCell addition and release delay is necessary to ensure the UE can meet performance requirements when dynamically managing dual connectivity under NR-DC. Specifically, this test evaluates the delay involved when adding a PSCell that was previously unknown to the UE and releasing it after it has been configured and CSI reporting has started.

In this test scenario, the UE begins with no prior knowledge of the PSCell and must complete RACH, CSI setup, and release within defined time windows.

The scenario uses two FR1 PCell and PSCell on separate carriers for less than 5 MHz:

- Cell 1: NR PCell at RF Channel 1 (10 MHz)

- Cell 2: NR PSCell at RF Channel 2 (3 MHz)

The PSCell addition and release delay TC scenario considers two cells (FR1 PCell and PSCell) on separate carriers. Where,

- Cell 1: NR PCell at RF Channel 1 (10 MHz)

- •	Cell 2: NR PSCell at RF Channel 2 (3 MHz).

Test configurations and parameters

As defined in the core requirements, the PSCell under test should be configured with a 3 MHz FDD channel, with an SCS of 15 kHz, as depicted in Table 4. Apart from that, the general parameters for NR PSCell addition and release should be reused here.

Table 4: Supported test configurations for FR1 PSCell Addition and Release with less than 5 MHz

| Configuration | Description |
| --- | --- |
| 1 | NR SCS 15 kHz, BW 3 MHz (12 PRB), FDD |

The PSCell addition and release delay with less than 5 MHz test cases are defined based on the test configuration shown in Table 4.

Furthermore, cell-specific parameters for PCell with 10 MHz CBW utilize the parameters specified in Table A.6.5.11.1-2. For the PSCell with a 3 MHz CBW, Table A.6.5.11.1-2 will serve as the baseline parameters and will be applied to Configuration 1, except for those specified in Table 5. These parameters are already defined in Rel-18 for less than 5 MHz.

Table 5: Cell Specific Parameters for FR1 PSCell Addition and Release with less than 5 MHz

| Parameter | Unit | Config | Cell 1 | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T3 |  | T4 |  |
| BWchannel | MHz | 1 | 3: NPRB,c = 12 |  |  |  |  |  |  |  |
| Data PRBs allocated |  | 1 | 12 |  |  |  |  |  |  |  |
| RMSI CORESET parameters |  | 1 | CR.1.2 FDD |  |  |  |  |  |  |  |
| Dedicated CORESET parameters |  | 1 | CCR.1.6 FDD |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1 | 15 |  |  |  |  |  |  |  |
| SSB configuration |  | 1 | SSB.13 FR1 |  |  |  |  |  |  |  |
| IoNote | dBm/3 MHz | 1 | TBD | N/A |  | TBD |  | TBD |  | TBD |
| NOTE : Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

The PSCell addition and release delay for the less than 5 MHz test case in Table A.6.5.11.1-2 will serve as the baseline parameter. This will be applied to configuration 1, except for those specified in Table 5.

Furthermore, if proposal 9 is approved, the total received power spectral density over the occupied bandwidth, Io, for cell 1 can utilize the value specified for 10 MHz, which is -59.78 dB. Otherwise, RAN4 should study and decide the Io for cell 1.

For cell 1 (NR PCell at RF Channel 1 with 10 MHz CBW) Io value can be set to -59.78. Otherwise, FFS.

For cell 2 (PSCell), as the CBW is 3 MHz, we need to study and agree on the Io value for NR less than 5MHz CA.

RAN4 should discuss and set the Io values for NR PSCell at RF Channel 2 with 3 MHz CBW.

Test requirements:

Clause A.6.5.11.1 defines and captures the NR PSCell addition and release delay. This test case for bandwidths under 5 MHz is an extension of Clause A.6.5.11.1. In most instances, the general test parameters align with the legacy requirements outlined in Table A.6.5.11.1-2 for NR-DC.

Cell-specific parameters for NR PSCell addition and release delay can be reused. The parameters are specified in Table A.6.5.11.1-2.

According to the agreed core CR [1] Additional delay formula should be revised as,

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + 3* T∆ + TPSCell_ DU + 2 ms with 12 PRB SSB

RAN4 to capture the following text for PSCell addition and release delay test requirement

- Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + 3* T∆ + TPSCell_ DU + 2 ms with 12 PRB SSB bandwidth.

Based on these discussions, proposals, and observations, we provide a test case draft. Depending on the outcome of the discussion on configuration aspects and common settings for NR-U test cases, other aspects of the test may need to be updated.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for NR CA/DC less than 5 MHz and made the following observations and proposals.

Observation 1	PCell should be on 5 MHz or 10 MHz CBW.

Observation 2	Scell should be on 3MHz CBW.

Observation 3	SCell Activation and deactivation of known and unknown SCell in FR1 is relevant for less than 5 MHz.

Observation 4	Cell-specific parameters for NR PSCell addition and release delay can be reused. The parameters are specified in Table A.6.5.11.1-2.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN 4 should agree on test cases for the following configurations for known and unknown SCell activation delay with less than 5 MHz

	PCC: 10 MHZ and SCC: 3 MHz in FR1.

Proposal 2	NR test cases for CA below 5 MHz should be defined for both 160 ms and 640 ms measurement cycles for a known SCell in FR1, under non-DRX conditions.

Proposal 3	The SCell activation delay with less than 5 MHz test case is defined based on the test configurations shown in Tables 1 and 2.

Proposal 4	The test parameters for activating known and unknown FR1 SCells with less than 5 MHz utilize the general parameters outlined in Table A.6.5.3.1-2, except for those specified in Table 3.

Proposal 5	RAN4 should discuss and agree on the time window T1, T2, and T3 for SCell activation in known and unknown test cases.

Proposal 6	RAN4 should discuss and decide the T1 value for the unknown FR1 SCell activation case

Option 1: T1=110ms

Option 2: T1=115ms.

Proposal 7	The PSCell addition and release delay TC scenario considers two cells (FR1 PCell and PSCell) on separate carriers. Where,

Cell 1: NR PCell at RF Channel 1 (10 MHz)

•	Cell 2: NR PSCell at RF Channel 2 (3 MHz).

Proposal 8	The PSCell addition and release delay with less than 5 MHz test cases are defined based on the test configuration shown in Table 4.

Proposal 9	The PSCell addition and release delay for the less than 5 MHz test case in Table A.6.5.11.1-2 will serve as the baseline parameter. This will be applied to configuration 1, except for those specified in Table 5.

Proposal 10	For cell 1 (NR PCell at RF Channel 1 with 10 MHz CBW) Io value can be set to -59.78. Otherwise, FFS.

Proposal 11	RAN4 should discuss and set the Io values for NR PSCell at RF Channel 2 with 3 MHz CBW.

Proposal 12	RAN4 to capture the following text for PSCell addition and release delay test requirement

Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + 3* T∆ + TPSCell_ DU + 2 ms with 12 PRB SSB bandwidth.

# References

- RP- 241622 “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel

- R4-2502608, Big CR for less than 5MHz RRM requirements in Rel-19, Intel Corporation.

- R4-2502586, WF on RRM requirements for NR_FR1_lessthan_5MHz_BW_Ph2, Intel Corporation

- R4-2504460, Draft CR for TC on SCell Activation Delay Requirement for Deactivated SCell Rel-19, Ericsson

- R4-2504459 Draft CR for TC on PSCell addition change delay in Rel-19