3GPP TSG RAN WG4 Meeting #114-bis	R4-2504464

Wuhan, China 07 April 2025 – 11 April 2025

Agenda Item:	7.12.5

Source: 	Ericsson

Title:	Discussions on RRM Performance Enhancement Requirements for Rel-19 ATG CA

Document for:	Discussion

# Introduction

A new Rel-19 Work Item on Air-to-Ground (ATG) for NR has been approved and revised [1]. RAN4 made significant progress in defining the core RRM requirements at the last meeting; however, the CRs have not yet been endorsed. In this contribution, we discuss our initial views on the performance requirements based on the agreements regarding core requirements and the open issues that remain.

# Discussion

In this section, we discuss the possible test configurations and test cases based on the core requirements outlined in the agreements.

## Test Configurations

In accordance with the ATG WID in [1] and the agreement outlined in [2], the Rel-19 ATG requirements apply to NR DL CA for both co-located intra-band and inter-band scenarios. Therefore, the new test cases developed for ATG will only be relevant to DL Co-existence FR1 CA.

New test cases developed for NR ATG CA will only be relevant to DL Co-existence FR1 CA.

## Reference measurements channels (RMCs)

The RRM ATG test cases use RMCs in different types of test cases to verify the core requirements. In rel-18, RMC values for ATG are revised based on legacy RMCs defined for NR. For ATG NR CA, we can reuse the RMCs defined for Rel-18 ATG for ATG test cases.

Rel-18 NR ATG RMCs can be reused for ATG NR CA test cases.

## OFDM Channel Noise Generator (OCNG)

In Rl-18, OCNG for FR1 is also defined in the RRM ATG test cases to model the usage of the radio resources. In Rel-19 NR ATG CA, similar OCNG pattern can be used that is defined in Rel-18 for ATG

Rel-18 NR ATG OCNG can be reused for ATG NR CA test cases.

## SCell Impact

In Rel-19 ATG CA, a single SCell is supported. Therefore, RAN4 should define new test cases related to a single SCC. FR1 co-located DL intra-band contiguous CA and FR1 co-located DL inter-band CA are agreed upon; the new test cases should capture only this criterion. However, since we are using legacy NR requirements as a baseline, legacy NR configurations should be used as a baseline for ATG NR CA. we believe no major modifications are needed.

RAN4 should reuse NR SCell test configurations for ATG DL CA.

## Test Case

So far, we have resolved most core performance issues; however, some remain open. In accordance with our agreement and the FFS from the WFs in Annex A, we assessed potential test cases. Not all core requirements necessitate new test cases; if the UE behavior is validated in one instance, further testing for the same behavior is not required.

In Rel-18 ATG, RAN4 introduced test cases for intra-frequency and inter-frequency cell reselection, RRC Connected state mobility, timing, RLM, BFD/LR, active BWP, and L1 RSRP/L1-SINR measurements.

For Rel-19, we need to agree on the test cases relevant to ATG DL CA. Below is a draft list of test cases for RAN4 discussion.

Table 1: Test case Lists for ATG CA in Rel-19

| No. | Item of Core Requirements | Type of Test Cases | Subclause | Note |
| --- | --- | --- | --- | --- |
| 1-1 | Interruption | Interruptions during measurements on deactivated NR SCC in FR1 | A.19.4.X | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.1 |
| 1-2 |  | Interruptions at NR SRS antenna port switching |  | Extend NR test cases for ATG-specific interruption behavior based on A.6.5.2.4 |
| 2-1 | SCell Activation Delay | SCell activation and deactivation of Scell for ATG | A.19.4.X | Modify Test case for SCell known, unknown based on A.6.5.3.1 / A.6.5.3.2 / A.6.5.3.3 |
| 3-1 | Beam Failure Recovery | Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | A.19.4.2.X | Modify test case based on A.6.5.5 |
| 4-1 | Active BWP Switch Delay | NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX | A.19.4.3.X | New test case required based on A.6.5.6.1.1 |
| 5-1 | L3 Measurement | CSI-RS based intra and inter frequency Measurements for event-triggered reporting tests without gap under non-DRX | A.19.X.X | New test case required based on A.6.6.10 and A.6.6.11 |

Furthermore, based on the agreements on the open issues we may need to introduce new test cases in future meetings. RAN4 should discuss and finalize the test cases based on Table 1 and decide on the work split during this meeting.

RAN4 should discuss and agree on the test cases based on Table 1.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM aspects for ATG UE DL CA and made the following observations.

Observation 1	New test cases developed for NR ATG CA will only be relevant to DL Co-existence FR1 CA.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Rel-18 NR ATG RMCs can be reused for ATG NR CA test cases.

Proposal 2	Rel-18 NR ATG OCNG can be reused for ATG NR CA test cases.

Proposal 3	RAN4 should reuse NR SCell test configurations for ATG DL CA.

Proposal 4	RAN4 should discuss and agree on the test cases based on Table 1.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420338, WF on NR ATG UE RF, CMCC

# Annex A

Table 2: Potential test cases assessment based on the agreements for ATG CA in Rel-19

| Item | Agreement Summary | FFS | Impacted Sections in TS 38.133 | Relevant Test Configurations/ Test cases | Test Case Modification/New Requirement |
| --- | --- | --- | --- | --- | --- |
| Scenario Definition | Only consider FR1 co-located DL intra-band contiguous CA and FR1 co-located DL inter-band CA. | - | 3.6.16 Applicability of requirements for ATG | A.19.1.2 | No test case required |
| Co-location Signaling | No need to provide co-location information to ATG UE at least in Rel-19. | - | 9.3D NR inter-frequency measurements for ATG | A.19.5.5.1 (CGI ID); validate measurement behavior without co-location info | No test case required |
| SCell Count | Only consider single SCell for R19 ATG CA. | - | 8.3D SCell Activation and Deactivation Delay for ATG | - | No test case required |
| UE Antenna Types | Same antenna type for intra-band; different or same for inter-band. | To differentiate Case 4 and Case 5, an additional UE capability to indicate common or separate Rx beam is needed. | 9.1D General measurement requirement for ATG 9.8D L1-SINR measurements for Reporting for ATG | - | No test case required |
| MRTD | 33 µs for the MRTD of ATG inter-band CA. | MRTD for ATG inter-band CA - FFS. | 7.6D (New) Maximum Receive Timing Difference for ATG | - | No test case required |
| MTTD | No need to define MTTD requirements. | - | 7.6D (New) Maximum Receive Timing Difference for ATG | Not applicable | No test case required |
| Cell Reselection | Introduce ΔRIB,c, the value refers to RF spec. | - | 4.2D Cell Re-selection for ATG | - | Can be verified by Rel-18 Cell Reselection tests |
| Interruption Requirements | Introduce interruption at SRS antenna port switching. | NR SRS carrier based switching interruption requirement. | 8.2D Interruption 8.6D Active BWP switch delay for ATG | A.6.5.2.1 A.6.5.2.4 | New test case required. NR test cases can be modified |
| SCell Activation Delay | Define SCell Activation Delay and reuse FR1. | SSB-less SCell; enhancement needed for band combinations, antenna types. | 8.3D SCell Activation and Deactivation Delay for ATG | A.6.5.3.1 A.6.5.3.2 A.6.5.3.3 | New test case needed for SCell known, unknown and CBM cases |
| SCell Addition/Activation Mechanism | - | Distance triggered SCell addition and/or activation. | 8.3D SCell Activation and Deactivation Delay for ATG | Requires new test case (distance-based triggering) | New test case needed, if agreed |
| Beam Failure Recovery | Introduce Beam failure recovery in SCell for ATG CA, legacy reused. | - | 8.5D Link Recovery Procedures for ATG | A.6.5.5 | New test case required for SCell in A.19.4.2 |
| Active BWP Switch Delay | Introduce and reuse legacy for multiple CCs. | - | 8.6D Active BWP switch delay for ATG | A.6.5.6.1.1 | New test case required in A.19.4.3 |
| Pre-configured measurement gap activation/deactivation delay | Introduce delay requirement upon SCell activation/deactivation, legacy reused. | - | 8.19D Pre-configured measurement gap activation/deactivation delay for ATG | A.6.6.17 | New test case required for Pre-MG activation/deactivation delay |
| Deactivated SCell Measurement | For all except Case 4: L1/L3 sharing factor not needed. | Case 4: L1/L3 sharing factor needed. | 9.5D L1-RSRP measurements for Reporting for ATG | - | - |
| Scheduling Restrictions | Scheduling restriction for intra/inter-band CA in TDD. | FFS for Case 4 UEs and different subcarrier spacing. | Section 8.3D, Section 9.1D | A.19.6.1.1 | No test case required |
| CSSF | Reuse the legacy scheme: PCC and SCC use different searchers. | - | 8.3D SCell Activation and Deactivation Delay for ATG or Measurement scheduling logic | - | No test case required |