3GPP TSG-RAN WG4 Meeting #114-bis 	R4-2504480

Wuhan, China, April 7th – 11th, 2025

Agenda Item: 7.23.3.2

Source: MediaTek inc.

Title: Discussion on SBFD Operation

Document for: Discussion

# Introduction

The work item (WI) for Evolution of NR duplex operation: Sub-band full duplex (SBFD) has the following objectives [1]:

The objectives are as follows: - For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In the last meeting, RRM group discussed several issues on the impacted RRM specification and whether new RRM requirements are needed to handle the new SBFD operation and CLI handling mechanism, where the issues and agreements are captured in the way forward (WF) [2] and collection of companies proposals in Topic summary [3]. In this contribution, we provide further discussion on the issues captured in the WF.

# Discussion on the RRM impact due SBFD operation

In SBFD operation UE remains a half-duplex node. This implies that in SBFD symbols, the UE can perform either DL reception (on DL subbands) or UL transmission (on UL subband), but not both. To avoid collisions between DL receptions and UL transmissions, the network must provide a way for UE to determine the link direction on SBFD symbols, i.e., whether the UE transmit or receive in a SBFD symbol. Therefore, several issues were brought up to the RRM discussion by multiple companies, which are captured in the WF.

# Discussion on the RRM impact due SBFD operation

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain:

- Agreements - Further discuss measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case - Option 1: existing measurement period for CSI-RS based L1 measurement apply, with the assumption that UE measures both DL subbands on each CSI-RS resource occasion. - Option 2: doubling existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion. - Option 3: Consider the UE measurement capability - FFS whether and how to account for the UE capability on CSI processing time. - Further discuss the accuracy requirements for the following cases - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD).

Measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case: First of all, RAN4 RRM should refer to RAN1 agreement on the CSI processing delay:

Agreement [RAN1#118b] For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

On one hand, provided that the first UE capability indicates a UE being able to follow legacy CSI processing, hence, the measurement period requirements for CSI-RS based L1 measurement in SBFD symbols with DUD case should be the same as legacy. On the other hand, the second UE capability increase the CSI processing delay by a factor of two, hence, RAN4 should check whether the new values of CSI processing delay would impact the CSI measurement requirements. Therefore, RRM group should check the existing spec in terms of CSI computation delay requirements as well as the CSI resource periodicity information, which are provided below:

From [38.214] Clause 5.4: Table 5.4-1: CSI computation delay requirement 1 Z1 [symbols]. Z1; Z'1. 0; 10; 8. 1; 13; 11. 2; 25; 21. 3; 43; 36. Table 5.4-2: CSI computation delay requirement 2 Z1 [symbols]; Z2 [symbols]; Z3 [symbols]. Z1; Z'1; Z2; Z'2; Z3; Z'3. 0; 22; 16; 40; 37; 22; X0. 1; 33; 30; 72; 69; 33; X1. 2; 44; 42; 141; 140; min(44,X3X3+ KB1); X2. 3; 97; 85; 152; 140; min(97, X3+ KB2); X3. 5; 388; 340; 608; 560; min(388, X5+ KB3); X5. 6; 776; 680; 1216; 1120; min(776, X6+ KB4); X6.

From [38.331] Clause 6.3.2: #### CSI-ResourcePeriodicityAndOffset The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources, and for periodic and semi-persistent reporting on PUCCH. both, the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, value slots5 corresponds to 5 slots, and so on. CSI-ResourcePeriodicityAndOffset information element -- ASN1START -- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START CSI-ResourcePeriodicityAndOffset ::= CHOICE { slots4 INTEGER (0..3), slots5 INTEGER (0..4), slots8 INTEGER (0..7), slots10 INTEGER (0..9), slots16 INTEGER (0..15), slots20 INTEGER (0..19), slots32 INTEGER (0..31), slots40 INTEGER (0..39), slots64 INTEGER (0..63), slots80 INTEGER (0..79), slots160 INTEGER (0..159), slots320 INTEGER (0..319), slots640 INTEGER (0..639) } -- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP -- ASN1STOP

From the existing CSI computation delay requirements [38.214], the maximum delay is approximately 4ms regardless of the numerology being used and regardless of the CSI resource periodicity. This can be seen aligned with CSI resource periodicity of minimum of 4ms. However, provided that the new UE capability increases the CSI computation delay by a factor of 2, hence, RAN4 requirements should be at least of 8 slots instead of 4 slots. Thus, RRM to introduce new requirements where the new requirements include a minimum threshold of 8ms on top of existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

Observation 1: The existing legacy CSI computation time can be up to 4ms.

Observation 2: The minimum periodicity for CSI resource configuration is 4 slots.

- For SBFD-aware UE with DUD case, RRM to introduce new requirements where the new requirements include a minimum threshold of 8ms on top of existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

Accuracy requirements:

Case 1: X1 >= X3 and X2 >= X3 has both DL subband satisfy the existing minimum requirements and hence there is no concern on supporting this case with no change in measurement nor accuracy requirements.

Observation 3: Case 1: X1 >= X3 and X2 >= X3 has both DL subband satisfy the existing minimum requirements.

- Existing accuracy requirements for Case 1 (X1 >= X3 and X2 >= X3) can be reused.

Case 2: X1 >= X3 and X2 < X3 has at least one DL subband satisfy the existing minimum requirements, i.e. one DL subband has CSI-RS that satisfy the minimum condition. Therefore, there is no concern on supporting this case with no change in measurement nor accuracy requirements.

Observation 4: Case 2: X1 >= X3 and X2 < X3 has at least one DL subband satisfy the existing minimum requirements, i.e. one DL subband has CSI-RS that satisfy the minimum condition.

- Existing accuracy requirements for Case 2 (X1 >= X3 and X2 < X3) can be reused.

Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 none of the two DL subband satisfy the existing minimum requirements, i.e. none of the two DL subband has sufficient CSI-RS resources to satisfy the minimum condition. Hence, it is not clear how to derive the requirements without conducting new simulations. Thus, Case 3 should be either deprioritized or RRM session shall conduct simulation to decide on the exact value of minimum PRB per each DL subband.

Observation 5: Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 none of the two DL subband satisfy the existing minimum requirements.

- RAN4 to deprioritize case 3, because existing accuracy requirements can’t be reused.

Issue 2-1-2: Requirements for CSI-RS based L3 measurement:

- Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

With the current existing measurement object (MO) configuration, a UE can’t differentiate whether the neighbouring cells are operating with DU/DUD configuration, also, the UE doesn’t know the exact location of two subband in DUD configuration. Therefore, CSI-RS based L3 measurement should be deprioritized.

Observation 6: The existing MO configures a UE with the location of measurement resources using the starting point and length of that measurement’s resources.

- RAN4 shall deprioritize CSI-RS based L3 measurements.

# Discussion on issues related to LSs from RAN1

RAN4 has the issues below, which are related to LS sent to RAN1 in [4]. RAN1 responded with answers and actions for each issue [5].

#### Issue 2-1-1a: CSI-RS L1 measurement – collision with UL - Recommended WF - It is suggested to wait for RAN1 reply. #### Issue 2-1-1c: CSI-RS L1 measurement – different symbol types - Recommended WF - It is suggested to wait for RAN1 reply.

Issue 2-1-1a: CSI-RS L1 measurement – collision with UL: RAN1 provided the following response:

Issue#1: Collision Case 2 that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements For collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) that involves CSI-RS resources used for L1-RSRP/SINR and RLM/BFD/CBD measurements, RAN1 would like to provide the following response: The intention of the mentioned RAN1 agreement is to follow the existing prioritization rules between semi-static DL channels/signals and dynamically scheduled UL transmission including semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement. The existing prioritization rules for serving-cell for collision Case 2 are as follows: - TS 38.213, Clause 11.1 “Slot configuration” “For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to receive a PDCCH, or a PDSCH, or a CSI-RS, or a DL PRS in a set of symbols of a slot, the UE receives the PDCCH, the PDSCH, the CSI-RS, or the DL PRS if the UE does not detect a DCI format that indicates to the UE to transmit a PUSCH, a PUCCH, a PRACH, or a SRS in at least one symbol of the set of symbols of the slot; otherwise, the UE does not receive the PDCCH, or the PDSCH, or the CSI-RS, or the DL PRS in the set of symbols of the slot.” - TS 38.213, Clause 11.1.1 “UE procedure for determining slot format” “If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols, the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.”.

Based on RAN1 response, the expected UE behaviour to prioritize the UL transmission for the collision scenario of the semi-statically configured DL, which involved CSI resources used for L1/RLM/BFD/CBD measurements, and dynamically scheduled UL transmission. Such behaviour is considered as legacy UE behaviour in TDD operation and RAN1 intention is to extend such UE behaviour to SBFD operation. Given that in TDD operation, RRM spec didn’t capture any specific requirements for such collision, hence there is no need to capture a new collision requirement in RRM spec for SBFD operation.

Observation 7: The intention of the mentioned RAN1 agreement is to follow the existing prioritization rules between semi-static DL channels/signals and dynamically scheduled UL transmission including semi-statically configured CSI-RS resources for L1-RSRP/SINR measurement and RLM/BFD/CBD measurement.

- There is no need to define new requirements in RAN4 RRM to capture the UE behaviour for the collision between DL CSI-RS and dynamically scheduled UL transmission.

Issue 2-1-1c: CSI-RS L1 measurement – different symbol types: RAN1 provided the following response:

Issue#2: Behaviour for CSI-RS based L1-RSRP/SINR and RLM/BFD/CBD measurements For resource configuration and/or reporting configuration for CSI-RS based L1-RSRP/SINR measurement and RLM/BFD/CBD measurement, RAN1 would like to provide the following response: - According to the following RAN1 agreements, Configuration 1 and Configuration 2 are not applicable to CSI-RS resources configuration and CSI-RS based RLM/BFD/CBD measurement. Agreement (RAN1#120) The configuration of Configuration 1 or 2 for DL BWP only applies to PDSCH receptions within the DL BWP. Agreement (RAN1#117) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only. - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols. - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. FFS whether support of configuration 2 is subject to UE capability. Agreement (RAN1#118bis) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.. - The following RAN1 agreements for CSI reporting are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Agreement (RAN1#118bis) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. Agreement (RAN1#120) For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation is configured in CSI-ReportConfig..

RAN1 differentiated the applicability of Configurations 1 and 2 based on the IE used for measurement/reporting. Where the first case is that the two configurations are not applicable to CSI-RS resources configuration based RLM/BFD/CBD measurements. In other words, there is no restrictions from NW configuration on whether to restrict the symbol types for CSI-RS resources. However, from RAN4 perspective the minimum number of CSI-RS resources in terms of contiguous PRBs shall satisfy the minimum requirements defined in RRM spec.

Observation 8: Based on RAN1 agreements, there is no restrictions from NW configuration on whether to restrict the symbol types for CSI-RS resources.

- RAN4 doesn’t need to define new requirements for different SBFD symbol type provided that the minimum number of CSI-RS resources in terms of contiguous PRBs shall satisfy the minimum requirements defined in RRM spec.

On the other hand, Configurations 1 and 2 are applicable to CSI-RS based L1-RSRP/SINR measurement/reporting. Provided that the CSI-RS based L1RSRP/SINR reporting is associated with specific reference signals, hence, no changes are expected from RAN4 RRM perspective.

- On the symbol type of CSI-RS based L1-RSRP/SINR measurement/reporting, RAN4 to follow RAN1 agreement and there no need to change RRM spec for L1-RSRP/L1-SINR.

# Discussion on remaining issues

Issue 1-2-4: Requirements for scheduling restriction:

- Agreements (from Meeting R4#113) - For DL/UL transition, e.g. after SSB measurement, further check RAN1 agreements and potential impact to RAN4 scheduling restriction. - Further discuss the impact to inter-frequency SSB measurement without gap case. - WF from the previous meeting - Recommended WF for UL/DL transition - Do not define new scheduling restriction requirements for DL/UL transition. - Recommended WF for inter-frequency SSB measurement outside MG - FFS whether additional scheduling restriction applies for inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band. - Option 1: yes, UE is not required to receive DL in DL subbands on SBFD symbols - Option 2: no, UE is required to receive DL in DL subbands on SBFD symbols

On the impact of UL/DL transition: the recommended way forward from the previous meeting is agreeable.

- For UL/DL transition, do not define new scheduling restriction requirements for DL/UL transition.

On the impact to inter-frequency SSB measurement without gap case, when the SSB is fully within DL active BWP but not completely contained in DL sub-band, UE is not required to receive DL in DL subbands on SBFD symbols.

- For inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band, UE is not required to receive DL in DL subbands on SBFD symbols.

Issue 1-2-5: Requirements for RACH requirements:

During RAN2#127bis meeting, RAN2 made an agreement regarding RACH resource selection as below:

RACH resource selection - Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

To our understanding, the above agreement being captured in RAN2 spec, hence, there is no need to duplicate mentioning this agreement in RAN4 spec. Perhaps it worth waiting for RAN2 spec to capture the above agreement, then RAN RRM can just refer to that section in RAN2 spec.

- RAN4 RRM spec should refer to the agreement on PRACH transmission re-attempt in RAN2 spec without a need to duplicate the same line in RRM spec.

# Summary

In this contribution, we have the following proposals:

Proposal 1: For SBFD-aware UE with DUD case, RRM to introduce new requirements where the new requirements include a minimum threshold of 8ms on top of existing measurement period, with the assumption that UE measures one DL subband on each CSI-RS resource occasion.

Proposal 2: Existing accuracy requirements for Case 1 (X1 >= X3 and X2 >= X3) can be reused.

Proposal 3: Existing accuracy requirements for Case 2 (X1 >= X3 and X2 < X3) can be reused.

Proposal 4: RAN4 to deprioritize case 3, because existing accuracy requirements can’t be reused.

Proposal 5: RAN4 shall deprioritize CSI-RS based L3 measurements.

Proposal 6: There is no need to define new requirements in RAN4 RRM to capture the UE behaviour for the collision between DL CSI-RS and dynamically scheduled UL transmission.

Proposal 7: RAN4 doesn’t need to define new requirements for different SBFD symbol type provided that the minimum number of CSI-RS resources in terms of contiguous PRBs shall satisfy the minimum requirements defined in RRM spec.

Proposal 8: On the symbol type of CSI-RS based L1-RSRP/SINR measurement/reporting, RAN4 to follow RAN1 agreement and there no need to change RRM spec for L1-RSRP/L1-SINR.

Proposal 9: For UL/DL transition, do not define new scheduling restriction requirements for DL/UL transition.

Proposal 10: For inter-frequency SSB measurement outside MG on SBFD symbols, when the SSB is fully within DL active BWP but not completely contained in DL sub-band, UE is not required to receive DL in DL subbands on SBFD symbols.

Proposal 11: RAN4 RRM spec should refer to the agreement on PRACH transmission re-attempt in RAN2 spec without a need to duplicate the same line in RRM spec.

# References

- RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June 2024.

- R4-2502699, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon, Athens, Greece, Feb 2025.

- R4-2500536, “Topic summary for [114][222] NR_duplex_evo”, Huawei, HiSilicon, Athens, Greece, Feb 2025.

- R4-2420165, “LS on CSI-RS measurement with SBFD operation”, RAN4, Orlando, FL, Nov 2024.

- R1-2501560, “Reply LS on CSI-RS measurement with SBFD operation”, RAN1, Athens, Greece, Feb 2024.