3GPP TSG-RAN WG4 Meeting #114bis	R4-2504498

Wuhan, Hubei, China, April 07-11 2025

Source:	ZTE Corporation, Sanechips

Title:	Some considerations on CA framework database

Agenda item:	9.1

Document for:	Approval

Introduction

Attributes of the band combination database

<Agreement>:

-	RAN4 agree that general notes from e.g. the table headers can be included to the general text of the specification and are not needed included to the per DL configuration JSON files.

-	RAN4 agree that supported bandwidths of a band and whether they are optional shall be captured only in the bandwidth-per-band table in the corresponding specification (e.g. Table 5.3.5-1 in 38.101-1)

-	RAN4 agree that JSON representations of individual band combinations shall include information on supported UL power classes per UL configuration

-	RAN4 agree that JSON representations of individual band combinations shall include needed additional information and restrictions, if any

-	RAN4 agree to use the new set of JSON files (New JSON format for Bands and schemas for Band and BandCombination) provided by Ericsson as baseline for further development of the band combination database

-	The understanding in RAN4 is that the band combination database is to reflect what is in the current specification. If changes are made to this and how band combinations are reflected this needs to be updated/aligned with the database. This until a transition are made to rely only on the database.

<Way forward>:

-	RAN4 shall consider whether the Excel sheets now used for requests and WIDs can be abolished by using the band combination database.

Timeline for the introduction of a band combination database

<Agreement>:

-	RAN4 will plan for a transition from MS Word table representation of band combinations to the ETSI MCC provided database either by the end of Rel-19 or during Rel-20

<Way forward>:

-	RAN4 shall discuss whether companies are to provide JSON files for inclusion to the band combination database directly or RAN4 would like to request ETSI MCC to develop a tool to generate these

-	This discussion shall include aspects on the process for updating and/or adding to the band combination database.

<Way forward>:

-	RAN4 shall consider whether or not additional “per band combination” requirements as “delta values” or MSD are not a part of the JSON files

In last RAN4 meeting, a progress update for a new tool to handle RAN4 CA band combination specification was proposed in [1]. The tool was tasked by RAN plenary at RAN#102 to develop a database to handle tens of thousands of band combinations specified in RAN4. Further updates on the progress of tool and WF on next step for CA framework database were proposed in [2, 3]. In this document we provide some considerations related to CA framework database.

Discussion

Issues related to WebApp

Based on the progress reported in [1], an initial version of the database has been created, and a WebApp access to the database has been developed for CA band combinations.

https://tnm.etsi.org/ca/

Following is an example of sorting inter-band CA configurations with specific bands such as including bands n1, n3 and n7. From the sorting results, we notice that it does not exactly match the spec as shown as below.

- The channel bandwidths in the configuration with single band are missing in the query result.

- The uplink configuration for BCS 4 and 5 for CA_n1A-n3A-n7A is not correct.

- No need to include the column for maximum aggregated BW for inter-band CA.

- No indication of “-” for the uplink configurations without CA such as uplink for CA_n1A-n3A-n7B.

- No indication of NOTEs for the support of HPUE configurations.

The initial WebApp access to the database does not exactly match the spec. For example, the missing channel bandwidth for the configuration with single band should be added. The indication of “-” for uplink configuration without CA should be added. The column for maximum aggregated BW should be removed from inter-band CA configurations.

For inter-band CA, it seems currently all configurations including FR2 band have not been supported yet. Following is the example for searching CA_n1-n257 in the database.

Currently in the initial WebApp, all configurations including FR2 band have not been supported.

When further looking into the intra-band CA configuration, we see that it has different format from inter-band CA configuration. The attribute “Maximum aggregated bandwidth” is needed for intra-band CA configuration. Furthermore, for FR2 intra-band CA configuration, one extra attribute “Fallback group” is necessary for the configuration.

FR1 intra-band contiguous CA & non-contiguous CA.

FR2 intra-band contiguous CA & non-contiguous CA.

There are different formats for the inter-band CA and intra-band CA configurations. These should be taken into account in the WebApp.

- Proposal 1:  It is proposed to take the following aspects into considerations for WebApp.

- Consider different attributes of “maximum aggregated BW”, “Fallback group” in inter-band and intra-band CA configurations.

- Add indications of “-” for the uplink configurations without CA.

- Add indications of NOTEs for HPUE configurations.

Issues related to band combination database

A new JSON format for bands and schemas for band and band combination has been provided in (New JSON format for Bands and schemas for Band and BandCombination) as a baseline for database. It captures the NOTEs in Table 5.3.5-1 by enabling the bandwidthList into an array of objects which define a bandwidth and additional options for UL and DL: “mandatory”, “optional”, “undefined”, “scell_only”. However, it is also noted that in current band combination database, there is no information for spec releases. The question is if we decide to maintain only one database for bands and band combinations for all releases in future? Actually, it is not the current status for RAN4 spec in docx format. When we searching a band combination in the database, we don't know from which release the band or configuration it supports. And for the NOTEs in the tables of band and band combination, it may also differs among different releases.

The current JSON format for bands and band combinations does not provide the information of spec release.

- Proposal 2:  It is proposed to introduce “spec release” in JSON format for bands and band combinations.

Another issue need to be mentioned here is related to the JSON file for band combination including FR2 bands. Let us take CA_n257G.json and CA_n257H.json files as an example.

https://forge.3gpp.org/rep/ran4/carrieraggregation

In CA_n257G.json file, the “ulConfigList” with “n257G” is matched with the configuration in 101-2 docx file, while for CA_n257H.json file, there is no “ulConfigList” listed. The uplink configurations of CA_n257G/H are missing in the CA_n257H.json. To look into the issue, the problem may result from the scripts importing the band combination specifications from 38.101-2/3 tables to JSON files. In the docx specification, the delimiter “/” is introduced in the uplink configurations for simplicity, in which “CA_n257G/H” stands for “CA_n257G” and “CA_n257H”. The handling of “/” in the scripts may be ignored.

In the current JSON file for band combinations including FR2 bands, most of “ulConfigList” with uplink configurations having delimiter “/” are missing.

- Proposal 3:  It is proposed to take consideration of delimiter “/” for uplink configurations including FR2 bands when importing the band combinations into the JSON files.

One more issue is for the JSON file name. Currently since only NR CA combinations are considered in the developing of database, the JSON file is named without prefix “CA_” for default, such as using “n1A-n3A.json” for CA_n1A-n3A. However, if we look into the future, when DC_n1A-n3A being supported, the “CA_” and “DC_” should be distinguished.

In the current CA framework database, the JSON file is named without prefix “CA_”, such as using “n1A-n3A.json” for CA_n1A-n3A.

- Proposal 4:  It is proposed to distinguish CA configurations with prefix “CA_” in the JSON file name.

Timeline for introducing band combination database

In [2], the timeline for introducing band combination database is proposed either by the end of Rel-19 or during Rel-20. RAN4 shall discuss whether companies are to provide JSON files for inclusion to the band combination database directly or RAN4 would like to request ETSI MCC to develop a tool to generate these.

To our understanding, considering the database is still under construction phase, many functions are still missing or immature, such as how to handle the NOTEs, to add new attributes to the database, to import the band combinations of different releases without error or distortion and further to support DC configurations, all these are time consuming and thus we suggest the timeline should be no earlier than Rel-20. Delegates need more time to be familiar with the new working procedure. Furthermore, we also expect the database could offload some rapporteur’s work to some extent, such as to use the database for requesting the new band combination and even for the status report. For drafting TP on band combination, extra workload should be avoided. It’s not good if proponent add JSON file in the TP and companies are required to flag not only on the TP itself, but also on the JSON file. For error checking for band combination, we expect the CR workload could be reduced by using the database approach. Finally, regarding to the issues of requirements for “delta values” or MSD as a part of the JSON files, more discussion in RAN4 are needed since RAN4 is currently discussing the MSD rules to simplify the MSD requirements.

- Proposal 5:  Many functions for the database tool are still missing or immature, it is suggested that the timeline for introducing the database should be no earlier than Rel-20.

- Proposal 6:  To offload the basket WI rapporteur’s work, the usage of database for requesting the new band combination and even for the status report are expected to be handled in the tool. For TP drafting and error checking CR, extra workload should be avoided.

Conclusion

In this contribution we provide some considerations on CA framework database. Issues related to WebApp and band combination database are discussed. The following observations and proposals have been made:

- The initial WebApp access to the database does not exactly match the spec. For example, the missing channel bandwidth for the configuration with single band should be added. The indication of “-” for uplink configuration without CA should be added. The column for maximum aggregated BW should be removed from inter-band CA configurations.

- Currently in the initial WebApp, all configurations including FR2 band have not been supported.

There are different formats for the inter-band CA and intra-band CA configurations. These should be taken into account in the WebApp.

- Proposal 1:  It is proposed to take the following aspects into considerations for WebApp.

- Consider different attributes of “maximum aggregated BW”, “Fallback group” in inter-band and intra-band CA configurations.

- Add indications of “-” for the uplink configurations without CA.

- Add indications of NOTEs for HPUE configurations.

The current JSON format for bands and band combinations does not provide the information of spec release.

- Proposal 2:  It is proposed to introduce “spec release” in JSON format for bands and band combinations.

In the current JSON file for band combinations including FR2 bands, most of “ulConfigList” with uplink configurations having delimiter “/” are missing.

- Proposal 3:  It is proposed to take consideration of delimiter “/” for uplink configurations including FR2 bands when importing the band combinations into the JSON files.

In the current CA framework database, the JSON file is named without prefix “CA_”, such as using “n1A-n3A.json” for CA_n1A-n3A.

- Proposal 4:  It is proposed to distinguish CA configurations with prefix “CA_” in the JSON file name.

- Proposal 5:  Many functions for the database tool are still missing or immature, it is suggested that the timeline for introducing the database should be no earlier than Rel-20.

- Proposal 6:  To offload the basket WI rapporteur’s work, the usage of database for requesting the new band combination and even for the status report are expected to be handled in the tool. For TP drafting and error checking CR, extra workload should be avoided.

- References

- R4-2501653, Progress update and next steps for the CA database, Nokia, Ericsson

- R4-2502976, WF on next step for CA framework database, Nokia

- RP-250109, Progress update and next steps for the CA database, Nokia