3GPP TSG-RAN WG4 Meeting #114bis	R4-2504500

Wuhan, China, April 7th – April 11th, 2025

Source:	Nokia

Title:	On L3 FBS requirements

Agenda item:	7.17.2.1

Document for:	Discussion

Introduction

In this contribution we continue the discussion about faster beam sweeping requirements for L3 measurements.

Discussion

Conditions to trigger FBS for L3 measurements

In the last RAN4 meeting the following agreement were made with the yellow highlighted FFS issues [1]:

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF Agreement in ad-hoc session: - Discuss if additional conditions are needed to activate FBS-based L3 measurements. - Do not consider additional conditions beside the agreements from RAN4#113 meeting. Agreement in RRM session (Monday): - Adopt FBS triggering condition of serving cell is worse than a threshold, including RSRP, RSRQ threshold. - Further discuss whether to limit the serving cell to FR2 serving cell. - Further discuss whether to add the hysteresis and timeToTrigger parameters. - Note: the scenario related to single carrier or CA/DC is discussed in another issue. - The signaling design is up to RAN2.

As it has been agreed not to introduce further conditions to trigger FBS than the configured signal based threshold, we focus on the yellow highlighted issues about the details of the configured condition.

It was earlier agreed that serving cell signal quality will trigger FBS. FBS applies only to FR2-1 cell if this is the only cell on this band. Furthermore, it has been agreed that CA scenarios are not supported. Hence, the scenarios where FBS may apply are:

- when the serving cell is an FR2-1 cell and CA is not configured, and

- when the serving cell is an FR1 cell and FR2-1 carrier is configured for inter-frequency measurements.

In the former case, it is clear that the serving FR2-1 PCell signal quality triggers FBS on the PCell, and hence the condition to trigger FBS is FR2-1 serving cell based. In the latter case, serving PCell is an FR1 cell and hence the condition would be FR1 serving cell based. We do not see a need to limit the feature to the former case, as the latter case may be useful for FR1  FR2-1 handover purposes. Therefore, we think the condition can be either FR1 or FR2-1 serving cell based condition. Under the agreed scope for this WI (no CA support), the serving cell that triggers FBS would be the PCell.

Support FBS triggering on a configured FR2-1 carrier based on both FR1 and FR2-1 serving cell condition.

How to implement reduced BSF

Multiple proposals of how to capture the reduced BSF in the specification were listed in the RAN4#114 meeting’s moderator summary under Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF.

How to capture the requirement depends on how the BSF reduction factor is signalled in the capability. Here the options are either to signal the reduced BSF (e.g. 2, 4, or 6) or a fraction that reduces the current beam sweeping factor 8 by multiplying it with the fraction (e.g. 1/4, 1/2, or 2/3). For signalling purposes, the former option where the BSF is signalled directly would be simpler and this is also what is used in the Rel-19 multi-Rx beam sweeping factor reduction capability. Therefore, in the following we assume this option when analyzing how to capture the delay in different requirements.

For handover delay, it is straightforward to capture the delay by defining N-value based on the signalled BSF:

N = [reduced beam sweeping factor] when the UE supports [Rel-19 reduced L3 beam sweeping capability]. Otherwise N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2.

For handover delay (scenario 5), define the value N to be dependent on the beam sweeping factor signalled in the UE capability, when the UE supports L3 FBS feature.

For RRC re-establishment requirements, in the last meeting it was discussed whether the reduced BSF applies for one or both sides of the max-function (e.g. MAX (1000 ms, 80 x TSMTC)). As we have shown in one of our previous contributions, if the reduced BSF only applies to the latter part of the max-function, the former part diminishes the achieved reduction. For example, if TSMTC=20 and Nreduced =1/2, the following values will be obtained for Tidentify_intra_NR, where the static part of the function overrides the reduction that is achieved.

| Serving cell | FR of target NR | Tidentify_intra_NR [ms] |  |
| --- | --- | --- | --- |
| SSB Ês/Iot (dB) | cell | Known NR cell | Unknown NR cell |
| ≥ -8 | FR2-1 | N/A | MAX (1000 ms, 800 ms) |

- For this reason, we think the BSF reduction needs to apply for the whole max function.

The fixed value in the max(.) function may overweight the reduced value of Tidentify_intra_NR and Tidentify_inter_NR obtained from BSF.

Furthermore, the function needs to take into account the additional processing delay 2 ms that was agreed to be included.

Apply the beam sweeping factor reduction for RRC re-establishment requirements (scenario 7) for the whole max-function (e.g. [reduced BSF]8 × MAX (1000 ms, 80 x TSMTC) + 2 ms).

For L3 measurement delay requirements, we propose to use the same approach as for RRC re-establishment delay i.e. to introduce a factor that takes into account the UE signalled BSF and the old N=8.

Under the agreed conditions, when FBS applies for FR2-1 L3 measurements (scenario 1-4), the existing measurement delay is multiplied by [reduced BSF]8.

Furthermore, we propose to add the conditions under which FBS applies in section 3, as was done for multi-Rx.

Introduce applicability of FBS requirements in section 3.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Support FBS triggering on a configured FR2-1 carrier based on both FR1 and FR2-1 serving cell condition.

Proposal 2: For handover delay (scenario 5), define the value N to be dependent on the beam sweeping factor signalled in the UE capability, when the UE supports L3 FBS feature.

For this reason, we think the BSF reduction needs to apply for the whole max function.

Observation 1: The fixed value in the max(.) function may overweight the reduced value of Tidentify_intra_NR and Tidentify_inter_NR obtained from BSF.

Proposal 3: Apply the beam sweeping factor reduction for RRC re-establishment requirements (scenario 7) for the whole max-function (e.g. [reduced BSF]/8 × MAX (1000 ms, 80 x TSMTC) + 2 ms).

Proposal 4: Under the agreed conditions, when FBS applies for FR2-1 L3 measurements (scenario 1-4), the existing measurement delay is multiplied by [reduced BSF]/8.

Proposal 5: Introduce applicability of FBS requirements in section 3.

- References

- R4-2502593, WF on RRM requirements for NR_RRM_Ph5_Part2, RAN4#114