3GPP TSG-RAN WG4 Meeting #114bis	R4-2504510

Wuhan, China, April 7 - 11, 2025

Source:	Nokia

Title:	Discussion on requirements for RedCap support in NR NTN

Agenda item:	7.29.4.1

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_Ph3). The most recent version of the WID can be found in [2].

In this document we provide our views on the 5th objective, namely the support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems, related to the RAN4 RRM impact considering the open issues identified in the latest WF from RAN4 #114 [3] and further detailed in the Topic summary [4].

Discussion

In the present document we will discuss the open issues presented in the WF [3], as listed below:

- Issue 1-1-1 Measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN

- Issue 1-1-2: SDT requirements

- Issue 1-2-3-1: Feature 25-1(parallelSMTC)

- Issue 1-2-3-1a: The proximity distance between two SMTCs

- Issue 1-2-3-3: The number of cells/SSBs/supported satellites

- Issue 1-2-3-5: Feature 25-3(parallelMeasurementGap)

- Issue 1-2-6: NCD-SSB

- Issue 1-2-7: Other features

- Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

- Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN

- Issue 2-1-3: SDT requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN

- Issue 2-4-1 L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Issue 3-1-1: General HD-FDD related impact

- Issue 3-2-1: The impact on NR Handover due to HD-FDD

- Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

General consideration on RRM requirements

RAN4 has discussed general aspects for RRM requirements for (e)RedCap UE with FR1-NTN.

E-UTRAN inter-RAT cell reselection requirements

Following agreement was achieved for E-UTRAN inter-RAT cell reselection requirements [3].

#### Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN? Online agreement: Agreement: - RAN4 to define measurement requirements in cell re-selection for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN.

In fact, measurement requirements for inter-RAT cell selection to EUTRAN TN for UE in NR NTN are defined in clause 4.2C.2.11, whilst these are defined for (e)RedCap UE in clause 4.2B.2.5. Thus, it is proposed to merge requirements from clause 4.2B.2.5 into clause 4.2C.2.11 for (e)RedCap UE in FR1-NTN as depicted below.

#### 4.2C.2.11 Measurements of inter-RAT E-UTRAN cells with TN carrier This clause applies for the inter-RAT cell re-selection for TN carriers, and NTN carriers if configured. The requirements in clause 4.2C.2.11 apply provided that network provides SIB19 and UE is configured with one or more TN carriers. This clause applies also to (e)RedCap UE in FR1-NTN. UE is allowed to skip TN neighbour cells measurement in an area where there is no coverage of the frequency based on the provided TN cell coverage information and UE GNSS position information. Otherwise, UE shall perform TN measurement if its estimated distance to tn-ReferenceLocation is smaller than tn-DistanceRadius. The requirements apply provided that the actual distance between UE to tn-ReferenceLocation is smaller than tn-DistanceRadius – 50m.This clause considers the inter-RAT cell reselection from NTN to E-UTRAN TN in FR1. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2. If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs. The requirements in this clause apply to inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause. The parameter NEUTRA_carrier, or NEUTRA_carrier_RedCap for (e)RedCap UE in FR1-NTN, is the number of EUTRA TN carriers indicated by the serving cell, except for the frequency carrier where there is no coverage of that frequency based on the provide TN cell coverage information and UE GNSS position information. If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, UE supports the enhanced inter-RAT E-UTRAN measurement requirements. The parameter NEUTRA_carrier for a UE, or NEUTRA_carrier_RedCap for (e)RedCap UE in FR1-NTN, configured with idle mode DC measurements (while T331 is running), is the combined total number of configured E-UTRA carriers in the neighbour frequency list and E-UTRA carriers configured for idle mode CA/DC measurements. - NOTE: Combined total number means that if a carrier is an E-UTRA carrier indicated by the serving cell for mobility and additionally a carrier configured for idle mode CA/DC measurements, it only counts as one carrier. The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period. For (e)RedCap UE in FR1-NTN at least two measurements shall be spaced by at least Tmeasure,EUTRAN_RedCap/2. An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

- the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band, and - the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band. - SCH conditions specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS 38.304 [1] within NEUTRA_carrier * Tdetect,EUTRAN, or within (NEUTRA_carrier_RedCap) * Tdetect,EUTRAN_RedCap for (e)RedCap UE in FR1-NTN, when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities. Cells which have been detected shall be measured at least every NEUTRA_carrier * Tmeasure,EUTRAN , or every (NEUTRA_carrier_RedCap) * Tdetect,EUTRAN_RedCap for (e)RedCap UE in FR1-NTN, when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ. When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN , or every Tmeasure,EUTRAN_RedCap for (e)RedCap UE in FR1-NTN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects, on an inter-RAT E-UTRAN carrier, a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell. The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell. For a cell that has been already detected, but has not been reselected to, the filtering shall be such that a UE not configured with eDRX_IDLE cycle shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within NEUTRA_carrier * Tevaluate,EUTRAN when Treselection = 0 as specified in table 4.2.2.5-1 and 4.2.2.5-2, or within (NEUTRA_carrier_RedCap) * Tevaluate,EUTRAN_RedCap for (e)RedCap UE in FR1-NTN when Treselection = 0 as specified in table 4.2B.2.5-1 and 4.2B.2.5-2, provided that the reselection criteria is met by a margin of at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities. If the Treselection timer has a non-zero value and an inter-RAT E-UTRA cell satisfies the reselection criteria defined in TS 38.304 [1], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect to this cell. When the distance between the UE and tn-ReferenceLocation is larger than tn-DistanceRadius +50m, the UE is allowed to not perform measurements on the TN frequency in the corresponding area. For UE not configured with eDRX_IDLE cycle, Tdetect,EUTRAN, Tmeasure,EUTRAN and Tevaluate, E-UTRAN are specified in table 4.2.2.5-1 and 4.2.2.5-2. For (e)RedCap UE in FR1-NTN, for 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap are specified in table 4.2B.2.5-1. For (e)RedCap UE in FR1-NTN, for 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap are specified in table 4.2B.2.5-2, where the requirements apply provided that the serving cell is configured with eDRX_IDLE lower or equal to 10.24s and the DRX cycle length is the same in all PTWs during any of Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap when multiple PTWs are used. ### 4.2C.3 Void

RAN4 to merge requirements from clause 4.2B.2.5 into clause 4.2C.2.11 for (e)RedCap UE in FR1-NTN.

CG-SDT requirements

Following agreement was achieved for CG-SDT requirements [3].

#### Issue 1-1-2: SDT requirements Online agreement: Agreement: - For SDT requirement in RRC_INACTIVE state for (e)RedCap with NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and RAN4 can refer to the requirements on TA validation for the PUR requirements for IoT-NTN, that is, - Define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Further discuss whether to send RAN2 LS

In fact, RAN4 has taken an earlier decision in Rel-18 to remove the RSRP change criterion for TA validation for UE in IOT-NTN performing CG-SDT in RRC_INACTIVE state, due to the limited near-far effect in NTN and the instability of the TA and its association with a validity timer and ephemeris. The functionality of TA validation for support of (e)RedCap UE in FR1-NTN should thus be aligned. RAN4 should notify RAN2 by LS that the RSRP change criterion is to be removed and hence thresholds for RSRP change criterion are not needed any longer to be signalled for (e)RedCap UE in FR1-NTN.

RAN4 to send LS to RAN2 informing them about the RAN4 agreement on the removal of the RSRP change criterion for TA validation in CG-SDT for (e)RedCap UE in FR1-NTN and on the reuse of the TA validation criteria as specified for PUR in IOT-NTN.

Parallel SMTCs

Following issue was discussed without conclusion [3].

##### Issue 1-2-3-1: Feature 25-1(parallelSMTC) Online discussion <Way Forward>: Options for further discussion: If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is: - Option 1: [1 SMTC or] 2 SMTCs not in parallel - Option 2: 2 parallel SMTCs - ‘Parallel’ means the SMTCs are TDM with proximity distance.

In our view, the same baseline capability for 1Rx and 2Rx RedCap UE should be defined, i.e. mandatory support for 2 SMTCs in parallel according to Option 2.

RAN4 to specify the same UE capability for Feature 25-1 (parallelSMTC), i.e. 1Rx and 2Rx RedCap UE both support 2 parallel TDM’ed SMTCs.

Proximity distance between two SMTCs

Following issue was discussed and two options left for further study [3].

##### Issue 1-2-3-1a: The proximity distance between two SMTCs Ad-hoc agreement Agreement: - RAN4 to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN. - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. - For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, - Option 1: No. RAN4 to reuse the legacy collision condition. - Option 2: Yes. RAN4 to consider relaxation for proximity, i.e., - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (1ms as starting point)

We prefer option 1, i.e. to reuse the same collision condition previously defined for regular UEs in NTN, as we do not support specifying different UE behaviours in this scenario.

RAN4 to reuse the legacy collision condition for (e)RedCap UEs in NTN.

Number of cells/SSBs/supported satellites

Following issue was discussed without conclusion [3].

##### Issue 1-2-3-3: The number of cells/SSBs/supported satellites <Way Forward>: - For the number of supported satellites, - Proposal 1: For the number of target LEO satellites the UE can monitor per carrier including serving satellite, legacy UE capability (Feature 25-7, maxNumber-LEO-SatellitesPerCarrier-r17) can be reused for RedCap UE. - Proposal 2: - RedCap UEs shall support at least one NGSO satellite per SMTC configured in a frequency layer in addition to the serving satellite. - The total number of supported satellites for measurement in a frequency layer is equal to N+1 where N is the number of configured SMTCs, if the serving satellite is to be measured in this frequency layer and N if the serving satellite is not to be measured in this frequency layer. - For the number of cells and SSB, - Proposal 1: RedCap NTN reuses the number of cells and SSB for NR NTN, i.e., - Proposal 1a: For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 8 identified cells, and

- 8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell is not smaller than the number of configured RLM-RS SSB resources. - 4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment. - In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

For the number of supported satellites, we prefer proposal 1, i.e. to reuse the legacy UE capability as defined by the Feature 25-7.

For the number of cells and SSBs we agree with proposal 1, i.e. to reuse the number of cells and SSBs for NR NTN.

RAN4 to reuse the legacy UE capability for number of supported satellites as defined by the Feature 25-7 for (e)RedCap UEs in NTN.

RAN4 to reuse the number of cells and SSBs for NR NTN for (e)RedCap UEs in NTN.

Parallel measurement gaps

Following issue was postponed to RAN4 #114bis [3].

##### Issue 1-2-3-5: Feature 25-3(parallelMeasurementGap) - Background The agreements on concurrent gap related issue in the last meeting: RAN4#113: < Agreement >: - Deprioritize parallel measurement gap for (e)RedCap UEs in NTN. - Come back to this issue at Apr 2025 meeting.. <Way Forward>: - For whether to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs, - Proposal 1: Support to define. - Proposal 1a: The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements. - Proposal 1b: The capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx.

We prefer Proposal 1a, since it keeps UE behaviour aligned for 1Rx and 2Rx (e)RedCap UE in NTN and no separate capabilities are needed.

RAN4 to specify the support of parallel measurement gaps (Feature 25-3) as optional for 1Rx and 2Rx (e)RedCap UEs in NTN.

NCD-SSB

Following issue was discussed without conclusion [3].

#### Issue 1-2-6: NCD-SSB <Way Forward>: - Proposal 1: - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB. - Proposal 2: - Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

We have a slight preference for Proposal 2, since it does not exclude applicability for NCD-SSB like Proposal 1.

In fact, NCD-SSB is specified for TN to account for the limited Rx bandwidth of (e)RedCap UEs of 20 MHz in FR1. Hence NCD-SSB in the dedicate BWP for (e)RedCap allows for fast time and frequency tracking and is used for mobility procedures such as handover. We believe that it is equally important for operation in NR NTN. Without NCD-SSB system performance is expected to degrade.

RAN4 to specify NCD-SSB support as mandatory for 1Rx and 2Rx (e)RedCap UEs in NTN.

Other features

Following issue was discussed without conclusion [3].

Issue 1-2-7: Other features <Way Forward>: - For SMTC adjustment in idle/inactive, - Option 1: Keep mandatory for RedCap UE. - Option 2: Keep optional for RedCap UE. - For Location-based measurement trigger, - Option 1: Keep optional for RedCap UE. - Option 2: RAN4 not to further discuss applicability of location-based measurement report trigger.

- SMTC adjustment in idle/inactive: We see this feature as a paramount feature for the correct functioning of NTN measurements. In connected mode the NW may recur to TA reports to adjust the measurement objects configuration for helping the UEs to track the neighbour cells belonging to different satellites. In Idle mode, this is not possible. If this feature is not present, the UE will not be capable to adjust the SMTC window to measure a neighbour satellite as the differential propagation delay changes over time compared to the serving satellite. This feature, therefore, has to be made mandatory.

SMTC adjustment in idle/inactive is mandatory in NTN and shall be kept mandatory for (e)RedCap UE in NTN.

- Location-based measurement report trigger: It is an optional feature for NR over NTN, and shall be kept optional for RedCap UEs, as it might facilitate the mobility procedures when the near-far effect is not very pronounced, which is likely to be the case in NTN.

Location-based measurement report trigger is kept optional for (e)RedCap UE in NTN, and RAN4 will define requirements for this feature.

Reduction in the number of UE Rx branches 	(RRC_IDLE/RRC_INACTIVE mode mobility)

Enhanced cell reselection measurement requirements

Following issue was discussed without conclusion [3].

#### Issue 2-1-1: Enhanced cell reselection measurement requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes are kept optional in Redcap over NTN. - Proposal 1a: Define enhanced cell reselection measurement requirement for RedCap UE not configured with eDRX_IDLE cycle, the requirement for normal UEs can be reused. - Proposal 1b: To define enhanced cell reselection requirements for RedCap UE, - Re-use the requirements for normal UE except that Tdetect is extended by [2] samples - Same delay requirements for 1Rx and 2Rx UEs. - Proposal 1c: - For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements - For 1 Rx (e)RedCap NTN UE, RAN4 to - Option 1: Reuse the existing enhanced cell reselection measurement requirements - Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined. - Proposal 1d: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands, - For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

Enhanced RRM requirements for measurements in Idle and Inactive is also an optional feature in NR over NTN, and shall be kept optional for RedCap devices.

The discussion refers to clauses 4.2C.2.3 and 4.2C.2.4 in TS 38.133. We support Proposal 1 to keep enhanced RRM requirements for measurements in idle and inactive modes optional for (e)RedCap in FR1-NTN. We also support proposal 1c with option 2 for 1 Rx UE. As for normal reselection some adjustments on e.g. cell reselection margin for 1 Rx versus 2 Rx (1 dB higher) are expected to be required.

RAN4 to keep enhanced RRM requirements for measurements in idle and inactive modes optional for (e)RedCap in FR1-NTN. 2Rx RedCap UE is expected to reuse measurement requirements without eDRX from non-RedCap UE, whilst for 1Rx RedCap UE relaxations, e.g. related to cell reselection margin, can be considered.

Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier

Following issue was discussed without conclusion [3].

Issue 2-1-2: Cell re-selection requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx/ 2Rx (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: The existing requirements for measurements of inter-RAT E-UTRAN cells with TN carrier can be used as baseline. - Proposal 2: For inter-RAT E-UTRAN TN carrier cell re-selection measurement requirements, reuse the legacy requirements defined for normal UEs for (e)Redcap UE not configured with eDRX_IDLE cycle, reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle. - Proposal 3: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands, - For 2Rx UE, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands. - For 1RX UE, reuse the same margin of RSRP/RSRQ as for 2Rx.

In our view, the legacy measurement requirements for (e)RedCap UE in TN should be reused for cell re-selection to an inter-RAT E-UTRAN cell with TN carrier, accounting for the different cell reselection margins for 1Rx and 2Rx          (e)RedCap UE.

For inter-RAT E-UTRAN TN carrier cell re-selection, RAN4 to reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-1 for (e)Redcap UE not configured with eDRX_IDLE cycle and Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle, accounting for the different cell reselection margins for 1Rx and 2Rx (e)RedCap UE.

SDT requirements

Following issue was discussed without conclusion [3].

#### Issue 2-1-3: SDT requirements for 1Rx/2Rx (e)RedCap UE with FR1-NTN <Way Forward>: - Proposal 1: For SDT requirements on Redcap NTN in RRC_INACTIVE state, RAN4 to revert the agreement reached in RAN4#113 and modify it as: - For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

In our view, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1Rx TN UE still applies for NTN UE, since the threshold is an offset to an absolute RSRP threshold. Thereagainst, the RSRP change is related to the relative threshold parameter cg-SDT-RSRP-ChangeThreshold-r17. So, we propose to keep the +1 dB offset as used for 1Rx RedCap UE in TN.

For SDT requirements for 1Rx (e)RedCap UE in FR1-NTN, RAN4 to keep the +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 as used for 1Rx (e)RedCap UE in TN.

L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs 	with FR1-NTN

L1/L3 measurement accuracy requirements

Following issue was discussed without conclusion [3].

#### Issue 2-4-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1: RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1: RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB.

We agree with Proposal 1 for 2Rx and 1Rx RedCap UE in RRC_CONNECTED mode.

RAN4 to agree the Way Forward on issue 2-4-1 (L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs).

Impact of HD-FDD

General HD-FDD related impact

Following issue was discussed without conclusion [3].

#### Issue 3-1-1: General HD-FDD related impact <Way Forward>: - Proposal 1: The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE in NTN scenario. - Proposal 1a: To following the TN, the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs. - Proposal 1c: - Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE during L1-RSRP measurement - Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period. - Proposal 1d:RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reach further conclusions.

- Proposal 2: For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side. - Proposal 3: For HD RedCap and eRedCap UEs, RAN4 to adopt a high level statement that the requirement does not count resources, channels, SSB, etc if they overlap with the other direction as specified a valid case in RAN1 specification. - Proposal 4: RAN4 should further discuss measurement behavior for collision between SMTC and UL when network indicates UL overriding DL in HD-FDD.

We agree to the way forward recommended by moderator [3] as a baseline for further work considering the ongoing discussion in RAN1. It is noted, in RAN1 there is an agreement on the collision case 3 (semi-persistent UL and semi-persistent DL colliding), and a working assumption for the collision case 4 (dynamic UL and dynamic DL colliding), the latter being under discussion related to the link direction to prioritize.

RAN4 to follow the recommended way forward for issue 3-1-1 considering ongoing discussion in RAN1:

Recommend to agree on:

- The legacy requirements and applicable conditions can be reused for defining requirements of RedCap UE in NTN scenario, and the following requirements will be affected by HD-FDD:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

To be discussed:

- Whether and how to define HD-FDD related requirements for the following:

  - Scheduling availability of UE performing intra/inter measurements

  - Scheduling availability of UE during L1-RSRP measurement

Impact on NR Handover due to HD-FDD

Following issue was discussed without conclusion [3].

#### Issue 3-2-1: The impact on NR Handover due to HD-FDD <Way Forward>: - Proposal 1: - For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - SSB is available at the UE once every SMTC period during Tsearch - One SSB is available during T∆ - One SSB is available during TIU. - For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added: - For RedCap NTN UE with HD-FDD, the requirements are met provided that - One SSB is available during T∆ - One SSB is available during TIU

When analysing the impact of HD-FDD on mobility requirements (HO, CHO, Satellite Switch with re-sync), several companies have proposed that:

“For RedCap NTN UE with HD-FDD, the requirements are met provided that the SSB is available at the UE once every SMTC period during Tsearch”

This requirement is derived from early HD-FDD applicability in terrestrial networks. But there is a technical concern in adapting this requirement directly to NTN scenarios.

Differently from TN, in NTN scenarios, if UEs support more than one satellite per SMTC, and therefore, if the UE cannot support simultaneously measurements from two different satellites (feature 25-5) within the same SMTC, then the SMTC is to be shared by multiple satellites (K_multi factor). This indicates that there might be further scheduling restriction when the UE is measuring neighbour satellites. Therefore, depending on how “availability” is defined, it cannot be guaranteed that one SSB will be available at the UE once every SMTC period.

For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side.

Others

UE capabilities for (e)RedCap UE with FR1-NTN bands

Following issue was discussed without conclusion [3].

#### Issue 4-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands? <Way Forward>: - Proposal 1: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17). - Proposal 2: Introduce new capability to declare support over RedCap over NTN. - Proposal 3: Both proposals, i.e., a new capability declaring (e) RedCap for NTN or combining (e)RedCap and NTN, are fine with us. It may be determined later.

Another point of discussion in the WF is the applicability of RedCap requirements under NTN needs further discussion. For a UE to support RedCap over NTN, it must support both RedCap and NTN features.

- Option A: Declare support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

- Option B: Declare explicitly – via a new capability – the support to RedCap over NTN.

Both options have their pros and cons (flexibility versus capability signalling overhead). The first option allows for less signalling overhead and less capabilities declaration. However, it is less flexible as it does not allow a UE that is NTN capable to be TN Redcap capable only. It also ensues from option B that if a UE supports RedCap over NTN, it must support RedCap over TN, which might not be always the case.

In order to leverage the most flexibility on the capability indication, we propose to adopt option B.

Introduce new capability to declare support for (e)RedCap over NTN.

Conclusion

This paper has analysed the open issues provided in the WF for this WI [3], and the following set of proposals are made:

- RAN4 to merge requirements from clause 4.2B.2.5 into clause 4.2C.2.11 for (e)RedCap UE in FR1-NTN.

RAN4 to send LS to RAN2 informing them about the RAN4 agreement on the removal of the RSRP change criterion for TA validation in CG-SDT for (e)RedCap UE in FR1-NTN and on the reuse of the TA validation criteria as specified for PUR in IOT-NTN.

RAN4 to specify the same UE capability for Feature 25-1 (parallelSMTC), i.e. 1Rx and 2Rx RedCap UE both support 2 parallel TDM’ed SMTCs.

RAN4 to reuse the legacy collision condition for (e)RedCap UEs in NTN.

RAN4 to reuse the legacy UE capability for number of supported satellites as defined by the Feature 25-7 for (e)RedCap UEs in NTN.

RAN4 to reuse the number of cells and SSBs for NR NTN for (e)RedCap UEs in NTN.

RAN4 to specify the support of parallel measurement gaps (Feature 25-3) as optional for 1Rx and 2Rx (e)RedCap UEs in NTN.

RAN4 to specify NCD-SSB support as mandatory for 1Rx and 2Rx (e)RedCap UEs in NTN.

SMTC adjustment in idle/inactive is mandatory in NTN and shall be kept mandatory for (e)RedCap UE in NTN.

Location-based measurement report trigger is kept optional for (e)RedCap UE in NTN, and RAN4 will define requirements for this feature.

RAN4 to keep enhanced RRM requirements for measurements in idle and inactive modes optional for (e)RedCap in FR1-NTN. 2Rx RedCap UE is expected to reuse measurement requirements without eDRX from non-RedCap UE, whilst for 1Rx RedCap UE relaxations, e.g. related to cell reselection margin, can be considered.

For inter-RAT E-UTRAN TN carrier cell re-selection, RAN4 to reuse the Redcap Inter-RAT cell reselection requirement defined in TS 38.133 Table 4.2B.2.5-1 for (e)Redcap UE not configured with eDRX_IDLE cycle and Table 4.2B.2.5-2 for (e)Redcap UE configured with eDRX_IDLE cycle, accounting for the different cell reselection margins for 1Rx and 2Rx (e)RedCap UE.

For SDT requirements for 1Rx (e )RedCap UE in FR1-NTN, RAN4 to keep the +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 as used for 1Rx (e)RedCap UE in TN.

RAN4 to agree the Way Forward on issue 2-4-1 (L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs).

RAN4 to follow the recommended way forward for issue 3-1-1 considering ongoing discussion in RAN1:

Recommend to agree on:

- The legacy requirements and applicable conditions can be reused for defining requirements of RedCap UE in NTN scenario, and the following requirements will be affected by HD-FDD:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

To be discussed:

- Whether and how to define HD-FDD related requirements for the following:

  - Scheduling availability of UE performing intra/inter measurements

  - Scheduling availability of UE during L1-RSRP measurement

For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side.

Introduce new capability to declare support for (e)RedCap over NTN.

The Draft CR for proposal 1 is submitted in [5].

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2502604, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #114

- R4-2500543, Topic summary for [114][229] NR_NTN_Ph3_Part1, Moderator (CATT), RAN4 #114

- R4-2504511, Draft CR38.133 Inter-RAT E-UTRAN in TN cell reselection for (e)RedCap over NR NTN, Nokia, RAN4 #114bis