3GPP TSG-RAN WG4 Meeting #114bis            	R4-2504537

Wuhan, China, 7th -11th, April, 2025

Agenda item:	7.10.4.2

Source:	Samsung

Title:	Discussion on demodulation requirement for NTN NGSO testing

Document for:	Discussion

- Introduction

In RAN#104 meeting, the revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved in [1]. As one of objective is NTN_testing_NGSO_channel_model with following

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel mode

According to the meeting agenda, and work plan in [2], this is the 1st meeting to discuss the UE demodulation performance requirements for NGSO for NR-NTN and IoT NTN in FR1-NTN bands and corresponding LTE bands based on the above channel model

In the last RAN4 meeting, the NGSO channel model for NTN testing was under discussion, the related agreements are captured in the WF [1] as following

- TE emulated channel model for satellite mobility - Satellite position/velocity estimation based on equation of motion - RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay based on the Kepler’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test. Specify both the satellite position/velocity calculation methods in TS 38.101-5/TS 36.102 - Note: the details models for Kepler’s equation and equation of motion can be referred in Appendix - RAN4 will introduce the accuracy requirement applied for two models compared with GMAT - Companies are encouraged to provide the calibration and comparison results with two models before RAN4#114bis meeting, based on calibration and comparison results, further discuss how to define the related accuracy requirement in the next meeting. Issue 1-2-2 Impact on RAN4 requirements - WF - Further discuss the impact on RAN4 requirement pending on channel model discussion progress. The impact of RF, RRM and demodulation requirement will be discussed in separate session. Issue 1-2-3 Impact on RAN4 demodulation requirements - Proposals - Option 1: - RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models - Option 2: - Once the channel model is determined, simulations can be immediately running to decide the value of the additional margin - Option 3 - Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

In this contribution, the view on the demodulation requirement impact for NR NTN and IoT NTN based on TE-emulated channel model with varying Doppler and delay shifts was provided.

# 2	RAN4 demodulation requirement impact

Rel-17 RAN4 RF/RRM/Demod requirements are specified based on the assumption with UE pre-compensation on doppler shift and time delay. From RAN4 core and performance requirements perspective, these requirements can be applicable for both non-zero/zero doppler/delay conditions; and which also irrespective to constant or time varied delay/ Doppler condition which related to Satellite motion.

Observation 1: RAN4 Rel-17 IoT over NR and NR NTN requirements (RRM, RF and performance) are specified based on the assumption that UE shall be capable of pre-compensation of doppler shift and time delay for Satellite access link; and requirements agonistic to test conditions of doppler and time delay due to Satellite motion.

Regarding the demodulation requirement, the requirements for NR PDSCH, eMTC PDSCH and NB-IoT are also defined based on the assumption that UE capable of pre-compensation of doppler shift and time delay. As specified in the TS 38.508, the NTN tests defined, corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test. Meanwhile, the EphemerisInfo-r17 values in SIB19 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 5.6.2 (kept constant throughout the duration of each measurement in the test).

Observation 2: NTN tests defined, corresponding to zero Doppler and constant delay for the type of satellite under test, and EphemerisInfo-r17 values in SIB19 kept constant throughput the duration of each measurement in the test.

The UE Doppler and propagation delay estimation, and pre-compensation will be verified before the baseband processing, where the estimation is not reference signal based. After the baseband, only residual frequency offset error or time offset can be handled by reference signal, such as TRS for NR and CRS for LTE eMTC/NB-IoT. So, to check whether the existing NR NTN and IoT NTN can be applied, we see one possible way is to add some residual frequency offset and time offset with considering the estimation/pre-compensation errors. Similar as IoT over NTN for SAN requirement, we suggest to consider the fixed frequency offset and time offset during the test.

Regarding the value of frequency error and timing error, the worst case of 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s can be considered to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply, compared to ideally pre-compensated, according to core requirement. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Table 7.1C.2-1: Te_NTN Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR1-NTN; 15; 15; 29*64*Tc. 30; 24*64*Tc. 60; N/A. 30; 15; 24*64*Tc. 30; 22*64*Tc. 60; N/A. Note 1: Tc is the basic timing unit defined in TS 38.211 [6].

Proposal 1: Considering worst case for frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply. Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

Test cases for evaluation

For NR NTN, the following cases were defined as following in Rel-17 considering different UE capability related with number of HARQ process, and HARQ disable.

Table 8.2.1.2.2.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD(1) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

And for IoT NTN, RAN4 introduced the demodulation performance requirements for UE category M1, and UE category NB1 and NB2, as following

Table 8.2.1.1.1.1-2: Minimum performance for single antenna port (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 70 | 10.4 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 70 | -4.2 | M1 |
| 3 | 1.4MHz QPSK 1/10 | R.3 FDD | OP.1 FDD | NTN-TDLA100-10 | 1x1 | 70 | -11.5 | M1 |

Table 8.3.1.1.1.1-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

Proposal 2: Apply the following test cases to check demodulation requirement impact with TE-emulated varying Doppler shift and propagation delay models

Table 8.2.1.2.2.1.1-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 0.3 |
| 1-2 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | 7.6 |
| 1-3 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | -0.4 |
| 1-4 | R.PDSCH.1-1.1 FDD(1) | 10 / 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | 1.1 |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Table 8.2.1.1.1.1-2: Minimum performance for single antenna port (FRC)

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 70 | 10.4 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 70 | -4.2 | M1 |
| 3 | 1.4MHz QPSK 1/10 | R.3 FDD | OP.1 FDD | NTN-TDLA100-10 | 1x1 | 70 | -11.5 | M1 |

Table 8.3.1.1.1.1-2: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 70% | -4.7 | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 FDD | 128 | NTN-TDLA100-10 | 1 | 1x1 | 70% | -10.6 | NB1, NB2 |

# 3	Conclusion

In this contribution, the view on the demodulation requirement impact with varying Doppler and delay shifts

is provided.

# References

R4-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

R4- 2412554, Workplan for enhanced requirements and conductive test methodology, Samsung

R4-2502314, Way Forward for [114][325] NTN_testing_NGSO_channel_model