3GPP TSG-RAN WG4 Meeting #114bis            	R4-2504538

Wuhan, China, 7th-11th, April, 2025

Agenda item:	7.10.3.6

Source:	Samsung

Title:	Discussion on NTN demodulation requirements for Less than 5MHz

Document for:	Discussion

- Introduction

In RAN#104 meeting, the revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved in [1]. As one of objective is less than 5MHz for NTN with following

Less than 5MHz for NTN (Core Part) - Specify the system parameters for channel bandwidth for less than 5MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives Less than 5MHz for NTN (Performance part) - Specify the necessary conformance requirements for SAN for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary RRM performance requirements and test cases for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary demodulation performance requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

According to the meeting agenda, and work plan in [2], this is the 1st meeting to discuss demodulation performance requirements for both SAN and UE to support less than 5MHz for NR-NTN in FR1-NTN bands

In this contribution, the view on NTN demodulation performance requirements for less than 5MHz was provided.

# 2	Test Scope

In general, the demodulation requirement specified in Rel-18 TN NR_FR1_less than _5MHz_BW can be leveraged to specify requirements for FR1 NR NTN.

For Less than 5MHz, based on RAN1 design, the main impact on physical layer channel is SSB/PBCH, PDCCH and the available channel bandwidth for both uplink and downlink data channel, where the available channel bandwidth is 12PRB or 15PRB pending on different bands.

## 2.1	Test Scope for UE demodulation

For PDCCH, according to RAN1 agreement 12PRB and 15PRB CORESET#0 is supported for 3MHz. For 12 PRB PDCCH no puncturing is expected, while upper 9 PRBs are punctured from 24 PRB PDCCH for 15 PRB PDCCH

For PBCH, according to RAN1 agreement 12PRB punctured PBCH is supported for 3 MHz channel bandwidth.

Base on Rel-18 RAN4 discussion, the demodulation requirements of PBCH with punctured PRB and the PDCCH with punctured has been introduced for UE supporting less than 5MHz.

For PDSCH and CSI part, since there is no physical enhancement are introduced, assuming there is no UE only support 3MHz channel bandwidth, there is no PDSCH and CSI reporting requirements introduced.

Observation 1:  RAN4 has already introduced the punctured PDCCH and punctured PBCH requirement in Rel-18 for NR supporting less than 5MHz. No new PDSCH and CSI reporting requirements introduced

For FR1 NR NTN, considering a UE to support NTN feature should meet the requirement defined for TN and NTN, specified in 38.101-4 and 38.101-5, respectively. And no requirement enhancement compared with TN scenario for PDCCH/PBCH/CSI reporting, So, RAN4 only define limited PDSCH performance requirement for NTN scenario.

Observation 2:  RAN4 has only introduced the limited PDSCH performance requirement for NTN scenario.

For FR1 NR NTN with supporting less than 5MHz, since the punctured PDCCH and punctured PBCH requirements has already introduced in TN with supporting less than 5MHz, similar as FR1 NR NTN, we think there is no necessary to define the corresponding requirement for FR1 NR NTN with supporting less than 5MHz.

For CSI reporting requirement, considering the large RTT delay, the CSI reporting may not be applied. Therefore, there is no related CSI reporting requirement for FR1 NTN scenario.

Proposal 1 No PBCH requirement introduced for NTN with less than 5MHz

Proposal 2 No PDCCH requirement introduced for NTN with less than 5MHz

Proposal 3 No CSI reporting requirement introduced for NTN with less than 5MHz

For PDSCH requirement, since there is no dedicated requirement specified for TN with less than 5MHz, in our view, it was necessary to introduce the related requirement for 3MHz under FR1 NTN scenario. Otherwise, the test coverage of less than 3MHz for FR1 NTN scenario cannot be guaranteed and tested, even based on the test applicability defined in 38.101-4 for TN and 38.101-5 for NTN scenario.

Proposal 4 RAN4 should introduce PDSCH requirement for FR1 NTN with less than 5MHz

## 2.2	Test Scope for SAN demodulation

For FR1 NR NTN, RAN4 has introduced SAN performance requirements in Rel-17 and Rel-18 as following

- 8.2 Performance requirements for PUSCH

  - 8.2.1 Requirements for PUSCH with transform precoding disabled

  - 8.2.2 Requirements for PUSCH with transform precoding enabled

  - 8.2.3 Requirements for UL timing adjustment

  - 8.2.4 Requirements for PUSCH repetition Type A

  - 8.2.5 Requirements for PUSCH with DM-RS bunding

- 8.3 Performance requirements for PUCCH

  - 8.3.1 DTX to ACK probability

  - 8.3.2 Performance requirements for PUCCH format 0

  - 8.3.3 Performance requirements for PUCCH format 1

  - 8.3.4 Performance requirements for PUCCH format 2

  - 8.3.5 Performance requirements for PUCCH format 3

  - 8.3.6 Performance requirements for PUCCH format 4

  - 8.3.7 Performance requirements for multi-slot PUCCH

- 8.4 Performance requirements for PRACH

  - Long sequence formats: format 0 and format 2

  - Short sequence formats: format B4 and format C2

In Rel-18 NR_FR1_less than _5MHz, RAN4 has lots of discussion whether to define the dedicated requirement for 3MHz for PUSCH, PUCCH and PRACH.

For PUSCH, it is RAN4 agreement that there are no BSs that supporting only less than 5MHz, and based on the existing test applicability rule for different CBWs, 3MHz requirements will never be tested. Therefore, RAN4 agreed that not to define PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement.

For PUCCH, based on companies’ evaluation, there is no obvious performance different observed in PUCCH format 0/1/3/4 compared with existing 5MHz requirements

Therefore, there is no necessary to introduce new requirements for these formats from performance reliability perspective, and only introduce the new demodulation requirement for PUCCH format 2 with 3MHz CBW. And meanwhile, to reduce the effort, a test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz was introduced.

For PRACH, its requirement is no differentiate with transmission BWs if its configurations can be fitted into the transmission BW, the minimum number of RB as 15 for 3MHz excepting for n100, and the minimum number of RB as 12 for 3MHz with n100, where the available number of subcarriers is 180 for 15 RB, 144 for 12RB, respectively. Then, the existing long PRACH format with LRA=839 and short PRACH formats with LRA=139 can be fitted into the transmission BW of 3MHz. Therefore, it was agreed that no new PRACH requirement for NR with Less than 5MHz, and adding an applicability rule that for BS supporting less than 5MHz carrier bandwidth, only test requirements relating to PRACH preamble format3 with 15KHz SCS with sequence length LRA=139, and PRACH format with 1.25kHz SCS with sequence length LRA=839 shall apply.

Observation 3:  No PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement were introduced in Rel-18 NR with less than 5MHz

Observation 4:  Only introduce new demodulation requirement for PUCCH format 2 with 3MHz and introduce the test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz only for format 2

Observation 5:  No new PRACH requirement for NR with less than 5MHz and introduce the applicability rule for BS supporting less than 5MHz carrier bandwidth.

Therefore, for NR NTN with supporting less than 5MHz, we would like to apply the similar principle as NR with supporting less than 5MHz for specifying the NR NTN SAN performance requirement with supporting less than 5MHz.

For SAN PUSCH requirement, no new SAN PUSCH requirement with 3MHz bandwidth for PUSCH with transform precoding disabled, with transform precoding enabled, UL timing adjustment, PUSCH repetition Type A and PUSCH with DM-RS bundling, if no SAN that supporting only less than 5MHz.

For SAN PUCCH requirement, further checking whether there is any obvious performance difference for different formats with 3MHz bandwidth under NTN channel condition compared the leac

For SAN PRACH requirement, no new PRACH requirement for NTN with less than 5MHz, and introducing the test applicability rule for SAN supporting less than 5MHz carrier bandwidth.

Proposal 5:  No New SAN PUSCH requirement with 3MHz bandwidth for PUSCH with transform precoding disabled, with transform precoding enabled, UL timing adjustment, PUSCH repetition Type A and PUSCH with DM-RS bundling if no SAN that supporting only less than 5MHz

Proposal 6:  FFS to introduce SAN PUCCH requirement with 3MHz bandwidth under NR NTN channel condition

Proposal 7:  No new SAN PRACH requirement for NTN with Less than 5MHz, and introducing the test applicability rule for SAN supporting less than 5MHz carrier bandwidth

# 3	Simulation Assumption

In this section, the simulation assumption is provided for both UE and SAN performance requirement.

## 3.1	Simulation Assumption for UE demodulation

Based on existing FR1 NTN test, there are four cases introduced with considering different UE capability for number of HARQ process and HARQ feedback disable.  In our view, they can be reused as a starting point to specify the related PDSCH requirement with supporting less than 5MHz as following

Table 1: Test cases for PDSCH with NTN supporting less than 5MHz

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| 1-2 | TBD | 3/ 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | TBD |
| 1-3 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | TBD |
| 1-4 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Proposal 8:  Reuse the existing FR1 NTN PDSCH test cases as a starting point to specify the related PDSCH requirement with supporting less than 5MHz

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| 1-2 | TBD | 3/ 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | TBD |
| 1-3 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | TBD |
| 1-4 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

## 3.2	Simulation Assumption for SAN demodulation

In order to evaluate the PUCCH performance with less than 5MHz under NTN channel condition, the existing PUCCH test cases for FR1 NTN can be considered as starting point.

PUCCH format 0

Table 2: Simulation assumption for PUCCH format 0 with NTN less than 5MHz

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |

PUCCH format 1

Table 3: Simulation assumption for PUCCH format 1 with NTN less than 5MHz

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

PUCCH format 2

Table 4: Simulation assumption for PUCCH format 2 with NTN less than 5MHz

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

PUCCH format 3

Table 5: Simulation assumption for PUCCH format 3 with NTN less than 5MHz

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2:11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |

PUCCH format 4

Table 6: Simulation assumption for PUCCH format 4 with NTN less than 5MHz

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Multi-slot PUCCH format 1

Table 7: Simulation assumption for PUCCH format 0 with NTN less than 5MHz

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

Proposal 9:  The following simulation assumption can be considered for evaluation SAN PUCCH performance with less than 5MHz

PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |

PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

PUCCH format 3

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2:11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |

PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

# 4	Conclusion

In this contribution, the view on test scope of demodulation requirement for NTN supporting less than 5MHz.

UE demodulation requirement

Observation 1:  RAN4 has already introduced the punctured PDCCH and punctured PBCH requirement in Rel-18 for NR supporting less than 5MHz. No new PDSCH and CSI reporting requirements introduced

Observation 2:  RAN4 has only introduced the limited PDSCH performance requirement for NTN scenario.

Proposal 1 No PBCH requirement introduced for NTN with less than 5MHz

Proposal 2 No PDCCH requirement introduced for NTN with less than 5MHz

Proposal 3 No CSI reporting requirement introduced for NTN with less than 5MHz

Proposal 4 RAN4 should introduce PDSCH requirement for FR1 NTN with less than 5MHz

SAN demodulation requirement

Observation 3:  No PUSCH requirements with 3MHz bandwidth for both non-HST and HST condition, as well as UL timing adjustment requirement were introduced in Rel-18 NR with less than 5MHz

Observation 4:  Only introduce new demodulation requirement for PUCCH format 2 with 3MHz and introduce the test applicability rule for new introduced UCI performance with 3MHz and legacy 5MHz only for format 2

Observation 5:  No new PRACH requirement for NR with less than 5MHz and introduce the applicability rule for BS supporting less than 5MHz carrier bandwidth.

Proposal 5:  No New SAN PUSCH requirement with 3MHz bandwidth for PUSCH with transform precoding disabled, with transform precoding enabled, UL timing adjustment, PUSCH repetition Type A and PUSCH with DM-RS bundling if no SAN that supporting only less than 5MHz

Proposal 6:  FFS to introduce SAN PUCCH requirement with 3MHz bandwidth under NR NTN channel condition

Proposal 7:  No new SAN PRACH requirement for NTN with Less than 5MHz, and introducing the test applicability rule for SAN supporting less than 5MHz carrier bandwidth

Proposal 8:  Reuse the existing FR1 NTN PDSCH test cases as a starting point to specify the related PDSCH requirement with supporting less than 5MHz

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| 1-2 | TBD | 3/ 15 | 16QAM, 0.48 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | TBD |
| 1-3 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLC5-200 | 1x2, ULA Low | 70 | TBD |
| 1-4 | TBD | 3/ 15 | QPSK, 0.30 | NTN-TDLA100-200 | 1x2, ULA Low | 70 | TBD |
| Note1: The Maximum throughput is based on the HARQ processes with HARQ feedback enabled. |  |  |  |  |  |  |  |

Proposal 9:  The following simulation assumption can be considered for evaluation SAN PUCCH performance with less than 5MHz

PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | Enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 12 for 2 symbols |

PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

PUCCH format 2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 3 |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |

PUCCH format 3

| Parameter | Test |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2:11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |

PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (Number of PRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Multi-slot PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | Option 1: The largest PRB index – (nrofPRBs – 1) Option 2: 11 |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |

# 5	 References

R4-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

R4- 2412554, Workplan for enhanced requirements and conductive test methodology, Samsung